| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0066_r2 |
| --- | --- |

| Title: | Non-CE1: Improved edge offset coding for SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akira Minezawa Kazuo Sugimoto Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Minezawa.Akira@ds.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

In this contribution, a modified edge offset coding for SAO is proposed. This scheme replaces unary binarization with truncated unary binarization for edge offset coding to remove coding redundancy in CABAC. The coding performance of the proposed scheme is the same compared with CE1 anchor. It is also reported that the truncated unary coding achieves about 0.01% to 0.08% coding gain on average compared with Exp. Golomb coding in VLC case.

# Introduction

In the current HM, unary binarization is used for coding of SAO edge offset in the case of interleaving (CABAC) mode although the edge offset value is bounded. In order to remove coding redundancy, we propose to replace unary binarization with truncated unary binarization for edge offset coding.

# Proposed scheme

The current HM uses unary binarization as follows for edge offset coding in CABAC case. (Note that the corresponding binarization description for edge offset is not correct in the current draft[1] .– See Appendix)

| Value of syntax element | Bin string |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (I_NxN) | 0 |  |  |  |  |  |
| 1 | 1 | 0 |  |  |  |  |
| 2 | 1 | 1 | 0 |  |  |  |
| 3 | 1 | 1 | 1 | 0 |  |  |
| 4 | 1 | 1 | 1 | 1 | 0 |  |
| 5 | 1 | 1 | 1 | 1 | 1 | 0 |
| … |  |  |  |  |  |  |

On the other hand, the edge offset shall be in the range of 0 to ( 1<< ( Min( bitDepth, 10 ) − 5 ) ) – 1. Therefore, 1-bin redundancy should be exist when an edge offset equal to ( 1<< ( Min( bitDepth, 10 ) − 5 ) ) – 1 is encoded. In order to remove coding redundancy, we propose to replace unary binarization with truncated unary binarization. An example of truncated unary binarization is shown in the following table in the case of max value of syntax element is equal to 5.

| Value of syntax element | Bin string |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (I_NxN) | 0 |  |  |  |  |  |
| 1 | 1 | 0 |  |  |  |  |
| 2 | 1 | 1 | 0 |  |  |  |
| 3 | 1 | 1 | 1 | 0 |  |  |
| 4 | 1 | 1 | 1 | 1 | 0 |  |
| 5 | 1 | 1 | 1 | 1 | 1 |  |

# Simulations & Results

The proposed scheme has been implemented on top of CE1 anchor software based on HM-6.0, and performance evaluation has been conducted using the common test configurations [2] with a modification SAOInterleaving=1, for LCU sizes of 64x64, 32x32 and 16x16. Table 1 to Table 3 show results of the evaluation on the proposed scheme compared with the CE1 anchor.

Table 1. BD bitrate savings of the proposed scheme on LCU=64

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | -0.01% | 0.00% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.09% | -0.13% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

Table 2. BD bitrate savings of the proposed scheme on LCU=32

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | -0.01% | -0.01% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.00% | 0.01% | 0.03% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.02% | 0.00% | 0.00% | 0.00% |
| Class D | -0.01% | -0.06% | 0.03% | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Overall | 0.00% | -0.01% | 0.02% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | -0.03% | 0.03% | 0.00% | 0.00% | 0.00% |
| Class F | -0.12% | 0.19% | -0.21% | 0.00% | -0.01% | -0.01% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3. BD bitrate savings of the proposed scheme on LCU=16

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | -0.01% | -0.01% | -0.01% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.01% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class B | 0.00% | -0.01% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | -0.01% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | -0.02% | -0.03% | -0.02% | -0.01% | -0.01% | -0.01% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.01% | 0.02% | 0.02% | 0.00% | 0.00% | 0.00% |
| Class C | -0.01% | -0.01% | 0.02% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | -0.01% | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | -0.01% | -0.03% | 0.00% | 0.00% | 0.00% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | -0.05% | -0.14% | -0.97% | -0.01% | -0.01% | -0.01% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Additional investigation on APS mode

In the current draft, SAO has APS syntax mode in addition to interleaving syntax mode. In the APS syntax mode, edge offset is VLC coded using Exp. Golomb code.

We have also investigated coding performance of truncated unary code compared with Exp. Golomb code for edge offset coding in the case of APS syntax mode(VLC).

We modified the CE1 anchor software by implementing the proposed method,and the performance evaluations of the proposed scheme have been conducted under the condition of the common test settings [2] with a modification SAOInterleaving=0. It is shown that Table 4 is the performance of the proposed scheme relative to the CE1 anchor.

Table 4. BD bitrate savings of the proposed scheme on APS mode

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.02% | -0.01% | -0.01% | 0.00% | 0.00% | 0.00% |
| Class B | -0.03% | -0.03% | -0.03% | -0.01% | -0.01% | -0.01% |
| Class C | -0.04% | -0.04% | -0.04% | -0.01% | -0.01% | -0.01% |
| Class D | -0.03% | -0.03% | -0.03% | -0.01% | -0.01% | -0.01% |
| Class E | -0.05% | -0.05% | -0.05% | -0.01% | -0.02% | -0.02% |
| Overall | -0.03% | -0.03% | -0.03% | -0.01% | -0.01% | -0.01% |
|  | -0.03% | -0.03% | -0.03% | -0.01% | -0.01% | -0.01% |
| Class F | -0.03% | -0.03% | -0.03% | 0.02% | 0.01% | 0.01% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.04% | -0.04% | -0.03% | -0.01% | -0.01% | -0.01% |
| Class B | -0.06% | -0.06% | -0.06% | -0.02% | -0.02% | -0.02% |
| Class C | -0.06% | -0.06% | -0.06% | -0.02% | -0.02% | -0.02% |
| Class D | -0.04% | -0.04% | -0.04% | -0.01% | -0.01% | -0.01% |
| Class E |  |  |  |  |  |  |
| Overall | -0.05% | -0.05% | -0.05% | -0.02% | -0.02% | -0.02% |
|  | -0.05% | -0.05% | -0.05% | -0.02% | -0.02% | -0.02% |
| Class F | -0.07% | -0.06% | -0.06% | 0.00% | -0.01% | -0.01% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.07% | -0.07% | -0.07% | -0.03% | -0.03% | -0.03% |
| Class C | -0.08% | -0.08% | -0.08% | -0.03% | -0.03% | -0.03% |
| Class D | -0.05% | -0.05% | -0.05% | -0.02% | -0.02% | -0.02% |
| Class E | -0.11% | -0.11% | -0.11% | -0.05% | -0.05% | -0.05% |
| Overall | -0.08% | -0.08% | -0.08% | -0.03% | -0.03% | -0.03% |
|  | -0.08% | -0.08% | -0.08% | -0.03% | -0.03% | -0.03% |
| Class F | -0.11% | -0.06% | -0.02% | -0.01% | -0.02% | -0.02% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusions

This contribution proposes to replace unary binarization with truncated unary binarization for coding of SAO edge offset to remove coding redundancy in CABAC case. Coding performance of the proposed scheme is comparable with the CE1 anchor. Additionally, we found that the proposed truncated unary coding achieves about 0.01% to 0.08% coding gain on average compared with Exp. Golomb coding in VLC case. We propose these schemes to be adopted to HM-7/DIS.

# References

- Benjamin Bross, “High Efficiency Video Coding (HEVC) text specification draft 6”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-H1003, 8th Meeting: San Jose, US, February, 2012.

- Frank Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-H1100, 8th Meeting: San Jose, US, February, 2012.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix: Normative descriptions to be reflected to WD text

A draft text of the proposed scheme is shown as follows, where the fixed descriptions of CD are highlighted in blue and the proposed changes are highlighted in yellow.

#### 7.3.2.8 Sample adaptive offset VLC syntax

| sao_offset_vlc( rx, ry, cIdx ) { | Descriptor |
| --- | --- |
| sao_type_idx[ cIdx ][ rx ][ ry ] | ue(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = =5 ) { |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | u(5) |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset[ cIdx ][ rx][ ry ][ i ] | se(v) |
| } else if( sao_type_idx[ cIdx ][ rx ][ ry ] != 0 ) |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset[ cIdx ][ rx][ ry ][ i ] | tu(v) |
| } |  |

#### 7.3.4.2 Sample adaptive offset CABAC syntax

| sao_offset_cabac( rx, ry, cIdx ) { | Descriptor |
| --- | --- |
| sao_type_idx[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = =5 ) { |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| for( i = 0; i < 4; i++ ) |  |
| sao_bo_offset[ cIdx ][ rx][ ry ][ i ] | ae(v) |
| } else if( sao_type_idx[ cIdx ][ rx ][ ry ] != 0 ) |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_eo_offset[ cIdx ][ rx][ ry ][ i ] | ae(v) |
| } |  |

#### 7.4.4.1 Sample adaptive offset unit CABAC semantics

...

sao_bo_offset[ cIdx ][ rx ][ ry ][ i ] indicates the offset value of i-th category of current coding treeblock at position rx and ry for the colour component cIdx.

The variable bitDepth is derived as follows.

- If cIdx is equal to 0, bitDepth is set equal to BitDepthY..

- Otherwise (cIdx is equal to1 or 2), bitDepth is set equal to BitDepthC.

It is a requirement of bitstream conformance that the values of sao_offset[ cIdx ][ rx ][ ry ][ i ] shall be in the range of −( 1<< ( Min( bitDepth, 10 ) − 5 ) ) to ( 1<< ( Min( bitDepth, 10 ) − 5 ) ) − 1, inclusive.

When sao_offset[ cIdx ][ rx ][ ry ][ i ] is not present, it is inferred as follows.

- If sao_merge_up_flag is equal to 1, sao_offset[ cIdx ][ rx ][ ry ][ i ] is set equal to sao_offset[ cIdx ][ rx ][ ry − 1 ][ i ].

- Otherwise, sao_offset[ cIdx ][ rx ][ ry ][ i ] is set equal to sao_offset[ cIdx ][ rx − 1 ][ ry ][ i ].

The array SaoOffsetVal is derived as follows.

SaoOffsetVal[ cIdx ][ rx ][ ry ][ 0 ] = 0		(7-1)

SaoOffsetVal[ cIdx ][ rx ][ ry ][ i + 1 ] = 
			sao_offset[ cIdx ][ rx ][ ry ][ i ] << ( bitDepth – Min( bitDepth, 10 ) ) 	(7-2)

sao_eo_offset[ cIdx ][ rx ][ ry ][ i ] indicates the offset value of i-th category of current coding treeblock at position rx and ry for the colour component cIdx.

The variable bitDepth is derived as follows.

- If cIdx is equal to 0, bitDepth is set equal to BitDepthY..

- Otherwise (cIdx is equal to1 or 2), bitDepth is set equal to BitDepthC.

It is a requirement of bitstream conformance that the values of sao_offset[ cIdx ][ rx ][ ry ][ i ] shall be in the range of 0 to ( 1<< ( Min( bitDepth, 10 ) − 5 ) ) − 1, inclusive.

When sao_offset[ cIdx ][ rx ][ ry ][ i ] is not present, it is inferred as follows.

- If sao_merge_up_flag is equal to 1, sao_offset[ cIdx ][ rx ][ ry ][ i ] is set equal to sao_offset[ cIdx ][ rx ][ ry − 1 ][ i ].

- Otherwise, sao_offset[ cIdx ][ rx ][ ry ][ i ] is set equal to sao_offset[ cIdx ][ rx − 1 ][ ry ][ i ].

The variable offsetSign is derived as follows.

- If i is larger than 1,offsetSign is set to −1.

- Otherwise (i is less than 2), offsetSign is set to 1.

The array SaoOffsetVal is derived as follows.

SaoOffsetVal[ cIdx ][ rx ][ ry ][ 0 ] = 0		(7-3)

SaoOffsetVal[ cIdx ][ rx ][ ry ][ i + 1 ] = 
			offsetSign*sao_offset[ cIdx ][ rx ][ ry ][ i ] << ( bitDepth – Min( bitDepth, 10 ) ) 	(7-4)

#### 9.2.1.1 Initialization process for context variables

…

Table 9-4 – Association of ctxIdx and syntax elements for each slice type in the initialization process

|  | Syntax element | ctxIdxTable | initialisationType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| sao_unit_cabac() | sao_merge_left_flag | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
|  | sao_merge_up_flag | Error: Reference source not found | 0 | 1 | 2 |
| sao_offset_cabac() | sao_type_idx | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | sao_bo_offset | Table | 0..2 | 3..5 | 6..8 |
|  | sao_eo_offset | Table 9-9 | 0..1 | 2..3 | 4..5 |

Table 9-8 – Values of variable initValue for sao_bo_offset ctxIdx

| Initialization variable | sao_bo_offset ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 247 | 154 | 244 | 215 | 154 | 169 | 215 | 154 | 169 |

Table 9-9 – Values of variable initValue for sao_eo_offset ctxIdx

| Initialization variable | sao_eo_offset ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 143 | 140 | 185 | 140 | 200 | 140 |

### 9.2.2 Binarization process

…

| Table 9-32 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element | initialisationType | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| sao_merge_left_flag | 0 | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | 1 |  | 0 | Error: Reference source not found | 3 |
|  | 2 |  | 0 | Error: Reference source not found | 6 |
| sao_merge_up_flag | 0 | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | 1 |  | 0 | Error: Reference source not found | 1 |
|  | 2 |  | 0 | Error: Reference source not found | 2 |
| sao_type_idx | 0 | FL, cMax = 3 | 1 | Error: Reference source not found | 0 |
|  | 1 |  | 1 | Error: Reference source not found | 2 |
|  | 2 |  | 1 | Error: Reference source not found | 4 |
| sao_band_position | 0 | FL, cMax = 5 | na | na | na, (uses Decode Bypass) |
|  | 1 |  | na | na | na, (uses Decode Bypass) |
|  | 2 |  | na | na | na, (uses Decode Bypass) |
| sao_bo_offset | 0 | as specified in subclause 9.2.2.7 | 2 | Table | 0 |
|  | 1 |  | 2 | Table | 3 |
|  | 2 |  | 2 | Table | 6 |
| sao_eo_offset | 0 | TU, cMax = ( 1<< ( Min( bitDepth, 10 ) − 5 ) ) − 1 | 1 | Table 9-9 | 0 |
|  | 1 |  | 1 | Table 9-9 | 2 |
|  | 2 |  | 1 | Table 9-9 | 4 |

### 9.2.2.6 Binarization process for cu_qp_delta

…

### 9.2.2.7 Binarization process for sao_bo_offset

Input to this process is a request for a binarization for the syntax element sao_bo_offset.

Output of this process is the binarization of the syntax element.

The bin string of this binarization process of a syntax element value synVal is specified by a process equivalent to the following pseudo-code.

absV = Abs( synVal ) 
if( absV = = 0 )
	put( 0 ) 
else {
	put( 1 ) 
	signV = ( synVal > 0 ) ? 0 : 1
	put( signV ) 
	cNum = absV − 1
	absVGreaterThan1Flag = ( absV = = 1 ) ? 0 : 1
	put( absVGreaterThan1Flag ) 
	if( absVGreaterThan1Flag ) { 
		while( cNum− − ) 
			put( 1 )
		put( 0 )
	}
}

#### 9.2.3.1 Derivation process for ctxIdx

…

| Table 9-37 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| sao_merge_left_flag | Error: Reference source not found | 0 | cIdx | na | na | na | na |
|  |  | 3 | cIdx | na | na | na | na |
|  |  | 6 | cIdx | na | na | na | na |
| sao_merge_up_flag | Error: Reference source not found | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |
| sao_type_idx | Error: Reference source not found | 0 | 0 | 1 | 1 | na | na |
|  |  | 2 | 0 | 1 | 1 | na | na |
|  |  | 4 | 0 | 1 | 1 | na | na |
| sao_bo_offset | Table | 0 | 0 | 1 | 2 | 2 | 2 |
|  |  | 3 | 0 | 1 | 2 | 2 | 2 |
|  |  | 6 | 0 | 1 | 2 | 2 | 2 |
| sao_eo_offset | Table 9-9 | 0 | 0 | 1 | 1 | 1 | 1 |
|  |  | 2 | 0 | 1 | 1 | 1 | 1 |
|  |  | 4 | 0 | 1 | 1 | 1 | 1 |