| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0069 |
| --- | --- |

| Title: | APS error resilience and partial updating |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela and Srikanth Manchenally Gopalakrishna Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

It is asserted that HEVC CD provides no means to detect a loss of an adaptation parameter set (APS) NAL unit, if the aps_id value of the lost APS NAL unit has been used previously by another APS NAL unit. It is claimed that in such a case the decoder would continue decoding using the previously received APS NAL unit, which would be likely to result into clearly visible artifacts. The contribution proposes such constraints on aps_id assignment that reportedly enable decoders to detect losses of APS NAL units.

The contribution also proposes a partial updating mechanism for adaptation parameter sets. The proposed syntax includes a possibility to copy any group of syntax elements (QM, deblocking filter, SAO, ALF) from either the same reference APS or from different APSes. In the proposal, the partial update references of an APS NAL unit are resolved at the time of decoding the APS NAL unit.

# Introduction

Adaptation Parameter Sets contain information which is likely to change relatively frequently, even from picture to picture, as it is likely to be adaptively created during the picture encoding process. At the moment Adaptation Parameter Sets can convey Quantization Matrices (a.k.a. scaling lists), Adaptive Loop Filtering (ALF) parameters, Sample Adaptive Offset (SAO) parameters, and deblocking filter parameters. It is expected that scalable extensions may need additional syntax elements groups to be included in the APS syntax. For example, the current H.264/AVC-based 3D video coding amendment draft specifies a new APS-like NAL unit type for depth range and view synthesis prediction parameters, which in the HEVC-based 3D video coding design could reside in the APS syntax.

There were several contributions for the San Jose meeting that proposed to have a partial updating mechanism for the Adaptation Parameter Set in order to reduce the size of APS NAL units and hence the spend a smaller bitrate for conveying APS NAL units. The proposals have been summarized in Ye-Kui Wang’s email to the JCT-VC email reflector on Feb 2nd, 2012 as follows:

| Doc. no. | Method summary | Cons |
| --- | --- | --- |
| H0069 | Linked APSs – conditionally referencing to another APS | Cross-APS dependency – error resilience problem. Not easy to specify. |
| H0070 | Conditional replacement | Cross-APS dependency – error resilience problem. Inefficiency when to-be-overwritten content may be re-usable. |
| H0255 - 1 | Multiple APS IDs in SH | Multiple APS IDs in SH. SH syntax not future-proof (syntax change needed when in the future stuff goes into APS ). |
| H0255 - 2 | Allowing explicitly signaling different types of APS parameters in SH that can override the one in APS | Complex SH syntax. Inefficiency when content added to SH may be re-usable. |
| H0381 | A “copy” flag for each type of APS params, which allows copying that type of APS parameters from another APS; Conditionally referencing to another APS for each type of APS params | Cross-APS dependency – error resilience problem. Complex APS syntax. |
| H0505 - 1 | Each APS NAL unit contains only one type of APS params; Grouping of multiple APS IDs and PPS ID in GPS; References only GPS ID in SH | Each APS NAL unit contains only one type of APS params. Needs GPS. |
| H0505 - 2 | Each APS NAL unit may contain multiple types of APS params; Grouping of multiple APS IDs and PPS ID in GPS; References only GPS ID in SH | Needs GPS. |

As one can see from the table above, error resilience is a problem in many of the proposals. What’s more, we believe that the HEVC CD and all the above-mentioned proposals fail to give decoder any means to detect a loss of an APS when the lost APS re-used an aps_id value that had already been used earlier by another APS. Consequently, any reference (from slice header or another APS NAL unit for partial updating of APS parameters) to the aps_id value used in a lost APS NAL unit points to the previous APS NAL unit using the same aps_id value. Therefore wrong syntax element values would be used in slice decoding process or in partial updating of APS parameters. Such use for wrong syntax element values typically has severe impacts in the decoding, e.g. clearly visible errors may be present in decoded pictures or decoding may fail altogether.

One option to overcome the loss resilience problem presented in the previous paragraph could be to avoid re-using of aps_id values in APS NAL units. However, this could potentially lead to a need for requiring decoders to support a large value range for APS identifier values and hence increased memory consumption, as decoders must be prepared for storing an APS for each value in the allowed aps_id value range.

# Summary of the Proposal

## Error Resilience

It is proposed that the encoder specifies the value range of aps_id values with the max_aps_id syntax element within the sequence parameter set. In other words, the value of aps_id may be in the range of 0 to max_aps_id, inclusive.

The encoder also specifies a range of aps_id values that are considered “used” and indicates that range to the decoder in max_aps_id_diff. The range is relative to the latest received APS NAL unit and hence specifies a kind of a sliding window of valid aps_id values. APS NAL units that have an aps_id value outside the sliding-window range are considered “unused” and a new APS NAL unit with the same aps_id value may be transmitted. Each received APS NAL unit updates the position of the sliding-window range of aps_id values considered “used”. It is recommended that encoders increment aps_id value by 1 relative to that in the previous APS NAL unit in decoding order. As aps_id values may wrap over, modulo arithmetic is used in determining the aps_id values within the sliding-window range.

Thanks to the controlled marking which aps_id values can be reused for new APS NAL units, the number of APSes is limited to (max_aps_id_diff + 1) and the above-mentioned error resilience problem is solved.

## Partial Updating

The proposed syntax includes a possibility to copy any group of syntax elements (QM, deblocking filter, SAO, ALF) from either the same APS or from different APSes, indicated by their aps_id value. The referred APSes must be marked “used”.

The partial update references are proposed to be resolved at the time of decoding the APS NAL unit, i.e. the APS is decoded by copying the referenced data from the indicated source APS into the APS being decoded. In other words, the references to other APS NAL units are resolved only once, which makes the decoding process simple.

# Proposed Specification Text

## Sequence Parameter Set

The following additions are proposed to sequence parameter set syntax and semantics.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| max_aps_id | ue(v) |
| if( max_aps_id > 0 ) |  |
| max_aps_id_diff | ue(v) |
| ... |  |

max_aps_id specifies the maximum allowed aps_id value.

max_aps_id_diff specifies the value range of aps_id values of adaptation parameter sets marked as “used”. max_aps_id_diff * 2 shall be less than max_aps_id.

## Adaptation Parameter Set

The following syntax and semantics are proposed for the adaptation parameter set.

- 7.3.2.5	Adaptation parameter set RBSP syntax

| aps_rbsp( ) { | Descriptor |
| --- | --- |
| aps_id | ue(v) |
| partial_update_flag | u(1) |
| if( partial_update_flag ) { |  |
| common_reference_aps_flag | u(1) |
| if( common_reference_aps_flag ) |  |
| common_reference_aps_id | ue(v) |
| } |  |
| aps_scaling_list_data_present_flag | u(1) |
| if( aps_scaling_list_data_present_flag ) { |  |
| if( partial_update_flag ) |  |
| aps_scaling_list_data_referenced_flag | u(1) |
| if( !partial_update_flag ¦ ¦ !aps_scaling_list_data_referenced_flag ) |  |
| scaling_list_param( ) |  |
| else if( !common_reference_aps_flag ) |  |
| aps_scaling_list_data_reference_aps_id | ue(v) |
| } |  |
| aps_deblocking_filter_flag | u(1) |
| if( aps_deblocking_filter_flag ) { |  |
| if( partial_update_flag ) |  |
| aps_deblocking_filter_referenced_flag | u(1) |
| if( !partial_update_flag ¦ ¦ !aps_deblocking_filter_referenced_flag ) { |  |
| disable_deblocking_filter_flag | u(1) |
| if( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } else if( !common_reference_aps_flag ) |  |
| aps_deblocking_filter_reference_aps_id | ue(v) |
| } |  |
| aps_sao_interleaving_flag | u(1) |
| if( !aps_sao_interleaving_flag ) { |  |
| aps_sample_adaptive_offset_flag | u(1) |
| if( aps_sample_adaptive_offset_flag ) { |  |
| if( partial_update_flag ) |  |
| aps_sao_referenced_flag | u(1) |
| if( !partial_update_flag ¦ ¦ !aps_sao_referenced_flag ) |  |
| aps_sao_param( ) |  |
| else if( !common_reference_aps_flag ) |  |
| aps_sao_reference_aps_id | ue(v) |
| } |  |
| aps_adaptive_loop_filter_flag | u(1) |
| if( aps_adaptive_loop_filter_flag ) { |  |
| if( partial_update_flag ) |  |
| aps_alf_referenced_flag | u(1) |
| if( !partial_update_flag ¦ ¦ !aps_alf_referenced_flag ) |  |
| alf_param( ) |  |
| else if( !common_reference_aps_flag ) |  |
| aps_alf_reference_aps_id | ue(v) |
| } |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

- 7.4.2.5	Adaptation parameter set RBSP semantics

aps_id specifies an identifier value that identifies the adaptation parameter set. The value of aps_id shall be in the range of 0 to max_aps_id, inclusive.

partial_update_flag equal to 0 specifies that no syntax element is included in this APS by reference. partial_update_flag equal to 1 specifies that syntax elements may be included in this APS by reference.

common_reference_aps_flag equal to 0 specifies that each group of syntax elements included by reference in this APS may have a different source APS identified by a different APS identifier value. common_reference_aps_flag equal to 1 specifies that each group of syntax elements included by reference in this APS are from the same source APS.

common_reference_aps_id specifies the APS identifier value for the source APS for all groups of syntax elements included in this APS by reference. When present, the adaptation APS NAL unit having aps_id equal to common_reference_aps_id shall be marked as “used” in the NAL unit decoding process specified in subclause 8.2.

aps_scaling_list_data_present_flag equal to 1 specifies that the scaling list parameters exist in this APS, equal to 0 specifies that scaling list parameters do not exist in this APS.

aps_scaling_list_data_referenced_flag equal to 0 specifies that the scaling list parameters are present in this aps_rbsp( ). aps_scaling_list_data_referenced_flag equal to 1 specifies that the scaling list parameters are included in this APS by reference.

aps_scaling_list_data_reference_aps_id specifies the APS identifier value for the APS from which the scaling list parameters are included in this APS by reference. When present, the adaptation APS NAL unit having aps_id equal to aps_scaling_list_data_reference_aps_id shall be marked as “used” in the NAL unit decoding process specified in subclause 8.2.

aps_deblocking_filter_flag equal to 1 specifies that deblocking parameters are present in the APS. aps_deblocking_filter_flag equal to 0 specifies that deblocking parameters do not exist in this APS.

aps_deblocking_filter_referenced_flag equal to 0 specifies that the deblocking parameters are present in this aps_rbsp( ). aps_deblocking_filter_referenced_flag equal to 1 specifies that the deblocking parameters are included in this APS by reference.

aps_deblocking_filter_reference_aps_id specifies the APS identifier value for the APS from which the deblocking parameters are included in this APS by reference. When present, the adaptation APS NAL unit having aps_id equal to aps_deblocking_filter_reference_aps_id shall be marked as “used” in the NAL unit decoding process specified in subclause 8.2.

aps_sao_interleaving_flag equal to 1 specifies that the SAO parameters are interleaved in slice data for slices referring to the current APS; equal to 0 specifies that the SAO parameters are in APS for slices referring to the current APS. When there is no active APS, aps_sao_interleaving_flag is inferred to be 0.

aps_sample_adaptive_offset_flag equal to 1 specifies that the SAO is on for slices referring to the current APS; equal to 0 specifies that the SAO is off for slices referring to the current APS.  When there is no active APS, the aps_sample_adaptive_offset_flag value is inferred to be 0.

aps_sao_referenced_flag equal to 0 specifies that the SAO parameters are present in this aps_rbsp( ). aps_sao_referenced_flag equal to 1 specifies that the SAO parameters are included in this APS by reference.

aps_sao_reference_aps_id specifies the APS identifier value for the APS from which the SAO parameters are included in this APS by reference. When present, the adaptation APS NAL unit having aps_id equal to aps_sao_reference_aps_id shall be marked as “used” in the NAL unit decoding process specified in subclause 8.2.

aps_adaptive_loop_filter_flag equal to 1 specifies that the ALF is on for slices referring to the current APS; equal to 0 specifies that the ALF is off for slices referring to the current APS. When there is no active APS, the aps_adaptive_loop_filter_flag value is inferred to be 0.

aps_alf_referenced_flag equal to 0 specifies that the ALF parameters are present in this aps_rbsp( ). aps_alf_referenced_flag equal to 1 specifies that the ALF parameters are included in this APS by reference.

aps_alf_reference_aps_id specifies the APS identifier value for the APS from which the ALF parameters are included in this APS by reference. When present, the adaptation APS NAL unit having aps_id equal to aps_alf_reference_aps_id shall be marked as “used” in the NAL unit decoding process specified in subclause 8.2.

aps_extension_flag equal to 0 specifies that no aps_extension_data_flag syntax elements are present in the picture parameter set RBSP syntax structure. aps_extension_flag shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value of 1 for aps_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for aps_extension_flag in a picture parameter set NAL unit.

aps_extension_data_flag may have any value. Its value does not affect decoder conformance to profiles specified in this Recommendation | International Standard.

## Decoding Process

The following yellow-highlighted additions are proposed to the NAL unit decoding process.

- 8.2	NAL unit decoding process

Inputs to this process are NAL units.

Outputs of this process are the RBSP syntax structures encapsulated within the NAL units.

The decoding process for each NAL unit extracts the RBSP syntax structure from the NAL unit and then operates the decoding processes specified for the RBSP syntax structure in the NAL unit as follows.

Subclause Error: Reference source not found describes the decoding process for NAL units with nal_unit_type equal to 1 and 5.

NAL units with nal_unit_type equal to 7 and 8 contain sequence parameter sets and picture parameter sets, respectively. Picture parameter sets are used in the decoding processes of other NAL units as determined by reference to a picture parameter set within the slice headers of each picture. Sequence parameter sets are used in the decoding processes of other NAL units as determined by reference to a sequence parameter set within the picture parameter sets of each sequence.

NAL units with nal_unit_type equal to 14 contain adaptation parameter sets, which are used in the decoding processes of other NAL units as determined by reference to an adaptation parameter set within the slice headers of each picture. When an adaptation parameter set NAL unit is decoded, the following ordered steps apply:

- Let currApsId be equal to the aps_id value of the adaptation parameter set NAL unit being decoded.

- When currApsId is greater than or equal to max_aps_id_diff, all adaptation parameter sets with aps_id value less than currApsId – max_aps_id_diff and greater than currApsId are marked as “unused”.

- When currApsId is smaller than max_aps_id_diff, all adaptation parameter sets with aps_id value greater than currApsId and less than or equal to max_aps_id – ( max_aps_id_diff – ( currApsId + 1 ) ) are marked as “unused”.

- When partial_update_flag is equal to 1 and aps_scaling_list_data_referenced_flag is equal to 1, the values of syntax elements in the scaling_list_param( ) syntax structure are inferred to have the same values as in the scaling_list_param( ) syntax structure for the APS NAL unit with aps_id equal to common_reference_aps_id, if present, or aps_scaling_list_data_reference_aps_id, otherwise.

- When partial_update_flag is equal to 1 and aps_deblocking_filter_flag is equal to 1, the values of disable_deblocking_filter_flag, beta_offset_div2, and tc_offset_div2 are inferred to have the same values, respectively, as disable_deblocking_filter_flag, beta_offset_div2, if present, and tc_offset_div2, if present, in the APS NAL unit with aps_id equal to common_reference_aps_id, if present, or aps_deblocking_filter_reference_aps_id, otherwise.

- When partial_update_flag is equal to 1, aps_sao_interleaving_flag is 0, and aps_sample_adaptive_offset_flag is equal to 1, the values of syntax elements in the aps_sao_param( ) syntax structure are inferred to have the same values as in the aps_sao_param( ) syntax structure for the APS NAL unit with aps_id equal to common_reference_aps_id, if present, or aps_sao_reference_aps_id, otherwise.

- When partial_update_flag is equal to 1 and aps_adaptive_loop_filter_flag is equal to 1, the values of syntax elements in the alf_param( ) syntax structure are inferred to have the same values as in the alf_param( ) syntax structure for the APS NAL unit with aps_id equal to common_reference_aps_id, if present, or aps_alf_reference_aps_id, otherwise.

- The adaptation parameter set NAL unit being decoded is marked as “used”.

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).