| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0070r1 |
| --- | --- |

| Title: | Nested hierarchy of tiles and slices through slice header prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela and Antti Hallapuro Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

It is observed in this document that the primary difference between a tile-shaped slice and a tile included in a slice (as one of many tiles included in the same slice) is the presence or absence of the slice header. In the HEVC CD, a tile may contain one or many complete slices, or a slice may contain one or many complete tiles.

The contribution intends to simplify the hierarchy of picture partitioning units and keep tile as picture partitioning and scan order unit only by separating the slice header prediction mechanism from the tile concept. To be more detailed, this contribution proposes the following items:

- A picture delimiter NAL unit may carry a slice header, which may be used for decoding of more than one slice of the picture.

- A slice header beyond the slice address need not be provided for any slice.

- A slice header may be selectably predicted from the previous slice in scan order or from a slice header carried within the picture delimiter NAL unit.

- A tile shall contain an integer number of slices. A slice cannot contain data from more than one tile.

Revision 1 of the contribution includes source code that implements the proposed changes and provides simulation results. When a slice size of about 36 LCUs of size 64x64 was used, the proposed slice header prediction provided about 1.5% BD-rate reduction on average in low-delay B main configuration when compared to HM6.0. When compared to HM6.0 with one slice per picture and a tile size about 6x6 LCUs of size 64x64, the proposal provided about 0.6% BD-rate increase on average in low-delay B main configuration.

# Summary of the Proposal

The proposal can be summarized to contain the following items. The main motivations for each one of the items are also given below.

- A picture delimiter NAL unit may carry a slice header, which may be used for decoding of more than one slice of the picture.

Motivation:

- Most encoders keep the slice header unchanged during the encoding of a coded picture. Hence, for many coded pictures there are many redundant copies of the slice header.

- In our experience, HEVC is vulnerable to transmission errors and any type of error concealment is likely to result into an increase in both the magnitude and the spatial area of the error over time. As an example, the results on APS losses were studied in JCTVC-H0132.

A good strategy to cope with transmission errors is to make use of temporal scalability as follows. If a slice was lost in picture having temporal_id equal to M, where M > 0, the decoder can choose to skip decoding of subsequent pictures having temporal_id equal to or greater than M until the next IDR or CRA picture. The decoder may also gradually increase the number of decoded and displayed temporal layers at each TLA picture providing a temporal up-switching point from the previously highest decoded temporal layer.

- Characteristic data for the coded picture, such as the used picture parameter set and referenced picture set, is given upfront in the access unit before SEI and other NAL units, which may be used in some environments for purposes such as fast-forward play.

- A slice header beyond the slice address need not be provided for any slice.

Motivation:

- The overhead for coding regular slices is reduced as they can refer to the slice header carried in the access unit delimiter.

- It is proposed that a tile always contains a positive integer number of slices. The design of not requiring slice headers enables coding of tiles in such a way that a slice header is not required.

- A slice header may be selectably predicted from the previous slice in scan order or from a slice header carried within the picture delimiter NAL unit.

Motivation:

- The selectable encoding enables tiles and slices to predict a slice header either from the access unit delimiter or from the previous slice having a slice header, both of which are valid options for encoding.

- The option for predicting from the previous slice having a slice header can be used particularly for an entropy slice.

- A tile shall contain an integer number of slices. A slice cannot contain data from more than one tile.

Motivation:

- In our opinion, the picture partitioning becomes conceptually simpler with the proposal, as a tile is clearly a superset of a slice.

- Thanks to the slice header prediction, the compression efficiency of tiles remains in the same level as in the HEVC CD with one slice per picture.

# Reported Benefits of the Proposal

We identified the following benefits of the proposal presented in this contribution:

- Simplification of the picture partitioning hierarchy.

The contribution intends to simplify the hierarchy of picture partitioning units such a way that a tile is clearly a superset of a slice, i.e. a tile contains an integer number of slices. A tile is kept as picture partitioning and scan order unit only by separating the slice header prediction mechanism from the tile concept.

- Packetization flexibility.

It is asserted that the contribution has the following additional benefits for packetization flexibility. In HEVC CD, a slice is the only unit for packetization, even if a slice may contain multiple tiles that are independently decodable if the slice header is available through some means. The encoder has to determine the slice size at the time of encoding, but in some applications the optimal packet size is not known at the time of encoding or the bitstream may be conveyed over different network connections having different optimal packet size. This contribution enables the packetizer to determine how many tiles are encapsulated in one packet, hence providing more flexibility to encapsulate an encoded bitstream even in many cases where slice size was selected sub-optimally for delivery. Due to parallel processing requirements in the encoding and/or decoding, it is not uncommon that the tile size is often considerably smaller than the maximum transfer unit (MTU) size of typical networks – hence, tiles can be a good elementary unit for packet size matching to the MTU size.

- Re-packetization for MTU size matching.

In heterogeneous transmission environments it may happen that the MTU size differs in different parts of the network. If several tiles per picture are available, they can be used as elementary units for MTU size matching performed in a media gateway. In other words, a media gateway may encapsulate a suitable number of tiles into one packet and modify the slice headers of the encapsulated tiles in such a manner that the first tile in a packet contains a full slice header while the remaining slice headers in a packet are predicted from the first one or from the access unit delimiter.

- Tile as a random access point for an intra-coded picture.

As a tile contains an integer number of coded slice NAL units, it can be used to provide random access capability within intra-coded pictures at configurable granularity. This allows fast decoding of specific parts of potentially huge images without the need to decode the whole image. Such functionality is useful for example in still-image applications where only a part of the image is decoded and displayed on the screen.

- Access unit delimiter as a “summary” of the coded picture.

Characteristic data for the coded picture, such as the used picture parameter set and referenced picture set, is given upfront in the access unit before SEI and other NAL units, which may be used in some environments for purposes such as fast-forward play.

- Slice headers need not be redundantly coded.

In many transmission environments, source-coding-level error resilience is not desirable for example due to error propagation and accumulation cause by error concealment. However, slices may still be needed for MTU size matching. The option to include slice header in access unit delimiter avoids the need for coding slice headers redundantly. For error-prone environments, a mechanism to repeat slice headers in the transport level can be used and hence the packetizer can control the number of redundant copies of the slice header.

# Changes to the Specification Text

## Background for Design Decisions

In this section, we point out some excerpts from the HEVC CD that are relevant to the contribution.

If there are multiple tiles within a slice, they are separated from each other using a 3-byte marker (entry_point_marker_two_3bytes) and additionally an address of the slice data that follows is provided by tile_idx_minus_1. Thus, no slice headers are needed for tiles embedded in a slice.

| slice_data( ) { | Descriptor |
| --- | --- |
| ... |  |
| do { |  |
| ... |  |
| if( moreDataFlag && ( ( tiles_or_entropy_coding_sync_idc = = 1 && TileId[ CtbAddrTS ] != TileId[ CtbAddrTS − 1 ] ) ¦ ¦ ( tiles_or_entropy_coding_sync_idc = = 2 && num_substream_minus1 > 0 && CtbAddrTS / PicWidthInCtbs <= num_substream_minus1 && CtbAddrTS % PicWidthInCtbs = = 0 ) ) ) { |  |
| rbsp_trailing_bits( ) |  |
| if( nextbits(24) = = 0x000002 ) { |  |
| entry_point_marker_two_3bytes | f(24) |
| tile_idx_minus_1 | u(v) |
| } |  |
| } |  |
| } while( moreDataFlag ) |  |
| } |  |

In the syntax presented further below, a tile always contains an integer number of slices. In the bitstream format, a coded slice NAL unit starts with a 3-byte start code, similar to the 3-byte marker used for tiles within a slice in the HEVC CD. However, slices are coded NAL units, which provides more flexibility for packetization among other things than marker-separated tiles within a slice as in the HEVC CD.

The design of entropy slices resembles that of tiles, when it comes to the syntax elements that are present. When the byte stream format is used, a slice starts with a start code. The address of the slice follows (first_slice_in_pic_flag and slice_address).

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag = = 0 ) |  |
| slice_address | u(v) |
| slice_type | ue(v) |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| ... |  |

As we made modifications to the slice header, we also aligned the syntax of entropy slices to that of slices with and without slice header prediction. The entropy slice functionality has remained unchanged compared to that in the HEVC CD.

## Used Notation

In the following syntax structures, the added text is indicated with highlighting only, while deleted text is indicated with strikethrough, red font color, and highlighting. For the semantics, only the proposed new syntax elements are specified.

## Access Unit Delimiter

### Syntax

| access_unit_delimiter_rbsp( ) { | Descriptor |
| --- | --- |
| pic_type | u(3) |
| idr_pic_flag | u(1) |
| au_delimiter_slice_header_pred_idc | ue(v) |
| if( au_delimiter_slice_header_pred_idc = = 1 ) |  |
| slice_header_data( 0, 0, 1 ) |  |
| else if(au_delimiter_slice_header_pred_idc = = 2 ) |  |
| slice_header_data( 0, 1, 1 ) |  |
| au_delimiter_extension_flag | u(1) |
| if( au_delimiter_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| au_delimiter_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

### New Semantics

au_delimiter_slice_header_pred_idc equal to 0 specifies that no slice_header_data( ) syntax structure is present. au_delimiter_slice_header_pred_idc equal to 1 and 2 specify that slice_header_data( ) syntax structure is present and control which syntax elements are present in the slice_header_data( ) syntax structure.

NOTE – Encoders may assign the value of au_delimiter_slice_header_pred_idc as follows.

au_delimiter_slice_header_pred_idc equal to 0 indicates that no slice header is included in the access unit delimiter and consequently at least one slice header must be present in the coded slice NAL units.

au_delimiter_slice_header_pred_idc equal to 1 indicates that the syntax elements of the slice header except for adaptive loop filter parameters and entry points for parallel processing are present in the slice header included in the access unit delimiter. Encoders may set au_delimiter_slice_header_pred_idc equal to 1 when slices and/or tiles are used and the syntax elements in the slice header remain unchanged for the coded picture.

au_delimiter_slice_header_pred_idc equal to 2 indicates that those syntax elements of the slice header that are not required for initializing the entropy decoding process are present in the slice header included in the access unit delimiter. Encoders may set au_delimiter_slice_header_pred_idc equal to 2 when entropy slices are used and most syntax elements in the slice header remain unchanged for the coded picture.

## Slice Header

### Syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag = = 0 ) |  |
| slice_address | ue(v) |
| slice_type | ue(v) |
| slice_header_pred_idc | ue(v) |
| if( slice_header_pred_idc ) { |  |
| entropy_slice_flag | u(1) |
| slice_header_pred_src_flag | u(1) |
| if( slice_header_pred_idc = = 1 ) |  |
| slice_header_data( 1, 0, 0 ) |  |
| else if( slice_header_pred_idc = = 2 ) |  |
| slice_header_data( 1, 1, 0 ) |  |
| else if( slice_header_pred_idc = = 3 ) |  |
| slice_header_data( 1, 1, 1 ) |  |
| } |  |
| else |  |
| slice_header_data( 0, 0, 0 ) |  |
| } |  |

### New Semantics

slice_header_pred_idc specifies which syntax elements are present in the slice_header_data( ) syntax structure.

NOTE – Encoders may assign the value of slice_header_pred_idc as follows.

slice_header_pred_idc equal to 0 indicates that the slice header is not predicted and hence all syntax elements are present in the slice header. slice_header_pred_idc equal to 0 may be used for conventional slices (i.e. slices including a full slice header).

slice_header_pred_idc equal to 1 may be used for entropy slices. Those syntax elements of the slice header that are not required for initializing the entropy decoding process are predicted.

slice_header_pred_idc equal to 3 may be used for tiles for which a slice header is not desired. All syntax elements of the slice header are predicted.

slice_header_pred_idc equal to 2 may be used for slices that predict syntax elements from the access unit delimiter. However, adaptive loop filter parameters and entry points for parallel processing, if any, are not predicted.

slice_header_pred_src_flag equal to 0 specifies that previous the slice header with slice_header_pred_idc equal to 0 is used to derive those syntax element values that are not present in the slice header of the current slice. slice_header_pred_src_flag equal to 1 specifies that the picture delimiter NAL unit is present in the same access unit as the current coded slice and that the slice_header_data( ) syntax structure of the picture delimiter NAL unit is used to derive those syntax element values that are not present in the slice header of the current coded slice.

The values of slice_header_pred_idc and slice_header_pred_src_flag shall be such that a value is present or inferred for each syntax element in the slice header. When a syntax element is present both in the current slice header and in the slice header pointed to by the value of slice_header_pred_src_flag, the syntax element in the current slice header is effective.

## Slice Header Data (New Subclause)

| slice_header_data( pred1Flag, pred2Flag, pred3Flag ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag = = 0 ) |  |
| slice_address | u(v) |
| slice_type | ue(v) |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag!pred1Flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( IdrPicFlag ) { |  |
| idr_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } else { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| delta_poc_lsb_lt[ i ] | ue(v) |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt_minus1[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_interleaving_flag | u(1) |
| slice_sample_adaptive_offset_flag | u(1) |
| if( slice_sao_interleaving_flag && slice_sample_adaptive_offset_flag ) { |  |
| sao_cb_enable_flag | u(1) |
| sao_cr_enable_flag | u(1) |
| } |  |
| } |  |
| if( scaling_list_enable_flag ¦ ¦ deblocking_filter_in_aps_enabled_flag ¦ ¦ ( sample_adaptive_offset_enabled_flag && !slice_sao_interleaving_flag ) ¦ ¦ adaptive_loop_filter_enabled_flag ) |  |
| aps_id | ue(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| if( lists_modification_present_flag ) { |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| } |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| } |  |
| if( !pred2Flag && cabac_init_present_flag && slice_type != I ) |  |
| cabac_init_flag | u(1) |
| if( !entropy_slice_flag!pred1Flag ) { |  |
| slice_qp_delta | se(v) |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_in_aps_enabled_flag ) |  |
| inherit_dbl_params_from_aps_flag | u(1) |
| if( !inherit_dbl_params_from_aps_flag ) { |  |
| disable_deblocking_filter_flag | u(1) |
| if( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( slice_type != I && ((collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0) ¦ ¦ (!collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0) ) |  |
| collocated_ref_idx | ue(v) |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| } |  |
| if( !pred2Flag && ( slice_type = = P ¦ ¦ slice_type = = B ) ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| if( !pred3Flag && adaptive_loop_filter_enabled_flag ) { |  |
| slice_adaptive_loop_filter_flag | u(1) |
| if( slice_adaptive_loop_filter_flag && alf_coef_in_slice_flag ) |  |
| alf_param( ) |  |
| if( slice_adaptive_loop_filter_flag && !alf_coef_in_slice_flag ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| if( !pred2Flag && seq_loop_filter_across_slices_enabled_flag && ( slice_adaptive_loop_filter_flag ¦ ¦ slice_sample_adaptive_offset_flag ¦ ¦ !disable_deblocking_filter_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| if( !pred3Flag && tiles_or_entropy_coding_sync_idc > 0 ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| } |  |

## Slice Data

| slice_data( ) { | Descriptor |
| --- | --- |
| ... |  |
| do { |  |
| ... |  |
| if( moreDataFlag && ( ( tiles_or_entropy_coding_sync_idc = = 1 && TileId[ CtbAddrTS ] != TileId[ CtbAddrTS − 1 ] ) ¦ ¦ ( tiles_or_entropy_coding_sync_idc = = 2 && num_substream_minus1 > 0 && CtbAddrTS / PicWidthInCtbs <= num_substream_minus1 && CtbAddrTS % PicWidthInCtbs = = 0 ) ) ) { |  |
| rbsp_trailing_bits( ) |  |
| if( nextbits(24) = = 0x000002 ) { |  |
| entry_point_marker_two_3bytes | f(24) |
| tile_idx_minus_1 | u(v) |
| } |  |
| } |  |
| } while( moreDataFlag ) |  |
| } |  |

# Experimental results

The contribution was implemented in HM6.0 software and simulations were run using common test conditions as a basis. The encoder was further configured to use tiles as listed in Table 1. The goal was to configure the encoder to use approximate tile size of 6x6 LCUs (which corresponds to 384x384 pixels with 64x64 pixel LCU size).

| Picture width | Picture height | Number of tiles | Tile widths in LCUs | Tile heights in LCUs |
| --- | --- | --- | --- | --- |
| 416 | 240 | 1 | 7 | 4 |
| 832 | 480 | 4 | 7 6 | 4 4 |
| 1024 | 768 | 6 | 5 5 6 | 6 6 |
| 1280 | 720 | 8 | 5 5 5 5 | 6 6 |
| 1920 | 1080 | 15 | 6 6 6 6 6 | 6 6 5 |

Table 1 - Tile configuration

The performance of the proposed method was then examined by running the following three simulations.

- Each picture is divided into ~6x6 LCU tiles and each tile consists of 1 slice.

- Each picture is divided into ~6x6 LCU tiles and each tile consists of 1 slice. Slice header prediction as proposed is used by sending slice header in Acces Unit Delimiter.

- Each picture consists of 1 slice that is divided into ~6x6 LCU tiles. Each tile is preceded by a tile marker.

The simulation in 2) was compared against simulations 1) and 3) in order to understand the impact of the proposed method on bitrate in different scenarios.  It should be noted that run-time measurement are not reliable due to heterogenous nature of the simulation cluster and impact of load on the cluster. Nevertheless, the proposed method should not introduce any measurable complexity to the codec.

The comparison between simulation 1) and 2) is shown in  Table 2. This comparison shows that ideally 1.5% bitrate savings can be achieved if impact of slice header on the bitrate is minimized.

|  | Low delay B Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -1,3% | -1,5% | -1,7% |
| Class C | -0,7% | -0,6% | -0,7% |
| Class D | 0,2% | 0,2% | 0,2% |
| Class E | -5,1% | -4,8% | -4,5% |
| Overall | -1,5% | -1,5% | -1,5% |
|  | -1,5% | -1,5% | -1,5% |
| Class F | -2,0% | -2,1% | -1,9% |
| Enc Time[%] | 82 % |  |  |
| Dec Time[%] | 99 % |  |  |

Table 2 - Test cases 1 and 2 compared

Simulations 3) and 2) are compared in Table 3. This comparison shows the difference in coding efficiency when slice header (with prediction) is used instead of tile markers. In this comparison a small loss on 0.6% is measured.

|  | Low delay B Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0,8% | 0,6% | 0,4% |
| Class C | 0,7% | 0,8% | 0,5% |
| Class D | 0,2% | 0,2% | 0,2% |
| Class E | 0,8% | 0,0% | 0,7% |
| Overall | 0,6% | 0,4% | 0,4% |
|  | 0,6% | 0,5% | 0,5% |
| Class F | 0,6% | 0,7% | 0,5% |
| Enc Time[%] | 83 % |  |  |
| Dec Time[%] | 112 % |  |  |

Table 3 -  test case 3 and 2 compared

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).