| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0071 |
| --- | --- |

| Title: | On deblocking filter parameter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaru Ikeda Lu Shuo Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2731 Masaru.Ikeda@jp.sony.com Shuo.Lu@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

In visual tests of HM5, sequence ‘riverbed’ drew attention because of its severe blocking artifacts.It reflects the problem that current deblocking filter may be not effective enough. This contribution proposes three solutions to strengthen deblocking filter. The simulation results shows that better visual quality can be achieved.

# Problem Statement

In HM6.0, as shown in table 1-1, two table-derived thresholds β and tc are used to control the properties of deblocking filter. The amount of filtering increases with these thresholds values. Encoder-selected offsets, beta_offset_div2 and tc_offset_div2, are implemented to adjust values of β and tc to achieve better subjective quality than default values.

For the sequence ‘riverbed’, when it is encoded by HM6.0 under the common test conditions (e.g., LP-Main, QP=37), here β=36 and tc=4 is used as specified in Table 1-1 with 37 as input. As shown in Figure 1-1 block noises are very clear. Figure 1-2 shows all filtering applied. Green lines mark where weak filter are applied, and red lines mark where strong filters are applied. The other places are not filtered. It is observed that most of block artifacts lie on boundaries with no filter or weak filter. Therefore, positive offsets should be set. With biggest offsets 13, the maximum thresholds values, β=64, tc=13, can be selected. Figure 1-4 shows the filtering applied under this setting. It can be noticed that block noises are reduced where filtering applied changed from weak filter to strong filter. But still block noises remain as shown in Figure 1-3. The result shows that the deblocking filter needs to be strengthened further, which means the value range of thresholds need to be expanded.

Figure 1-1 riverbed, LP-Main, QP=37, Frame num=33

Figure 1-2 filtering applied to riverbed, LP-Main, QP=37, Frame num=33

The configuration of deblocking filter used to for the result of Figure 1-2

#=========== Deblock Filter =====

DeblockingFilterControlPresent  :1

LoopFilterOffsetInAPS         :0

LoopFilterDisable             :0

LoopFilterBetaOffset_div2      :13

LoopFilterTcOffset_div2       :13

#============================

Figure 1-3 riverbed, LP-Main, QP=37, Frame num=33, offsets = 13

Figure 1-4 filtering applied to riverbed, LP-Main, QP=37, Frame num=33, offsets = 13

Table 1–1 Derivation of threshold variables β and tC from input Q in HM6.0

| Q | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| β | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 7 | 8 |
| tC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| Q | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 |
| β | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 |
| tC | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 4 | 4 | 4 |
| Q | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 |  |  |  |
| β | 38 | 40 | 42 | 44 | 46 | 48 | 50 | 52 | 54 | 56 | 58 | 60 | 62 | 64 | 64 | 64 |  |  |  |
| tC | 5 | 5 | 6 | 6 | 7 | 8 | 9 | 9 | 10 | 10 | 11 | 11 | 12 | 12 | 13 | 13 |  |  |  |

rovided000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000

# Proposed solutions

## Solutions:

Since β plays main role at making decision for Luma deblocking filter as specified in 8.7.1[1], this contribution is proposed to enable encoders to select the value range of β. Through expanding β, the amount of strong filter is able to increase further. Referring to implementation, three methods are introduced as below.

Method1:

It is proposed to modify the clip operation of iIndexB addressing β (2-1), and to provide additional values of β when the index exceeding the maximum QP 51. Providing larger β can improve subjective quality on content where visible blocking noise remains. When iIndexB is less than 0, clip operation to 0 is applied after (2-1).

As shown in (2-2), by using one parameter beta_k_log2, it enables encoder to control the rate of change in additional β value.

iIndexB = qPL + ( beta_offset_div2 << 1 ) 		       				(2-1)

If iIndexB > 51,

β = β [51] + ((iIndexB –51) << beta_k_log2)					(2-2)

, where β[Q] is β value specified as Table 1-1 with the input Q.

Otherwise,

same as the current draft / HM6.0.

Method2:

In HM6, there is strong correlation between the quantization parameter QP and the value of β. As shown in Figure 2-1, where the x-axis is QP and the y-axis is the value of β specified as Table 1-1, β-QP forms a piecewise linear function as (2-3).

iIndexB = Clip3(0, 51, qPL + ( beta_offset_div2 << 1 ))

β = 0,                           for QP∊[ 0,16)

= β [16] + ((iIndexB –16) << 0),   for QP∊[16,28]

= β [28] + ((iIndexB –28) << 1),   for QP∊(28,51]	 		(2-3)

Based on current β-QP relation, it is proposed to enable encoder to control the slopes of β-QP. As shown in (2-4), by using nonnegative parameters beta_k0_log2 and beta_k1_log2, encoder can control the increment of β along with QP. When beta_k0_log2 is equal to 0 and beta_k1_log2 is equal to 1 as default value, β-QP relation is consistent with HM6. With bigger beta_k0_log2 or/and beta_k1_log2, bigger β can be provided to increase the amount of filtering.

β = 0,                               	   	for QP∊[ 0,16)

= β [16] + ((iIndexB –16) << beta_k0_log2),	for QP∊[16,28]

= β [28] + ((iIndexB –28) << beta_k1_log2), 	for QP∊(28,51]		(2-4)

Figure 2-1 β-QP in HM6

Method3:

In HM6 parameter table, β is determined by QP position which is controlled by the average quantization parameter QP of the adjacent two blocks and one offset. The effect of the offset is to increase the amount of filtering by choosing bigger β than default one, while to decrease the amount of filtering by using smaller β than default one. But it limits the adjustment only within the values linking to current QPs.

Instead of adjusting QP position, as shown in (2-5) it is proposed to add the offset value directly to default β. It can increase or decrease the value of β more flexibly. By using beta_k_log2 to control the granularity of the adjustment amount, it enables encoder to select proper value range of β depending on video content.

β = β [QP] + (beta_offset << beta_k_log2)						(2-5)

, where β[Q] is β value specified as Table 1-1 with the input Q.

## Syntax modifications:

As described before, proposed Method1 and Method3 will introduce a syntax element in deblocking filter as shown below:

Note – In Method3, beta_offset_div2 should be renamed as beta_offset because the value is not divided by 2 but divided by (1<<beta_k_log2).

Adaptation parameter set RBSP syntax

| aps_rbsp( ) { | Descriptor |
| --- | --- |
| aps_id | ue(v) |
| aps_scaling_list_data_present_flag | u(1) |
| if( aps_scaling_list_data_present_flag ) |  |
| scaling_list_param( ) |  |
| aps_deblocking_filter_flag | u(1) |
| if(aps_deblocking_filter_flag) { |  |
| disable_deblocking_filter_flag | u(1) |
| if( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| beta_k_log2 | ue(v) |
| } |  |
| } |  |
| … | u(1) |

Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_in_aps_enabled_flag ) |  |
| inherit_dbl_params_from_aps_flag | u(1) |
| if( !inherit_dbl_params_from_aps_flag ) { |  |
| disable_deblocking_filter_flag | u(1) |
| if( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| beta_k_log2 | ue(v) |
| } |  |
| } |  |
| } |  |
| … |  |

Proposed Method2 will introduce two syntax elements as shown below:

Adaptation parameter set RBSP syntax

| aps_rbsp( ) { | Descriptor |
| --- | --- |
| aps_id | ue(v) |
| aps_scaling_list_data_present_flag | u(1) |
| if( aps_scaling_list_data_present_flag ) |  |
| scaling_list_param( ) |  |
| aps_deblocking_filter_flag | u(1) |
| if(aps_deblocking_filter_flag) { |  |
| disable_deblocking_filter_flag | u(1) |
| if( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| beta_k0_log2 | ue(v) |
| beta_k1_log2 | ue(v) |
| } |  |
| } |  |
| … | u(1) |
| } |  |

Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_in_aps_enabled_flag ) |  |
| inherit_dbl_params_from_aps_flag | u(1) |
| if( !inherit_dbl_params_from_aps_flag ) { |  |
| disable_deblocking_filter_flag | u(1) |
| if( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| beta_k0_log2 | ue(v) |
| beta_k1_log2 | ue(v) |
| } |  |
| } |  |
| } |  |
| … |  |

# Results

## Subjective quality improvement

The simulation results are generated under the following conditions as representative. Proposed methods all enable encoder to adjust the value range of β flexibly by both offset and new parameter. It provides the encoder designers high flexibility to optimize the subjective quality according to different contents and demands.

Proposed methods are working as optional configurations, and only need to turn on when blocking noises appear. Sequence riverbed is tested under LP-Main while sequence Kimono is tested under AI-Main. That is because under these conditions, block noises can be observed. General encode settings are specified as Table 3-1.

For each method, results of using one example β value are shown as below to explain the effectiveness of expanded β.

Table 3-1 general encode condition

| riverbed | Test condition | LP-Main | Kimono | Test condition | AI-Main |
| --- | --- | --- | --- | --- | --- |
|  | Frame number to be encoded | 250 |  | Frame number to be encoded | 240 |
|  | Frame rate | 25 |  | Frame rate | 24 |
|  | QP | 37 |  | QP | 37 |

Table 3-2 parameter configurations

| Configuration | HM6.0 offset0 (common test condition) | HM6.0 offset13 | Method1 | Method2 | Method3 |
| --- | --- | --- | --- | --- | --- |
| DeblockingFilterControlPresent | 0 | 1 | 1 | 1 | 1 |
| LoopFilterBetaOffset_div2 | 0 | 13 | 12 | 3 | 13 |
| LoopFilterTcOffset_div2 | 0 | 13 | 13 | 13 | 13 |
| LoopFilterBetaK_log2 | N/A | N/A | 3 | 0 | 3 |
| LoopFilterBetaK1_log2 | N/A | N/A | N/A | 3 | N/A |

Table 3-3   β value corresponding to each parameter in Table 3-2

| QP | HM6.0 offset0 (common test condition) | HM6.0 offset13 | Method1 | Method2 | Method3 |
| --- | --- | --- | --- | --- | --- |
| 37 | 36 | 64 | 144 | 138 | 140 |

Both objective and subjective results are reported as below. Through adapting the parameter configurations as specified in Table 3-2, similar β value is selected for all three methods. As a result, the decoded results have similar visual quality. For the areas where filtering applied changed from weak filter to strong filter, the block noises decrease accordingly as expected. Please refer to Figure 3-1 to 3-16 for details.

The objective results are shown in Table 3-4. It shows that proposed methods can improve visual quality while keep or improve the objective evaluations.

Table 3-4 coding result

| sequence | QP | Condition | Bitrate [kbps] | Ypsnr [db] | Upsnr [dB] | Vpsnr [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| riverbed | 37 | HM6.0 offset 0 | 2910.48 | 31.71 | 37.91 | 40.73 |
|  |  | HM6.0 offset13 | 2903.44 | 31.73 | 37.91 | 40.73 |
|  |  | Method1 | 2897.95 | 31.73 | 37.91 | 40.72 |
|  |  | Method2 | 2898.35 | 31.73 | 37.91 | 40.72 |
|  |  | Method3 | 2898.75 | 31.73 | 37.90 | 40.73 |
| Kimono | 37 | HM6.0 offset 0 | 3795.37 | 36.51 | 40.05 | 41.13 |
|  |  | HM6.0 offset13 | 3794.66 | 36.50 | 40.05 | 41.13 |
|  |  | Method1 | 3793.88 | 36.47 | 40.05 | 41.13 |
|  |  | Method2 | 3793.90 | 36.47 | 40.05 | 41.13 |
|  |  | Method3 | 3793.88 | 36.47 | 40.05 | 41.13 |

Method1:

Figure 3-1 riverbed, LP-Main, QP=37, Frame num=33, offset=12, Method1: LoopFilterBetaK_log2=3

Figure 3-2 riverbed, LP-Main, QP=37, Frame num=33, offset=12, Method1: LoopFilterBetaK_log2=3

Method2:

Figure 3-3 riverbed, LP-Main, QP=37, Frame num=33, offset=3

Method2: LoopFilterBetaK_log2=0, LoopFilterBetaK1_log2=3

Figure 3-4 riverbed, LP-Main, QP=37, Frame num=33, offset=3

Method2: LoopFilterBetaK_log2=0, LoopFilterBetaK1_log2=3

Method3:

Figure 3-5 riverbed, LP-Main, QP=37, Frame num=33, offset=13, Method3: LoopFilterBetaK_log2=3

Figure 3-6 riverbed, LP-Main, QP=37, Frame num=33, offset=13, Method3: LoopFilterBetaK_log2=3

HM6.0

Figure 3-7 Kimono, AI-Main, QP=37, Frame num=19

Figure 3-8 Kimono, AI-Main, QP=37, Frame num=19

HM6.0 with offset13

Figure 3-9 Kimono, AI-Main, QP=37, Frame num=19, offset=13

Figure 3-10 Kimono, AI-Main, QP=37, Frame num=19, offset=13

Method1:

Figure 3-11 Kimono, AI-Main, QP=37, Frame num=19, offset=12, Method1: LoopFilterBetaK_log2=3

Figure 3-12 Kimono, AI-Main, QP=37, Frame num=19, offset=12, Method1: LoopFilterBetaK_log2=3

Method2:

Figure 3-13 Kimono, AI-Main, QP=37, Frame num=19, offset=3

Method2: LoopFilterBetaK_log2=0, LoopFilterBetaK1_log2=3

Figure 3-14 Kimono, AI-Main, QP=37, Frame num=19, offset=3

Method2: LoopFilterBetaK_log2=0, LoopFilterBetaK1_log2=3

Method3:

Figure 3-15 Kimono, AI-Main, QP=37, Frame num=19, offset=13, Method3: LoopFilterBetaK_log2=3

Figure 3-16 Kimono, AI-Main, QP=37, Frame num=19, offset=13, Method3: LoopFilterBetaK_log2=3

# Conclusion

The core motivation of proposed solution above is to provide additional β values where is not covered by current HM6. For the tested sequence ‘riverbed’, all of the 3 methods can remove visible blocking noises more effectively than HM6.0 offset 0 and offset 13. Simulation results show that comparing with HM6, the proposed methods have negligible influence on Bitrate/SNR.

Among these three methods, Method3 can control deblocking filter flexibly and simply, and it is also easy to modify the other variable tc following the same rule if necessary. It is recommended to adopt this one.

# References

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: WD modification

Replace the following part,

8.7.1.4.1

A variables βis specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 51, qPL + ( beta_offset_div2 << 1 ) ) as input.

to the following part.

Method1:

A variable β is derived as follows:

- Luma quantization parameter is derived as follow:

iIndexB = max(0,qPL + ( beta_offset_div2 << 1 ))

- If iIndexB is equal or less than 51, β is specified as Table8-12 with iIndexB as input.

- Otherwise (iIndexB is greater than 51), the following ordered steps apply:

- β [51] is specified as Table 8-12 with input 51.

- The β value is derived as follow:

β = Clip3 (0, 255, β [51]+ ( ( iIndexB –51) << beta_k_log2) )

where beta_k_log2 determines the rate of change in β value along with the change of iIndexB.

Method2:

A variable β is derived as follows:

- Luma quantization parameter is derived as follow:

iIndexB = Clip3( 0, 51, qPL + ( beta_offset_div2 << 1 ) )

- If iIndexB is equal to or greater than 16, the following applies:

- If iIndexB ∊[16, 28], β is derived as follow:

β = Clip3 (0, 255, 6 + ((iIndexB –16) << beta_k0_log2))

- Otherwise(iIndexB ∊(28, 51]), β is derived as follow:

β = Clip3 (0, 255, β [28] + ((iIndexB –28) << beta_k1_log2)),

where β[28] = Clip3 (0, 255, 6 + ((28 –16) << beta_k0_log2))

beta_k0_log2 and beta_k1_log2 determines the rate of change in β value along with the change of iIndexB∊[16,28], and iIndexB∊(28, 51] respectively.

- Otherwise (iIndexB is less than 16), β is set equal to 0.

Method3:

A variable β is derived as follow:

β = Clip3 (0, 255, β [qPL] + ( beta_offset << beta_k_log2) )

where β[qPL] is specified as Table 8-12 with luma quantization parameter qPL as input.