| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0072 |
| --- | --- |

| Title: | Usage of cropping rectangle and sample aspect ratio to support 2D-compatible services in “Frame compatible Plano-Stereoscopic 3DTV” within the HEVC specification. |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marco Arena, RAI Paola Sunna, RAI | Email: Email: | marco.arena@rai.it p.sunna@rai.it |
| Source: | RAI |  |  |

_____________________________

# Abstract

This contribution proposes a way to use HEVC signalling to distribute 3D stereoscopic frame compatible video in a 2D compatible manner.

# Introduction

Frame compatible formats are the key technology that is being adopted in the first generation of 3DTV broadcasting services.

Squeezing the Left and Right images in a single High Definition frame allows the service provider to reuse part of the existing production infrastructure and the whole of the existing distribution infrastructure. In most cases, the set top boxes already deployed at the user’s premises for HD services can also be reused.

The MPEG2 (ISO/IEC 13818-2) and AVC (ISO/IEC 14496-10) specifications provide suitable signalling for these formats.

Due to the geometrical layout of the composite frame, that contains both the left and the right views, the resulting video will not be service (meaning backwards or 2D) compatible: legacy HD decoders will be able to decompress the video, but 2D TV-sets or displays will display the composite picture and not, as it would be desirable, one of the two composing images (L or R).

This puts an undesirable burden on the service operator, since an additional 2D signal has to be simulcast, requiring double capacity occupation on the broadcast multiplex.

This is becoming even more relevant since, progressive conversion of services from SD to HD (with the related increase of coding bitrate) and the possibility for a second digital dividend (UHF 700 MHz band) already prospected at ITU WRC-12 after 2015, are making RF spectrum resources even more precious.

Having suitable technical means enabling delivery of “2D service compatible” frame compatible formats would be of extreme interest for those operators not willing to activate dedicated channels for 3D services but, nonetheless, intending to broadcast stereoscopic content on a “per event” basis.

A “2D service compatible” 3D system is defined as a 3D transmission mode which may be decoded by 2D receivers to deliver 2D video to 2D displays. It is assumed that the 2D service compatible mode transmits the same 3D video frames as the frame compatible mode. The main advantage of this mode is that a single video signal is sufficient to support 2D and 3D vision, thus it is preferable by terrestrial broadcasters often facing a scarcity of frequency resources.

Since the transmitted video frames are the same, in both modes new 3D ready decoders may be 2D service compatible and frame compatible at the same time (i.e., depending on the connected display and on the user choices, the decoder can deliver a 2D or a 3D service).

A 2D-service compatible mode of operation for frame compatible formats has already been defined within the DVB Frame Compatible Plano-Stereoscopic 3DTV (DVB-3DTV http://www.etsi.org/deliver/etsi_ts/101500_101599/101547/01.01.01_60/ts_101547v010101p.pdf) specification (available here), leveraging on signalling already existing within the AVC specification (the frame cropping rectangle parameter).

Since there are no theoretical limitations to the values this parameter can assume (any meaningful values within the boundaries of the image), the cropping rectangle can be used to output any rectangular region of the input frame.

In particular, if the input frame is a composite frame used for the distribution of 3D stereoscopic video (in the so called Side by Side or Top and Bottom Frame Compatible modes), the cropping rectangle can be used to delimit an area within the frame containing one of the two 2D views: as illustrated in the drawings, a H.264 compliant decoder, decompressing such a video would only output the view contained within the cropping rectangle.

The output video, properly scaled, provides a 2D-compatible format.

3DTV receivers are required to ignore the cropping rectangle parameter when SEI frame packing arrangement messages indicate that a frame compatible format is being decoded, so as to be able to use also the part of the decoded picture outside the cropping rectangle bounds (needed to present the second view).

Please, note that this behavior, even though it works in practical implementations, is probably not fully conformant to the intended meaning of the AVC specification, where the cropping rectangle should contain the whole frame compatible format.

At the same time, it should be noted that, since this feature is deemed to be a powerful tool, regional standardization bodies such as HD Forum Italia have decided to make its support mandatory for decoders coming to the Italian market.

# Scope of this document

Scope of this document is to propose a possible technical method to implement a 2D service compatible mode of operation for frame compatible video within the HEVC specification. The authors are ready to analyze any other technical solutions that MPEG members may propose.

As already anticipated, HEVC is one of the tools needed to cope with the forthcoming second digital dividend and should, therefore, provide all the tools that service operators (especially broadcasters) would need to speed up the progressive adoption of new service models.

In particular, since the introduction of 3DTV services will be slow and, probably, event-centered, more than based on day-to-day production (also due to the scarcity of content and the low penetration of 3D decoders on the market), most service operators will have limited resources (especially in terms ob bitrate) available. Being able to distribute 3D content in 2D compatible manner with a low overhead (10-15% of the bitrate needed for 2D encoding), would be for most operators (in particular over digital terrestrial networks) the only feasible way to introduce 3D services.

There is, therefore, a definite interest in having the possibility to use frame compatible formats in HEVC together with the cropping rectangle so as to obtain 2D compatibility in the distribution of stereoscopic streams.

The behavior to enable is schematically described in the following flowchart: based on the knowledge of the display it is connected to, a decoder can be asked to either apply the cropping rectangle as it is defined in the sequence parameter set (if the display is 2D), or, knowing that the display is 3D and that the decoded stream contains a frame compatible format, to skip it and feed the display with the whole decoded content.

no

Please, note that the required signalling structures (SEI messages for frame packing arrangement or, better, a placeholder thereof, and cropping rectangle in the sequence parameter set) are already defined in HEVC.

What is currently missing is a way to indicate to a decoder willing to extract the two views that part of the “legal” video content is outside the cropping rectangle.

This might be done in several different ways:

- Adding a second cropping rectangle to enclose the whole stereoscopic format (this would increase the signalling overhead)

- Defining suitable frame_packing_arrangement_type SEI messages to describe a format in which only one of the two views is enclosed by the cropping rectangle

- Defining a suitable content_interpretation_type value.

# Patent right declaration

RAI does not have any current or pending patent rights relating to the technology described in this contribution.