#### Modifications to the working draft highlighted in yellow.

#### 7.3.2.8 Sample adaptive offset VLC syntax

| sao_offset_vlc( rx, ry, cIdx ) { | Descriptor |
| --- | --- |
| sao_type_idx[ cIdx ][ rx ][ ry ] | ue(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = =5 ) { |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | u(5) |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset_sign[ cIdx ][ rx][ ry ] | u(1) |
| sao_offset[ cIdx ][ rx][ ry ][ i ] | ue(v) |
| } else if( sao_type_idx[ cIdx ][ rx ][ ry ] != 0 ) |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset[ cIdx ][ rx][ ry ][ i ] | u(1) |
| } |  |

#### 7.3.4.2 Sample adaptive offset CABAC syntax

| sao_offset_cabac( rx, ry, cIdx ) { | Descriptor |
| --- | --- |
| sao_type_idx[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = =5 ){ |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset_sign[ cIdx ][ rx][ ry ] | ae(v) |
| sao_offset[ cIdx ][ rx][ ry ] | ae(v) |
| } else if( sao_type_idx[ cIdx ][ rx ][ ry ] != 0 ) |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset[ cIdx ][ rx][ ry ][ i ] | ae(v) |
| } |  |

#### 7.4.2.8 Sample adaptive offset VLC semantics

sao_type_idx[ cIdx ][ rx ][ ry ] indicates the offset type as specified in Table 7-1 of current coding treeblock at position rx and ry for the colour component cIdx.

When sao_type_idx[ cIdx ][ rx ][ ry ] is not present, it is inferred as follows.

- If sao_merge_up_flag is equal to 1, sao_type_idx[ cIdx ][ rx ][ ry ] is set equal to sao_type_idx[ cIdx ][ rx ][ ry − 1 ].

- Otherwise, sao_type_idx[ cIdx ][ rx ][ ry ] is set equal to sao_type_idx[ cIdx ][ rx − 1 ][ ry ].

Table 7-1 – Specification of the edge type for SAO

| - sao_type_idx[ cIdx ][ rx ][ ry ] | - Edge type (informative) |
| --- | --- |
| - 0 | - Not applied |
| - 1 | - 1D 0-degree edge |
| - 2 | - 1D 90-degree edge |
| - 3 | - 1D 135-degree edge |
| - 4 | - 1D 45-degree edge |
| - 5 | - Band |

sao_band_position[ cIdx ][ rx ][ ry ] indicates the displacement of the band offset of the pixel range when sao_type_idx[ cIdx ][ rx ][ ry ] is equal to 5.

When sao_band_position[ cIdx ][ rx ][ ry ] is not present it is inferred as follows.

- If sao_merge_up_flag is equal to 1, sao_band_position[ cIdx ][ rx ][ ry ] is set equal to sao_band_position[ cIdx ][ rx ][ ry − 1 ].

- Otherwise, sao_band_position[ cIdx ][ rx ][ ry ] is set equal to sao_band_position[ cIdx ][ rx − 1 ][ ry ].

sao_offset_sign[ cIdx ][ rx ][ry] indicates the sign of the band offset value of i-th category of current coding treeblock at position rx and ry for the colour component cIdx

sao_offset[ cIdx ][ rx ][ ry ][ i ] indicates the offset value of i-th category of current coding treeblock at position rx and ry for the colour component cIdx.

The variable bitDepth is derived as follows.

- If cIdx is equal to 0, bitDepth is set equal to BitDepthY..

- Otherwise (cIdx is equal to1 or 2), bitDepth is set equal to BitDepthC.

It is a requirement of bitstream conformance that when sao_type_idx[ cIdx ][ rx ][ ry ] is not equal to 5, the values of sao_offset[ cIdx ][ rx ][ ry ][ i ] shall be in the range of 0 to 1 ( 1<< ( Min( bitDepth, 10 ) − 5 ) ) − 1, inclusive and otherwise, the values of sao_offset[ cIdx ][ rx ][ ry ][ i ] shall be in the range of 0 −( 1<< ( Min( bitDepth, 10 ) − 5 ) ) to ( 1<< ( Min( bitDepth, 10 ) − 5 ) ) − 1, inclusive. A further requirement of bitstream conformance is that the values of sao_offset shall be such that the pixels of the sao modified picture buffer are within 0 to (1<<BitDepth)-1.

When sao_offset[ cIdx ][ rx ][ ry ][ i ] is not present, it is inferred as follows.

- If sao_merge_up_flag is equal to 1, sao_offset[ cIdx ][ rx ][ ry ][ i ] is set equal to sao_offset[ cIdx ][ rx ][ ry − 1 ][ i ].

- Otherwise, sao_offset[ cIdx ][ rx ][ ry ][ i ] is set equal to sao_offset[ cIdx ][ rx − 1 ][ ry ][ i ].

The variable offsetSign is derived as follows.

- If sao_type_idx[ cIdx ][ rx ][ ry ] is less than 5 and i is larger than 1,offsetSign is set to −1.

- Otherwise (sao_type_idx[ cIdx ][ rx ][ ry ] is equal to 5 or i is less than 2), offsetSign is set to 1.

The array SaoOffsetVal is derived as follows when sao_type_idx[ cIdx ][ rx ][ ry ] is equal to 5.

SaoOffsetVal[ cIdx ][ rx ][ ry ][ 0 ] = 0		(7-1)

SaoOffsetVal[ cIdx ][ rx ][ ry ][ 1 ] = 
			_sao_offset_sign[ cIdx ][ rx ][ ry ]*(1 << sao_offset[ cIdx ][ rx ][ ry ]) ( bitDepth – Min( bitDepth, 10 ) ) 		(7-2)

The array SaoOffsetVal is derived as follows when sao_type_idx[ cIdx ][ rx ][ ry ] is in the range of 1 to 4:

SaoOffsetVal[ cIdx ][ rx ][ ry ][ 0 ] = 0		(7-3)

SaoOffsetVal[ cIdx ][ rx ][ ry ][ i + 1 ] = 
			offsetSign*(sao_offset[ cIdx ][ rx ][ ry ][ i ] << (bitDepth-8))( bitDepth – Min( bitDepth, 10 ) ) 	(7-4)

##### Modification process for luma and chroma samples

Inputs to this process are:

–	a variable cIdx specifying colour component index,

–	a sample position ( xC, yC ),

–	a pair of variables ( rx, ry ) specifying the coding treeblock location,

–	an array saoValueArray specifying offset values,

–	a block size nS.

Output of this process is a modified picture buffer for the colour component cIdx.

Let recSaoPicture represents the processed sample array of the current picture of colour component cIdx and saoTypeIdx is set equal to sao_type_idx[ cIdx ][ rx ][ ry ].

Variable bitDepth is set equal to BitDepthY if cIdx is equal to 0, otherwise, set equal to BitDepthC. A variable bitDepthIncr is set equal to bitDepth minus 8.

Depending on the value of saoTypeIdx, the following applies:

–	If saoTypeIdx is equal to one of the values of 1, 2, 3 or 4, the following ordered steps apply:

- Arrays hPos[2] and vPos[2] are specified in Table 8-2.

- A variable edgeIdx is derived as follows.

edgeIdx = 2 + ∑k( Sign(( recPicture[ xC + i, yC + j ]>>bitDepthIncr) –

(recPicture[ xC + i + hPos[ k ], yC + j + vPos[ k ] ]>>bitDepthInr) ) ) with k = 0..1	(8-5)

- When any of the following conditions is true, edgeIdx is set equal to 2.

- The pixel at position  (xC + i + hPos[ k ], yC + j + vPos[ k ]) is ouside picture boundary

- slice_loop_filter_across_slices_enabled_flag is equal to 0 and the pixel at position (xC + i + hPos[ k ], yC + j + vPos[ k ]) belongs to different slice.

- loop_filter_across_tiles_enabled_flag is equal to 0 and the pixel at position (xC + i + hPos[ k ], yC + j + vPos[ k ]) belongs to different tile.

- The reconstructed picture buffer is modified as follows:

- If pcm_loop_filter_disable_flag is equal to 1 and the recPicture[ xC + i, yC + j ] is a reconstructed sample of an I_PCM block or qpprime_y_zero_transquant_bypass_flag is equal to 1 and QP’Y is equal to 0, the recSaoPicture[ xC + I yC + j ] is derived as

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ]	(8-6)

with i = 0..nS−1 and j = 0..nS−1 where 	edgeTable[5] = { 1, 2, 0, 3, 4}.

- Otherwise,

recSaoPicture[ xC + i, yC + j ] = Clip3(0, (1<<bitDepth)-1, recPicture[ xC + i, yC + j ] + saoValueArray[ edgeTable[ edgeIdx ] ]	)(8-7)

with i = 0..nS−1 and j = 0..nS−1 where 	edgeTable[5] = { 1, 2, 0, 3, 4}.

- Otherwise, , if saoTypeIdx is equal to 5, the following ordered steps apply:

- If cIdx is equal to 0, the variable bandShift is set equal to BitDepthY − 5

- Otherwise (cIdx is not equal to 0), bandShift is set equal to BitDepthC − 5.

- The variable saoLeftClass is set equal to sao_band_position[ cIdx ][ rx ][ ry ].

- The vector bandTable is defined with 32 elements and all elements are initially set to 0. Then, 4 of its elements (indicating the position of bands for which an offset is explicitly transmitted) are modified as follows:

for( i = 0; i < 4; i++ ) 
	bandTable[ (i + saoLeftClass) & 0x1F ]  = i + 1		(8-8)

if(saoLeftClass >0)

bandTable[ (i + saoLeftClass - 1) & 0x1F ]  = 1

- The reconstructed picture buffer recSaoPicture is modified as follows.

bandIdx = ( recPicture[ xC + i, yC + j ] >> bandShift )
recSaoPicture[ xC + i, yC + j ] = Clip3(0, (1<<bitDepth)-1, recPicture[ xC + i, yC + j ] + saoValueArray[ bandTable[ bandIdx ] ])	(8-9)
	with i = 0..nS−1 and j = 0..nS−1

–	Otherwise (sao_type_idx[ cIdx ][ rx ][ ry ] is equal to 0), the following applies:

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] with i = 0..nS−1 and j = 0..nS−1	(8-10)

Table 8-2 – Specification of hPos[2] and vPos[2] according to the type of sample adaptive offset process

| sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| hPos[0] | −1 | 0 | −1 | 1 |
| hPos[1] | 1 | 0 | 1 | −1 |
| vPos[0] | 0 | −1 | −1 | −1 |
| vPos[1] | 0 | 1 | 1 | 1 |