| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0073_r1 |
| --- | --- |

| Title: | AHG 6: SAO simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenneth Andersson, Per Wennersten, Rickard Sjöberg Färögatan 6 16480 Stockholm | Tel: Email: | +46 10 7141538 kenneth r andersson @ ericsson com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution claims that similar coding efficiency and subjective quality as HM6.0 CE1 conditions anchors can be achieved (BDR luma 0.0%, chroma -1.2%) using about 50% less memory for SAO parameters in worst case. The contribution proposes to use one single band offset with a larger width of the band compared to HM6.0, band offset magnitudes restricted to a power of 2 and having edge offsets restricted to be equal to 0 or equal to a fixed value depending on which edge type and bit depth that is used so that edge offsets not abruptly can change the edge characteristics. Other claimed features of the proposal are that clamping (clipping) operations can be avoided and that band offsets can be implemented with sign operations instead of table lookup. WD draft modifications and software are attached with the proposal.

# Introduction

In HEVC SAO [1] can be of two kinds, either edge offsets or band offsets. Edge offsets can be used in one of 4 directions; horizontal, vertical, diagonal from top left to bottom right, or diagonal from bottom left to top right. The specific direction is determined by sao_type_idx = 1 .. 4.  sao_type_idx = 5 is used for band offsets. sao_type_idx = 0 correspond to no SAO filtering at all.

When band offset is selected (e.g. sao_type_idx is 5) four offsets are used for four consecutive bands. The band position for the first band is indicated by sao_band_position which is sent in the bitstream with 5 bits accuracy (0..31).

When edge offset is selected (e.g. sao_type_idx is in the range of 1 to 4) four offsets are used.

To make the SAO band filtered value stay within the allowed range according to the bit depth, a clamping  operation is required. This is not specified in the working draft HEVC [1] but it exists in HM6.0 software where a table with same number of entries as the 2bit depth - 1 plus the range of offset values is used for clamping.

# SAO simplification

## Simplification of band offsets

It is proposed to reduce the number of band offset from 4 to 1 and double the width of the band selected by band position while maintaining the accuracy of the band position at 5 bits. The single band offset is either positive or negative and never zero when the proposed single band offset has been selected. Thus we have chosen to encode the sign separately from the offset magnitude. Furthermore, to reduce the cost of coding the single band offset it is restricted to be a power of 2, e.g. ..-4, -2, -1, 1, 2, 4 …

The RDO works similar as HM6.0 with storing deviations between original pixel values and reconstructed pixel values after deblocking. Since the width of the band is doubled in the proposal statistics from two neighbouring bands are merged to one band. The RDO selects band position and offset with least cost similar as is HM6.0 but using only a single offset.

An additional feature of the band offset simplification is that it can be implemented with sign operations instead of table lookup which could be preferred in some implementations. It is shown in the code how to do it but it is turned off by default:

bandIdx = ( recPicture[ xC + i, yC + j ] >> bandShift )-band_position

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] + (sign(bandIdx +R1)+sign(-bandIdx+R2)) << sao_offset

where R1=1 and R2=0 when the saoOffset is positive, an R1=0 and R2=-1 when the saoOffset is negative.

BandIdx is the band of the current pixel adjusted by the band position.

## Simplification of edge offsets

It is proposed to restrict edge offsets to 0 or (1<<(bitDepth-8)) when edgeIdx is equal to 0 or 1, and restrict edge offsets to 0 or –(1<<(bitDepth-8)) when edgeIdx is equal to 3 or 4. This simplification ensures for bitDepth equal to 8 that the edge characteristic will not drastically change from for example a valley with edgeIdx=0 to a hill.

Example for 8 bit video:

Consider a neighbourhood of a-b-c, where b is the centre pixel. If b is a hill the proposed offset is -1 or 0 thus b cannot become less than a or c and thus it cannot go below 0. Similar reasoning can be made for other edge types.

Furthermore no clamping is needed for bitDepth equal to 8.

To enable similar properties for bitDepth higher than 8 it is proposed to modify the determination of edgeIdx as follows:

edgeIdx = 2 + Sign((recPicture[x,y] >>bitDeptIncr)−(recPicture[x+ hPos[k],y+ vPos[k]]>>bitDeptIncr)) with k=0..1

where bitDepthIncr is equal to bitDepth-8.

The RDO works similar as HM6.0 with the difference that the offset values are restricted as indicated above and that each offset is represented by one bit.

## Requirement on bitstream conformance

It is proposed to add a requirement on offsets values that they shall be such that pixel values of the picture buffer stays within the range 0 to (1<<bitDepth)-1, where bitDepth is the bit depth at which the video is encoded. Thus no clamping operation is needed for either edge or band offsets.

For band offsets this is achieved by restricting offsets in the encoder in both ends of the band range.

# Results

Results are presented for random access with SAOInterleaving equal to 0 for both anchor and test, and for low delay with SAOInterleaving equal to 1 for both anchor and test. LCU size is 64x64.

Visual inspection indicates that the subjective quality of the proposal is similar as the anchor.

The results in the table below indicate that the overall BDR difference to HM6.0 with CE1 conditions [2] including low delay P for both BO and EO simplifications is 0.0% for luma and -1.2% for chroma with similar decoding time as the anchor. For only BO simplifications the BDR difference is 0.0% for luma and -0.3% for chroma with similar decoding time as the anchor. Negative BDR corresponds to gain.

The encoding was performed on a linux cluster thus encoding times are unreliable. The encoding time is expected be similar as the anchor.

|  | All Intra Main |  |  |  | All Intra HE10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V |  | Y | U | V |
| Class A |  | 0,1% | -1,1% | -1,4% |  | 0,2% | -0,6% | -0,7% |
| Class B |  | 0,0% | -0,6% | -0,8% |  | 0,2% | -0,5% | -0,8% |
| Class C |  | -0,1% | -0,3% | -0,1% |  | 0,1% | -0,5% | -0,5% |
| Class D |  | 0,0% | -0,6% | -0,6% |  | 0,1% | -0,7% | -1,0% |
| Class E |  | 0,0% | -0,9% | -1,5% |  | 0,2% | -0,3% | -1,2% |
| Overall |  | 0,0% | -0,7% | -0,9% |  | 0,1% | -0,5% | -0,8% |
|  |  | 0,0% | -0,7% | -0,9% |  | 0,1% | -0,5% | -0,8% |
| Class F |  | 0,6% | 0,2% | 0,8% |  | 0,7% | -0,5% | -0,1% |
| Enc Time[%] |  | 72% |  |  |  | 69% |  |  |
| Dec Time[%] |  | 101% |  |  |  | 100% |  |  |
|  |  | Random Access Main |  |  |  | Random Access HE10 |  |  |
|  |  | Y | U | V |  | Y | U | V |
| Class A |  | 0,2% | -0,5% | -0,6% |  | 0,5% | 0,7% | 0,6% |
| Class B |  | 0,1% | -0,4% | -0,5% |  | 0,4% | 0,5% | 0,4% |
| Class C |  | 0,0% | 0,0% | 0,2% |  | 0,3% | 0,3% | 0,5% |
| Class D |  | -0,1% | -0,4% | -0,2% |  | 0,1% | 0,1% | -0,1% |
| Class E |  |  |  |  |  |  |  |  |
| Overall |  | 0,0% | -0,3% | -0,3% |  | 0,3% | 0,4% | 0,3% |
|  |  | 0,0% | -0,3% | -0,3% |  | 0,3% | 0,4% | 0,3% |
| Class F |  | 0,6% | 0,4% | 1,5% |  | 1,0% | 0,4% | 1,5% |
| Enc Time[%] |  | 100% |  |  |  | 82% |  |  |
| Dec Time[%] |  | 101% |  |  |  | 100% |  |  |
|  |  | Low delay B Main |  |  |  | Low delay B HE10 |  |  |
|  |  | Y | U | V |  | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |
| Class B |  | 0,0% | -1,7% | -2,4% |  | 0,0% | -1,7% | -2,3% |
| Class C |  | 0,0% | -0,9% | -0,8% |  | -0,2% | -1,2% | -1,3% |
| Class D |  | -0,2% | -1,1% | -1,6% |  | -0,3% | -2,0% | -1,7% |
| Class E |  | 0,3% | -3,0% | -3,3% |  | 0,1% | -0,9% | -2,2% |
| Overall |  | 0,0% | -1,6% | -1,9% |  | -0,1% | -1,5% | -1,9% |
|  |  | 0,0% | -1,6% | -1,9% |  | -0,1% | -1,5% | -1,8% |
| Class F |  | 0,4% | -0,5% | -0,2% |  | -0,1% | -0,5% | -1,4% |
| Enc Time[%] |  | 69% |  |  |  | 76% |  |  |
| Dec Time[%] |  | 101% |  |  |  | 101% |  |  |
|  |  | Low delay P Main |  |  |  | Low delay P HE10 |  |  |
|  |  | Y | U | V |  | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |
| Class B |  | -0,3% | -2,5% | -2,9% |  | -0,1% | -2,5% | -2,8% |
| Class C |  | 0,0% | -1,2% | -1,3% |  | -0,2% | -2,0% | -2,1% |
| Class D |  | -0,1% | -1,5% | -2,3% |  | -0,4% | -2,2% | -2,4% |
| Class E |  | -0,2% | -3,6% | -4,6% |  | 0,1% | -2,4% | -3,4% |
| Overall |  | -0,2% | -2,1% | -2,7% |  | -0,2% | -2,3% | -2,6% |
|  |  | -0,2% | -2,0% | -2,6% |  | -0,2% | -2,2% | -2,5% |
| Class F |  | 0,5% | -0,6% | -0,4% |  | 0,0% | -0,6% | -1,4% |
| Enc Time[%] |  | 74% |  |  |  | 72% |  |  |
| Dec Time[%] |  | 101% |  |  |  | 101% |  |  |

Table 1 Coding efficiency of the BO and EO simplification compared to HM6.0 CE1

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,1% | -0,6% | -0,9% | 0,1% | -0,6% | -1,1% |
| Class B | 0,0% | -0,1% | -0,2% | 0,1% | -0,2% | -0,5% |
| Class C | 0,0% | 0,2% | 0,4% | 0,0% | -0,1% | 0,0% |
| Class D | 0,0% | 0,0% | -0,1% | 0,0% | -0,1% | -0,2% |
| Class E | 0,1% | -0,1% | -0,6% | 0,1% | -0,4% | -0,8% |
| Overall | 0,0% | -0,1% | -0,2% | 0,1% | -0,3% | -0,5% |
|  | 0,0% | -0,1% | -0,3% | 0,1% | -0,3% | -0,5% |
| Class F | 0,2% | 0,2% | 0,8% | 0,2% | -0,2% | 0,3% |
| Enc Time[%] | 82% |  |  | 86% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,1% | 0,0% | 0,0% | 0,0% | 0,0% | -0,4% |
| Class B | 0,1% | 0,2% | 0,2% | 0,1% | 0,2% | 0,1% |
| Class C | 0,0% | 0,5% | 0,6% | 0,0% | 0,2% | 0,4% |
| Class D | 0,0% | -0,1% | 0,1% | 0,0% | 0,0% | -0,1% |
| Class E |  |  |  |  |  |  |
| Overall | 0,1% | 0,2% | 0,2% | 0,0% | 0,1% | 0,0% |
|  | 0,1% | 0,2% | 0,2% | 0,0% | 0,1% | 0,0% |
| Class F | 0,5% | 0,7% | 1,6% | 0,3% | 0,4% | 1,5% |
| Enc Time[%] | 109% |  |  | 90% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,0% | -0,1% | -0,6% | 0,0% | -0,7% | -0,9% |
| Class C | 0,0% | 0,0% | 0,4% | -0,1% | -0,1% | -0,4% |
| Class D | -0,1% | 0,1% | 0,2% | -0,1% | -0,5% | -0,6% |
| Class E | 0,0% | -0,8% | -0,9% | 0,0% | -0,5% | -1,4% |
| Overall | 0,0% | -0,2% | -0,2% | -0,1% | -0,5% | -0,8% |
|  | 0,0% | -0,2% | -0,2% | -0,1% | -0,5% | -0,9% |
| Class F | 0,3% | -0,7% | 0,3% | 0,0% | 0,2% | -0,4% |
| Enc Time[%] | 75% |  |  | 82% |  |  |
| Dec Time[%] | 101% |  |  | 98% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,0% | -0,3% | -0,6% | 0,0% | -0,6% | -1,5% |
| Class C | 0,0% | 0,2% | 0,3% | 0,0% | -0,2% | -0,1% |
| Class D | -0,1% | 0,0% | -1,0% | -0,1% | -0,4% | -0,4% |
| Class E | 0,0% | -0,5% | -1,6% | -0,1% | -1,2% | -1,7% |
| Overall | 0,0% | -0,1% | -0,7% | -0,1% | -0,6% | -0,9% |
|  | 0,0% | -0,1% | -0,7% | -0,1% | -0,6% | -0,9% |
| Class F | 0,3% | 0,4% | 0,1% | -0,1% | -0,5% | -0,8% |
| Enc Time[%] | 65% |  |  | 66% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

Table 2 Coding efficiency of the BO simplification compared to HM6.0 CE1

# Conclusions

We have shown how to simplify both band and edge offsets so that about 50% less memory in worst case is required for storing offset values and at the same time showing similar luma BDR and some benefit in chroma BDR and similar subjective quality compared to current HEVC software. Additional benefits of the proposal are that neither band nor edge offset needs clamping operations as a final stage and that edge offsets not abruptly can change edge characteristics.

We propose to include the simplification into HEVC specification.

# References

- “WD6: Working Draft 6 of High-Efficiency Video Coding”, JCTVC-H1003_dK, USA, February 2012

- “Description of Core Experiment 1 (CE1): Sample adaptive offset filtering”, JCTVC-H1101r2, USA, February 2012

# Patent rights declaration(s)

Ericsson AB may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).