| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0075r3 WG11 Number: m24314 |
| --- | --- |

| Title: | AHG7: A restriction of motion vector for small PU size |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | +81-44-549-2152 takeshi.chujoh@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

An experimental result of restriction of motion vector for small PU size is reported.  This is a technology to reduce memory bandwidth for motion compensation that does not change any current syntax, semantics and decoding process.  The worst cases of memory bandwidth of interpolation process are two-dimensional interpolation positions for both luma and chroma of bi-prediction PU.  Therefore, in order to reduce the worst case of memory bandwidth, for example, an encoding method that at least one motion vector of L0 or L1 is restricted to the integer position for both luma and chroma is introduced.  As an experimental result, the loss of coding efficiency is an average of 0.44% and this value is smaller than the result of prohibition of both 4x8 and 8x4 bi-prediction.  Their worst memory bandwidths are almost the same.  This restriction should be applied to levels equal to or higher than 3.

# A restriction of motion vector for small block size

The interpolation filters on current HM are specified 8 or 7-tap for luminance and 4-tap for chrominance.  Considering block based processing, memory access of smaller block is increased.  Therefore, in order to reduce memory bandwidth, the restriction of small prediction block size is a useful method that does not any change syntax, semantics and decoding process.

When color format is 4:2:0, the resolutions of interpolation of HM are quarter for luminance signal and eighth chrominance signal.  In Figure 1, “A” indicates the integer position for luminance.  White “A” is the integer position for both luminance and chrominance.  Light gray blocks are the non-two-dimensional interpolation positions for either luminance or chrominance.  The worst cases of memory bandwidth of interpolation process are two dimensional interpolation positions for both luminance and chrominance of bi-prediction block.  They are dark gray blocks.

Therefore, if we restrict motion vectors in order to reduce memory bandwidth, we should consider the fractional position for chrominance.

Figure 1  Relationship of interpolation position for luminance and chrominance (4:2:0)

In this contribution, we test an example of MV restriction as follows.

- 4x8 bi-prediction is prohibited.

- One motion vector of L0 or L1 shall indicate the integer position for chroma when PU size is 8x4 bi-prediction.

- When merge mode is selected, above two condition is prohibited.

# Experimental results

The simulation was conducted for all sequences of WQVGA, WVGA, 720p, 1080p and cropped 4kx2k based on the recommended simulation common conditions, JCTVC-H1100 [1].  Table 1 indicates the average results of coding efficiency compared to each anchor.  According to JCTVC-H1100, the anchor is HM60.  Used platform is that the OS is Windows 7 64-bit; the CPU is Core™i7-2600, 3.4GHz and the compiler is Visual C++ 2008.

Previous CE3 [2] conducted computational complexity and memory bandwidth analyses for each proposal.  At this time, memory bandwidth for each block size and for each single and bidirectional prediction are measured.  This proposal can reduce worst case of memory bandwidth.

Table 1	Average results of RA, RA10, LB and LB10

|  | Proposal |  |  | 8x4 and 4x8 bi-prediction prohibition |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.17% | 0.21% | 0.21% | 0.25% | 0.34% | 0.24% |
| Class B | 0.30% | 0.07% | 0.20% | 0.32% | 0.15% | 0.33% |
| Class C | 0.59% | 0.62% | 0.56% | 0.63% | 0.67% | 0.62% |
| Class D | 0.82% | 0.64% | 0.60% | 0.87% | 0.65% | 0.68% |
| Class E | 0.34% | -0.05% | 0.12% | 0.43% | 0.28% | 0.46% |
| Class F | 0.28% | 0.26% | 0.51% | 0.32% | 0.26% | 0.65% |
| Overall | 0.44% | 0.33% | 0.41% | 0.49% | 0.40% | 0.52% |
| Enc Time[%] | 95.36% |  |  | 91.69% |  |  |
| Dec Time[%] | 99.36% |  |  | 99.79% |  |  |
| MemBand(2D:4x2) Worst [%] | -35.23% |  |  | -36.36% |  |  |
| MemBand(2D:8x2) Worst [%] | -42.19% |  |  | -43.75% |  |  |
| MemBand(1D:4x1) Worst [%] | -31.25% |  |  | -31.25% |  |  |
| MemBand(1D:8x1) Worst [%] | -50.00% |  |  | -50.00% |  |  |

# Conclusions

The simulation results of restriction of motion vector for small PU size have been shown.  As experimental results the degradation of coding efficiency is small and the worst case of memory bandwidth is improved.  When the worst case of memory bandwidth is reduced, motion vectors should be restricted, considering chroma fractional positions.  Considering to experimental results, if proposed restriction is applied to levels equal to or higher than 3, the coding losses are sufficiently small.

# References

[1] 	F. Bossen, “Common test conditions and software reference configurations,” Joint Collaborative Team on Video Coding, JCTVC-H1100, San Jose, Feb. 2012.

[2] 	T. Chujoh and E. Alshina, “CE3: Motion compensation,” Joint Collaborative Team on Video Coding, JCTVC-F903, Torino, July 2011.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex

Change in A4.2.

g) If the value of 1 for RestBiPUFlag is specified, the following conditions shall be kept.

- When inter_pred_flag is equal to Pred_BI and log2Cb is equal to 3, PartMode, PART_Nx2N shall not be selected.

- When inter_pred_flag is equal to Pred_BI, log2Cb is equal to 3 and PartMode is equal to PART_2NxN, xFracC and yFracC of either one of predSampleLXs shall be equal to 0.

- When merge_flag is equal to 1, merge_idx of above two predictions shall not be selected.

Table A-2 – Main profile and xxx profile level limits

| Level number | SliceRate | RestBiPUFlag | inter_4x4_enabled_flag |
| --- | --- | --- | --- |
| 1 | - | - | 0 |
| 2 | - | - | 0 |
| 3 | 22 | 1 | 0 |
| 3.1 | 60 | 1 | 0 |
| 4 | 60 | 1 | 0 |
| 4.1 | 24 | 1 | 0 |
| 4.2 | 24 | 1 | 0 |
| 4.3 | 24 | 1 | 0 |
| 5 | 24 | 1 | 0 |
| 5.1 | 24 | 1 | 0 |
| 5.2 | 24 | 1 | 0 |
| 6 | 24 | 1 | 0 |
| 6.1 | 24 | 1 | 0 |
| 6.2 | 24 | 1 | 0 |