| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0076r1 |
| --- | --- |

| Title: | AHG15: Signalling Long-term Reference Picture Set |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Byeong Moon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130 Korea | Tel: Email: | - hendry.hendry@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes a method for signalling sets of long-term reference pictures in SPS. It is suggested that the long-term reference picture set (LTRPS) may be useful when the pictures that should be selected as long-term reference picture are predictable. The proposed LTRPS contains parameters that are used to compute POC of long-term reference pictures that shall be available in DPB prior to decoding a slice. It is claimed that since the proposed LTRPS contains parameters to compute the POC of required long-term reference pictures rather than the POCs themselves, the proposed LTRPS does not required to be frequently updated, thus, it is friendly to system that signal all necessary parameters for decoding only once in the beginning. The bit-count comparison for signalling long-term reference pictures for the scenarios described in the common conditions for reference picture marking and list construction proposals (JCTVC-H0725) reports that the proposed method reduces the number of required bits by about 84% to 87%. It is suggested that the intent of the proposed LTRPS is not for replacing the current mechanism of signalling long-term reference pictures in slice header, rather, it is proposed to complement it.

# Introduction

As described in [1], the method of signalling long-term reference pictures (LTRPs) is as follow:

- There is a flag in SPS, called “long_term_ref_pics_present_flag”, controlling whether or not long-term reference picture is used in the HEVC bitstream.

- If long_term_ref_pics_present_flag is set then, the list of LTRPs that must exist in decoded picture buffer (DPB) shall present in slice header.

- Only LSB difference between POC of the LTRP and POC of current slice is signaled.

- To avoid confusion when there might be two same delta LSB, MSB cycle information may also be signaled.

While the signalling method works, it may still be improved considering that there are cases where:

- Pictures that are selected as LTRP can be predicted

- Occurrence of LTRP has certain cycle or period.

In such cases, rather than simply signalling delta POC LSB of the LTRP in slice header, it might be better to define sets of LTRP in SPS and then let each slice header tells which set of LTRP it needed. Such concept / method is actually is similar to the concept of reference picture set (RPS) that is used for signalling short-term reference pictures.

In the remaining of this document, method of signalling long-term reference picture set (LTRPS) is described. We propose that LTRPS is considered to be an additional LTRP signalling method to the current signalling method.

# Long-term Reference Picture Set

## Concept

The following figure illustrates the concept of LTRPS. LTRPS contains a set of parameterized LTRP that are signalled in SPS. Slice header can choose whether to refer to LTRPS or use current signalling method.

Each LTRP in LTRPS is parameterized with the following:

- Cycle information

- Offset information.

Those two parameters is used to compute POC of LTRP as follow:

POC of LTRP = ((current POC / cycle) * cycle) + offset

Figure 1 – Concept of LTRPS (slice header are read from left to right, top to bottom)

Use case scenarios 2.6 and 3.3 described in [2] may serve as a good example of the application of LTRPS. For the sake of simplicity to describe the concept of LTRPS, the use case 2.6 is used as example below.

As illustrated in the below figure, the entire scene of use case 2.6 can simply be considered as of 2 scenes, that are, 20 seconds for shooting interviewer and 60 seconds for shooting interviewee, and the rest is simply just repetition of those two scenes. For such condition, if target frame rate is 60 frames-per-second, the selection of LTRP is as follow:

- Pictures from POC 0 – 1200 shall not use any LTRP

- Pictures from POC 1201 – 4792 shall keep reference picture POC 1192 as LTRP

- Pictures from POC 4793 – 4799 shall not use any LTRP

- Picture with POC 4800 shall use reference picture POC 1192 as LTRP

- Pictures from POC 4801 – 5992 shall keep reference picture POC 4792 as LTRP

- So forth…

Figure 2 – Illustration of two scenes in use case 2.6

Signalling LTRP for the above condition can be done as follow:

- In SPS, define 2 LTRPS

  - LTRPS index 0 contains 1 LTRP

    - Cycle 4800

    - Offset 1192

  - LTRPS index 1 contains 1 LTRP

    - Cycle 4800

    - Offset -8

- In slice header

  - For POC 0 – 1200

    - Not use LTRPS

    - num_long_term_pics = 0

  - For POC 1201 – 4792

    - Use LTRPS index 0

  - For POC 4793 – 4799

    - Not use LTRPS

    - num_long_term_pics = 0

  - For POC 4800

    - Not use LTRPS

    - Code LTRP 1192 by using current signaling method (i.e., signal delta POC LSB)

  - For POC 4801 – 5992

    - Use LTRPS index 1

…

Note that similar approach can be applied when for different scene length (i.e. 10s, 30s, 40s, etc) and also to solve signaling LTRP for use case scenario 3.3.

## Proposed LTRPS Syntax and Semantics

The modification to SPS, slice header and decoding process to accommodate the proposed LTRPS is as follow. Note that only semantics of newly added element is described while semantics of related but existing elements can be referred from [1].

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| long_term_ref_pics_present_flag | u(1) |
| if (long_term_ref_pics_present_flag == 1) |  |
| { |  |
| num_long_term_ref_pic_sets | ue(v) |
| if (num_long_term_ref_pic_sets > 0) |  |
| { |  |
| log2_max_lt_cycle_minus4 | ue(v) |
| for (i = 0; i < num_long_term_ref_pic_sets; i++) |  |
| { |  |
| num_lt_ref_pics_minus1 [ i ] | ue(v) |
| for (j = 0; j <= num_lt_ref_pics_minus1 [i]; j++) |  |
| { |  |
| lt_cycle_base_minus1 [ i ][ j ] | u(n) |
| lt_offset_sign [ i ][ j ] | u(1) |
| lt_offset [ i ][ j ] | u(n) |
| used_by_curr_pic_lt_flag [ i ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

log2_max_lt_cycle_minus4 plus 4 specifies the bits required to represent the syntax elements lt_cycle_base_minus1 [ i ][ j ] and lt_offset [ i ][ j ].

lt_cycle_base_minus1 [ i ][ j ] plus 1 specifies the cycle base to calculate POC of the j-th long-term reference picture in the i-th long-term reference picture set. The lt_cycle_base_minus1 [ i ][ j ] shall be represented by log2_max_lt_cycle_minus4 + 4 bits.

lt_offset_sign [ i ][ j ] equals 0 indicates that the offset to calculate POC of the j-th long-term reference picture in the i-th long-term reference picture set is a negative offset. lt_offset_sign [ i ][ j ] equals 1 indicates otherwise.

lt_offset [ i ][ j ] specifies the offset to calculate POC of the j-th long-term reference picture in the i-th long-term reference picture set. The lt_offset [ i ][ j ] shall be represented by log2_max_lt_cycle_minus4 + 4 bits.

The variable LtOffset [ i ][ j ] is defined as follow:

LtOffset[ i ][ j ] = (lt_offset_sign [ i ][ j ] == 0) ? –lt_offset[ i ][ j ] : lt_offset[ i ][ j ]

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( long_term_ref_pics_present_flag ) { |  |
| long_term_ref_pic_set_sps_flag | u(1) |
| if (long_term_ref_pic_set_sps_flag) |  |
| { |  |
| long_term_ref_pic_set_idx | u(v) |
| } |  |
| else |  |
| { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| delta_poc_lsb_lt[ i ] | ue(v) |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt_minus1[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

long_term_ref_pic_set_sps_flag equal to 1 specifies that the long-term reference picture set of the current picture shall be created using syntax elements in the active sequence parameter set. long_term_ref_pic_set_sps_flag equal to 0 specifies that the long-term reference picture set of the current picture shall be created using syntax elements presents in the slice header.

long_term_ref_pic_set_idx equal to 1 specifies the index to the list of the long-term reference picture sets specified in the active sequence parameter set that shall be used for creation of the reference picture set of the current picture. The syntax element long_term_ref_pic_set_idx shall be represented by Ceil( Log2( num_long_term_ref_pic_sets ) ) bits. The value of long_term_ref_pic_set_idx shall be in the range of 0 to num_long_term_ref_pic_sets − 1, inclusive, where num_long_term_ref_pic_sets is the syntax element from the active sequence parameter set.

The variable LtRpsIdx is derived as follows.

if( long_term_ref_pic_set_sps_flag )
LtRpsIdx = long_term_ref_pic_set_idx

else
LtRpsIdx = num_long_term_ref_pic_sets

Decoding Process to Derive POC of LTRP for Each Slice

The following process shall be applied in order to derive the POC of LTRP sets in for each slice.

if (long_term_ref_pic_set_sps_flag)

{

for( i = 0, j = 0, k = 0; i < num_lt_ref_pics[ LtRpsIdx ]; i++ )

if (used_by_curr_pic_lt_flag [LtRpsIdx ][ i ])

PocLtCurr[ j++ ] = ( ( PicOrderCntVal / ( lt_cycle_base_minus1 [LtRpsIdx ][ i ] + 1 ) ) * ( lt_cycle_base_minus1 [LtRpsIdx ][ i ] + 1) ) + LtOffset [LtRpsIdx ][ i ]

else

PocLtFoll[ k++ ] = ( ( PicOrderCntVal / ( lt_cycle_base_minus1 [LtRpsIdx ][ i ] + 1 ) ) * ( lt_cycle_base_minus1 [LtRpsIdx ][ i ] + 1 ) ) ) + LtOffset [LtRpsIdx ][ i ]

NumPocLtCurr = j

NumPocLtFoll = k

}

else

{

for( i = 0, j = 0, k = 0; i < num_long_term_pics; i++ )
if( delta_poc_msb_present_flag[ i ] )
  if( used_by_curr_pic_lt_flag[ i ] )
    PocLtCurr[ j++ ] = ( ( PicOrderCntVal − DeltaPocLt[ i ] + MaxPicOrderCntLsb ) % 
	MaxPicOrderCntLsb ) – ( DeltaPocMSBCycleLt[ i ] )* MaxPicOrderCntLsb
  else
    PocLtFoll[ k++ ] = ( ( PicOrderCntVal − DeltaPocLt[ i ] + MaxPicOrderCntLsb ) %
	MaxPicOrderCntLsb ) – ( DeltaPocMSBCycleLt[ i ] ) * MaxPicOrderCntLsb
else
  if( used_by_curr_pic_lt_flag[ i ] )
    PocLtCurr[ j++ ] = ( PicOrderCntVal − DeltaPocLt[ i ] + MaxPicOrderCntLsb ) % 
	MaxPicOrderCntLsb
  else
    PocLtFoll[ k++ ] = ( PicOrderCntVal − DeltaPocLt[ i ] + MaxPicOrderCntLsb ) %
	MaxPicOrderCntLsb

NumPocLtCurr = j
NumPocLtFoll = k

}

# Analysis and Discussion

## Bit-count Analysis

The following tables tabulate the bit-count comparison of signalling LTRP with current signalling method and proposed signalling method for use case scenarios 2.6 and 3.3. In average, for all those test cases, the propose signalling method reduces the amount of required bits by about 84% to 87%.

Detail bit-count analysis for each case is given in the attached spreadsheet file.

| Case 2.6 (10s -- 30s -- 10s -- 30s -- 4800 frames) |  |  |
| --- | --- | --- |
| Field | Required bits |  |
|  | Current method | Proposed method |
| SPS | 1 | 126 |
| PPS | 0 | 0 |
| Slice Header (1st half) | 38673 | 6595 |
| Slice Header (2nd half) | 49270 | 7191 |
| Total in Slice Header | 87,943 | 13,786 |
| Total Bit-count | 87,944 | 13,912 |
| Reduction (%) | 84.18% |  |
| Case 2.6 (20s -- 60s -- 20s -- 60s -- 9600 frames) |  |  |
| Field | Required bits |  |
|  | Current method | Proposed method |
| SPS | 1 | 136 |
| PPS | 0 | 0 |
| Slice Header (1st half) | 83875 | 13195 |
| Slice Header (2nd half) | 106998 | 14391 |
| Total in Slice Header | 190,873 | 27,586 |
| Total Bit-count | 190,874 | 27,722 |
| Reduction (%) | 85.48% |  |
| Case 3.3 (200ms RTT -- 600 frames) |  |  |
| Field | Required bits |  |
|  | Current method | Proposed method |
| SPS | 1 | 92 |
| PPS | 0 | 0 |
| Slice Header (1st half) | 5872 | 902 |
| Slice Header (2nd half) | 6024 | 900 |
| Total in Slice Header | 11,896 | 1,802 |
| Total Bit-count | 11,897 | 1,894 |
| Reduction (%) | 84.08% |  |
| Case 3.3 (1000ms RTT -- 600 frames) |  |  |
| Field | Required bits |  |
|  | Current method | Proposed method |
| SPS | 1 | 132 |
| PPS | 0 | 0 |
| Slice Header (1st half) | 7376 | 902 |
| Slice Header (2nd half) | 8220 | 900 |
| Total in Slice Header | 15,596 | 1,802 |
| Total Bit-count | 15,597 | 1,934 |
| Reduction (%) | 87.60% |  |

  - Applicability

The attractive points of the proposed method are as follow:

- LTRPS does not contain POC of LTRPs but contains parameters that can be used to compute POC of the required LTRPs. Thus, it has no problem with system that only sends its coding parameters (SPS and PPS) once in the beginning and never updates them.

- The proposed method works well together with current signalling method.

For conditions where the occurrence of LTRP is less predictable, several more LTRPS can be generated in SPS in anticipation. For example, it might be possible that a variation of the above use case scenario happens, that is, instead of fix 20s and 60s, the interval of when camera will switch is not known in advance. For such condition instead of defining only two set of LTRPS, several more sets of LTPRS can be defined for intervals such as: 10s, 20s, 30s, 40s, 50s, 60s, etc.

Having said the above, the proponent believes that the proposed LTRPS is applicable to various applications such as random access, low delay, teleconferencing, etc.

# Conclusion

A method for signalling sets of long-term reference pictures is proposed. It is suggested that the long-term reference picture set (LTRPS) may be useful when the pictures that should be selected as long-term reference picture are predictable. The intent of the proposed LTRPS is not for replacing the current mechanism of signalling long-term reference pictures in slice header, rather, it is proposed to complement it. We recommend the group to consider adopting the proposed signaling of long-term reference picture sets.

# References

[1] JCTVC-H1003, “High efficiency video coding (HEVC) text specification draft 6,” 8th JCT-VC Meeting, San Jose, US.

[2] JCTVC-H0725, “Common conditions for reference picture marking and list construction proposals,” 8th JCT-VC Meeting, San Jose, US.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).