| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0078 |
| --- | --- |

| Title: | AHG4: Single-core decoder friendly WPP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Byeong Moon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130 Korea | Tel: Email: | - hendry.hendry@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

It is assessed that the current ordering of coding tree in the bitstream when WPP is used might not be friendly for single-core decoder since it has to jump forth and back within the bitstream to the correct location for parsing. One way to avoid this problem is to force the number of WPP substream to be maximum, that is, one LCU line is one WPP substream, so that the order of coding tree is in the normal picture raster scan order. However, such hard constraint to always force using maximum number of substream might not be always desired as it is further assessed that the current coding tree order is useful if the bitstream is really intended for multi-core decoder.

This contribution proposes to add a flag either in SPS or PPS to indicate whether or not coding tree is reordered when WPP is used. It is suggested by proponent that the flag gives flexibility to encoder to determine to which side the coded bitstream will be friendlier to, that is, if the flag is set, then the coded bitstream is friendlier to multi-core decoder, else then the coded bitstream is friendlier to single-core decoder.

# Problem Statement

According to current HEVC specification [1], when wave front parallel processing (WPP) is used, the order of LCU within bitstream is reordered in which LCUs that belong to the same substream are sent sequentially as illustrated in Figure 1. With such arrangement, if the bitstream is decoded by decoder with multi-cores, parsing and decoding process will be easy and straightforward. On contrary, a single-core decoder will have to jump forth and back in order to parse and decode the coded bitstream correctly.

- -- Decoding order of coding tree block when WPP substreams are decoded with multi-cores decoder

- -- Decoding order of coding tree block when WPP substreams are decoded with single-core decoder

Figure 1 – Coding of bitstream coded with WPP (multiple-cores vs. single-core)

One solution to make WPP friendlier to single-core decoder is by mandating that if WPP is used, the number of substream must be maximum, meaning that the number of substream is the same as the number of LCU rows. In this way, the order of LCU in the bitstream will be naturally in picture raster scan order and a single-core decoder does not have to jump forth and back within the bitstream to go to appropriate bitstream location for parsing and decoding.

However, mandating maximum number of substream might cause WPP not friendly for multi-cores decoder as it has to jump to correct bitstream location to find the next LCU row to be parsed and decoded as illustrated in Figure 2.

- – Max number of substream parsed and decoded with single-core decoder

- – max number of substream parsed and decoded with multi-cores decoder (#substream > #cores)

Figure 2 – Coding of bitstream coded with WPP (multiple-cores vs. single-core)

Given the above assessment that the arrangement of LCU can only be friendly for either single-core or multi-cores decoder, we do not recommend to the constraint that mandates the number of WPP substreams maximum. We recommend letting encoder to decide whether or not LCU in the bitstream is reordered and signal a flag to notify decoder.

# Proposed Flag

To determine whether or not LCU within slice / picture is reordered when WPP is used, we propose to add a flag in either SPS / PPS called ‘ctb_reordering_flag’ with the following semantic:

- If the flag is false then syntax element num_substreams_minus1 is not present in PPS. The number of substream is each slice inferred to be the same as num_entry_point_offsets.

- If the flag is true then syntax element num_substreams_minus1 is present in PPS

The syntax and semantic of the flag is as the following

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| … |  |
| } else if( tiles_or_entropy_coding_sync_idc = = 2 ) |  |
| ctb_reordering_flag | u(1) |
| if (ctb_reordering_flag) |  |
| num_substreams_minus1 | ue(v) |
| … |  |
| } |  |

ctb_reordering_flag equals 1 indicates that syntax element num_substreams_minus1 is present and the coded treeblock in the bitstream might not be in picture raster scan order. Ctb_reordering_flag equals 0 indicates that syntax element num_substreams_minus1 is not present and coded treeblocks is in picture raster scan order. When ctb_reordering_flag equals 0, the number of substreams in each slice is inferred to be the same as the value of num_entry_point_offsets.

The following changes (highlighted) is proposed to the semantic of num_entry_point_offsets in slice header.

num_entry_point_offsets specifies the number of entry_point_offset[ i ] syntax elements in the slice header. When tiles_or_entropy_coding_sync_idc is equal to 1, the value of num_entry_point_offsets shall be in the range of 0 to ( num_tile_columns_minus1 + 1 ) * ( num_tile_rows_minus1 + 1 ) − 1, inclusive. When tiles_or_entropy_coding_sync_idc is equal to 2 and ctb_reordering_flag is equal 1, the value of num_entry_point_offsets shall be in the range of 0 to num_substreams_minus1, inclusive. When tiles_or_entropy_coding_sync_idc is equal to 2 and ctb_reordering_flag is equal 0, the value of num_entry_point_offsets shall be in the range of 0 to PicHeightInCtbs - 1, inclusive. When not present, the value of num_entry_point_offsets is inferred to be equal to 0.

It is worthy to note that picture coded with one or more WPP substreams can be decoded by decoder with single-core or more multiple-cores. However, in term of easiness of implementation, there can be difference such as:

- When ctb_reordering_flag is false then the bitstream can be considered single-core decoding friendly, on the other hand,

- When ctb_reordering_flag is true then the bitstream can be considered more multi-cores decoding friendly.

# Conclusion

This contribution proposes to add a flag either in SPS or PPS to indicate whether or not coding tree is reordered when WPP is used. The proposed flag can give flexibility to encoder to determine to which side the coded bitstream will be friendlier to, that is, if the flag is set, then the coded bitstream is friendlier to multi-core decoder, else then the coded bitstream is friendlier to single-core decoder. We recommend the JCTVC group to consider adopting the proposed flag.

# References

[1] JCTVC-H1003, “High efficiency video coding (HEVC) text specification draft 6,” 8th JCT-VC Meeting, San Jose, US.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).