| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0079 |
| --- | --- |

| Title: | AHG4: Simplified CABAC Initialization for WPP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Byeong Moon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130 Korea | Tel: Email: | - hendry.hendry@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

Currently when WPP is used, CABAC probability table of the first LCU, starting from the second LCU rows, is initialized from that of the 2nd LCU of the previous LCU row. It is assessed that this initialization mechanism requires a buffer for storing the states of CABAC probability table before it is used. This contribution reports a study on possibility to reset CABAC probability table of the 1st LCU of every LCU row when WPP is used in order to avoid the need to provide buffer for storing the states of CABAC probability table. It is reported that resetting CABAC probability table at every first LCU causes luma loss at average 0.1% for AI-MAIN, 0.1%Y for AI-HE10, 0.2% for RA-MAIN, 0.2% for RA-HE10, 0.7% for LB-MAIN, and 0.7% for LB-HE10.

It is suggested by the proponent that the idea proposed in this contribution can be combined with the idea proposed in I0078 – AHG4: Single-core decoder friendly WPP, that is, reset CABAC probability table of the 1st LCU of every LCU row when the proposed ctb_reordering_flag is not set so that the coded bitstream is even friendlier for parsing and decoding with single-core decoder. Further, the proponent would support the inclusion of this version of WPP (i.e., reset CABAC probability table of the 1st LCU of every LCU row and mandate that ctb_reordering_flag is not set) to the main profile of HEVC.

# Problem Statement

As per Committee Draft of HEVC specification, when Wavefront Parallel Processing (WPP) is used, CABAC probability table of the first LCU of every LCU row, except for the first LCU of the first LCU row, shall be initialized from the last state of the CABAC probability table after the 2nd LCU of the previous row has been processed. Below figure illustrates the initialization of CABAC probability table when WPP is used. As depicted, CABAC probability table of the first LCU of the first LCU row is initialized from default; however, CABAC probability table of the first LCU of the remaining LCU row is initialized from the CABAC probability table after the 2nd LCU of the previous row has been processed, that is:

- CABAC probability table of LCU #5 is initialized from the CABAC probability table after LCU #1 has been processed.

- CABAC probability table of LCU #10 is initialized from the CABAC probability table after LCU #6 has been processed.

- The similar process is continued as many as LCU rows available in the slice / picture

Figure 1 – CABAC probability table update when WPP is used

There are two major issues with the above scheme of initializing CABAC probability table, they are:

- There is requirement at both encoder and decoder to provide memory buffer to keep CABAC probability table

- Complicated process for storing and loading CABAC probability table.

While the above two issues is valid for systems (ie., encoders and decoders) with and without parallel processing capability, it is worse for systems without parallel processing (i.e., systems with single core processing). The following figure illustrates the above two issues for system with single core processing. As depicted, the system (i.e., encoder or decoder) has allocated specific memory as CABAC buffer and the system shall do the following:

- After processing LCU #1, store the state of CABAC probability table into the CABAC buffer

- Before processing LCU #5, load CABAC probability table from the CABAC buffer

- After processing LCU #6, store the state of CABAC probability table into the CABAC buffer

- Before processing LCU #10, load CABAC probability table from the CABAC buffer

- The similar process is continued as many as LCU rows available in the slice / picture

Figure 2 – The need of CABAC buffer and complicated process for CABAC initialization when WPP is processed with single core system

# Resetting CABAC probability table every LCU row

## Concept

To avoid the abovementioned two issues, we have studied the possibility to reset CABAC probability table of the 1st LCU of every LCU row when WPP is used as illustrated in Figure 3. As depicted, CABAC probability table of LCU #0, #5, and #10 is initialized from default initialization value. If there are more LCU rows exist in the slice / picture, the first LCU in the LCU row shall be treated as suggested above (i.e., its CABAC probability table value shall be initialized from default values).

(a)

(b)

Figure 3 –CABAC initialization is reset every LCU row

The benefits of the idea compared to the current method for CABAC initialization when WPP is used are as follow:

- There is no need for memory allocation for CABAC buffer

- There is no need for complicated storing and loading CABAC buffer.

- There is no need for scheduling storing and loading values of CABAC probability table into and from CABAC buffer.

## Impact to coding performance

The performance loss because of resetting CABAC probability table of the 1st LCU of every LCU row on average is 0.1% for AI-MAIN, 0.1%Y for AI-HE10, 0.2% for RA-MAIN, 0.2% for RA-HE10, 0.7% for LB-MAIN, and 0.7% for LB-HE10. The following tables tabulate the detail experiment results.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.2% | -0.2% | -0.3% | 0.1% | -0.3% | -0.5% |
| Class B | 0.2% | -0.1% | -0.2% | 0.2% | 0.0% | -0.4% |
| Class C | 0.1% | 0.0% | 0.1% | 0.1% | 0.2% | 0.1% |
| Class D | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class E | 0.3% | -0.9% | -1.1% | 0.2% | -0.8% | -1.1% |
| Overall | 0.1% | -0.2% | -0.3% | 0.1% | -0.1% | -0.3% |
|  | 0.1% | -0.2% | -0.2% | 0.1% | -0.1% | -0.3% |
| Class F | 0.2% | 0.0% | 0.1% | 0.1% | -0.6% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | 0.1% | 0.0% | 0.1% | -0.2% | -0.1% |
| Class B | 0.2% | 0.2% | 0.0% | 0.2% | 0.3% | 0.0% |
| Class C | 0.2% | 0.3% | 0.3% | 0.2% | 0.3% | 0.4% |
| Class D | 0.3% | 0.2% | 0.0% | 0.2% | 0.4% | 0.4% |
| Class E |  |  |  |  |  |  |
| Overall | 0.2% | 0.2% | 0.1% | 0.2% | 0.3% | 0.2% |
|  | 0.2% | 0.2% | 0.1% | 0.2% | 0.3% | 0.2% |
| Class F | 0.3% | 0.1% | 0.2% | 0.2% | 0.3% | 0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.5% | 0.4% | 0.4% | 0.5% | 1.1% | 0.8% |
| Class C | 0.3% | 0.5% | 0.4% | 0.3% | 0.5% | 0.6% |
| Class D | 0.6% | 0.2% | -0.2% | 0.5% | 0.6% | 0.6% |
| Class E | 1.8% | 1.2% | 1.4% | 1.6% | 0.2% | -0.6% |
| Overall | 0.7% | 0.5% | 0.4% | 0.7% | 0.7% | 0.4% |
|  | 0.7% | 0.7% | 0.5% | 0.7% | 0.7% | 0.4% |
| Class F | 0.8% | 1.3% | 1.0% | 0.9% | 0.1% | 1.4% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

# Recommendation

By resetting CABAC probability table state of the 1st LCU of every LCU row, decoder implementation can avoid the requirement to provide buffer for storing the CABAC state. We consider this benefit is very significant. However, the impact of the action to coding performance is also relatively significant, particularly to low delay configuration. Thus, we recommend that the benefit versus impact of the above idea to be further discussed during the meeting.

Further, the idea above can be combined with the idea proposed in [2] that proposes to add a flag to determine whether or not LCU are reordered. If the flag is not set, then the number of WPP substream is maximum which makes it friendly for processing with single-core decoder. The two ideas can be combined to produce even friendlier version of WPP for single-core decoder such that the state of CABAC probability table of the 1st LCU of every LCU row is reset if ‘ctb_reordering_flag’ is not set.

In our opinion, the combination would make WPP very lightweight so that it can be easily implemented. Therefore, we would support the inclusion of this version of WPP in the main profile of HEVC.

# Conclusion

This contribution reports the result of resetting the state of CABAC probability table of the 1st LCU of every LCU row to avoid the requirement to provide additional buffer for storing states of CABAC probability table. While the benefit of the idea is clear, the impact of the idea to coding performance is relatively significant particularly for low delay configuration. Thus, we recommend that the benefit versus impact of the above idea to be further discussed during the meeting. Further, the idea of resetting CABAC probability table every LCU row can seem be combined with idea proposed in [2]. Therefore, we also recommend that the idea of resetting CABAC probability table of the 1st LCU of every LCU row is applied when ‘ctb_reordering_flag’ is not set.

# References

[1] JCTVC-H1003, “High efficiency video coding (HEVC) text specification draft 6,” 8th JCT-VC Meeting, San Jose, US.

[2] JCTVC-I0078, “AHG4: Single-core decoder friendly WPP,” 9th JCT-VC Meeting, Geneva, CH.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).