| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0081 |
| --- | --- |

| Title: | APS partial update – APS update with one or multiple references |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Byeong Moon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130 Korea | Tel: Email: | - hendry.hendry@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes two schemes to allow APS partial updates, that is, APS partial updates with one APS reference and APS partial updates with multiple APS references. The APS partial updates with one APS reference is similar to the scheme proposed in H0069 with the different that the propose scheme add flags to indicate whether or not the information that is not present in the current APS exist in reference APS. This is claimed important to avoid worst case where decoder has to search till the end of linked-list of APS only to find that particular information does not exist. The APS partial updates with multiple APS references is similar to the schemes proposed in H0381 with improvements.

It is further assessed that partial updates with one reference might be simpler for implementation but may have problem with long linked-list of APS. On the other hand, partial updates with multiple references may be a bit more complicated but will have shorter link-list of APS, making it more preferred by the proponent.

# Introduction

APS partial updates seems to be a desired feature to be added into HEVC specification, however, until the last meeting in San Jose, U.S, there was still no agreed APS partial update scheme adopted.

In this document, we propose two schemes for APS partial update mechanism. The two proposed APS partial update schemes share the following concept:

- An APS may carry all information (i.e., scaling list data, deblocking data, sample adaptive offset data, and adaptive loop filter data) itself.

- If one or more information (i.e., scaling list data, deblocking data, sample adaptive offset data, and adaptive loop filter data) is not present in an APS, that information can be obtained / referred from the same type of information carried in previously sent / received APS.

- Based on the above point 1 and 2, the following definition is applied:

  - Intra APS: An APS that does not need reference(s) to other APS. If one or more information is not present in the APS, it can automatically be inferred as not exists.

  - Inter APS: An APS that need reference(s) to one or more APS. If one or more information is not present in the APS, it may still be referred from other APS.

The different between the two proposed schemes are as follow:

Scheme 1:

- If the APS is an inter APS, then it can have at maximum one reference APS only

- Benefit of having only one reference APS only:

  - Simple design.

Scheme 2:

- If the APS is an inter APS, then it can have one or more reference APS

  - Each type of information in APS (i.e., scaling list data, deblocking data, sample adaptive offset data, and adaptive loop filter data) can have its own reference

- Benefit of having one or more reference APS:

  - Flexible design

  - Less / shorter chain of APS update chain.

# APS Partial Update Schemes

## Scheme with one APS reference

In this method, the idea of APS partial update is as follow:

- APS can refer to other APS.

- If APS refers to other APS, it can refer to at maximum one reference APS.

The following figure illustrates the concept of the proposed APS partial update method. Information that is illustrated with bold, italic font is not present within the APS but can be obtained from reference APS.

Figure 1 – APS partial update with one APS reference

The proposed scheme is similar to previously proposed H0069, however, in this proposal, we add some flags to avoid decoder searching till the end linked-list of for information that is not existed. Considered the situation illustrated in Figure 2. The figure illustrates the linked-list of APS starting from APS ID 3 then APS ID 2, then APS ID 1 and finally closed by APS ID 0 (i.e., intra APS). In each APS ID 3 to 1, deblocking data is not present so that by scheme proposed in H0069 it is inferred that it might be available in the reference APS. However, until the end of the linked-link, which is APS ID 0, deblocking data is not present (i.e., illustrated by crossed text of Deblocking in APS ID 0).

Figure 2 – Worst case condition when a particular data (e.g., deblock data) is not present and it has to be traced back to the end of link list of APS.

The syntax and semantics of APS table to accommodate the proposed scheme 1 is as the following.

| aps_rbsp( ) { | Descriptor |
| --- | --- |
| aps_id | ue(v) |
| aps_intra_flag | u(1) |
| if (!aps_intra_flag) |  |
| aps_ref_id | ue(v) |
| aps_scaling_list_data_present_flag | u(1) |
| if( aps_scaling_list_data_present_flag ) |  |
| scaling_list_param( ) |  |
| else |  |
| aps_scaling_list_data_from_ref_flag | u(1) |
| aps_deblocking_data_present_flag | u(1) |
| if(aps_deblocking_data_present_flag) |  |
| { |  |
| disable_deblocking_filter_flag | u(1) |
| if( !disable_deblocking_filter_flag ) |  |
| { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| else |  |
| { |  |
| aps_deblocking_data_from_ref_flag | u(1) |
| } |  |
| aps_sample_adaptive_offset_data_present_flag | u(1) |
| if( aps_sample_adaptive_offset_data_present_flag ) |  |
| { |  |
| aps_sample_adaptive_offset_flag | u(1) |
| if( aps_sample_adaptive_offset_flag ) |  |
| aps_sao_param( ) |  |
| } |  |
| else |  |
| { |  |
| aps_sampe_adaptive_offset_data_from_ref_flag | u(1) |
| } |  |
| aps_adaptive_loop_filter_data_present_flag | u(1) |
| if( aps_adaptive_loop_filter_data_present_flag ) |  |
| alf_param( ) |  |
| else |  |
| aps_adaptive_loop_filter_data_from_ref_flag | u(1) |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

aps_id identifies the adaptation parameter set that is referred to in the slice header.

aps_intra_flag equals 1 indicates this adaptation parameter set does not refer to any other adaptation parameter set. aps_intra_flag equals 0 indicates otherwise.

aps_ref_id specifies the aps_id of the reference adaptation parameter set that is refer to by this adaptation parameter set.

aps_scaling_list_data_present_flag equal to 1 specifies that scaling list parameters exist in this adaptation parameter set, equal to 0 specifies that scaling list parameters do not exist in this adaptation parameter set.

aps_scaling_list_data_from_ref_flag equal to 1 specifies that scaling list parameters may exist in the  adaptation parameter set referred to by current adaptation parameter set, equal to 0 specifies that scaling list parameters do not exist in the adaptation parameter set referred to by current adaptation parameter set.

aps_deblocking_data_present_flag equal to 1 specifies that deblocking parameters are present in the adaptation parameter set, equal to 0 specifies that deblocking parameters do not exist in this adaptation parameter set.

aps_deblocking_data_from_ref_flag equal to 1 specifies that deblocking parameters may exist in the  adaptation parameter set referred to by current adaptation parameter set, equal to 0 specifies that deblocking parameters do not exist in the adaptation parameter set referred to by current adaptation parameter set.

aps_sample_adaptive_offset_data_present _flag equal to 1 specifies that the sample adaptive offset parameters are present in this adaptation parameter set, equal to 0 specifies that sample adaptive offset parameters are not present in this adaptation parameter.

aps_sample_adaptive_offset_data_from_ref_flag equal to 1 specifies that sample adaptive offset parameters may exist in the  adaptation parameter set referred to by current adaptation parameter set, equal to 0 specifies that sample adaptive offset parameters do not exist in the adaptation parameter set referred to by current adaptation parameter set. If sample adaptive offset parameters are not present in current adaptation parameter set or in reference adaptation parameter set, sample adaptive offset parameters are interleaved in slice data.

aps_sample_adaptive_offset_flag equal to 1 specifies that the sample adaptive offset is on for slices referring to the current adaptation parameter set; equal to 0 specifies that the sample adaptive offset is off for slices referring to the current APS.  When there is no active adaptation parameter set, the aps_sample_adaptive_offset_flag value is inferred to be 0.

aps_adaptive_loop_filter_data_present_flag equal to 1 specifies that the adaptive loop filter is on for slices referring to the current adaptation parameter set; equal to 0 specifies that the adaptive loop filter may be off for slices referring to the current adaptation parameter set. When there is no active adaptation parameter set, the aps_adaptive_loop_filter_flag value is inferred to be 0.

aps_adaptive_loop_filter_data_from_ref_flag equal to 1 specifies that the activation of adaptive loop filter for slices referring to the current adaptation parameter set depends on the aps_adaptive_loop_filter_data_present_flag in the reference adaptation parameter set, equal to 0 specifies that the adaptive loop filter is off for slices referring to the current adaptation parameter set.

aps_extension_flag equal to 0 specifies that no aps_extension_data_flag syntax elements are present in the picture parameter set RBSP syntax structure. aps_extension_flag shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value of 1 for aps_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for aps_extension_flag in a picture parameter set NAL unit.

aps_extension_data_flag may have any value. Its value does not affect decoder conformance to profiles specified in this Recommendation | International Standard.

## Scheme with multiple APS references

In this method, the idea of APS partial update is as follow:

- APS can refer to other APS.

- If APS refers to other APS, it can refer to one or more reference APS.

- It should be restricted / constrained that the reference APS for any parameters shall not in turn refer to other APS for the same type of parameter. This constraint helps the scheme to have shorter length of APS linked-list.

The following figure illustrates the concept of the proposed APS partial update method. Information that is illustrated with bold, italic font is not present within the APS but can be obtained from reference APS whose ID is indicated in the bracket. For example, deblocking parameter is not present in APS ID 1 but can be find the in APS ID 0.

Figure 3 – APS partial update with multiple APS references

The following figure illustrates the problem of referring to other APS that in turns needs other for reference. In the example below, scaling in APS ID 3 refers to the scaling in APS ID 2. However, scaling parameter is not present in APS ID 2, instead, it refer to APS ID 1. In such situation, actually, scaling in APS ID 3 must refer to APS ID 1 instead of APS ID 2.

Figure 4 – Illusration of the proposed constraint

To accommodate APS partial update with multiple references, the following changes modifications are made:

- Add new syntax element called ‘aps_intra_flag’ that indicates whether or not the APS needs other APS as reference

- For each types of parameters (i.e., scaling list, deblocking, sample adaptive offset, adaptive loop filer), add the following syntax elements:

  - xxx_data_from_ref_flag, where xxx is replaced by one of the following: aps_scaling_list, aps_deblocking, aps_sample_adaptive_offset, aps_adaptive_loop_filter.

  - aps_ref_id_xxx, where xxx is replaced by one of the following: scaling, deblocking, sample_adaptive_offset, adaptive_loop_filter

The following table tabulates the proposed syntax element of APS.

| aps_rbsp( ) { | Descriptor |
| --- | --- |
| aps_id | ue(v) |
| aps_intra_flag | u(1) |
| aps_scaling_list_data_present_flag | u(1) |
| if( aps_scaling_list_data_present_flag ) |  |
| scaling_list_param( ) |  |
| else if (!aps_intra_flag) |  |
| { |  |
| aps_scaling_list_data_from_ref_flag | u(1) |
| if ( aps_scaling_list_data_from_ref_flag ) |  |
| aps_ref_id_scaling | ue(v) |
| } |  |
| aps_deblocking_data_present_flag | u(1) |
| if(aps_deblocking_data_present_flag) |  |
| { |  |
| disable_deblocking_filter_flag | u(1) |
| if( !disable_deblocking_filter_flag ) |  |
| { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| else if ( !aps_intra_flag ) |  |
| { |  |
| aps_deblocking_data_from_ref_flag | u(1) |
| if ( aps_deblocking_data_from_ref_flag ) |  |
| aps_ref_id_deblocking | ue(v) |
| } |  |
| aps_sample_adaptive_offset_data_present_flag | u(1) |
| if( aps_sample_adaptive_offset_data_present_flag ) |  |
| { |  |
| aps_sample_adaptive_offset_flag | u(1) |
| if( aps_sample_adaptive_offset_flag ) |  |
| aps_sao_param( ) |  |
| } |  |
| else if ( !aps_intra_flag ) |  |
| { |  |
| aps_sample_adaptive_offset_data_from_ref_flag | u(1) |
| if ( aps_sample_adaptive_offset_data_from_ref_flag ) |  |
| aps_ref_id_sample_adaptive_offset | ue(v) |
| } |  |
| aps_adaptive_loop_filter_data_present_flag | u(1) |
| if( aps_adaptive_loop_filter_data_present_flag ) |  |
| alf_param( ) |  |
| else if ( !aps_intra_flag ) |  |
| { |  |
| aps_ adaptive_loop_filter_data_from_ref_flag | u(1) |
| if ( aps_adaptive_loop_filter_data_from_ref_flag ) |  |
| aps_ref_id_ adaptive_loop_filter | ue(v) |
| } |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

aps_id identifies the adaptation parameter set that is referred to in the slice header.

aps_intra_flag equals 1 indicates this adaptation parameter set does not refer to any other adaptation parameter set. aps_intra_flag equals 0 indicates otherwise.

aps_ref_id specifies the aps_id of the reference adaptation parameter set that is refer to by this adaptation parameter set.

aps_scaling_list_data_present_flag equal to 1 specifies that scaling list parameters exist in this adaptation parameter set, equal to 0 specifies that scaling list parameters do not exist in this adaptation parameter set.

aps_scaling_list_data_from_ref_flag equal to 1 specifies that scaling list parameters may exist in the  adaptation parameter set referred to by current adaptation parameter set, equal to 0 specifies that scaling list parameters do not exist in the adaptation parameter set referred to by current adaptation parameter set.

aps_ref_id_scaling specifies the aps_id of the reference adaptation parameter set that contains scaling list parameters.

aps_deblocking_data_present_flag equal to 1 specifies that deblocking parameters are present in the adaptation parameter set, equal to 0 specifies that deblocking parameters do not exist in this adaptation parameter set.

aps_deblocking_data_from_ref_flag equal to 1 specifies that deblocking parameters may exist in the  adaptation parameter set referred to by current adaptation parameter set, equal to 0 specifies that deblocking parameters do not exist in the adaptation parameter set referred to by current adaptation parameter set.

aps_ref_id_deblocking specifies the aps_id of the reference adaptation parameter set that contains deblocking parameters.

aps_sample_adaptive_offset_data_present _flag equal to 1 specifies that the sample adaptive offset parameters are present in this adaptation parameter set, equal to 0 specifies that sample adaptive offset parameters are not present in this adaptation parameter.

aps_sample_adaptive_offset_data_from_ref_flag equal to 1 specifies that sample adaptive offset parameters may exist in the  adaptation parameter set referred to by current adaptation parameter set, equal to 0 specifies that sample adaptive offset parameters do not exist in the adaptation parameter set referred to by current adaptation parameter set. If sample adaptive offset parameters are not present in current adaptation parameter set or in reference adaptation parameter set, sample adaptive offset parameters are interleaved in slice data.

aps_ref_id_sample_adaptive_offset specifies the aps_id of the reference adaptation parameter set that contains sample adaptive offset parameters.

aps_sample_adaptive_offset_flag equal to 1 specifies that the sample adaptive offset is on for slices referring to the current adaptation parameter set; equal to 0 specifies that the sample adaptive offset is off for slices referring to the current APS.  When there is no active adaptation parameter set, the aps_sample_adaptive_offset_flag value is inferred to be 0.

aps_adaptive_loop_filter_data_present_flag equal to 1 specifies that the adaptive loop filter is on for slices referring to the current adaptation parameter set; equal to 0 specifies that the adaptive loop filter may be off for slices referring to the current adaptation parameter set. When there is no active adaptation parameter set, the aps_adaptive_loop_filter_flag value is inferred to be 0.

aps_adaptive_loop_filter_data_from_ref_flag equal to 1 specifies that the activation of adaptive loop filter for slices referring to the current adaptation parameter set depends on the aps_adaptive_loop_filter_data_present_flag in the reference adaptation parameter set, equal to 0 specifies that the adaptive loop filter is off for slices referring to the current adaptation parameter set.

aps_ref_id_adaptive_loop_filter specifies the aps_id of the reference adaptation parameter set that contains adaptive loop filter parameters.

aps_extension_flag equal to 0 specifies that no aps_extension_data_flag syntax elements are present in the picture parameter set RBSP syntax structure. aps_extension_flag shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value of 1 for aps_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for aps_extension_flag in a picture parameter set NAL unit.

aps_extension_data_flag may have any value. Its value does not affect decoder conformance to profiles specified in this Recommendation | International Standard.

# Conclusion

In this contribution, two proposes schemes to allow APS partial updates, that is, APS partial updates with one APS reference and APS partial updates with multiple APS references are presented. The APS partial updates with one APS reference is similar to the scheme proposed in H0069 with the different that the propose scheme add flags to indicate whether or not the information that is not present in the current APS exist in reference APS. This is claimed important to avoid worst case where decoder has to search till the end of linked-list of APS only to find that particular information does not exist. On the other hand, the APS partial updates with multiple APS references is similar to the schemes proposed in H0381 with improvements.

In our analysis, partial updates with one reference might be simpler for implementation but may have problem with long linked-list of APS. On the other hand, partial updates with multiple references may be a bit more complicated but will have shorter link-list of APS, thus, we think that APS partial update with multiple references is more preferred.

We recommend the JCTVC group to consider adopting one of the proposed schemes.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).