| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0082 |
| --- | --- |

| Title: | APS partial update – APS buffer management |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Byeong Moon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130 Korea | Tel: Email: | - hendry.hendry@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes buffer management mechanisms for APS. It is claimed that if APS partial update is possible, then there must be mechanism to manage APS to guarantee that the referring and update mechanism can work, such that, there is no case where the APS needed as reference is not available in the memory, unless it is lost during transmission. The contribution proposes three mechanisms:

- Sliding window with reference copy

- Buffer reset with intra APS is received

- Combination of the first two proposed mechanisms.

It is assessed that the proposed mechanism 1 is simple by may have error resiliency issue since it does not require intra APS to be signalled in certain interval of period; proposed mechanism 2 is also simple by may require bigger memory to store all APS until an intra APS is received; lastly, the proposed mechanism 3 addresses the issues of the both proposed mechanism 1 and 2, thus, the proponent recommend this mechanism to be adopted.

Note: this contribution can be ignored if APS partial update feature is not used for HEVC.

# Introduction

If APS partial update mechanism is used, there must be management system to guarantee that partial update and referring mechanism work. APS buffering mechanism shall be able to guarantee that for any APS that exist in the buffer, all of its reference APS shall also exists in the buffer, unless there is APS lost during data transmission. The following figure illustrates the situation that should not happen. In the illustration, APS ID 0 is not in the buffer which makes all other APS cannot be perfectly decoded because they are directly or indirectly refer to the missing APS.

Figure 1 – APS need for reference is not available in buffer

# Proposed APS Buffer Managements

## Scheme 1 -- Sliding window with reference copy

The concept of the proposed method is as follow:

- The first APS in the bitstream shall be an intra APS (i.e., APS that does not refer to any other APS)

- Maximum of reference distance must be defined. Such distance indicates how many previously received APS between current APS and its reference APSs

- The maximum reference distance also serves as information to determine the minimum size of memory to buffer received APS.

- Insertion of newly received APS and removal of existing APS from buffer are managed by first-in-first-out (FIFO) concept. The FIFO concept for insertion and removal process is illustrated in the following figure. In the initial situation, the APS buffer capacity has been full. When a new APS is received (i.e., APS ID 4), one APS must be removed from the APS buffer. The APS that must be removed from the buffer is APS ID 0 since it is the earliest received APS among any other APS in the APS buffer.

Figure 2 – Insertion and Removal of APS from buffer

- When an APS is received and decoded, all reference must be resolved, that is, if there is parameter that is not present in the APS but available by reference from other APS, that parameter must be copied from the reference and the parameter is marked as present in the APS.

Figure 3 – Solving reference prior / during insertion of newly received APS into buffer

Advantages of the proposed method 1:

- Easy buffer management.

- Can be implemented with small / minimum size of APS buffer

- Never have buffer overflow problem

Disadvantages of the proposed method 1:

- Not good for error resilience, APS buffer is never reset so that if there is any APS lost which can make APS referencing mechanism fails, the problem will propagate.

Signalling Maximum APS Reference Distance

As described in the concept of the proposed method, there is requirement to specify maximum reference distance. It is proposed to signal such information in HEVC Sequence Parameter Set (SPS) as follow:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| max_aps_ref_distance | ue(v) |
| … |  |
| } |  |

max_aps_ref_distance specifies the maximum distance between current / active APS to any of previously received APS that it uses for reference(s). The value of max_aps_ref_distance shall be in the range of 0. max_aps_ref_distance equals 0 specifies that all APS present in the bitstream shall not need reference to other APS.

NOTE: It is required that at the system provide a buffer to keep at least max_aps_ref_distance + 1 APS.

## Scheme 2 – Buffer reset upon receiving intra APS

The concept of the proposed method is as follow:

- The first APS in the bitstream shall be an intra APS (i.e., APS that does not refer to any other APS)

- Maximum distance between the occurrences of two intra APS must be defined.

- The maximum distance also serves as information to determine the minimum size of memory to buffer received APS.

- It is required by this proposed method that any non-intra APS shall not refer to any APS received before the last received intra APS. The following figures illustrate the restriction.

Figure 4 – Cases where reference is ok and not ok

- When an intra APS is received:

  - All existing APS in the buffer is removed (i.e., buffer is made empty)

  - Insert the intra APS into the buffer

Advantages of the proposed method 2:

- Easy buffer management.

- Good for error resilience since at certain interval, APS buffer is always reset so that if there is any APS lost which can make APS referencing mechanism fails, it will always be reset at the occurance of intra APS.

Disadvantages of the proposed method 2:

- May require bigger memory buffer compared to proposed method 1.

- Buffer may overflow when intra APS is lost.

Signalling Maximum Intra APS Distance

As described in the concept of the proposed method, there is requirement to specify maximum distance between the occurrence of two intra APS. It is proposed to signal such information in HEVC Sequence Parameter Set (SPS) as follow:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| max_intra_aps_distance | ue(v) |
| … |  |
| } |  |

max_intra_aps_distance specifies the maximum distance the occurrence between two APS that do not require reference to other APS. The value of max_intra_aps_distance shall be in the range of 0 to TBD. max_intra_aps_distance equals 0 specifies that all APS present in the bitstream shall not need reference to other APS.

NOTE: It is required that at the system provide a buffer to keep at least max_intra_aps_distance + 1 APS.

## Scheme 3 – Hybrid scheme

This proposed method combines the above two proposed method (i.e., proposed method 1 – sliding window with reference copy and proposed method 2 – buffer reset when intra APS is received)

The concept of the proposed method is as follow:

- The first APS in the bitstream shall be an intra APS (i.e., APS that does not refer to any other APS)

- Maximum of reference distance must be defined. Such distance indicates how many previously received APS between current APS and its reference APSs

- The maximum reference distance also serves as information to determine the minimum size of memory to buffer received APS.

- Insertion of newly received APS and removal of existing APS from buffer are managed by first-in-first-out (FIFO) concept. The FIFO concept for insertion and removal process is illustrated in the following figure. In the initial situation, the APS buffer capacity has been full. When a new APS is received (i.e., APS ID 4), one APS must be removed from the APS buffer. The APS that must be removed from the buffer is APS ID 0 since it is the earliest received APS among any other APS in the APS buffer.

Figure 5 – Insertion and Removal of APS from buffer

- When an APS is received and decoded, all reference must be resolved, that is, if there is parameter that is not present in the APS but available by reference from other APS, that parameter must be copied from the reference and the the parameter is marked as present in the APS.

Figure 6 – Solving reference prior / during insertion of newly received APS into buffer

- It is required by this proposed method that any non-intra APS shall not refer to any APS received before the last received intra APS. The following figures illustrate the restriction.

Figure 7 – Cases where reference is ok and not ok

- When an intra APS is received:

  - All existing APS in the buffer is removed (i.e., buffer is made empty)

  - Insert the intra APS into the buffer

Advantages of the proposed method 2:

- Can be implemented with relatively small / minimum buffer size.

- Good for error resilience since at certain interval, APS buffer is always reset so that if there is any APS lost which can make APS referencing mechanism fails, it will always be reset at the occurance of intra APS.

- Never have buffer overflow problem

Disadvantages of the proposed method 2:

- Complicated buffer management relative to the proposed method 1 and 2.

Signalling Maximum APS Reference Distance

As described in the concept of the proposed method, there is requirement to specify maximum reference distance. It is proposed to signal such information in HEVC Sequence Parameter Set (SPS) as follow:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| max_aps_ref_distance | ue(v) |
| … |  |
| } |  |

max_aps_ref_distance specifies the maximum distance between current / active APS to any of previously received APS that it uses for reference(s). The value of max_aps_ref_distance shall be in the range of 0. max_aps_ref_distance equals 0 specifies that all APS present in the bitstream shall not need reference to other APS.

NOTE: It is required that at the system provide a buffer to keep at least max_aps_ref_distance + 1 APS.

# Conclusion

In this contribution three buffer management mechanisms for APS are presented. The proposed mechanism is important if APS partial update is possible to guarantee that the referring and update mechanism can work, such that, there is no case where the APS needed as reference is not available in the memory, unless it is lost during transmission.

Advantages and disadvantages of the proposed mechanisms are also presented. In our opinion, the proposed mechanism 3 gives the best trade-off so that we recommend that the JCTVC group consider adopting the proposed mechanism 3, if APS partial updates feature is supported in HEVC.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).