| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0083 |
| --- | --- |

| Title: | Error resilience improvements for APS partial updates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Byeong Moon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130 Korea | Tel: Email: | - hendry.hendry@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

It is assessed that if APS partial updates mechanism is allowed, in which, some APS is allowed to refer to other APS, there is a risk of incorrect APS resolution / decoding if the APS needed for reference is not available. This contribution proposes some recommendation to improve HEVC bitstream robustness against APS lost.

Note: this contribution can be ignored if APS partial update feature is not used for HEVC.

# Introduction & problem statement

If APS partial update is applied, in which, some APS is allowed to refer to parameters in other APS instead of carries all parameters it needs to carry, there is a risk of incorrect APS resolution / decoding if the APS needed for reference is not available. The unavailability of reference APS can be due to various reason, for example, APS lost during transmission. Thus, some techniques / methods are needed to improve the error resiliency of HEVC bitstream when APS partial update is used.

Problem #1 – Error propagation of incorrect parameters

The problem of propagation of incorrect parameters is illustrated in the following figure. When an APS that is needed for reference is not available (e.g., due to lost during transmission), all other APS that directly or indirectly refer to it will not be perfectly decidable.

Figure 1 – Error propagation of incorrect parameters

Problem #2 –Phantom Error

We define phantom error as the error that occurs when an APS, which refers to other APS(es) that are actually available), referred by one or more slice(s) is not available. The following figure illustrates condition that creates phantom error. In such condition, when the slice that refer to the inavailable APS is being decoded, the decoder cannot find the APS that is referred by the slice thus, it will simply assume that all parameters (i.e., scaling list, deblocking, SAO, ALF) for the slice are not available. However, it might not be true since actually those parameters are available in other APS.

Figure 2 – Phantom error which actually can perfectly be concealed

Problem #3 –Ambiguous reference identification

The range of APS ID is limited. For example, in HEVC main profile, the range of APS ID is restricted to be from 0 to 63. When the maximum APS ID has been reached, the ID will be wrapped around and started from 0 again.

This wrapped around characteristic of APS ID may be a problem when burst error happens. When burst error happen, it is possible that there are two APS with the same ID exists in the APS buffer and thus create confusion for slice header. The following figure illustrates this problem.

Figure 3 – Ambiguous reference of APS due to burst data lost

In the following sections, some techniques / methods / restrictions are proposed to improve error resiliency of HEVC bitstreams against error due to unavailability of APS that are needed for reference.

# Proposed improvements

## Addressing the problem #1

To stop propagation of incorrect parameters, the following restriction shall be mandated:

- At some interval, intra APS (i.e., APS that do not refer to any other APS) must present in the bitstream)

- Any non-intra APS shall not refer to any APS received before the last received intra APS.

- The occurrence interval between two intra APS shall not be bigger that the occurrence interval between two IDR slice or CRA slice.

- IDR slice and CRA slice shall only refer to intra APS

## Addressing the problem #2

To deal with phantom error of APS, the following syntax elements are proposed to be added in slice header

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( scaling_list_enable_flag ¦ ¦ deblocking_filter_in_aps_enabled_flag ¦ ¦ ( sample_adaptive_offset_enabled_flag && !slice_sao_interleaving_flag ) ¦ ¦ adaptive_loop_filter_enabled_flag ) |  |
| { |  |
| aps_id | ue(v) |
| scaling_data_from_ref_flag | u(1) |
| deblocking_data_from_ref_flag | u(1) |
| sao_data_from_ref_flag | u(1) |
| alf_data_from_ref_flag | u(1) |
| } |  |
| … |  |
| } |  |

scaling_data_from_ref_flag equals to 1 indicates that scaling list parameters for current slice may present in APS other the APS indicated by the aps_id, equals to 0 indicates that scaling list parameters for current slice only presents in the APS indicated by the aps_id

deblocking_data_from_ref_flag equals to 1 indicates that deblocking parameters for current slice may present in APS other the APS indicated by the aps_id, equals to 0 indicates that deblocking parameters for current slice only presents in the APS indicated by the aps_id

sao_data_from_ref_flag equals to 1 indicates that SAO parameters for current slice may present in APS other the APS indicated by the aps_id, equals to 0 indicates that SAO parameters for current slice only presents in the APS indicated by the aps_id

alf_data_from_ref_flag equals to 1 indicates that ALF parameters for current slice may present in APS other the APS indicated by the aps_id, equals to 0 indicates that ALF parameters for current slice only presents in the APS indicated by the aps_id

With the above elements, the phantom error can be detected easily. The following figure illustrates how decoder detects the occurrence of phantom error. When APS ID 2 is not available, during decoding of the slice, decoder can check the values of the flags and find out that all the parameters are actually may be available in other APS.

Figure 4 – Detection of phantom error

## Addressing the problem #3

To solve the problem of ambiguous reference identification, the following method is proposed.

During decoding of a slice, if the active APS that is referred by the slice is duplicated in APS buffer, the following actions should be taken:

- Sort the duplicated APS(s) in descending order of their receiving timestamp, that is, the APS that are recently received shall occupy smaller index in the array of sorted APS.

- Use the APS in the first index (i.e., index 0)

By using the proposed method, there will be no ambiguity when there are more than one APS with the same APS ID exists in APS buffer.

# Conclusion

In this contribution some proposed methods for improving HEVC bitstreams robustness against APS lost are described. We recommend the JCTVC group to discussed the proposed methods and adopt them.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).