| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0084 |
| --- | --- |

| Title: | Reduced number of checking scaled MV candidates in AMVP list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongbing Lin Lingzhi Liu | Tel: Email: | +86 10 60612142 yblin@huawei.com Lingzhi.liu@huawei.com |
| Source: | HiSilicon Technologies, Huawei Technologies |  |  |

_____________________________

# Abstract

One modification is proposed to simplify spatial AMVP derivation by reducing number of checking scaled MV candidates. The number of candidate positions used for deriving the scaled MV candidate is reduced from 5 to 2. Simulation results show that the proposed modification achieves coding efficiency of 0.0%, 0.0% and 0.0% for both random access and low-delay configurations under common test conditions. It is recommended to adopt the proposal.

# Introduction

In HM6, total five spatial candidate positions are used for spatial AMVP candidate derivation. These candidate positions are also exploited for deriving scaled MV candidate during spatial AMVP list construction [1]. As shown in Fig.1, two candidate positions (left-below and left, i.e. A0, A1) are firstly scanned to derive the scaled MV candidate. If the scaled MV candidate is not available, the remain three positions (Above-right, above, above-left, i.e. B0, B1, B2) will be further scanned to derive the scaled MV candidate. Total five candidate positions need to be checked for deriving the scaled MV candidate in the worst case.

Figure 1. Candidate positions for deriving the scaled MV candidate

In this document, the number of candidate positions used for deriving the scaled MV candidate is reduced from 5 to 2. As shown in Table 1, only the left position is checked during the first scanning and the above position for the second scanning.

Table 1. Candidate positions used for deriving the scaled MV candidate

|  | HM6 | Proposal |
| --- | --- | --- |
| In first scanning | Left-below (A0) Left (A1) | Left (A1) |
| In second scanning | Above-right (B0) Above (B1) Above-left (B2) | Above (B1) |

# Experimental results

The simulation of this proposal is based on HM6.0 software under common test conditions specified in [2]. The experimental results are summarized in Table 2. The proposed method shows coding efficiency of 0.0%, 0.0%, 0.0% for both random access and low-delay configurations.

Table 2. Experimental results of the proposed method comparing to HM6.0

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.2% | 0.0% | 0.2% | 0.0% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.2% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.1% |
| Class D | 0.1% | 0.2% | 0.2% | 0.0% | -0.2% | -0.3% |
| Class E | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% |
| Class F | -0.1% | -0.2% | -0.1% | 0.0% | 0.1% | 0.5% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

# Conclusion and recommendation

One modification is proposed to simplify spatial AMVP derivation by reducing the number of checking scaled MV candidates. No coding efficiency loss is observed for the proposed modification. It is recommended to adopt the proposal.

# References

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 6,” JCT-VC Document, JCTVC-H1003, San Jose, US, Feb. 2012

- F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-H1100, San Jose, US, Feb. 2012

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed text changes to WD6

#### 8.5.2.1.6 Derivation process for motion vector predictor candidates

…

The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps:

- Let a set of two sample locations be (xAk, yAk), with k = 0, 1, specifies sample locations with xAk = xP − 1, yA0 = yP + nPSH and yA1 = yA0 - MinPuSize. The set of sample locations ( xAk, yAk ) represent the sample locations immediately to the left side of the left partition boundary and it’s extended line.

- Let the availability flag availableFlagLXA be initially set equal to 0 and the both components of mvLXA are set equal to 0.

- If one of the following conditions is true, the variable isScaledFlagLX is set equal to 1.

- the prediction unit covering luma location ( xA0, yA0 ) is available and PredMode is not MODE_INTRA.

- the prediction unit covering luma location ( xA1, yA1 ) is available and PredMode is not MODE_INTRA.

- For ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 − MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1 and the reference index refIdxLX[ xAk ][ yAk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXA is set equal to 1 and the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ] and ListA is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1 and PicOrderCnt( RefPicListLY( refIdxLY[ xAk ][ yAk ] ) ) is equal to PicOrderCnt( RefPicListLX( refIdxLX ) ), availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ] ,  ListA is set equal to LY and mvLXA is set equal to mvLXA.

- When availableFlagLXA is equal to 0,  ( xAk, yAk ) is set equal to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ], ListA is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ], ListA is set equal to LY.

- When availableFlagLXA is equal to 1, mvLXA is derived as specified below.

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td (8-1)

DistScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 ) (8-2)

mvLXA = Clip3( −8192, 8191.75, Sign( DistScaleFactor * mvLXA ) * 			( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 ) ) (8-3)

where td and tb are derived as

td = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListListA( refIdxA ) ) ) (8-4)

tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListLX( refIdxLX ) ) ) (8-5)

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

- Let a set of three sample location (xBk, yBk), with k = 0,1,2, specifies sample locations with xB0 = xP + nPSW, xB1 = xB0− MinPuSize , xB2 = xP − MinPuSize and yBk = yP − 1. The set of sample locations ( xBk, yBk ) represent the sample locations immediately to the upper side of the above partition boundary and its extended line.

- If yP−1 is less than yCtb, the following applies.

xB0 = (xB0>>Log2MinCbSize)<<Log2MinCbSize) + ((xB0>>Log2MinCbSize)&1)*(MinPuSize*2−1)	(8-6)
xB1 = (xB1>>Log2MinCbSize)<<Log2MinCbSize) + ((xB1>>Log2MinCbSize)&1)*(MinPuSize*2−1)	(8-7)
xB2 = (xB2>>Log2MinCbSize)<<Log2MinCbSize) + ((xB2>>Log2MinCbSize)&1)*(MinPuSize*2−1)	(8-8)

where MinPuSize = 1 << ( Log2MinCbSize − 1)

- Let the availability flag availableFlagLXB be initially set equal to 0 and the both components of mvLXB are set equal to 0.

- For ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP + nPSW, xB1 = xB0 − MinPuSize , and xB2 =  xP − MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, and the reference index refIdxLX[ xBk ][ yBk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXB is set equal to 1 and the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ] and ListB is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1 and PicOrderCnt( RefPicListLY( refIdxLY[ xBk ][ yBk ] ) ) is equal to PicOrderCnt( RefPicListLX( refIdxLX ) ), availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ],  refIdxB is set equal to refIdxLY[ xBk ][ yBk ],  and ListB is set equal to LY.

- When isScaledFlagLX is equal to 0 and availableFlagLXB is equal to 1,  mvLXA is set equal to mvLXB and refIdxA is set equal to refIdxB and availableFlagLXA is set equal to 1.

- When isScaledFlagLX is equal to 0, availableFlagLXB is set equal to 0 and  ( xBk, yBk ) is set equal to ( xB1, yB1 ) where xB1 = xP + nPSW - MinPuSize, the following applies:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ], ListB is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ], ListB is set equal to LY.

- When availableFlagLXB is equal to 1 and PicOrderCnt( RefPicListListB( refIdxB ) ) is not equal to PicOrderCnt( RefPicListLX( refIdxLX ) ), mvLXB is derived as specified below.

…