| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0085 |
| --- | --- |

| Title: | On chroma intra prediction mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongbing Lin Lingzhi Liu | Tel: Email: | +86 10 60612142 yblin@huawei.com Lingzhi.liu@huawei.com |
| Source: | HiSilicon Technologies, Huawei Technologies |  |  |

_____________________________

# Abstract

This document proposes a modified chroma intra prediction mode coding when LM mode is enabled. Similar to luma intra prediction mode coding, both DM and LM modes are treated as MPMs and the other 4 remaining modes are bypass coded. The proposed method could unify the parsing procedure for Luma and Chroma mode coding. Simulation results reveal that this method also achieves the coding efficiency of 0.0%, -0.2% and -0.2% for both AI-HE10 and AI-main with LM enabled. It is recommended to adopt the proposal.

# Introduction

In the last meeting, fixed length coding for remaining intra chroma modes has been adopted. The binarization of Intra chroma modes in HM6 is shown in Table 1. For DM and LM use two contexts respectively while the remaining 4 modes use fixed-length code.

Table 1 binarization for intra chroma prediction modes in HM6.0

| Mode Idx | LM enabled | LM disabled |
| --- | --- | --- |
| DM | 0 | 0 |
| LM | 10 | x |
| mode 0 | 1100 | 100 |
| mode 1 | 1101 | 101 |
| mode 2 | 1110 | 110 |
| mode 3 | 1111 | 111 |

In this document, the binarization is modified as shown in Table 2. As similar as luma prediction mode coding, both DM and LM modes are treated as MPMs and other 4 intra modes become remaining modes. The first bin is used to indicate MPM or remaining mode, and is coded with context. In case of zero value of the first bin, the second bin is also coded with a separated context. These remaining 4 modes use bypass coding. It is also observed that the bin numbers of remaining mode is reduced from 4 to 3, which is helpful for parsing in the worst case.

Table 2 The proposed binarization for intra chroma prediction modes

| Mode Idx | LM enabled | LM disabled |
| --- | --- | --- |
| DM | 00 | 0 |
| LM | 01 | x |
| mode 0 | 100 | 100 |
| mode 1 | 101 | 101 |
| mode 2 | 110 | 110 |
| mode 3 | 111 | 111 |

# Experimental results

The simulations of this document use HM6.0 software. The common test conditions and reference configurations specified in [2] are followed. Noted that LM mode is turned on for main configuration in the experiment. The experimental results are summarized in Table 3. Coding efficiency of 0.0%, -0.2% and -0.2% is observed for both all intra HE10 and all intra main with LM enabled.

Table 3. Experimental results for the proposal in HE10 and main with LM enabled

|  | All Intra Main with LM enabled |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.4% | -0.3% | 0.0% | -0.4% | -0.4% |
| Class B | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.2% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |
| Class E | 0.0% | -0.2% | -0.3% | 0.0% | -0.2% | -0.2% |
| Overall | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.2% |
|  | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.2% |
| Class F | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusion and recommendation

It is recommended to adoption of the proposal due to its unification of intra mode coding between luma and chroma, friendly parsing and improved performance.

# References

- F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-H1100, San Jose, US, Feb. 2012

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 6,” JCT-VC Document, JCTVC-H1003, San Jose, US, Feb. 2012

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed text changes to WD6

#### 9.2.2.9 Binarization process for intra_chroma_pred_mode

Input to this process is a request for a binarization for the syntax element intra_chroma_pred_mode and the chroma_pred_from_luma_enabled_flag.

Output of this process is the binarization of the syntax element.

Table 9-1 – Specifcation of intra_chroma_pred_mode depending on chroma_pred_from_luma_enabled_flag

| Value of intra_chroma_pred_mode | chroma_pred_from_luma_enabled_flag = 1 | chroma_pred_from_luma_enabled_flag = 0 |
| --- | --- | --- |
| 5 | 00 | n/a |
| 4 | 01 | 0 |
| 0 | 100 | 100 |
| 1 | 101 | 101 |
| 2 | 110 | 110 |
| 3 | 111 | 111 |

| Table 9- – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| intra_chroma_pred_mode (chroma_pred_from_luma_enabled_flag == true) | Table 9-2 | 0 | 0 | 1 | na | na | na |
|  |  | 2 | 0 | 1 | na | na | na |
|  |  | 4 | 0 | 1 | na | na | na |
| intra_chroma_pred_mode_flag (chroma_pred_from_luma_enabled_flag == true) | Table 9-2 | 0 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |
|  |  | 4 | 0 | na | na | na | na |
| intra_chroma_pred_mode_DM_LM (chroma_pred_from_luma_enabled_flag == true) | Table 9-2 | 0 | na | 1 | na | na | na |
|  |  | 2 | na | 1 | na | na | na |
|  |  | 4 | na | 1 | na | na | na |
| intra_chroma_pred_mode (chroma_pred_from_luma_enabled_flag == false) | Table 9-2 | 0 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |
|  |  | 4 | 0 | na | na | na | na |

Table 9-2 – Values of variable initValue for intra_chroma_pred_mode ctxIdx

| Initialization variable | intra_chroma_pred_mode ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 63 | 63 | na | na | na | na |