| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0086 |
| --- | --- |

| Title: | Cross-check report of virtual NSQT (JCTVC-I0146) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yongbing Lin | Tel: Email: | +86 10 60612142 yblin@huawei.com |
| Source: | HiSilicon Technologies |  |  |

_____________________________

# Abstract

This document reports cross-check results of LG’s proposal on virtual NSQT (JCTVC-I0146). When virtual non square transform is applied, luma CBF of the TU is derived as zero. The rest results show all bit-rate and PSNR values match exactly with those reported by proponent, and runtime ratios are very similar.

# Introduction

This document reports cross-check results of LG’s proposal on virtual NSQT [1]. The virtual NSQT whose size is larger than maximum transform size is discussed. When virtual non square transform is applied, luma CBF of the TU is derived as zero. The concept of virtual NSQT makes it possible to apply 32x8 TU for chroma components.

# Test results

We got the source code and independently compiled the code with VS2005, and ran in environment of Windows server 2003 R2 64bits. The common test conditions and software reference configurations [2] are followed in our simulation. HM6.1 is used as anchor. The test results are summarized in Table 1. It shows coding efficiency of 0.0%, -0.2%, -0.1% in RA-HE10 and 0.1%, -0.4%, -0.7% in LB-HE10. It is confirmed that all bit-rate and PSNR values match exactly with those reported by proponent, and encoding/decoding run time has similar character.

Table 1. Test results for virtual NSQT, comparing to HM6.1

|  | Random Access HE10 |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.3% | -0.1% |  |  |  |
| Class B | 0.1% | -0.2% | -0.2% | 0.1% | -0.6% | -0.8% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.5% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.6% |
| Class E |  |  |  | 0.2% | -1.3% | -1.0% |
| Overall | 0.0% | -0.2% | -0.1% | 0.1% | -0.4% | -0.7% |
|  | 0.0% | -0.2% | -0.1% | 0.1% | -0.5% | -0.6% |
| Class F | 0.0% | -0.2% | -0.2% | -0.1% | -1.2% | -0.6% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

# Recommendation

The test results reported in JCTVC-I0146 is verified.

# References

- Jungsun Kim, Byeongmoon Jeon, “Virtual non square transform unit over maximum transform size with zero CBF,” JCT-VC Document, JCTVC-I0146, Geneva, CH, April, 2012

- F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-H1100, San Jose, US, Feb. 2012