| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0087 |
| --- | --- |

| Title: | Comments on Reference Picture Lists Combination Syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Yong Joon Jeon Seung Wook Park Byeong Moon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130, Korea Ying Chen Ye-Kui Wang Wei-Jung Chien 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | - hendry.hendry@lge.com yongjoon.jeon@lge.com seungwook.park@lge.com bm.jeon@lge.com 1-858-845-6589 cheny@qualcomm.com 1-858-651-8345 yekuiw@qualcomm.com 1-858-845-1795 wchien@qualcomm.com |
| Source: | LG Electronics and Qualcomm Incorporated |  |  |

_____________________________

# Abstract

It was reported that there are some redundancies on the reference picture list combination syntax in the current HEVC WD. More specifically when the ref_pic_list_combination_flag is equal to 0, it indicates that two reference picture lists are identical, but both lists are still signaled separately. In this document, it is proposed that the location of this flag be moved to an earlier place of the slice header, such that the redundancy can be removed by not signaling the reference picture list 1 when ref_pic_list_comibination_flag is equal to 0. Furthermore, the name of this flag is changed to avoid confusion.

# Introduction

In the current HEVC WD, ref_pic_list_comibination_flag is signalled for a B slice and when this flag equals to 1, either implicit reference picture lists combination or explicit reference picture list combination is used to generate a combined list; when this flag equal to 0, reference picture list 0 (RefPicList0) and reference picture list 1 (RefPicList1) are identical thus no list combination is needed.

Based on the current design, there are two ways to decide whether RefPicList0 and RefPicList1 are identical thus no reference picture list combination is needed. One way is to check the ref_pic_list_combination_flag, and the other way is to compare these two lists.

However, when ref_pic_list_combination_flag is equal to 0, there is no need to signal the RefPicList1. Such a redundancy is reported in the last JCTVC meeting in Geneva.

# Proposal

It is proposed that the ref_pic_list_combination_flag be moved to an earlier place of the slice header to avoid the redundancy and the semantics of this flag can be kept unchanged.

Note that the modifications are highlighted in green.

## Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| … |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( slice_type = = B ) |  |
| identical_lists_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( !identcial_lists_flag ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| … |  |
| } |  |

## Reference picture list modification syntax

| ref_pic_list_modification( ) { | Descriptor |
| --- | --- |
| if( slice_type % 5 != 2 && slice_type % 5 != 4 ) { |  |
| ref_pic_list_modification_flag_l0 | u(1) |
| if( ref_pic_list_modification_flag_l0 ) |  |
| do { |  |
| modification_of_pic_nums_idc | ue(v) |
| if( modification_of_pic_nums_idc = = 0 ¦ ¦ modification_of_pic_nums_idc = = 1 ) |  |
| abs_diff_pic_num_minus1 | ue(v) |
| else if( modification_of_pic_nums_idc = = 2 ) |  |
| long_term_pic_num | ue(v) |
| } while( modification_of_pic_nums_idc != 3 ) |  |
| } |  |
| if( slice_type % 5 = = 1 && !identical_lists_flag ) { |  |
| ref_pic_list_modification_flag_l1 | u(1) |
| if( ref_pic_list_modification_flag_l1 ) |  |
| do { |  |
| modification_of_pic_nums_idc | ue(v) |
| if( modification_of_pic_nums_idc = = 0 ¦ ¦ modification_of_pic_nums_idc = = 1 ) |  |
| abs_diff_pic_num_minus1 | ue(v) |
| else if( modification_of_pic_nums_idc = = 2 ) |  |
| long_term_pic_num | ue(v) |
| } while( modification_of_pic_nums_idc != 3 ) |  |
| } |  |
| } |  |

## Reference picture lists combination syntax

| ref_pic_list_combination( ) { | Descriptor |
| --- | --- |
| if( slice_type % 5 = = 1 ) {// b slice |  |
| ref_pic_list_combination_flag | u(1) |
| if( !identical_lists_flag ref_pic_list_combination_flag ) { |  |
| num_ref_idx_lc_active_minus1 | ue(v) |
| ref_pic_list_modification_flag_lc | u(1) |
| if( ref_pic_list_modification_flag_lc ) |  |
| for ( i =0; i <= num_ref_idx_lc_active_minus1; i++ ) { |  |
| pic_from_list_0_flag | u(1) |
| ref_idx_list_curr | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |

## Slice header semantics

identical_lists_flag ref_pic_list_combination_flag equal to 1 0 indicates that the reference picture list 0 and the reference picture list 1 are combined to be an additional reference picture lists combination used for the prediction units being uni-directional predicted. This flag equal to 0 1 indicates that the reference picture list 0 and reference picture list 1 are identical thus reference picture list 0 is used as the reference picture lists combination. The reference picture lists combination is set to be empty at the start of the loop defined in this the reference picture lists combination syntax table.

Note that no matter the idential_lists_and_combine_flag (the previous ref_pic_list_combination_flag) is 0 or 1, there is a combined list. The difference is that when idential_lists_and_combine_flag is equal to 1, the combined list is set to reference picture list 0 and when idential_lists_and_combine_flag is equal to 0, the combined list is actually combined by the explicit mechanism or implicit mechanism based on the ref_pic_list_modification_flag_lc.

# Discussion

Similar changes may be made for the prediction weights table syntax and the collocated_from_l0_flag syntax in the slice header based on the same spirit as the proposal.

We don’t include in the proposal part of this document but we recommend the following syntax modifications to be examined and considered by the group.

## Prediction weight table syntax

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| luma_log2_weight_denom | ue(v) |
| if( chroma_format_idc != 0 ) |  |
| chroma_log2_weight_denom | ue(v) |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) { |  |
| … |  |
| } |  |
| if( slice_type = = B && !identical_lists_flag ) |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) { |  |
| luma_weight_l1_flag | u(1) |
| if( luma_weight_l1_flag ) { |  |
| luma_weight_l1[ i ] | se(v) |
| luma_offset_l1[ i ] | se(v) |
| } |  |
| if( chroma_format_idc != 0 ) { |  |
| chroma_weight_l1_flag | u(1) |
| if( chroma_weight_l1_flag ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| chroma_weight_l1[ i ][ j ] | se(v) |
| chroma_offset_l1[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |

## Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !entropy_slice_flag ) { |  |
| … |  |
| if( slice_type = = B && !identical_lists_flag ) |  |
| collocated_from_l0_flag | u(1) |
| … |  |
| } |  |
| … |  |
| } |  |

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).