| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0091-r1 |
| --- | --- |

| Title: | AHG13: Cross-verification report of JCTVC-I0117 on sample-based angular intra prediction for HEVC lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Keiichi Chono 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5439 chono@ct.jp.nec.com |
| Source: | NEC Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results of JCTVC-I0117 on sample-based angular intra prediction for HEVC lossless coding. The cross-verification task was successfully completed and it is confirmed that proponent's results are accurate.

- Simulation results

The software provided by TI was successfully compiled and run on Window-based computing platform. Table1 shows the simulation results, where Reference and Tested are HM-6.0 and HM-6.0 using TI's lossless coding, respectively. Table2 shows additional simulation results, where Tested is lossy coding based on HM-6.0 with/without TI's lossless coding tool. It was confirmed that all results are identical to TI's results.

Table1: HM-6.0 (lossless) versus TI's lossless coding

|  | All Intra HE |  |  | All Intra LC |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Comp. ratio |  | bit-saving % | Comp. ratio |  | bit-saving % |  |  |  |
|  | Reference | Tested |  | Reference | Tested |  |  |  |  |
| Class A | 2.21 | 2.46 | -10.26% | 2.20 | 2.46 | -10.48% |  |  |  |
| Class B | 2.09 | 2.18 | -4.52% | 2.08 | 2.18 | -4.69% |  |  |  |
| Class C | 1.97 | 2.11 | -6.49% | 1.97 | 2.11 | -6.76% |  |  |  |
| Class D | 1.83 | 2.01 | -8.41% | 1.82 | 2.01 | -8.67% |  |  |  |
| Class E | 2.90 | 3.23 | -10.30% | 2.89 | 3.23 | -10.47% |  |  |  |
| Class F | 4.51 | 5.21 | -12.23% | 4.48 | 5.20 | -12.55% |  |  |  |
| Overall | 2.58 | 2.86 | -8.30% | 2.57 | 2.85 | -8.53% |  |  |  |
| Enc Time[%] | 103% |  |  | 103% |  |  |  |  |  |
| Dec Time[%] | 93% |  |  | 94% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access 10 |  |  |
|  | Comp. ratio |  | bit-saving % | Comp. ratio |  | bit-saving % | Comp. ratio |  | bit-saving % |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class A | 2.72 | 2.80 | -2.99% | 2.72 | 2.80 | -3.07% | 2.03 | 2.08 | -2.69% |
| Class B | 2.38 | 2.41 | -0.94% | 2.38 | 2.41 | -0.99% | 2.38 | 2.41 | -0.94% |
| Class C | 2.55 | 2.59 | -1.69% | 2.55 | 2.59 | -1.76% |  |  |  |
| Class D | 2.67 | 2.73 | -2.07% | 2.66 | 2.72 | -2.13% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F | 28.38 | 30.90 | -7.11% | 28.24 | 30.82 | -7.28% |  |  |  |
| Overall | 7.99 | 8.55 | -2.85% | 7.95 | 8.54 | -2.93% | 2.48 | 2.52 | -0.02 |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 96% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |  |  |  |
|  | Comp. ratio |  | bit-saving % | Comp. ratio |  | bit-saving % |  |  |  |
|  | Reference | Tested |  | Reference | Tested |  |  |  |  |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 2.38 | 2.40 | -0.77% | 2.38 | 2.40 | -0.80% |  |  |  |
| Class C | 2.55 | 2.58 | -1.45% | 2.54 | 2.58 | -1.51% |  |  |  |
| Class D | 2.66 | 2.71 | -1.66% | 2.66 | 2.71 | -1.72% |  |  |  |
| Class E | 4.15 | 4.16 | -0.28% | 4.15 | 4.16 | -0.30% |  |  |  |
| Class F | 47.02 | 49.50 | -5.72% | 46.82 | 49.36 | -5.84% |  |  |  |
| Overall | 11.66 | 12.18 | -2.00% | 11.62 | 12.15 | -2.06% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |  |  |  |

Table2: HM-6.0 (lossless) versus HM-6.0 (lossy)

|  | All Intra HE |  |  | All Intra LC |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Comp. ratio |  | bit-saving % | Comp. ratio |  | bit-saving % |  |  |  |
|  | Reference | Tested |  | Reference | Tested |  |  |  |  |
| Class A | 2.21 | 2.06 | 7.20% | 2.20 | 2.07 | 6.56% |  |  |  |
| Class B | 2.09 | 1.87 | 11.79% | 2.08 | 1.87 | 11.40% |  |  |  |
| Class C | 1.97 | 1.74 | 13.15% | 1.97 | 1.74 | 12.80% |  |  |  |
| Class D | 1.83 | 1.66 | 10.64% | 1.82 | 1.66 | 10.35% |  |  |  |
| Class E | 2.90 | 2.55 | 13.54% | 2.89 | 2.58 | 12.05% |  |  |  |
| Class F | 4.51 | 3.78 | 22.91% | 4.48 | 3.76 | 23.09% |  |  |  |
| Overall | 2.58 | 2.27 | 13.67% | 2.57 | 2.27 | 13.24% |  |  |  |
| Enc Time[%] | 199% |  |  | 117% |  |  |  |  |  |
| Dec Time[%] | 108% |  |  | 110% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access 10 |  |  |
|  | Comp. ratio |  | bit-saving % | Comp. ratio |  | bit-saving % | Comp. ratio |  | bit-saving % |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class A | 2.72 | 2.39 | 13.78% | 2.72 | 2.40 | 13.12% | 2.03 | 2.37 | -15.20% |
| Class B | 2.38 | 2.05 | 16.24% | 2.38 | 2.03 | 17.38% | 2.38 | 2.10 | 13.75% |
| Class C | 2.55 | 2.16 | 17.99% | 2.55 | 2.16 | 17.97% |  |  |  |
| Class D | 2.67 | 2.26 | 17.92% | 2.66 | 2.29 | 16.74% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F | 28.38 | 23.13 | 18.88% | 28.24 | 23.61 | 16.83% |  |  |  |
| Overall | 7.99 | 6.59 | 17.26% | 7.95 | 6.69 | 16.81% | 2.48 | 2.20 | 0.01 |
| Enc Time[%] | 132% |  |  | 106% |  |  | 132% |  |  |
| Dec Time[%] | 98% |  |  | 111% |  |  | 96% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |  |  |  |
|  | Comp. ratio |  | bit-saving % | Comp. ratio |  | bit-saving % |  |  |  |
|  | Reference | Tested |  | Reference | Tested |  |  |  |  |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 2.38 | 2.04 | 16.82% | 2.38 | 2.02 | 18.19% |  |  |  |
| Class C | 2.55 | 2.15 | 18.56% | 2.54 | 2.14 | 18.49% |  |  |  |
| Class D | 2.66 | 2.24 | 18.92% | 2.66 | 2.27 | 17.32% |  |  |  |
| Class E | 4.15 | 3.19 | 30.00% | 4.15 | 3.26 | 27.15% |  |  |  |
| Class F | 47.02 | 41.36 | 15.22% | 46.82 | 43.38 | 12.14% |  |  |  |
| Overall | 11.66 | 10.14 | 19.24% | 11.62 | 10.55 | 18.21% |  |  |  |
| Enc Time[%] | 127% |  |  | 107% |  |  |  |  |  |
| Dec Time[%] | 116% |  |  | 96% |  |  |  |  |  |

# Patent rights declarations

NEC Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).