| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0094 |
| --- | --- |

| Title: | Improvement of the rate control based on pixel-based URQ model for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyomin Choi, Junghak Nam, Jonghun Yoo and Donggyu Sim Kwangwoon University 447-1, Wolgye-Dong, Nowon-Gu, Seoul, Korea Ivan V. Bajić School of Engineering Science Simon Fraser University 8888 University Drive Burnaby, BC, V5A 1S6 Canada | Tel: Email: | +82 (02) 940-5470 {hyomin06, qejixfyza, mmyjh, dgsim }@kw.ac.kr +1-778-782-7159 ibajic@ensc.sfu.ca |
| Source: | KWU (Kwangwoon University) SFU (Simon Fraser University) |  |  |

_____________________________

# Abstract

This contribution presents improvement of rate control algorithm based on the proposed rate control in JCTVC-H0213. This document includes two additional conditions for the rate control implemented on HM6.1. One is initial QP setting for beginning of a sequence, the other is boundary clipping for target bits in the frame level rate control. The initial QP is automatically calculated based on bit per pixel (bpp) with target bitrate. Therefore, the QP value in the configuration file is ignored. The boundary for target bits of a frame is used to conform to the hypothetical reference decoder (HRD) requirement, so the target bits are clipped by lower and upper bounds. In addition, the rate control in JCTVC-H0213 is modified according to experts’ comments in the last San Jose meeting, especially using Quantization Step (Q-Step) in rate-quantization model (RQ model) instead of using directly QP value. The refinement of the rate control is implemented on HM6.1 and evaluated on diverse conditions.

# Introduction

A rate control algorithm for HEVC, JCTVC-H0213, was introduced and adopted to implement in HM at the last San Jose meeting. However, a problem in the rate control is that the QP value for beginning of a sequence is affected by the configuration file setting. For practical rate control algorithm, QP value at the beginning of the sequence is automatically calculated by predefined condition because we do not have an idea how many bits are generated by the manually defined QP value for every time. Therefore, we propose the condition to calculate the initial QP value based on bpp with target bitrate. Furthermore, the introduced rate control doesn’t take care about boundary of frame bits. In real time processing, it may or may not cause under- or over-flow of a coded picture buffer (CPB). Therefore, we propose a simple equation that clips target bit for a frame with considering HRD conformance. In addition, we have reflected experts’ comments to the rate control that Q-Step should be used for RQ model instead of using directly QP value. The final refinement model is implemented on HM6.1 and evaluated on several conditions.

# Rate control scheme

The proposed rate control creates bitstream within the available channel bandwidth. The algorithm works with given information such as an available channel bandwidth, coding structure, frame rate, GOP size on the GOP, frame, and unit level. Specially, the proposed rate control employs a pixel-based unified rate quantization (URQ) model. It is fairly suitable for characteristics of HEVC which have diverse coding block size.

## GOP-level rate control

The GOP level rate control manages total bits in a GOP. Especially, the proposed rate control takes into account the bits of the first frame with initial buffer status over the rate control scheme. Over the rate control scheme, the occupancy for the first frame as an initial buffer status are controlled as follows

Ii(j)= Oij , i = 0, j = 1                          (1)

Oij = bij−Rijf                              (2)

where Ii(j) describes the status of the initial buffer at the j-th frame in the i-th GOP, Oi(j) represents a occupancy of j-th frame in the i-th GOP, Ri(j) is the available bandwidth and f indicates predefined frame rate, and bi(j) is the actual generated bits. Ai(j1) represents the adjustment bits that should be added or subtracted depending of the occupancy bits and initial buffer status. Once the initial occupancy bits for the first frame is calculated when the 1st frame in the 0th GOP. It should be updated after encoding every frame. The adjustment bits Ai(j1) reflected to the initial buffer is calculated by formula (3)

Ai(j−1) = c1×Oi(j−1) , if I1j−1 > 0 and Oij−1 < 0c1×Iij−1 , if I1j−1 < 0 and Oij−1 > 0                     (3)

where c1 can be a constant value less than 1.0. In this contribution we set the c1 equal to 0.60. After updating the Ii(j), each term that enters the calculation of the adjustment bits Ai(j1) should be updated according to Ii(j1) and Oi(j1). The calculated The Ai(j1) is always less than or equal to 0. The initial occupancy buffer keeps trying to reach to 0 until the end of encoding. The total bit budget Bi(j) in a GOP should be distributed according to the various parameters, and is calculated from the formula (3)

Bij=Rijf×NGOP−Vij, if j=1          (4)

where NGOP is the number of frames in a GOP and Vi(j) is the occupancy of the virtual buffer. At the beginning, the total bit budget is calculated from the equation (4) with j set to 1. The occupancy of the virtual buffer Vi(j) can be defined by formula (5) and (6). Therefore, at the beginning of the encoding of the first frame in the first GOP, the total budget is calculated by the available channel bandwidth over the frame rate and multiplied by the number of frames in a GOP. For other frames in the GOP, Bi(j) is accumulated using the actual generated bits bi(j1). In addition, in the VBR case where the available bandwidth Ri(j) is changing, Bi(j) can take that into account for the remaining frames in the GOP. After finishing the encoding process for a GOP, the total bit budget should be recalculated using the previous occupancy of the virtual buffer.

Vi1=0Vi−1NGOPi=1otherwise                           (5)

Vij=Vij−1+bij−1−Rij−1f ∓ Aij−1j=2,3,...,NGOP            (6)

where the Ai(j1) is calculated by formula (3).

## Frame-level rate control

For the frame level rate control, QP allocation is conducted before encoding. After encoding, all the statuses of buffers should be updated along with the URQ model and the linear model for the MAD prediction, both to be described later in this document. To calculate a proper QP value for the frame is a key issue of rate control. There are three cases that need to be considered when calculating QP: 1) the first frame of a GOP, 2) reference frame except for the first frame in a GOP, and 3) non-reference frame.

### Strategy for reference frames

- The first frame in a GOP

At the beginning of a sequence, the frame of the zeroth GOP, initial QP value is derived. For deriving the QP, bppinit should be calculated with input target bitrate by formula (7).

bppinit = R01f×Npixels                               (7)

where Npixels is number of pixels for a frame. The initial QP is set by the value of bppinit as follows,

QPi1=12 0.7780 <bppinit 17 0.3200 <bppinit ≤ 0.778022 0.1220 <bppinit ≤ 0.320027 0.0469 <bppinit ≤ 0.122032 0.0213 <bppinit ≤ 0.046937 0.0102 <bppinit ≤ 0.021342 0.0049 <bppinit ≤ 0.010247 0.0024 <bppinit ≤ 0.0049                     (8)

The first frame in subsequent GOPs employs QP value calculated by formula (9) and the value can be modified by formula (10)

QPi1=maxQPi−1NGOP−2,minQPi−1NGOP+2,k = jrefQPi−1kNref         (9)

where Nref indicates number of reference frames in a GOP, and the index k is a frame index among the reference frames. Equation (9) represents clipping of QPi(1) to [QPi1(NGOP)2, QPi1(NGOP)+2], so that it is within 2 of the QP of the last frame in the previous GOP, for smoothness. After computing QPi(1), this value is modified according to the condition that the absolute value of Vi(j) + Ii(j) be less than the average bits per frame as follows:

QPi1=QPi1 + 2 if Vij+Iij > 0                   (10)

- Other reference frames

The QP computation for other reference frames is derived from the proposed pixel-based URQ model. The URQ model needs a predicted frame complexity value, based on Mean Absolute Difference (MAD), and a target bit budget for a frame. The target bit budget is calculated by several elements such as occupancy of the virtual buffer, average weighting parameter of reference and non-reference frames, target buffer level, remaining total bit budget in a GOP, and the initial buffer status. Formula (11) and (12) show the way to get the target buffer level. The target buffer level is used to compute the target bit budget based on status of buffer occupancy. The target buffer level LBuf,i(2) is initialized for each GOP with the occupancy of virtual buffer Vi(2).

LBuf,i2=Vi2                                  (11)

After encoding a frame, the target buffer level is updated by formula (12). It reflects average weighting factors of reference frames and non-reference frames calculated by actual generated bits and QP to the target buffer level.

LBuf,ij+1=LBuf,ij−LBuf,i(2)Nref−1+Wi(j)−1×Ri(j)f              (12)

Wi(j)=Wref,i(j)×M+1Wref,i(j)+Wnon−ref,i(j)×M                     (13)

where Nref presents the number of reference frames in a GOP. Wref,i(j) and Wnon−ref,i(j) are the average weighting factors calculated by formulae (14) and (15), respectively. M denotes the number of non-reference frames between consecutive reference frames. The M value is equal to 1 when using a hierarchical reference structure. For the low-delay case, it is set to 0.

Wref,ij=bij×QPij8+7×Wref,ij−18                       (14)

Wnon−ref,ij=bi(j)×QPi(j)8+7×Wnon−ref,ij−18                   (15)

The final target bit budget is a weighted average for two target bit budgets: one is the target bit budget based on the total bit budget in a GOP, and the other is the target bit budget based on the status of buffer occupancy, target buffer level, and initial buffer level. The final target bit budget is computed by formula (16).

Ti(j)=β×Ti(j)+(1−β)×Ti(j)                        (16)

where β is set to 0.6 for the random access case and to 0.9 for the low delay case. The target bit budget Ti(j) based on the total bit budget is calculated by formula (17). It also involves average weighting factors. Because the target bit budget is calculated for reference frames, each portion for the reference and non-reference frames in the total bit budget should be appropriately weighted.

Tij=Wref,i(j−1)×Bi(j)Wref,i(j−1)×Nleftref+Wnonref,i(j−1)×Nleftnonref             (17)

where Nleft_ref and Nleft_non_ref represent numbers of not-yet-coded reference frames and not-yet-coded non-reference frames in a GOP, respectively. The other target bit budget, Tij, is calculated by formula (18) based on several buffer statuses.

Tij=Rijf+γ×Lij−Vij−Iijf                   (18)

where γ is set to 0.25 for the random access case and to 0.5 for the low delay case. The final target bits calculated by (16)-(18) is clipped by

Ti(j) = minUbound,ij,maxLbound,ij,Ti(j)                 (19)

Lbound,ij=Rijf−Vij+Ii(j) j=1                   (20)

Ubound,ij=Bij×ω j=1                (21)

where ω is 0.9. To compute QP value for a frame with the proposed pixel-based URQ model, target bit computed by (16)-(21) and frame complexity are needed. The frame complexity, represented here by the Mean Absolute Difference (MAD) is calculated by linear model using formula (22).

MADpred,ij = a1×MADactual,ij−1−M+a2                  (22)

where a1 is set to 1 and a2 is set to 0 at the initial stage. After coding a frame, those factors are updated using the actual MAD values. The pixel-based URQ model is shown in formula (23).

TijNpixels,ij=α×MADpred,ijQstepij+β×MADpred,ijQstepi2j                 (23)

where Npixels,i(j) is number of pixels in a frame, and Ti(j)/Npixels,i(j) is the average bit rate in bits per pixels (bpp) of the j-th frame in the i-th GOP. To compute Qstep value with equation (23), it needs a predicted MAD MADpred,i(j), number of pixels in a frame Npixels,i(j), and the target bit Ti(j). After computing (23), the QP value is adjusted for consecutive frames by smoothing using formula (24). However, if the target bit Ti(j) is less than 0, the QP value is set to the QP value of the previous frame plus 1.

QPij=QPij−M−1 + 1 if Tij < 0        (24)

### Strategy for non-reference frames

The proposed rate control derives QP value for non-reference frame without buffer status using formula (25).

QPij+1=QPij+QPij+2+22 if QPij≠QPij+2QPij+2Otherwise              (25)

All the derived QPs for either reference or non-reference frame are clipped by formula (26)

QPij=max0,min51, QPij                          (26)

## LCU-level rate control

In this contribution, we assume that a unit is equal to a LCU. In HEVC, the LCU might not be aligned with the frame boundary. Therefore, the right- and bottom parts of some LCUs may have different number of pixels compare to other LCUs. At the unit level, the number of pixels is an important factor in the proposed rate control. Multi-slice or CU level rate control could be derived using a similar pixel-based approach, but in this contribution, we simply apply the idea to the LCU unit level.

### Strategy for QP and Buffer

At the unit level, the proposed rate control scheme calculates the QP according the buffer status with the goal of consuming the bits allocated as target bits for the whole frame. The average bits available for each LCU should be calculated based on the number of pixels in that LCU. The available bits for each LCU are equal to the number of pixels in a LCU multiplied by bpp. For the bpp, the target bit budget for a frame is divided by the number of pixels in the frame using formula (27).

bpp=TijNpixels                                    (27)

At the unit level, the total bits budget should be controlled. Therefore, the total bit budget at the m-th unit of the j-th frame, Bjm, is calculated by formula (28). The initial total bit budget is equal to the target bit budget for the frame, and the budget is updated after encoding each unit:

Bjm=bpp×Npixels , j m=0                    (28)

where bj(m1) represents the actual generated bits of (m1)-th unit in the j-th frame. Nunits is the number of units in the frame. The occupancy of the virtual buffer in the frame also works with the actual generated bits for the m-th unit in the j-th frame by formula (29).

Vjm=0Vjm−1+bjm−1−bpp×Npixels,jm−1m=0otherwise            (29)

The target bit budget for the unit, Tj(m), is calculated by weighted averaging of two other target bit numbers by formula (30):

Tj(m)=β×Tj(m)+(1−β)×Tj(m)                        (30)

where β is set to 0.5, Tj(m) is the target bit number based on buffer status, calculated by formula (31),

Tjm=bpp×Npixels,jm−VjmNleftunits                      (32)

Npixels,j(m) is the number of pixels of m-th unit in a j-th frame, Nleft_units the number of remaining units in a frame, and Tj(m) is the target bit number based on the remaining bits, calculated by formula (33),

Tjm=BjmNleftpixels , j×Npixels , jm                        (33)

where Nleft_pixels,j represents the remaining number of pixels in the j-th frame. Collocated MAD value in the previous reference frame is used for the current unit MAD. The equation for pixel-based URQ model for a unit is the same as the URQ model for the frame, shown in equation (23), with Npixels,i(j) now representing the number of pixels in the unit. After encoding a unit, the factors of the URQ model are updated. As a initial factors for the URQ model, those values are shown in Table 1.

Table 1. URQ model factors according slice types

| Slice Type | Low QSTEP range (<= 33) |  | High QSTEP range (>= 33) |  |
| --- | --- | --- | --- | --- |
|  | α | β | α | β |
| B-Slice | 16.429 | -1.1494 | 4.9371 | 0.0922 |

### Lambda updating

The proposed rate control scheme using the URQ model gives one QP value for each unit. After that, conventional RDO mode decision can be applied to each unit. However, the lambda () factor that combines distortion and rate in the Lagrangian cost function is important in the RDO process. This value is affected by the QP value. Therefore, each unit has to calculate its own lambda value from the assigned QP value.

# Experiments

The proposed rate control is estimated by CBR and VBR condition. For the estimation, target bitrates are set to output bitrates by HM6.1 with JCT-VC common test conditions and several bits defined by “Joint Call for proposals on Video Compression Technology”, N11113. Defined target rate is shown in Table 2. For experiments, we only test rows of ‘Rate 4’ and ‘Rate 5’. Furthermore, we take into account how the rate control does work well for various LCU size for the experiments. The results are described in 3.2 and 3.3 section.

Table 2. Target rate defined by document N11113, which is Joint Call for proposals on Video compression technology

| Class | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 5 |
| --- | --- | --- | --- | --- | --- |
| A: 2560x1600p30 | 2.5 Mbit/s | 3.5 Mbit/s | 5 Mbit/s | 8 Mbit/s | 14 Mbit/s |
| B1: 1080p24 | 1 Mbit/s | 1.6 Mbit/s | 2.5 Mbit/s | 4 Mbit/s | 6 Mbit/s |
| B2: 1080p50-60 | 2 Mbit/s | 3 Mbit/s | 4.5 Mbit/s | 7 Mbit/s | 10 Mbit/s |
| C: WVGAp30-60 | 384 kbit/s | 512 kbit/s | 768 kbit/s | 1.2 Mbit/s | 2 Mbit/s |
| D: WQVGAp30-60 | 256 kbit/s | 384 kbit/s | 512 kbit/s | 850 kbit/s | 1.5 Mbit/s |
| E: 720p60 | 256 kbit/s | 384 kbit/s | 512 kbit/s | 850 kbit/s | 1.5 Mbit/s |

The experiments show how many bits apart from the target bitrate as well as for PSNR. In addition, we attach diverse plots which show an accumulated occupancy of virtual buffer with deviation for the occupancy, frame bits and PSNR on frame by frame. In this section, we attach average values for each common test conditions. Full results are shown in ‘JCTVC-I0094_experiments_HM6.1_VS_HM6.1_
INIT_RC_VS_HM6.1_Proposed_RC.xls’. Furthermore, additional test results using defined target rate by Table 2 are shown in ‘JCTVC-I0094_experiments_HM6.1_Proposed_RC_on_Targetrate.xls’. To show how many bits apart from the target bitrate, we defined Δkbps with formula (34)

ΔKbps='bitsRC'−'bitsHM6.1''bitsHM6.1'×100                         (34)

In addition, ΔPSNR is defined by absolute difference between the proposed rate control and HM6.1 with rate control as following formula (35). Minus value for ΔPSNR means PSNR of rate control is less than that of HM6.1 with rate control.

ΔPSNR='PSNRRC'−'PSNRHM5.0'                         (35)

## HM6.1 Rate control VS. HM6.1 Rate control with Proposal

In this section, we evaluate Δkbps and ΔPSNR for all sequences on JCT-VC common test conditions. Table 3 shows Δkbps and ΔPSNR of HM6.1 rate control described by JCTVC-H0213, compared with HM6.1. It shows all results for random access (RA), low delay (LB), and low delay P (LP) cases. For RA case, the rate control implemented on HM6.1 shows about 99.17% and 1.37dB degradation. Relatively less degradations are shown in both LB and LP, compared to RA case. The accuracy by Δkbps for LB and LP are about 99.78%.

Table 3. Δkbps and ΔPSNR of HM6.1 rate control compared with HM6.1 for RA, LB, and LP cases

| Main profile | Random access (RA) |  | Low delay (LB) |  | Low delay P (LP) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Δkbps | ΔPSNR | Δkbps | ΔPSNR | Δkbps | ΔPSNR |
| ClassA | 1.16% | -1.13 |  |  |  |  |
| ClassB | 0.96% | -1.33 | 0.15% | -0.98 | 0.20% | -0.89 |
| ClassC | 0.61% | -1.65 | 0.33% | -0.60 | 0.09% | -0.51 |
| ClassD | 0.57% | -1.36 | 0.11% | -0.46 | 0.11% | -0.45 |
| ClassE |  |  | 0.35% | -0.71 | 0.43% | -0.66 |
| Avg | 0.83% | -1.37 | 0.23% | -0.69 | 0.21% | -0.63 |

Table 4 shows Δkbps and ΔPSNR of HM6.1 rate control on the proposed condition. All the accuracy of each case are a little bit higher than HM6.1 rate control, but degradation for ΔPSNR of LB and LP get worse about 0.3 dB, excluding RA mode. However, the rate control on proposed condition shows better buffer status and fluctuation for frame bits than HM6.1 rate control without target bits clipping. We attach diverse plots which show the buffer status and fluctuation of frame bits and PSNR for each case and different LCU size. Even though the LCU sizes are different, we can see a similar fashion for all the curves thanks to the proposed URQ model.

Table 4. Δkbps and ΔPSNR of HM6.1 rate control on proposed condition compared with HM6.1 for RA, LB, and LP cases

| Main profile | Random access (RA) |  | Low delay (LB) |  | Low delay P (LP) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Δkbps | ΔPSNR | Δkbps | ΔPSNR | Δkbps | ΔPSNR |
| ClassA | 1.16% | -1.15 |  |  |  |  |
| ClassB | 0.58% | -0.74 | 0.09% | -1.09 | 0.12% | -0.99 |
| ClassC | 0.53% | -1.11 | 0.06% | -0.60 | 0.31% | -0.54 |
| ClassD | 0.68% | -1.02 | 0.10% | -0.50 | 0.10% | -0.48 |
| ClassE |  |  | 0.19% | -1.34 | 0.14% | -1.24 |
| Avg | 0.74% | -1.01 | 0.09% | -1.09 | 0.12% | -0.99 |

Figures in this section show three kinds of figures, which are accumulated occupancy of virtual buffer status, frame bits fluctuation, and PSNR fluctuation. In addition, each plot includes three different curves, which are by HM6.1 with red colors, HM6.1 rate control with green lines, and HM6.1 rate control on proposed condition with blue colors. For the accumulated occupancy of virtual buffer, best results are that the water-line is shown nearby zero around X-axis. The rate control on the proposed condition is fairly shown around X-axis, compared to two others. Frame fluctuation plot in second row shows the frame bits are clipped well by the proposed condition. It affects the coded picture buffer not to be over- or under flowing. PNSR fluctuation is also shown in third row. Relatively, the entire PSNR curve by proposed one is lowe0than the others. All attached figures shows similar fashion in three different each plot. Figure 1, 2, 3, and 4 show the plots in RA case, Fig. 5, 6, 7, and 8 are for LB case, and Fig 9, 10, 11, and 12 are for LP case.

Figure 1. Accumulated occupancy of virtual buffer, frame bits and PSNR fluctuation for NebutaFestival with QP 22 on HM6.1, HM6.1 rate control and HM6.1 rate control on the proposed condition, respectively.

Figure 2. Accumulated occupancy of virtual buffer, frame bits and PSNR fluctuation for Cactus with QP 27 on HM6.1, HM6.1 rate control and HM6.1 rate control on the proposed condition, respectively.

Figure 3. Accumulated occupancy of virtual buffer, frame bits and PSNR fluctuation for BasketballDrill with QP 32 on HM6.1, HM6.1 rate control and HM6.1 rate control on the proposed condition, respectively.

Figure 4. Accumulated occupancy of virtual buffer, frame bits and PSNR fluctuation for BlowingBubbles with QP 37 on HM6.1, HM6.1 rate control and HM6.1 rate control on the proposed condition, respectively.

Figure 5. Accumulated occupancy of virtual buffer, frame bits and PSNR fluctuation for BasketballDrive with QP 22 on HM6.1, HM6.1 rate control and HM6.1 rate control on the proposed condition, respectively.

Figure 6. Accumulated occupancy of virtual buffer, frame bits and PSNR fluctuation for BQMall with QP 27 on HM6.1, HM6.1 rate control and HM6.1 rate control on the proposed condition, respectively.

Figure 7. Accumulated occupancy of virtual buffer, frame bits and PSNR fluctuation for BQSquare with QP 32 on HM6.1, HM6.1 rate control and HM6.1 rate control on the proposed condition, respectively.

Figure 8. Accumulated occupancy of virtual buffer, frame bits and PSNR fluctuation for FourPeople with QP 37 on HM6.1, HM6.1 rate control and HM6.1 rate control on the proposed condition, respectively.

Figure 9. Accumulated occupancy of virtual buffer, frame bits and PSNR fluctuation for BQTerrace with QP 22 on HM6.1, HM6.1 rate control and HM6.1 rate control on the proposed condition, respectively.

Figure 10. Accumulated occupancy of virtual buffer, frame bits and PSNR fluctuation for PartyScene with QP 27 on HM6.1, HM6.1 rate control and HM6.1 rate control on the proposed condition, respectively.

Figure 11. Accumulated occupancy of virtual buffer, frame bits and PSNR fluctuation for RaceHorses with QP 32 on HM6.1, HM6.1 rate control and HM6.1 rate control on the proposed condition, respectively.

Figure 12. Accumulated occupancy of virtual buffer, frame bits and PSNR fluctuation for Johnny with QP 37 on HM6.1, HM6.1 rate control and HM6.1 rate control on the proposed condition, respectively.

## Diverse LCU sizes with HM6.1 Rate control with Proposal on CBR

One advantage of the proposed URQ model is that the model can be employed regardless of LCU size because the model is underlying the average bits per pixel. Therefore, we evaluate the URQ model truly works with diverse LCU size while keeping reasonable rate control performance. Table 5 to 7 shows that the average output bitrate and PSNR by HM6.1 rate control on the proposed condition with LCU 64, 32, 16. For evaluation, we exploited the bitrate defined by Table 2 as a target bitrate for each class. Table 5, Table 6, and Table 7 show for RA, LB, and LP case, respectively. Even though the deviations of output bitrate and PSNR among different LCU sizes exist for all case, but it is not big difference. Furthermore, the average output bitrate meets to the target bitrate with accuracy. We attach plots to compare accumulated occupancy status of virtual buffer, frame bits fluctuation, and PSNR fluctuation among different LCU sizes.

Table 5. Bitrate and PSNR of HM6.1 rate control on proposed condition with LCU 64, 32, and 16 for RA

| Main profile | Target bitrate (Kbit/s) | LCU 64 |  | LCU 32 |  | LCU 16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Bitrate | Y-PSNR | bitrate | Y-PSNR | bitrate | Y-PSNR |
| ClassA | 8000 | 7992.07 | 36.53 | 7955.72 | 36.29 | 7907.63 | 34.50 |
|  | 14000 | 13853.92 | 38.51 | 13798.24 | 38.29 | 13764.59 | 36.69 |
| ClassB1 (1080p 24) | 4000 | 3970.26 | 39.24 | 3965.06 | 39.15 | 3973.16 | 38.56 |
|  | 6000 | 5904.12 | 40.32 | 5950.16 | 40.36 | 6000.07 | 39.95 |
| ClassB2 (1080p50-60) | 7000 | 6979.86 | 36.31 | 6982.52 | 36.12 | 6968.27 | 34.10 |
|  | 10000 | 9980.05 | 37.04 | 9981.12 | 37.04 | 9973.57 | 36.89 |
| ClassC | 1200 | 1194.07 | 33.25 | 1193.15 | 33.02 | 1193.83 | 32.73 |
|  | 2000 | 1990.29 | 35.28 | 1989.33 | 35.23 | 1989.64 | 34.99 |
| ClassD | 850 | 845.73 | 35.48 | 845.11 | 35.35 | 844.17 | 35.97 |
|  | 1500 | 1492.78 | 38.37 | 1493.12 | 38.44 | 1489.39 | 38.97 |

Table 6. Bitrate and PSNR of HM6.1 rate control on proposed condition with LCU 64, 32, and 16 for LB

| Main profile | Target bitrate (Kbit/s) | LCU 64 |  | LCU 32 |  | LCU 16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | bitrate | Y-PSNR | bitrate | Y-PSNR | bitrate | Y-PSNR |
| ClassB1 (1080p 24) | 4000 | 4001.87 | 38.71 | 4005.34 | 38.42 | 4014.39 | 37.97 |
|  | 6000 | 6005.63 | 40.07 | 6009.60 | 39.84 | 6042.63 | 39.39 |
| ClassB2 (1080p50-60) | 7000 | 7006.86 | 35.81 | 7006.69 | 35.78 | 7011.94 | 35.15 |
|  | 10000 | 10005.19 | 36.77 | 10007.84 | 36.66 | 10007.60 | 36.22 |
| ClassC | 1200 | 1200.67 | 33.01 | 1200.65 | 33.04 | 1202.64 | 32.82 |
|  | 2000 | 2000.73 | 35.17 | 2001.27 | 35.28 | 2000.84 | 34.82 |
| ClassD | 850 | 850.48 | 35.27 | 850.43 | 35.28 | 850.34 | 35.09 |
|  | 1500 | 1500.22 | 38.07 | 1500.34 | 38.06 | 1500.72 | 37.95 |
| ClassE | 850 | 848.75 | 39.79 | 848.00 | 40.46 | 848.84 | 39.98 |
|  | 1500 | 1500.51 | 40.79 | 1499.65 | 41.64 | 1500.47 | 41.36 |

Table 7. Bitrate and PSNR of HM6.1 rate control on proposed condition with LCU 64, 32, and 16 for LC

| Main profile | Target bitrate (Kbit/s) | LCU 64 |  | LCU 32 |  | LCU 16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | bitrate | Y-PSNR | bitrate | Y-PSNR | bitrate | Y-PSNR |
| ClassB1 (1080p 24) | 4000 | 4008.19 | 38.52 | 4005.18 | 38.27 | 4013.39 | 37.75 |
|  | 6000 | 6007.21 | 39.81 | 6004.38 | 39.66 | 6033.54 | 39.14 |
| ClassB2 (1080p50-60) | 7000 | 7005.90 | 35.60 | 7002.69 | 35.51 | 7000.10 | 35.04 |
|  | 10000 | 10006.32 | 36.53 | 10008.08 | 36.41 | 10006.37 | 35.96 |
| ClassC | 1200 | 1200.94 | 32.89 | 1200.66 | 32.86 | 1200.89 | 32.63 |
|  | 2000 | 2000.95 | 34.94 | 2007.53 | 35.05 | 2001.00 | 34.58 |
| ClassD | 850 | 850.36 | 34.91 | 850.31 | 34.94 | 850.42 | 34.74 |
|  | 1500 | 1500.65 | 37.55 | 1500.45 | 37.59 | 1500.83 | 37.44 |
| ClassE | 850 | 849.71 | 39.45 | 849.62 | 40.22 | 850.46 | 39.75 |
|  | 1500 | 1500.78 | 40.63 | 1498.70 | 41.35 | 1501.02 | 41.02 |

Figure 13. Accumulated occupancy of virtual buffer, frame bits and PSNR fluctuation for Traffic with 8Mbps on HM6.1 rate control on the proposed condition with LCU 64, 32 and 16, respectively.

Figure 14. Accumulated occupancy of virtual buffer, frame bits and PSNR fluctuation for Traffic with 14Mbps on HM6.1 rate control on the proposed condition with LCU 64, 32 and 16, respectively.

Figure 15. Accumulated occupancy of virtual buffer, frame bits and PSNR fluctuation for Cactus with 7Mbps on HM6.1 rate control on the proposed condition with LCU 64, 32 and 16, respectively.

Figure 16. Accumulated occupancy of virtual buffer, frame bits and PSNR fluctuation for Cactus with 10Mbps on HM6.1 rate control on the proposed condition with LCU 64, 32 and 16, respectively.

Figure 17. Accumulated occupancy of virtual buffer, frame bits and PSNR fluctuation for BasketballDrill with 1.2Mbps on HM6.1 rate control on the proposed condition with LCU 64, 32 and 16, respectively.

Figure 18. Accumulated occupancy of virtual buffer, frame bits and PSNR fluctuation for BasketballDrill with 2Mbps on HM6.1 rate control on the proposed condition with LCU 64, 32 and 16, respectively.

Figure 19. Accumulated occupancy of virtual buffer, frame bits and PSNR fluctuation for BlowingBubbles with 850Kbps on HM6.1 rate control on the proposed condition with LCU 64, 32 and 16, respectively.

Figure 20. Accumulated occupancy of virtual buffer, frame bits and PSNR fluctuation for BlowingBubbles with 1.5Mbps on HM6.1 rate control on the proposed condition with LCU 64, 32 and 16, respectively.

Figure 21. Accumulated occupancy of virtual buffer, frame bits and PSNR fluctuation for KristenAndSara with 850Kbps on HM6.1 rate control on the proposed condition with LCU 64, 32 and 16, respectively.

Figure 22. Accumulated occupancy of virtual buffer, frame bits and PSNR fluctuation for KristenAndSara with 1.5Mbps on HM6.1 rate control on the proposed condition with LCU 64, 32 and 16, respectively.

## Diverse LCU sizes with HM6.1 Rate control with Proposal on VBR

The proposed rate control also is implemented with consider VBR scenario. Therefore, we try to evaluate how it works well for VBR case. Furthermore, we test the VBR scenario with diverse LCU size. Figure 23 and Fig. 24 shows frame bit fluctuation, each plots include curves by LCU 64, 32, and 16 with red, green, and blue, respectively.

Figure 23. Variable bit rate condition from 850Kbps to 1.5Mbps for BlowingBubbles with diverse LCU sizes

Figure 24. Variable bit rate condition from 850Kbps to 1.5Mbps for BQSquare with diverse LCU sizes

# Reference

- H. Choi, J. Nam, J. Yoo, D. Sim, and I. V. Bajić, “Rate control based on unified RQ model for HEVC,” JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCT-VC H0213 (m23088), San José, CA, USA, Feb. 2012

- ITU-T Q6/16 Visual Coding and ISO/IEC JTC1/SC29/WG11 Coding of Moving Pictures and Audio, “Joint call for proposals on video compression technology,” ISO/IEC JTC1/SC29/WG11 N11113, Kyoto, Jan. 2010

- F. Bossen, “Common test conditions and software reference configurations,” JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCT-VC H1100 (m24011), San José, CA, USA, Feb. 2012

- K. Suehring, D. Flynn, and F. Bossen, “HEVC software guidelines,” JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCT-VC H1001 (m24003), San José, CA, USA, Feb. 2012

- K.-P. Lim, G. Sullivan, and T. Wiegand, “Text description of joint model reference encoding methods and decoding concealment methods,” JVT of ISO/IEC MPEG and ITU-T VCEG, JVT-N046, Hongkong, CN, Jan. 2005

# Patent rights declaration(s)

KWU and SFU do not have any current or pending patent rights relating to the technology described in this contribution.