| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 Apr–7 May, 2012 | Document: JCTVC-I0098_r1 WG11 Number: m24337 |
| --- | --- |

| Title: | CE2.c.1-1: Test of maximum number of BA filters of 10 (JCTVC-I0250) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage Takayuki Itoh Takashi Watanabe | Email: | tomoo.yamakage@toshiba.co.jp takayuki2.itoh@toshiba.co.jp takashi39.watanabe@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

Test results of CE2.c.1-1(Reduction of the number of filter classes to 10 in BA mode by a non-normative means (i.e., encoder restriction), JCTVC-I0250) are reported. The current specification allows up to 16 classes. Based on the CE2 HE10 test condition that refers to the common test condition, the simulation was conducted. An average of 0.1% loss was observed by restricting to up to 10 filter classes.

# Introduction

This contribution is a report of test results of reduction of the maximum number of filter classes to 10 in BA mode by non-normative means (i.e., encoder restriction). The current specification allows up to 16 classes. Reduction of the maximum number of filter classes will save the buffer size of ALF coefficients. If BA mode is adopted, switching of filter coefficient set may occur every 4x4 block. Therefore, it would be preferable to reduce the maximum number of filters.

Based on the CE2 HE10 test condition that refers to the common test condition, the simulation was conducted. BD-rate losses for AI/RA/LDB/LDP are 0.0%/0.1%/0.1%/0.3% for luma, respectively.

# Experimental Results

Table 1  Experimental results for the maximum number of filter classes of 10

| Test case | HE10 |  |  |  |
| --- | --- | --- | --- | --- |
|  | IO | RA | LDB | LDP |
| Class A | 0.0% | 0.1% |  |  |
| Class B | 0.0% | 0.1% | 0.2% | 0.4% |
| Class C | 0.0% | 0.0% | 0.1% | 0.2% |
| Class D | 0.0% | 0.0% | 0.0% | 0.1% |
| Class E | 0.0% |  | 0.1% | 0.5% |
| Overall | 0.0% | 0.1% | 0.1% | 0.3% |
| Class F | 0.1% | 0.1% | 0.2% | 0.3% |
| Enc time | 99% | 101% | 98% | 97% |
| Dec time | 100% | 100% | 96% | 96% |

There is no mismatch between encoder and decoder.

# Conclusion

This contribution is a report of test results of reduction of the maximum number of filter classes to 10 in BA mode by non-normative means (i.e., encoder restriction). Based on the CE2 HE10 test condition that refers to the common test condition, the simulation was conducted. An average of 0.1% loss was observed by restricting to up to 10 filter classes.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).