| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0101 |
| --- | --- |

| Title: | Simplification on default quantization matrix signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sung-Chang Lim Hui Yong Kim Jinho Lee Jin Soo Choi 218 Gajeong-ro, Yuseong-gu, Daejeon, 305-700, Korea | Tel: Email: | +82-42-860-5772 {sclim, hykim5, jinosoul, jschoi} @etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

# Abstract

This contribution presents a simplification on default quantization matrix signaling. In HEVC CD, the usage of default matrix is signaled by setting the first quantization matrix coefficient, which is equal to 0. In order to simplify the parsing process for quantization matrix coefficient, the proposed method signals the usage of default matrix by coding scaling_list_pred_matrix_id_delta with the value equal to 0.

# Introduction

In order to control the subjective quality of video sequence, quantization matrices are supported in HEVC. HEVC quantization matrix is based on AVC-style signaling and representation of quantization matrix and has the following newly introduced features [1].

- Downsampled matrix signaling for 16x16 and 32x32 quantization matrices

- Downsampled default matrices for 16x16 and 32x32 quantization matrices

- Matrix copy from a previous matrix with same sizeID.

- Separate coding of DC matrix coefficient for 16x16 and 32x32 quantization matrices

In HEVC, a value of quantization matrix coefficient must be in the range of 1 to 255. So, it is possible to signal the usage of default matrix with an unused quantization matrix coefficient, which is equal to 0. Specifically, in order to signal the usage of default matrix, the first matrix coefficient is set equal to 0 and signaled using the syntax element scaling_list_delta_coef or DC matrix coefficient is set equal to 0 and signaled using the syntax element scaling_list_dc_coef_minus8 in Table 1, respectively. For example, when 4x4 quantization matrix needs to be set to the default matrix, the first scaling_list_delta_coef is set equal to -8 and signaled. And, for another example, when 16x16 quantization matrix needs to be set to the default matrix, scaling_list_dc_coef_minus8 is set equal to -8 and signaled.

Table 1. Scaling list syntax in HEVC CD

| scaling_list( scalingList, sizeID , matrixID ) { | Descriptor |
| --- | --- |
| nextCoef = 8 | u(1) |
| coefNum = Min( 64, ( 1 << ( 4 + ( sizeID << 1) ) ) ) |  |
| UseDefaultScalingMatrix = 0 |  |
| if( sizeID > 1 ) { |  |
| scaling_list_dc_coef_minus8[ sizeID − 2 ][ matrixID ] | se(v) |
| if( scaling_list_dc_coef_minus8[ sizeID − 2 ][ matrixID ] = = −8 ) |  |
| UseDefaultScalingMatrixFlag = 1 |  |
| } |  |
| if( UseDefaultScalingMatrixFlag = = 0 ) { |  |
| stopNow = 0 |  |
| for( i=0; i < coefNum && !stopNow; i++) { |  |
| scaling_list_delta_coef | se(v) |
| nextCoef = ( nextCoef + scaling_list_delta_coef + 256 ) % 256 |  |
| if( sizeID < 2 ) { |  |
| useDefaultScalingMatrixFlag = (i = = 0 && nextCoef = = 0 ) |  |
| if( useDefaultScalingMatrixFlag ) |  |
| stopNow = 1 |  |
| } |  |
| if( !stopNow ) |  |
| scalingList[ i ] = nextCoef |  |
| } |  |
| } |  |
| } |  |

In order to copy coefficients of the current matrix from coefficients of the reference matrix, scaling_list_pred_matrix_id_delta in Table 2 is used to indicate the reference matrix ID.

Table 2. Scaling list data syntax in HEVC CD

| scaling_list_param( ) { | Descriptor |
| --- | --- |
| scaling_list_present_flag | u(1) |
| if( scaling_list_present_flag ) |  |
| for( sizeID = 0; sizeID < 4; sizeID++ ) |  |
| for( matrixID = 0; matrixID < (sizeID = = 3) ? 2 : 6; matrixID++ ) { |  |
| scaling_list_pred_mode_flag | u(1) |
| if( !scaling_list_pred_mode_flag ) |  |
| scaling_list_pred_matrix_id_delta | ue(v) |
| else |  |
| scaling_list( ScalingList[ sizeID ][ matrixID ], sizeID , matrixID ) |  |
| } |  |
| } |  |

Note that scaling_list_pred_matrix_id_delta should have a positive value and is coded using unsigned exponential-Golomb code, ue(v) and the reference matrix ID is calculated by scaling_list_pred_matrix_id_delta and the equation in Table 3. It can be noted that RefMatrixID can never be identical with MatrixID.

Table 3. Semantics of scaling_list_pred_matrix_id_delta in HEVC CD

scaling_list_pred_matrix_id_delta specifies the target reference matrix to copy the value of scaling list. When scaling_list_pred_mode_flag is equal to 0, scaling_list_pred_matrix_id_delta specifies which matrix should be used in the current matrix by the following: RefMatrixID = MatrixID – ( 1 + scaling_list_pred_matrix_id_delta ) (7-1) where MatrixID is specified in Error: Reference source not found.

# Proposal

In a way similar to the signaling of default matrix using the unused quantization matrix coefficient in HEVC CD, it is possible to signal the usage of default matrix with an unused reference matrix ID, which is equal to the current matrix ID, because the reference matrix ID cannot be identical with the current matrix ID. That is, in order to signal the usage of default matrix, scaling_list_pred_matrix_id_delta is coded with the value equal to 0. Table 4 shows the proposed semantics in order to signal the usage of default matrix using scaling_list_pred_matrix_id_delta.

Table 4. Proposed semantics of scaling_list_pred_matrix_id_delta

scaling_list_pred_matrix_id_delta specifies the target reference matrix to copy the value of scaling list. When scaling_list_pred_mode_flag is equal to 0, scaling_list_pred_matrix_id_delta specifies which matrix should be used in the current matrix by the following: RefMatrixID = MatrixID – scaling_list_pred_matrix_id_delta (7-2) where MatrixID is specified in Error: Reference source not found. The variable RefMatrixID equal to MatrixID specifies that scaling list for MatrixID is set equal to the default scaling list as specified from Error: Reference source not found to Error: Reference source not found.

By using the proposed method, there is no need to signal the usage of default matrix with an unused quantization matrix. As can be seen from Table 5, the parsing process for quantization matrix coefficient can be simplified and the number of operations needed to identify the usage of default matrix can be reduced as compared with HEVC CD.

Table 5. Proposed scaling list syntax

| scaling_list( scalingList, sizeID , matrixID ) { | Descriptor |
| --- | --- |
| nextCoef = 8 | u(1) |
| coefNum = Min( 64, ( 1 << ( 4 + ( sizeID << 1) ) ) ) |  |
| if( sizeID > 1 ) |  |
| scaling_list_dc_coef_minus8[ sizeID − 2 ][ matrixID ] | se(v) |
| for( i=0; i < coefNum; i++) { |  |
| scaling_list_delta_coef | se(v) |
| nextCoef = ( nextCoef + scaling_list_delta_coef + 256 ) % 256 |  |
| scalingList[ i ] = nextCoef |  |
| } |  |
| } |  |

# Experimental result

The proposed method was implemented on HM6.0 software. The experiments are conducted on the platform of Intel Core i7 CPU 990X @ 3.46GHz with 24GB RAM. Table 6 shows the summary result of the proposed method compared to HM6.0 anchor which is generated with using the modified quantization matrix file "scaling_list_symmetry1_rd.txt", contained in the zip file, under common test conditions [2]. The modified quantization matrix file is based on the quantization matrix file "scaling_list_symmetry1.txt" in CE4 description at the previous meeting [3]. The reason of modification is that HM6.0 encoder only takes 64 quantization matrix coefficients per matrix and DC matrix coefficient as separate value from AC matrix coefficients. Also, the quantization matrix files in CE4 description at the previous meeting does not use any matrix as default matrix. Thus, they are modified to set 16x16 and 32x32 luma quantization matrices equal to default matrices in order to verify the proposed method.

Table 6. Summary result of the proposed method using modified "scaling_list_symmetry1.txt"

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

The proposed method does not cause any noticeable change in terms of BD-rates because HM6.0 signals the quantization matrix one time in each bitstream. So, the comparison of actual coded bits for HM6.0 and the proposed method is tabulated in Table 7. Three modified quantization matrix files "scaling_list_symmetry1_rd.txt", "scaling_list_symmetry7_rd.txt", and "scaling_list_asymmetry7_rd.txt", used in experiments are contained in the zip file. An average coded bits for quantization matrices of -0.86% is increase by the proposed method over three different kinds of quantization matrices. That is, actually, the proposed method reduces the number of coded bits for quantization matrices as compared to HM6.0. The reason is that, in HM6.0, the usage of default matrix is signaled by coding either scaling_list_delta_coef equal to -8 or scaling_list_dc_coef_minus8 equal to -8 with se(v). In the proposed method, the usage of default matrix is signaled by coding scaling_list_pred_matrix_id_delta equal to 0 with ue(v). Thus, 8 bits per default matrix is saved by using the proposed method. However, when scaling_list_pred_matrix_id_delta is used to indicate the reference matrix ID in matrix copy mode, at most 2 bits per matrix copy mode is increased by the proposed method as compared to HM6.0.

Table 7. Comparison of the numbers of coded bits for quantization matrices

| Quantization matrix file | Number of coded bits for quantization matrices |  | Bits increase |
| --- | --- | --- | --- |
|  | HM6.0 | Proposed method |  |
| Modified " scaling_list_symmetry1.txt" | 1469 bits | 1457 bits | -0.82% |
| Modified "scaling_list_symmetry7.txt" | 2107 bits | 2087 bits | -0.95% |
| Modified "scaling_list_asymmetry7.txt" | 2459 bits | 2439 bits | -0.81% |

The comparison of the numbers of coded bits for quantization matrices in the best case and the worst case are tabulated in Table 8. Note that, by the proposed method, the more default matrix is used, the less bits are needed to code the quantization matrices, and the more matrix copy mode is used, the more bits are needed to code the quantization matrices. However, when all of the quantization matrices uses the default matrix, scaling_list_present_flag is coded with the value equal to 0 and only 1 bit is needed. Thus, this is not the best case. If the only one matrix is a non-default and the other matrices are the default matrices, the maximum coded bits decrease is achieved by the proposed method, i.e. the best case. In the best case scenario, 152 bits can be saved. On the other hand, all the matrices use the matrix copy mode except the matrices with matrixID equal to 0, the maximum coded bits increase is achieved by the proposed method, i.e. the worst case. In the worst case scenario, 32 bits can be wasted.

Table 8. The comparison of the numbers of coded bits for quantization matrices in the best case and the worst case

| The best case |  |  |  | The worst case |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number of non-default matrices | Number of default matrices (size) | Number of matrix copy modes | Total coded bits decrease | Number of non-default matrices | Number of default matrices | Number of matrix copy modes | Total coded bits increase |
| 1 | 5 (4x4) + 6 (8x8) + 6 (16x16) + 2 (32x32) = 19 | 0 | 152 bits (= 19 * 8 bits) | 4 | 0 | 5 (4x4) + 5 (8x8) + 5 (16x16) + 1 (32x32) = 16 | 32 bits (= 16 * 2 bits) |

# Conclusion

In this contribution, simplification on default quantization matrix signaling is proposed. In order to simplify the parsing process for quantization matrix coefficient, the proposed method signals the usage of default matrix by coding scaling_list_pred_matrix_id_delta with the value equal to 0. It is recommended to adopt the proposed method into HM.

# Reference

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, and T. Wiegand, "High Efficiency Video Coding (HEVC) text specification draft 6," JCTVC-H1003, Feb. 2012.

- F. Bossen, "Common test conditions,", JCTVC-H1100, Feb. 2012.

- K. Sato, R. Joshi, S. Liu, M. Zhou, "Core Experiment 4 (CE4): Quantization matrices," JCTVC-H1104, Feb. 2012.

# Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex: Proposed text changes based on JCTVC-H1003_dK

- 7.3.2.4	Scaling list syntax

| scaling_list( scalingList, sizeID , matrixID ) { | Descriptor |
| --- | --- |
| nextCoef = 8 | u(1) |
| coefNum = Min( 64, ( 1 << ( 4 + ( sizeID << 1) ) ) ) |  |
| UseDefaultScalingMatrix = 0 |  |
| if( sizeID > 1 ) { |  |
| scaling_list_dc_coef_minus8[ sizeID − 2 ][ matrixID ] | se(v) |
| if( scaling_list_dc_coef_minus8[ sizeID − 2 ][ matrixID ] = = −8 ) |  |
| UseDefaultScalingMatrixFlag = 1 |  |
| } |  |
| if( UseDefaultScalingMatrixFlag = = 0 ) { |  |
| stopNow = 0 |  |
| for( i=0; i < coefNum && !stopNow; i++) { |  |
| scaling_list_delta_coef | se(v) |
| nextCoef = ( nextCoef + scaling_list_delta_coef + 256 ) % 256 |  |
| if( sizeID < 2 ) { |  |
| useDefaultScalingMatrixFlag = (i = = 0 && nextCoef = = 0 ) |  |
| if( useDefaultScalingMatrixFlag ) |  |
| stopNow = 1 |  |
| } |  |
| if( !stopNow ) |  |
| scalingList[ i ] = nextCoef |  |
| } |  |
| } |  |
| } |  |

- 7.4.2.3	Scaling list data semantics

scaling_list_pred_matrix_id_delta specifies the target reference matrix to copy the value of scaling list. When scaling_list_pred_mode_flag is equal to 0, scaling_list_pred_matrix_id_delta specifies which matrix should be used in the current matrix by the following:

RefMatrixID = MatrixID – ( 1 +  scaling_list_pred_matrix_id_delta )	(7-3)

where MatrixID is specified in Error: Reference source not found.

The variable RefMatrixID equal to MatrixID specifies that scaling list for MatrixID is set equal to the default scaling list as specified from Error: Reference source not found to Error: Reference source not found.

- 7.4.2.4	Scaling list semantics

scaling_list_delta_coef specifies the difference of the matrix coefficient from the previous matrix coefficient, when pred_mode is equal to 1.

When useDefaultScalingMatrixFlag is derived to be equal to 1, the scaling list is inferred from  the default scaling list as specified from Error: Reference source not found to Error: Reference source not found.