| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0103 |
| --- | --- |

| Title: | Non-CE3: On scanIdxC derivation for chroma LM mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hui Yong Kim, *Kyung Yong Kim, *Gwang Hoon Park, Sung-Chang Lim, Jinho Lee, and Jin Soo Choi ETRI, 218 Gajeong-ro, Yuseong-gu, Daejeon, 305-700, Korea *Media Lab., Kyung Hee University 1, Seochun, Kiheung, Youngin, Gyonggi, 446-701, Korea | Tel: Email: | +82-42-860-5069 {hykim5, sclim, jinosoul, jschoi}@etri.re.kr *+82-31-201-3680 *{kimky, ghpark} @khu.ac.kr |
| Source: | ETRI and Kyung Hee University |  |  |

_____________________________

# Abstract

In this contribution a scan type derivation method for LM mode chroma residuals is proposed. While the current HM uses only the diagonal scan for LM mode chroma residuals, the proposed method determine the scan type adaptively to the luma intra prediction mode. It is reported that average BD-rate reduction of 0.1% for each chroma component is consistently observed over all the classes in the All-Intra HE10 configuration.

# Introduction

In the current HM, mode-dependent coefficient scanning is used for intra coded luma and chroma residuals, as specified in the sub-clause 7.3.9 and 7.4.9 of the WD6_dK text [1]. Figure 1 summarizes the scan type derivation process of the current HM. Note in the current HM, the scan type for LM mode chroma residuals is fixed to the diagonal scan.

Figure 1. Mode dependent scan type derivation of HM 6

# Proposal

In this proposal, we proposed to use luma intra prediction mode for deriving the scan type of the LM mode chroma residuals. Since LM mode prediction uses reconstructed luma sample values, it is more logical to use luma intra prediction mode for the residual scan type. Figure 2 shows the proposed scan type derivation process, where the modified part is highlighted with yellow color. Note that the modification is minimal and does not increase the worst case complexity of the parsing process.

Figure 2. Mode dependent scan type derivation of the proposed method

The followings are the required text modification for the proposed method. Note that the suggested modifications are highlighted in yellow color.

In 7.3.9 Transform unit syntax:

transform_unit( x0, y0, xC, yC, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) {

......

if( PredMode = = MODE_INTRA ) {

scanIdx = ScanType[ log2TrafoSize − 2 ][ IntraPredMode ]

scanIdxC = ( IntraPredModeC = = Intra_FromLuma ) ? scanIdx : ScanType[ log2TrafoSize − 2 ][ IntraPredModeC ]

......

}

In 7.4.9 Transform unit semantics:

Table 7-1 – Specification of ScanType[ log2TrafoSize − 2 ][ IintraPredMode ]

| IintraPredMode | log2TrafoSize − 2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 0 | 0 | 0 | 0 | 0 |
| ... | ... | ... | ... | ... |
| 31 – 354 | 0 | 0 | 0 | 0 |

# Experimental results

The proposed method was implemented on HM6.0 software. Table 1 shows the summary results of the proposed method against HM 6.0 anchor under common test conditions [2]. Note that slight but consistent chroma gain is observed in the table.

Table 1. Summary result of the Proposal

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% |
| Class B | 0.0% | -0.1% | -0.2% |
| Class C | -0.0% | -0.2% | -0.2% |
| Class D | 0.0% | -0.2% | -0.1% |
| Class E | 0.0% | -0.1% | 0.0% |
| Overall | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -0.1% | -0.1% |
| Class F | -0.0% | -0.3% | -0.2% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusions

This contribution proposed the luma intra prediction mode dependent scanIdxC derivation for the LM chroma mode. Experimental results showed average BD-rate reduction of 0.1% for each chroma component. We recommend adopting the proposed solution for the next version of HM.

# Reference

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand (Editors), “High Efficiency Video Coding (HEVC) text specification draft 6,” JCTVC-H1003, Feb. 2012.

- F. Bossen, "Common test conditions and software reference configurations," JCTVC-H1100, Feb. 2012.

# Patent rights declaration(s)

ETRI and Kyung Hee University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: Required SW changes

The required software change is just one line of code, as shown in the following code-capture.