| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0106 |
| --- | --- |

| Title: | AHG7: Level definition to limit memory bandwidth of MC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo and Teruhiko Suzuki Sony City Osaki, 2-10-1 Osaki Shinagawa-ku, Tokyo Japan | Tel: Email: | +81-50-3750-2744 kenji.kondo@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to limit maximum memory bandwidth of motion compensation (MC). The two simple solutions are introduced. One of the solutions is to restrict small PU block, another solution is to limit number of motion vectors in a LCU. This method is applied in H.264/AVC standard [1]. For small block restriction, the impact of coding efficiency is shown.

# Introduction

The memory bandwidth especially maximum of MC process is big issue for implementation. It was mentioned in [2]. We showed that small block and bi-prediction consume memory bandwidth in the [2]. Furthermore, in [3] we discussed what desirable restriction is. We mentioned that the memory bandwidth should be limited in relevant small region (e.g. CU and/or LCU) in the [3].

# Proposed methods

To limit memory bandwidth, we propose to restrict a small PU restriction or to limit the number of motion vectors in a LCU. Both methods can limit maximum memory bandwidth in small region (CU or LCU).

## The removal of “inter_4x4_enabled_flag”

In [5], inter_4x4_enabled_flag is defined. The semantics is “specifies whether inter prediction can be applied to blocks having the size of 4 by 4 luma samples”. The common test condition sets disabling 4x4. Unfortunately, the restriction is not enough for large image application such as 4k2k. So, more restriction is proposed in this contribution. If there is more PU restriction, inter_4x4_enabled_flag is not necessary. So, we propose to remove the inter_4x4_enabled_flag.

## PU restriction

We show the small block restriction case in Table 1. In this table, R0 – R4 are the restriction case. The R4 means that bi-pred 4x4, 4x8, 8x4, 8x8 and uni-pred 4x4, 4x8 and 8x4 are restricted. The R0 does not restrict anything. In right column, the worst memory bandwidth are shown compared with anchor condition (4x4 block is restricted). The percentage of the bandwidth for R1 is 0% because the next worst block type is bi-pred 4x8 than uni-pred 4x4.

Table 1 PU restriction case

|  | Prohibition mode (X mark means the mode is not to be used.) |  |  |  |  | Worst B/W, Worst mode |
| --- | --- | --- | --- | --- | --- | --- |
|  | Bi-4x4 | Bi-4x8, Bi-8x4 | Uni-4x4 | Bi-8x8 | Uni-4x8, Uni-8x4 |  |
| Anchor | X |  | X |  |  | 0%, Bi-4x8 |
| R0 |  |  |  |  |  | +52%, Bi-4x4 |
| R1 | X |  |  |  |  | 0%, Bi-4x8 |
| R2 | X | X | X |  |  | -44%, Bi-8x8 |
| R3 | X | X | X | X |  | -50%, Uni-4x8 |
| R4 | X | X | X | X | X | -58%, Bi-8x16 |

## Number of motion vectors in a LCU

The one of solutions to reduce the bandwidth is to limit number of motion vectors. In H.264/AVC standard, the max number of motion vectors per two consecutive macro blocks (MaxMvsPer2Mb) is defined. By this level definition, it was prohibited that small block such as 4x4 block of bi-prediction is not always continually used. Actually, this limitation is meaningful for implementation. For hardware, engineers can design the memory bus including throughput and latency to satisfy maximum memory bandwidth.

In H.264/AVC standard, the MaxMvsPer2Mb is 32 and 16 for restriction B3 and over 3.1. If we extend the level definition to HEVC, it may be shown in Table 2. Where the restriction B1 of this table is corresponding to restriction B3 of H.264/AVC and the restriction B2 is corresponding to level 3.1 and above of H.264/AVC. The size of MB is 16x16 and two MBs can have 32 motion vectors for restriction B3 of H.264/AVC, so 16 MVs can be calculated per one MB.

We think that the strong restriction R3 or R4 is needed for 4k2k 60P application.

Table 2 the max number of motion vectors per a LCU

| LCU size | Restriction B1 | Restriction B2 | Restriction B3 |
| --- | --- | --- | --- |
| 16x16 | 16 | 8 | 4 |
| 32x32 | 64 | 32 | 16 |
| 64x64 | 256 | 128 | 64 |

## A relation between PU restriction and number of motion vectors

Above both methods has relation because to restrict small block size is to limit number of motion vectors easily. For example, when the R1 (small block restriction) is guaranteed, also the B1 (number of MVs) is guaranteed. Thus, the PU restriction includes limiting number of MVs as shown in Table 3. The limitation of number of MVs has flexibility than PU restriction because it can have a chance to use smaller PU. But the encoder complexity will increase. In many AVC encoders, we believe that the PU restriction is used to simply guarantee the level (number of MVs).

Table 3 a relation between PU restriction and number of MVs

| PU restriction | Number of MVs |
| --- | --- |
| R1 | B1 |
| R2 | B2 |
| R3 | B2 |
| R4 | B3 |

# Experiment

## Test conditions

The anchor encoding followed a common test condition [4].

## Restriction setting

Since a reason in section 2.1, the option “Disableinter4x4” of HM6.0 software set 0. Other test condition is following a common test condition [4]. The HM6.0 software is modified as restrict as Table 1.

# Test results

We show the test results for each restriction case in Table 4, Table 5, Table 6, Table 7 and Table 8 for R0, R1, R2, R3 and R4.

## Restriction R0

Table 4 the test results for R0

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class C | -0.1% | -0.3% | -0.3% | -0.1% | -0.3% | -0.4% |
| Class D | -0.3% | -0.4% | -0.5% | -0.2% | -0.3% | -0.5% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | -0.2% |
|  | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | -0.2% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 114% |  |  | 112% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.3% | -0.2% | 0.0% | -0.2% | -0.2% |
| Class C | -0.2% | -0.3% | -0.3% | -0.2% | -0.1% | -0.4% |
| Class D | -0.3% | -1.0% | -0.1% | -0.3% | -1.1% | -1.0% |
| Class E | 0.1% | -0.4% | -0.1% | 0.0% | -0.6% | 0.1% |
| Overall | -0.1% | -0.5% | -0.2% | -0.1% | -0.5% | -0.4% |
|  | -0.1% | -0.5% | -0.2% | -0.1% | -0.4% | -0.4% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 116% |  |  | 114% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |

## Restriction R1

Table 5 the test results for R1

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% | -0.1% |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class C | -0.1% | -0.2% | -0.2% | 0.0% | -0.2% | -0.2% |
| Class D | -0.1% | -0.4% | -0.3% | -0.1% | -0.2% | -0.3% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | -0.2% |
|  | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | -0.2% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 109% |  |  | 107% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.5% | -0.3% | 0.0% | 0.0% | 0.2% |
| Class C | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% | -0.3% |
| Class D | 0.0% | -0.1% | 0.3% | -0.1% | -0.5% | -0.7% |
| Class E | 0.0% | 0.0% | 0.4% | 0.1% | 0.3% | -0.3% |
| Overall | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | -0.2% |
|  | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% | -0.2% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 109% |  |  | 108% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

## Restriction R2

Table 6 the test results for R2

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.3% | 0.3% | 0.3% | 0.2% | 0.3% | 0.2% |
| Class B | 0.3% | 0.2% | 0.3% | 0.2% | 0.1% | 0.1% |
| Class C | 0.6% | 0.6% | 0.5% | 0.4% | 0.5% | 0.5% |
| Class D | 0.8% | 0.8% | 0.8% | 0.8% | 0.7% | 0.5% |
| Class E |  |  |  |  |  |  |
| Overall | 0.5% | 0.5% | 0.5% | 0.4% | 0.4% | 0.3% |
|  | 0.5% | 0.5% | 0.5% | 0.4% | 0.4% | 0.3% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 92% |  |  | 94% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.4% | 0.1% | 0.4% | 0.3% | 0.3% | 0.0% |
| Class C | 0.9% | 0.8% | 1.0% | 0.7% | 0.7% | 0.8% |
| Class D | 1.2% | 1.0% | 1.3% | 1.0% | 0.6% | 0.3% |
| Class E | 0.7% | 0.4% | 0.6% | 0.3% | 0.4% | 0.4% |
| Overall | 0.8% | 0.6% | 0.8% | 0.6% | 0.5% | 0.4% |
|  | 0.8% | 0.6% | 0.8% | 0.6% | 0.5% | 0.3% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 91% |  |  | 92% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

## Restriction R3

Table 7 the test results for R3

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.9% | 0.8% | 0.9% | 0.6% | 0.6% | 0.6% |
| Class B | 1.3% | 0.7% | 0.8% | 1.0% | 0.6% | 0.6% |
| Class C | 1.7% | 1.9% | 2.1% | 1.4% | 1.6% | 1.7% |
| Class D | 2.2% | 2.0% | 2.3% | 1.8% | 1.8% | 1.9% |
| Class E |  |  |  |  |  |  |
| Overall | 1.5% | 1.3% | 1.5% | 1.2% | 1.1% | 1.2% |
|  | 1.5% | 1.3% | 1.5% | 1.2% | 1.1% | 1.2% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 89% |  |  | 91% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1.6% | 0.2% | 0.5% | 1.3% | 0.3% | 0.5% |
| Class C | 2.4% | 2.3% | 2.5% | 2.0% | 2.1% | 2.1% |
| Class D | 2.7% | 1.7% | 2.6% | 2.4% | 1.5% | 2.1% |
| Class E | 1.6% | 1.1% | 1.8% | 0.9% | 0.5% | 0.6% |
| Overall | 2.1% | 1.3% | 1.8% | 1.7% | 1.1% | 1.3% |
|  | 2.1% | 1.3% | 1.8% | 1.6% | 1.1% | 1.3% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 86% |  |  | 89% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |

## Restriction R4

Table 8 the test results for R4

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 1.3% | 1.9% | 1.7% | 0.9% | 1.4% | 1.1% |
| Class B | 1.7% | 1.3% | 1.4% | 1.3% | 1.0% | 1.0% |
| Class C | 3.1% | 3.6% | 4.1% | 2.2% | 2.6% | 2.9% |
| Class D | 4.4% | 4.8% | 5.3% | 3.4% | 3.7% | 3.8% |
| Class E |  |  |  |  |  |  |
| Overall | 2.6% | 2.8% | 3.0% | 1.9% | 2.1% | 2.1% |
|  | 2.6% | 2.8% | 3.0% | 1.9% | 2.1% | 2.1% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 75% |  |  | 80% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 2.2% | 0.7% | 1.0% | 1.6% | 0.7% | 0.8% |
| Class C | 4.3% | 4.8% | 4.9% | 3.2% | 3.7% | 4.0% |
| Class D | 6.0% | 6.5% | 6.8% | 4.5% | 4.4% | 4.8% |
| Class E | 2.2% | 2.1% | 2.0% | 1.1% | 0.3% | 1.1% |
| Overall | 3.7% | 3.4% | 3.6% | 2.7% | 2.3% | 2.7% |
|  | 3.7% | 3.5% | 3.7% | 2.6% | 2.3% | 2.6% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 73% |  |  | 77% |  |  |
| Dec Time[%] | 97% |  |  | 99% |  |  |

# Conclusion

This contribution proposed two methods that is a restriction of small PU and limiting number of motion vectors in a LCU to reduce worst memory bandwidth. For PU restriction, four restriction cases were shown. And the impact of coding efficiency was reported.

# References

[1] ITU-T Recommendation H.264 and ISO/IEC 14496-10 (MPEG-4 part 10) Advanced Video Coding (AVC)

[2] K. Kondo, “Non-CE3: Report on a restriction for small block”, JCTVC-G392, JCT-VC 7th meeting, Geneva, 2011

[3] K. Kondo, “AHG7: Level definition to limit memory bandwidth of MC”, JCTVC-H0600, JCT-VC 8th meeting, San Jose, 2011

[4] F. Bossen, “Common test conditions and software reference configurations”, JCTVC-H1100, JCT-VC 8th meeting, Geneva 2011

[5] B. Bross, et al., “High efficiency video coding (HEVC) text specification draft 6”, JCTVC-H1003, JCT-VC 7th meeting, Geneva, 2011

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).