| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0107_r1 |
| --- | --- |

| Title: | AHG7: Modification of merge candidate derivation to reduce MC memory bandwidth |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo, Teruhiko Suzuki (Sony)*, Tomoyuki Yamamoto, Tomohiro Ikai (Sharp)** * Sony City Osaki, 2-10-1 Osaki Shinagawa-ku, Tokyo Japan **1-9-2 Nakase, Mihama-ku, Chiba Japan | Tel: Email: | *+81-50-3750-2744 **+81-43-299-8526 kenji.kondo@jp.sony.com teruhikos@jp.sony.com yamamoto.tomoyuki@sharp.co.jp |
| Source: | Sony and Sharp |  |  |

_____________________________

# Abstract

This contribution proposes to replace the bi-prediction of merge candidates to uni-prediction when block size is small (e.g. 4x4, 4x8, 8x4 and 8x8). This technique aims to avoid the coding efficiency loss by restricted bi-prediction for small block. To restrict bi-prediction and small block is simple way to limit maximum memory bandwidth. When a bi-prediction for small block is restricted by level, the encoder cannot choose merge candidates for bi-prediction. It makes it difficult to use merge and skip mode, and introduces coding efficiency loss. This proposal makes possible to use merge and skip mode by replacing the prediction direction from bi-pred to L0 uni-pred. For restriction (4x4, bi-pred 4x8, 8x4 and 8x8) case, without proposed method the BD% is 1.5, 1.2, 2.1 and 1.7% for RA-Main, RA-HE10, LB-Main and LB-HE10. With proposed method, the BD% is 1.0, 0.8, 1.1 and 0.9%. It was observed that the coding efficiency can be recovered by this proposal.

# Introduction

The memory bandwidth of MC could often be bottleneck for throughput of hardware implementation. We believe that many implementations will prohibit a bi-prediction when block size is small to reduce memory bandwidth. However it makes it difficult to use merge mode and skip because merge candidates are often bi-prediction. To avoid the problem we propose a modification of marge candidate derivation.

In HM6.0, the inter_4x4_enabled_flag is sent. This flag can restrict smallest inter PU (e.g. 4x4) and it can limit maximum memory bandwidth. However, this limitation is not enough for large image application such as 4k2k and/or high frame rate. This will be depend on profile and level as same as above. It will change which block size should be restricted.

# Proposed method

In San Jose meeting, this technique was proposed in [1, 2]. The basic technique is to replace the bi-prediction of merge candidates to uni-prediction. The [1] and [2] had slight difference about how to choose a set of small block, but it is combined in this contribution.

This contribution proposes also to send a signal disable_bip_small_mrg_idc in sps to choose the size set of small block. With this signal, the encoder implementation can have flexibility.

## Modification of merge candidate derivation

When merge candidates are bi-prediction for small block, the bi-prediction is replaced to L0 prediction. Where the size set of small block is defined with disable_bip_small_mrg_idc as below

- disable_bip_small_mrg_idc:

  - 0: no modification

  - 1: 4x4

  - 2: 4x4, 8x4, and 4x8

  - 3: 4x4, 8x4, 4x8, and 8x8

The disable_bip_small_mrg_idc is sent in sps.

# Experiment

## Test conditions

The anchor encoding followed a common test condition [3].

## Overhead check

This proposal has negligible overhead due to send an information “ disable_bip_small_mrg_idc” in sps. To confirm, the impact is tested with the disable_bip_small_idc is equal to 0 on common test condition.

## Restriction setting

To see the benefit of this technique, we compared on and off on the same restriction. Where the restriction cases (R0 – R4) set as follow

|  | Prohibition mode (X mark means the mode is not to be used.) |  |  |  |  | Worst B/W, Worst mode |
| --- | --- | --- | --- | --- | --- | --- |
|  | Bi-4x4 | Bi-4x8, Bi-8x4 | Uni-4x4 | Bi-8x8 | Uni-4x8, Uni-8x4 |  |
| Anchor | X |  | X |  |  | 0%, Bi-4x8 |
| R0 |  |  |  |  |  | +52%, Bi-4x4 |
| R1 | X |  |  |  |  | 0%, Bi-4x8 |
| R2 | X | X | X |  |  | -44%, Bi-8x8 |
| R3 | X | X | X | X |  | -50%, Uni-4x8 |

## Test cases

| Restriction | Without proposal | With proposal |
| --- | --- | --- |
| R0 | Case-A | Case-E disable_bip_small_mrg_idc=0 |
| R1 | Case-B | Case-F disable_bip_small_mrg_idc=1 |
| R2 | Case-C | Case-G disable_bip_small_mrg_idc=2 |
| R3 | Case-D | Case-H disable_bip_small_mrg_idc=3 |

# Test results

## Overhead check

The overhead for additional syntax was checked as following table. As we can see, the impact is negligible.

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

## Restriction R0

### Case A

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class C | -0.1% | -0.3% | -0.3% | -0.1% | -0.3% | -0.4% |
| Class D | -0.3% | -0.4% | -0.5% | -0.2% | -0.3% | -0.5% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | -0.2% |
|  | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | -0.2% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 114% |  |  | 112% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.3% | -0.2% | 0.0% | -0.2% | -0.2% |
| Class C | -0.2% | -0.3% | -0.3% | -0.2% | -0.1% | -0.4% |
| Class D | -0.3% | -1.0% | -0.1% | -0.3% | -1.1% | -1.0% |
| Class E | 0.1% | -0.4% | -0.1% | 0.0% | -0.6% | 0.1% |
| Overall | -0.1% | -0.5% | -0.2% | -0.1% | -0.5% | -0.4% |
|  | -0.1% | -0.5% | -0.2% | -0.1% | -0.4% | -0.4% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 116% |  |  | 114% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |

### Case E

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class C | -0.1% | -0.3% | -0.3% | -0.1% | -0.3% | -0.4% |
| Class D | -0.3% | -0.4% | -0.5% | -0.2% | -0.3% | -0.5% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | -0.2% |
|  | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | -0.2% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 114% |  |  | 112% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.3% | -0.2% | 0.0% | -0.2% | -0.2% |
| Class C | -0.2% | -0.3% | -0.3% | -0.2% | -0.1% | -0.4% |
| Class D | -0.3% | -1.0% | -0.1% | -0.3% | -1.1% | -1.0% |
| Class E | 0.1% | -0.4% | -0.1% | 0.0% | -0.6% | 0.1% |
| Overall | -0.1% | -0.5% | -0.2% | -0.1% | -0.5% | -0.4% |
|  | -0.1% | -0.5% | -0.2% | -0.1% | -0.4% | -0.4% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 116% |  |  | 114% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

## Restriction R1

### Case-B

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% | -0.1% |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class C | -0.1% | -0.2% | -0.2% | 0.0% | -0.2% | -0.2% |
| Class D | -0.1% | -0.4% | -0.3% | -0.1% | -0.2% | -0.3% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | -0.2% |
|  | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | -0.2% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 109% |  |  | 107% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.5% | -0.3% | 0.0% | 0.0% | 0.2% |
| Class C | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% | -0.3% |
| Class D | 0.0% | -0.1% | 0.3% | -0.1% | -0.5% | -0.7% |
| Class E | 0.0% | 0.0% | 0.4% | 0.1% | 0.3% | -0.3% |
| Overall | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | -0.2% |
|  | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% | -0.2% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 109% |  |  | 108% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

### Case-F

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class C | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | -0.3% |
| Class D | -0.2% | -0.3% | -0.4% | -0.1% | -0.3% | -0.3% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.2% |
|  | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.2% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 110% |  |  | 108% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.3% | -0.2% | -0.1% | -0.1% | 0.0% |
| Class C | -0.1% | -0.1% | -0.3% | -0.1% | -0.2% | -0.1% |
| Class D | -0.2% | -0.2% | -0.1% | -0.2% | -1.1% | -0.8% |
| Class E | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% | 0.3% |
| Overall | 0.0% | -0.2% | -0.2% | -0.1% | -0.4% | -0.2% |
|  | 0.0% | -0.1% | -0.1% | -0.1% | -0.3% | -0.3% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 109% |  |  | 108% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |

## Restriction R2

### Case-C

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.3% | 0.3% | 0.3% | 0.2% | 0.3% | 0.2% |
| Class B | 0.3% | 0.2% | 0.3% | 0.2% | 0.1% | 0.1% |
| Class C | 0.6% | 0.6% | 0.5% | 0.4% | 0.5% | 0.5% |
| Class D | 0.8% | 0.8% | 0.8% | 0.8% | 0.7% | 0.5% |
| Class E |  |  |  |  |  |  |
| Overall | 0.5% | 0.5% | 0.5% | 0.4% | 0.4% | 0.3% |
|  | 0.5% | 0.5% | 0.5% | 0.4% | 0.4% | 0.3% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 92% |  |  | 94% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.4% | 0.1% | 0.4% | 0.3% | 0.3% | 0.0% |
| Class C | 0.9% | 0.8% | 1.0% | 0.7% | 0.7% | 0.8% |
| Class D | 1.2% | 1.0% | 1.3% | 1.0% | 0.6% | 0.3% |
| Class E | 0.7% | 0.4% | 0.6% | 0.3% | 0.4% | 0.4% |
| Overall | 0.8% | 0.6% | 0.8% | 0.6% | 0.5% | 0.4% |
|  | 0.8% | 0.6% | 0.8% | 0.6% | 0.5% | 0.3% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 91% |  |  | 92% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

### Case-G

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.3% | 0.0% | 0.1% | 0.0% |
| Class B | 0.2% | 0.2% | 0.2% | 0.2% | 0.1% | 0.2% |
| Class C | 0.4% | 0.4% | 0.4% | 0.3% | 0.4% | 0.3% |
| Class D | 0.7% | 0.6% | 0.6% | 0.6% | 0.5% | 0.5% |
| Class E |  |  |  |  |  |  |
| Overall | 0.4% | 0.3% | 0.4% | 0.3% | 0.3% | 0.2% |
|  | 0.4% | 0.3% | 0.4% | 0.3% | 0.3% | 0.3% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 93% |  |  | 94% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | -0.3% | -0.1% | 0.2% | 0.1% | 0.1% |
| Class C | 0.5% | 0.6% | 0.4% | 0.4% | 0.4% | 0.3% |
| Class D | 0.7% | 0.6% | 0.8% | 0.7% | 0.0% | 0.2% |
| Class E | 0.2% | 0.1% | 0.8% | 0.1% | -0.1% | -0.3% |
| Overall | 0.4% | 0.2% | 0.4% | 0.3% | 0.1% | 0.1% |
|  | 0.4% | 0.2% | 0.5% | 0.3% | 0.2% | 0.1% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 91% |  |  | 93% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

## Restriction R3

### Case-D

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.9% | 0.8% | 0.9% | 0.6% | 0.6% | 0.6% |
| Class B | 1.3% | 0.7% | 0.8% | 1.0% | 0.6% | 0.6% |
| Class C | 1.7% | 1.9% | 2.1% | 1.4% | 1.6% | 1.7% |
| Class D | 2.2% | 2.0% | 2.3% | 1.8% | 1.8% | 1.9% |
| Class E |  |  |  |  |  |  |
| Overall | 1.5% | 1.3% | 1.5% | 1.2% | 1.1% | 1.2% |
|  | 1.5% | 1.3% | 1.5% | 1.2% | 1.1% | 1.2% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 89% |  |  | 91% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1.6% | 0.2% | 0.5% | 1.3% | 0.3% | 0.5% |
| Class C | 2.4% | 2.3% | 2.5% | 2.0% | 2.1% | 2.1% |
| Class D | 2.7% | 1.7% | 2.6% | 2.4% | 1.5% | 2.1% |
| Class E | 1.6% | 1.1% | 1.8% | 0.9% | 0.5% | 0.6% |
| Overall | 2.1% | 1.3% | 1.8% | 1.7% | 1.1% | 1.3% |
|  | 2.1% | 1.3% | 1.8% | 1.6% | 1.1% | 1.3% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 86% |  |  | 89% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |

### Case-H

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.5% | 0.5% | 0.6% | 0.2% | 0.6% | 0.4% |
| Class B | 0.9% | 0.5% | 0.6% | 0.7% | 0.3% | 0.4% |
| Class C | 1.2% | 1.3% | 1.4% | 1.0% | 1.0% | 1.2% |
| Class D | 1.7% | 1.5% | 1.6% | 1.5% | 1.4% | 1.5% |
| Class E |  |  |  |  |  |  |
| Overall | 1.0% | 0.9% | 1.0% | 0.8% | 0.8% | 0.8% |
|  | 1.0% | 0.9% | 1.1% | 0.8% | 0.8% | 0.8% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 91% |  |  | 93% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.8% | -0.3% | 0.0% | 0.6% | -0.1% | 0.0% |
| Class C | 1.3% | 1.1% | 0.9% | 1.1% | 1.1% | 1.2% |
| Class D | 1.6% | 0.9% | 0.8% | 1.5% | 0.4% | 1.2% |
| Class E | 0.5% | 0.2% | 0.9% | 0.2% | -0.1% | 0.1% |
| Overall | 1.1% | 0.4% | 0.6% | 0.9% | 0.3% | 0.6% |
|  | 1.1% | 0.4% | 0.6% | 0.9% | 0.4% | 0.6% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 88% |  |  | 90% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |

# Working draft text

This text modification for working draft is based on section 7.3 of JCTVC-H1003. For detail, we attached the document file.

#### 7.3.2.1	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| ... |  |
| if( log2_min_coding_block_size_minus3 = = 0 ) |  |
| inter_4x4_enabled_flag | u(1) |
| disable_bip_small_mrg_idc | u(2) |
| num_short_term_ref_pic_sets | ue(v) |
| ... |  |
| } |  |

#### 7.4.2.1	Sequence parameter set RBSP semantics

disable_bip_small_mrg_idc indicates sizes of prediction blocks where bi-prediction is restricted. The relation between the value of disable_bip_small_mrg_idc and sizes of prediction blocks where bi-prediction is restricted is shown in Table 7-x.

Table 7-x – Specification of disable_bip_small_mrg_idc

| - disable_bip_small_mrg_idc | - Sizes of prediction blocks where bi-prediction is restricted |
| --- | --- |
| - 0 | - - |
| - 1 | - 4x4 |
| - 2 | - 4x4, 8x4, 4x8 |
| - 3 | - 4x4, 8x4, 4x8, 8x8 |

# Conclusion

To reduce MC memory bandwidth, this contribution showed the technique that replace bi-prediction of merge candidate with uni-prediction when the block size is small. By send a signal disable_bip_small_mrg_idc, an encoder can choose the set of small block size among (4x4), (4x4, 4x8, 8x4) or (4x4, 4x8, 8x4, and 8 x 8). In experiment, the benefit of this proposal was checked based on several restriction cases. For restriction (4x4, bi-pred 4x8, 8x4 and 8x8) case, without proposed method the BD% is 1.5, 1.2, 2.1 and 1.7% for RA-Main, RA-HE10 LB-Main and LB-HE10. With proposed method, the BD% is 1.0, 0.8, 1.1 and 0.9%. It was observed that the coding efficiency can be recovered by this proposal.

# References

[1] T. Ikai, “AHG7: Controllable memory bandwidth reduction with bi-pred to uni-pred conversion”, JCTVC-H0096, JCT-VC 8th meeting, Geneva 2011

[2] K. Kondo, “AHG7: Modification of merge candidate derivation to reduce MC memory bandwidth”, JCTVC-H0221, JCT-VC 8th meeting, Geneva 2011

[3] F. Bossen, “Common test conditions and software reference configurations”, JCTVC-H1100, JCT-VC 8th meeting, Geneva 2011

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).