| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0108 |
| --- | --- |

| Title: | High Bit Depth Considerations in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Pierre Andrivon Philippe Bordes | Tel: Email: | +33-2-99-27-38-07 pierre.andrivon@technicolor.com philippe.bordes@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution relates a shallow analysis of WD6 (HEVC Working Draft 6) and HM6.0 (HEVC Test Model v6.0) with respect to high bit depth as well as different bit depth coding experiment results. These latter deal with comparisons of coding efficiency between JM18.3 (H.264/AVC Joint Model v18.3) and  HM6.0 for different bit depth coding; namely 8, 10, 12, 14 bits. Proposed test material is six class B (1080p) videos retrieved from SVT sequences and adapted to 8, 10, 12, 14 bits format. This document states that, for the proposed test material, HM6.0 outperforms JM18.3 HP (High Profile) in objective measure (BD-rate) of around 18% luma, 29%, Cb, 22% Cr for AI-HE (All Intra High Efficiency) 8, 10, 12 bits configuration and around 33% luma, 50% Cb, 35% Cr for RA-HE (Random Access High Efficiency), 8, 10, 12 bits configuration. AI-HE 14 bits experiment reportedly shows performances regression from 18% to 13% luma in favour of HM6.0. It is reported that RA-HE 14 bits experiment could not exploit results due to JM18.3 issues. Finally, this contribution proposes to continue high bit depth HEVC support preparatory work through an Ad Hoc Group.

# Introduction

According to Table A-5 of [1] High Efficiency Video Coding (HEVC) is deemed appropriate to code video sequences from SQCIF (128x96) up to 7680x4320. As of 8th JCT-VC San José meeting, discussions start about HEVC interest within DVB. During the 33rd CM DVB-AVC that was held on the 11th and 12th of April in Geneva, HEVC discussions started for Commercial Requirements. Through DVB, MPEG-2 and H.264/AVC are widely deployed as video broadcast standards for television [2], respectively for Digital Television and HDTV thus Ultra High Definition Television (UHDTV) may be a direct application of coming HEVC standard.

In parallel, ITU is drafting UHDTV parameters values for a new recommendation ([3], Table 1 from [4]) that reportedly shows 10 bits and 12 bits precision for 4k and 8k pixel raster.

| Parameter | Value |  |
| --- | --- | --- |
| Aspect Ratio | 16x9 |  |
| Pixel Raster | 3840x2160 (4k), 7680x4320 (8k) |  |
| Pixel aspect ratio | 1:1 (square pixels) |  |
| Scan | Progressive |  |
| Frame Rate | 120, 60, 60/1.001, 50, 30, 30/1.001, 25, 24, 24/1.001 |  |
| Precision | 10 bits | 12 bits |
| Code for Reference Black | 64 | 256 |
| Code for Reference White | 940 | 3760 |
| Value for Reference Black (cd/m2) | unspecified |  |
| Value for Reference White (cd/m2) | unspecified |  |
| Non-linear coding | Linear+Gamma 2.2 (same as Rec.709) |  |

Table 1, Parameters in draft UHDTV signal format, courtesy from [4]

Besides, [5] reports that the D-Cinema distribution master used the contrast sensitivity based on Barten’s equation to determine the bit depth to be used: 12 bits [6]. Indeed, in described conditions, 12-bit encoding modulation is reported to stand below the visual modulation threshold for the entire luminance range and to not show contouring. Thus, according to [5], 12-bit would suit to 7680x4320 resolution for Super Hi-Vision system. We might speak of the beginning of a convergence between D-Cinema new format (future DCI DCSS 2.0) and future UHDTV.

Some equipments all along the production and distribution chains are already available to provide high quality content. Concerning professional acquisition, Sony F65 (8k), Red One (4k), Arri Alexa (2k)... high definition cameras catch up to 14 f-stops. Besides, Panasonic broadcast cameras already supports AVC-Intra video coding that supports 10 bits intra coding. Production and post-production must keep high accuracy during the whole processing chain. H.264/AVC [12] defines profiles (annex A) for contribution networks in order to reduce exchanged file size and bandwidth like the High 4:4:4 Predictive Profile (Hi444PP) able to encode content up to 14 bits. Finally, from the home consumer viewpoint, flat panels now supports 10-bit bit depth and can be fed by HDMI 1.4a as well as DisplayPort that both support at least 12-bit per component. We may assume that future high quality contribution supports like Blu-Ray successor will also profit of new renderer capabilities.

# Short analysis

## WD6

Considering [1], much of the more than 8-bit bit depth specificities taken into account are inherited from H.264/AVC (sometimes with adaptations or extensions) as follows:

- Clipping functions

- High level syntax (bit_depth_luma_minus8, bit_depth_chroma_minus8 up to 14 bits)

- Weighted Predictions (up to 14 bits)

- Some intra sample prediction process (8.4.3.1.1)

- Motion Compensation process (up to 14 bits)

- Quantization Parameters derivation and binarization process

Some particularities can be noticed:

- High level syntax of PCM and PCM considerations:

pcm_sample_bit_depth_luma_minus1, pcm_sample_bit_depth_chroma_minus1 from  [13]

- Sample Adaptive Offset (new HEVC tool) considers bit depth

- Unlike H.264/AVC, deblocking does not seem to take into account bit depth

- Intra_FromLuma prediction mode (new HEVC tool) considers bit depth

- Motion Compensation process is thought so that each data yielded at intermediary stage holds in 16-bit memory ([14]). It is reported that samples up to 14 bits are considered as an input of this process.

The here above descriptions are informative and not exhaustive.

## HM6.0

Basically, it is claimed that no major discrepancy was found between software and above considerations. As core functionalities of HM6.0 basically seem to support up to 14 bits processing, it is proposed, in the following section, to run HM6.0 for 8, 10, 12 and 14 bits and compare results with H.264/AVC reference software (JM18.3) in order to identify potential prohibitive bottlenecks and limitations.

# Experiments

## Setup

Four experiments have been conducted in order to test the maturity of HM6.0 with respect to high bit depth. New test material is used in order to assess 8, 10, 12 and 14-bit encoding consistency of HM6.0. Original content was retrieved from public SVT multi format test set [7]. Interest of this content is that it deals with 16-bit full range format [8] and complexity of scenes is somehow different (Cf. chapter 5.9 in [8]). We retrieved six sequences that come from a 3840x2160p50 scan. These 16-bit linear sequences are converted to ‘sgi16’ (interleaved RGB), 16-bit full range format. These later are downsampled to 1080p (Class B) thanks to a Lanczos filter before being converted in Y’CbCr (compliant with [9] and [10]). The 1080p Y’CbCr 4:4:4 16-bit contents are then chroma downsampled with a Lanczos-3 filter to obtain finally six 1080p Y’CbCr 4:2:0 16-bit sequences that are truncated to 8, 10, 12, 14-bit in order to be used as inputs of each experiment (Table 2). Transformed sequences can be provided on demand.

| Sequence name | Coding difficulty | Snapshot |
| --- | --- | --- |
| CrowdRun | Difficult |  |
| ParkJoy | Difficult |  |
| DucksTakeOff | Difficult |  |
| InToTree | Easy |  |
| OldTownCross | Easy |  |
| Seeking | Medium |  |

Table 2, High Bit Depth test material used

250 first frames (5s) of each sequence are used for experiments

JM18.3 High Profile with same bit depth (8, 10, 12, 14) as HM6.0 is used as reference.

For HM6.0, two configuration files are modified: AI-HE10 and RA-HE10 (provided with HM6.0). Actually InternalBitDepth and InputBitDepth are set to 8, 10, 12, 14 respectively for 8-bit, 10-bit, 12-bit, 14-bit experiment.

For JM18.3, “HM-like” equivalent configuration files as used in [11] are employed with modifications as follows:
- In “defines.h” IMGTYPE is set to 1 (unsigned short).
- SourceBitDepthLuma, SourceBitDepthChroma, OutputBitDepthLuma, OutputBitDepthChroma are set to 8, 10, 12, 14 respectively for 8-bit, 10-bit, 12-bit, 14-bit experiment.
- In AI-HE configuration file, CRA is set to 0 and ReferenceReorder is set to 1.

The source codes have been run on a cluster grid with the following characteristics:

- OS: Linux (RedHat 5.5)

- CPU: 2 x Intel(R) Xeon(R) E5680 @ 3.33GHz, 6 cores.

- RAM: 96 GB per 6 cores.

## Results

### PSNR calculation remark

First of all, it is stated that objective measure (PSNR) is computed differently in JM18.3 and HM6.0 for bit depth over 8 bits. Actually the difference stands in the maximum value used in PSNR calculation.

JM18.3 takes into account the full bit depth range and computes the maximum value as follows:
max_value = (1 << bitdepth) - 1

HM6.0 computes the maximum value as follows:
max_value = 255 x (1<<(bitdepth-8)

Indeed, following [15] contribution on IBDI, it was agreed that the maximum value corresponds in principle to extended bit depth video. However, this is not exactly the case for native high bit depth content that may obey to [9] and [10] constraints or not (full range signal). For consistency’s sake and further comparisons purpose, it is suggested that JM and HM should harmonize the way PSNR is computed. It is proposed to align PSNR computation with the maximum value the HEVC current WD6 admits as an input (full range).

### Experiments results

Experiments results are presented in following tables. It is proposed to compare 8-bit results with [11], in order to check their consistency. It is reported in [11] that class B AI-HE results (-23,2% for Y, -20.6% for U and -19.3% for V) are quite equivalent to Table 3 results (-18.3% for Y, -29,6% for U and -23.3% for V) however one can notice that, HM version and test material are different and there is a variance in results due to each sequence coding difficulty. It is also reported in [11] that class B RA-HE results (-39,7% for Y, -37.5% for U and -36.7% for V) are quite above Table 4, for luma (-33.2% for Y, -51.1% for U and -40.1% for V). However the variance in RA-HE results is important, luma result is -19.6% for Parkjoy sequence and -50.0% for OldTownCross. Indeed this last sequence is made up of multiple details of Stockholm city with an aerial view. The adaptive coding structures (CU, PU, TU) of HEVC may be assets for this sequence. It is related that sequences like CrowdRun and Parkjoy have less favorable results that those exposed in [11] for RA-HE 8 bits test condition where luma results are -33.2% in favor of HM6.0 minimum while our sequence Parkjoy have a result of -19.6% for luma and CrowdRun -22.6% for the same component. Thus, three out of six of our sequences have worse results than those presented in [11]. Same remark for AI-HE 8 bits test conditions where three out of six of our sequences are under the minimum luma result of [11]. Another interesting analysis about test material can be found in [16].

It is stated that each component trend across bit depth is a very low decrease (less than 1% for Y and less than 4% for U and less than 9% for V with respect to BD-rate) with a unhooking for 14-bit AI-HE test condition. It would mean that there is a limitation in HM6.0 with respect to 14-bit AI-HE encoding. It is claimed by the authors that these results are encouraging as they have a tendency to show that limited efforts (in comparison with a “from scratch” approach) would have to be undertaken in order to support high bit depth until 14-bit in HM.

| Class B 1080p | Sequence name | Y | U | V |
| --- | --- | --- | --- | --- |
|  | CrowdRun | -15,8% | -19,1% | -16,7% |
|  | DucksTakeOff | -20,0% | -31,6% | -45,0% |
|  | Parkjoy | -15,0% | -18,6% | -15,9% |
|  | InToTree | -22,2% | -53,0% | -22,9% |
|  | Seeking | -19,2% | -32,7% | -21,6% |
|  | OldTownCross | -17,6% | -23,0% | -17,9% |
|  | Average | -18,3% | -29,6% | -23,3% |

Table 3 JM18.3 vs HM6.0 All Intra HE – 8 bits

| Class B 1080p | Sequence name | Y | U | V |
| --- | --- | --- | --- | --- |
|  | CrowdRun | -22,6% | -33,7% | -29,7% |
|  | DucksTakeOff | -28,4% | -59,4% | -65,8% |
|  | Parkjoy | -19,6% | -38,7% | -24,2% |
|  | InToTree | -45,1% | -79,0% | -45,6% |
|  | Seeking | -33,6% | -46,2% | -33,3% |
|  | OldTownCross | -50,0% | -49,6% | -41,8% |
|  | Average | -33,2% | -51,1% | -40,1% |

Table 4 JM18.3 vs HM6.0 Random Access HE – 8 bits

| Class B 1080p | Sequence name | Y | U | V |
| --- | --- | --- | --- | --- |
|  | CrowdRun | -15,4% | -17,9% | -14,9% |
|  | DucksTakeOff | -19,5% | -30,7% | -44,0% |
|  | Parkjoy | -14,5% | -17,9% | -14,5% |
|  | InToTree | -21,4% | -52,0% | -17,9% |
|  | Seeking | -18,6% | -31,6% | -19,8% |
|  | OldTownCross | -16,8% | -20,0% | -14,5% |
|  | Average | -17,7% | -28,4% | -20,9% |

Table 5 JM18.3 vs HM6.0 All Intra HE – 10 bits

| Class B 1080p | Sequence name | Y | U | V |
| --- | --- | --- | --- | --- |
|  | CrowdRun | -22,3% | -28,9% | -24,6% |
|  | DucksTakeOff | -31,5% | -60,1% | -60,2% |
|  | Parkjoy | -19,9% | -36,8% | -12,9% |
|  | InToTree | -43,1% | -76,9% | -33,3% |
|  | Seeking | -33,6% | -41,6% | -28,1% |
|  | OldTownCross | -47,9% | -42,1% | -35,3% |
|  | Average | -33,0% | -47,7% | -32,4% |

Table 6 JM18.3 vs HM6.0 Random Access HE – 10 bits

| Class B 1080p | Sequence name | Y | U | V |
| --- | --- | --- | --- | --- |
|  | CrowdRun | -15,1% | -17,6% | -14,9% |
|  | DucksTakeOff | -19,3% | -30,5% | -44,1% |
|  | Parkjoy | -14,3% | -17,8% | -14,3% |
|  | InToTree | -20,9% | -52,3% | -19,0% |
|  | Seeking | -18,3% | -31,5% | -19,4% |
|  | OldTownCross | -16,4% | -20,0% | -14,1% |
|  | Average | -17,4% | -28,3% | -21,0% |

Table 7 JM18.3 vs HM6.0 All Intra HE – 12 bits

| Class B 1080p | Sequence name | Y | U | V |
| --- | --- | --- | --- | --- |
|  | CrowdRun | -21,8% | -28,4% | -24,1% |
|  | DucksTakeOff | -30,9% | -60,0% | -59,8% |
|  | Parkjoy | -19,4% | -36,7% | -12,0% |
|  | InToTree | -42,6% | -76,9% | -31,2% |
|  | Seeking | -33,5% | -41,1% | -27,5% |
|  | OldTownCross | -47,2% | -40,0% | -46,5% |
|  | Average | -32,6% | -47,2% | -31,4% |

Table 8 JM18.3 vs HM6.0 Random Access HE – 12 bits

| Class B 1080p | Sequence name | Y | U | V |
| --- | --- | --- | --- | --- |
|  | CrowdRun | -11,5% | -11,9% | -9,2% |
|  | DucksTakeOff | -14,0% | -23,2% | -32,5% |
|  | Parkjoy | -9,7% | -10,9% | -7,6% |
|  | InToTree | -15,2% | -43,8% | -5,2% |
|  | Seeking | -14,3% | -25,6% | -13,5% |
|  | OldTownCross | -13,5% | -13,8% | -6,5% |
|  | Average | -13,0% | -21,5% | -12,4% |

Table 9 JM18.3 vs HM6.0 All Intra HE – 14 bits

| Class B 1080p | Sequence name | Y | U | V |
| --- | --- | --- | --- | --- |
|  | CrowdRun | x | x | x |
|  | DucksTakeOff | x | x | x |
|  | Parkjoy | x | x | x |
|  | InToTree | -40,7% | -68,9% | -25,0% |
|  | Seeking | x | x | x |
|  | OldTownCross | x | x | x |
|  | Average | x | x | x |

Table 10 JM18.3 vs HM6.0 Random Access HE – 14 bits

RA-HE 14 bits results (Table 10) are not directly exploitable as JM18.3 encountered issues for encoding test material with this test condition. However, results from HM6.0 with this test condition tend to show that results are coherent with 8, 10 and 12 values (Cf. attached excel datasheet). For instance, a comparison between HM6.0 RA-HE 12 bits and HM6.0 RA-HE 14 bits shows that luma is -2.3%, Cb -8.5%, Cr -4.9% in favor of 12 bits test conditions.

# Conclusion

This contribution relates a short analysis of WD6 and HM6 software with respect to high bit depth support. Four experiments were conducted to identify current status and potential bottlenecks of HM software. It consisted in a comparison between HM6.0 and JM18.3 thru encoding campaigns of six 8, 10, 12 and 14 bits contents for RA-HE and AI-HE test conditions. It is reported that results are coherent for 8, 10 and 12 bits with a small decrease trend. Some limitations may appear for 14–bit content and AI test condition. It is stated that JM18.3 has issues with RA-HE 14 bits test conditions. It is suggested to perform a thorough study of HEVC (WD+HM) intra modules then inter modules as further studies and to extend test conditions (low delay cases, other than class B test material). It is suggested to harmonize JM and HM PSNR calculation. It is also recommended to propose an Ad Hoc Group (AhG) in order to prepare High Bit Depth support in HEVC.

# References

- JCTVC-H1003_dk, “High efficiency video coding (HEVC) text specification draft 6”, San José, CA, USA, Feb. 2012.

- ETSI TS 101 154 v1.9.1, “Digital Video Broadcasting (DVB); Specification for the use of Video and Audio Coding in Broadcasting Applications based on MPEG-2 Transport Stream”, Sept. 2009.

- ITU-R WP6C/564-E Annex 4, “Preliminary draft new Recommendation ITU-R BT.[IMAGE-UHDTV] – Parameters values for UHDTV systems for production and international programme exchange”, Oct. 2011.

- M. Sugawara, C. Todd, “Rethinking Color for Next-Generation TV”, Hollywood Post Alliance, Inc. presentation, 17 February 2012 (http://www.hpaonline.com/mc/page.do?sitePageId=131455)

- Y. Nishida et al., “Super Hi-Vision System Offering Enhanced Sense of Presence and New Visual Experience”, Amsterdam, IBC 2011.

- Digital Cinema Initiatives, LLC, “Digital Cinema System Specification version 1.2”, 7 March 2008.

- ftp://vqeg.its.bldrdoc.gov/HDTV/SVT_MultiFormat/

- “The SVT High Definition Multi Format Test Set”,
 ftp://vqeg.its.bldrdoc.gov/HDTV/SVT_MultiFormat/SVT_MultiFormat_v10.pdf, v1.0,, Feb. 2006.

- ITU-R BT.709-5, “Parameter values for the HDTV standards for production and international programme exchange”, April 2002.

- ITU-R BT.1361-1, “Worldwide unified colorimetry and related characteristics of future television and imaging systems”

- JCTVC-I0409, “Comparison of Compression Performance of HEVC Working Draft 6 with AVC High Profile”, Geneva, Switzerland, April 2012.

- ITU-T Rec H.264, “Advanced video coding for generic audiovisual services”, March 2010.

- JCTVC-E192, “Proposal of enhanced PCM coding in HEVC”, Geneva,  Switzerland, March 2011.

- JCTVC-F537, “On the motion compensation process”, Torino, Italia, July 2011.

- JCTVC-D152, “Adaptive scaling for bit depth compression on IBDI”, Daegu, Republic of Korea, Jan. 2011.

- JCTVC-I0407, “Compression performance of HEVC WD6 based on ITU-R Rec. BT.1210”, Geneva, Switzerland, April 2012.