| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0109 |
| --- | --- |

| Title: | New results of implicit weighted prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Akiyuki Tanizawa Takeshi Chujoh Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 akiyuki.tanizawa@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This document reports results for the implicit weighted prediction on IBBBP (M=4) coding structure. AVC-based weighted prediction scheme was adopted in HM and WD. The explicit WP can achieve large coding gain for fading sequences with illumination variation. However, the gain of the implicit WP compared to the explicit WP on current coding structures (RA and LDB) is not so high. The main reason is that AVC based implicit WP focuses on IBBP (M=3) coding structure originally and the current coding structures of HM common test conditions are not effective. For example, in the case of RA coding structure, the some weighting factors deriving from the implicit WP are identical to the default weighting factor. In this contribution, it is reported that the implicit WP on IBBBP (M=4) coding structure is tested and reported. AVC-based implicit WP can get 3.3% BD-rate gain on average for black-fade/white-fade sequences.

# Implicit Weighted Prediction

At JCT-VC Torino meeting, AVC-based weighted prediction (AVCWP) scheme [1][2] was adopted in HM4 and WD4 [3]. The explicit WP can achieve large coding gain for fading sequences with illumination variation. However, the gain of the implicit WP compared to the explicit WP is not so high. The main reason is that AVC-based implicit WP is not useful for current coding structures (such as RA).

For example, we assume that RA coding structure such as figure 1 is utilized. If the temporal distance among the target picture and two reference pictures are same, then the weighting factors of WL0 and WL1 are set to 1/2, respectively. This means these parameters are identical to the default weighting factor which not using WP (just averaging).

Figure 1. an example of implicit WP on RA coding structure

AVC based implicit WP was originally developed for the conventional IBBP (M=3) coding structure. However, current HM common test conditions don’t support such coding structures, because current HM software does not support IBBP (M=3) coding structure so far.

In this contribution, we tested IBBBP (M=4) coding structure to evaluate the coding efficiency of AVC-based implicit WP.

# Experimental Results

HM software version 6.0 (HM-6.0) with the common configuration Error: Reference source not found is used in these experiments. Test sequences with black/white-fade are provided by WP AHG [5].

The current HM software has two types of cost functions for motion estimation (ME), a default cost function and a weighted cost function. In the case of WP, the weighted cost function is used. When the implicit WP is utilized, the weighting factor is always set to the default weighting factor and the weighted cost function is used. This may cause the computational complexity increase. In our simulations, both conditions are tested, the default cost function and the weighted cost function.

Error: Reference source not found1 and Table 2 show BD-Rate and the relative average encoding/decoding time for implicit WP and the implicit WP without weighted ME compared to HM6.0 anchor. The negative value of BD-Rate indicates gain. Table 1 shows the results for black-fade sequences and Table 2 shows the results for white-fade sequences. It observed that the implicit WP can achieve 3.3% to 3.6% BD-rate gain on average. Moreover, this scheme can reduce the decoding time. Through the experiments, we found that it is not necessary to use weighted motion estimation for implicit WP.

Table 1. BD-Rate[%] and relative encoding/decoding time[%] of implicit WP and implicit WP without weighted ME compared to HM6.0 anchor for black-fade sequences on IBBBP coding structure

| Black-fade case | IBBBP (M=4) Main |  |  |  |  | IBBBP (M=4) HE10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc[%] | Dec[%] | Y | U | V | Enc[%] | Dec[%] |
| ImplicitWP | -3.3% | -2.8% | -3.2% | 130% | 92% | -3.4% | -3.5% | -3.5% | 123% | 93% |
| ImplicitWP_noWPME | -3.3% | -2.8% | -3.1% | 94% | 92% | -3.4% | -3.5% | -3.5% | 92% | 93% |

Table 2. BD-Rate[%] and relative encoding/decoding time[%] of implicit WP and implicit WP without weighted ME compared to HM6.0 anchor for white-fade sequences on IBBBP coding structure

| White-fade case | IBBBP (M=4) Main |  |  |  |  | IBBBP (M=4) HE10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc[%] | Dec[%] | Y | U | V | Enc[%] | Dec[%] |
| ImplicitWP | -3.6% | -3.3% | -3.7% | 134% | 91% | -3.6% | -4.1% | -4.0% | 126% | 92% |
| ImplicitWP_noWPME | -3.5% | -3.3% | -3.7% | 94% | 90% | -3.6% | -4.1% | -4.0% | 91% | 92% |

# Conclusion

In this contribution, the new results of implicit WP on IBBBP (M=4) coding structure are reported. Experimental results show that the gain of the implicit WP can achieve the average BD-rate gain of 3.3% to 3.6% and the weighted motion estimation is not needed for implicit WP.

AVC-based implicit WP might be useful for a certain coding structure such as IBBP (M=3).

# References

[1]	P. Bordes, "Weighted Prediction," Joint Collaborative Team on Video Coding, JCTVC-F265, Torino, July 2011.

[2]	A. Tanizawa, "Explicit Weighted Prediction with simple WP parameter estimation," Joint Collaborative Team on Video Coding, JCTVC-F326, Torino, July 2011.

[3]	AHG report, " WD4: Working Draft 4 of High-Efficiency Video Coding," Joint Collaborative Team on Video Coding, JCTVC-F803, Torino, July 2011.

[4]	F. Bossen ," Common test conditions and software reference configurations ", Joint Collaborative Team on Video Coding, JCTVC-H1100, San Jose, Feb. 2012.

[5]	AHG report, " JCT-VC AHG report: Weighted prediction (AHG 18)," Joint Collaborative Team on Video Coding, JCTVC-F018, Torino, July 2011.

# Patent rights declaration(s)

TOSHIBA Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).