| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0110 |
| --- | --- |

| Title: | Improved implicit weighted prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akiyuki Tanizawa Takeshi Chujoh Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 akiyuki.tanizawa@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This document presents an improved implicit weighted prediction. Weighted prediction (WP) scheme consists of two modes, an explicit WP and an implicit WP. An implicit WP is a method of not explicitly encoding WP parameters, but implicitly deriving them from information of reference pictures. In AVC-based implicit WP, only weighting factors can be derived from the temporal distances among the target picture and two reference pictures used for bi-prediction in B-slices. However, the offset value is always set to 0. Therefore, there are issues in that the conventional implicit WP cannot compensate the pixel value additively and cannot be applicable to uni-prediction. In order to cope with these issues, the derivation process for implicit WP parameters based on image characteristics of the reference pictures stored in decoded picture buffer is proposed. The experimental results in HM software version 6.0 including AVC-based implicit WP are reported. The BD-rate gain of the proposed method for black/white-fade sequences is 16% to 34% compared to HM common condition anchor. It is reported that this scheme can reduce the decoding time.

# Implicit Weighted Prediction

At JCT-VC Torino meeting, AVC-based weighted prediction (AVCWP) scheme [1][2] was adopted in HM4 and WD4 [3]. The AVCWP can achieve large coding gain (up to 60%) for fading sequences with illumination variation. However, the gain of the implicit WP compared to the explicit WP is not so high. Although in the implicit WP of AVCWP, weighting factors are implicitly derived from the temporal distances among the target picture and two reference pictures used for bi-prediction, the offset is always set to 0. This is one reason of coding efficiency decrease. Furthermore, there is an issue in that the implicit WP cannot be applicable to uni-prediction for both P- and B-slices, because it needs two reference pictures for bi-prediction. In this document, the improved implicit WP method using image characteristics of the local decoded pictures is proposed.

## Algorithm description for proposed implicit WP

The WP is a method of an adaptive weighting of inter prediction signal to compensate illumination variation between the reference pictures and the target one. It can be applied in uni-prediction (1) or in bi-prediction (2) for the explicit WP and in bi-prediction (2) for the implicit WP [4].

if( logWDC >= 1 ) 
	predSamples[ x, y ] = Clip1( ( ( predSamplesLX [ x, y ] * w0C + 2logWDC − 1 ) >> logWDC ) + oXC ) 
else
	predSamples[ x, y ] = Clip1( predSamplesLX [ x, y ] * wXC + oXC )                              (1)

predSamples [ x, y ] = Clip1( ( ( predSamplesL0 [ x, y ] * w0C + predSamplesL1 [ x, y ] * w1C

+ ( ( o0C + o1C + 1 ) << logWDC ) ) >> ( logWDC + 1 ) )                                   (2)

In order to derive optimal WP parameters for implicit WP, WP parameter estimation method based on alpha-blending model is introduced in [2] as a derivation process. This method uses only spatial image characteristics, which are robust with respect to motion.

We assume that the following relationship between two fading picture at different times.

Y’( t ) = α( t ) / α( t – 1 ) * Y ( t – 1 ) + ( 1 – α( t ) / α( t – 1 ) ) * C,				(3)

where Y ( t ) is the original picture at time t before fading and Y’ ( t ) is the fading picture. C is a tagert pixel value (such as 16 for black-fade and 235 for white-fade) and α( t ) means the alpha-blending parameter whose range is 0 to 1. The weighting factor w0 is α( t ) / α( t – 1 ) and the offset factor o0 is ( 1 – α( t ) / α( t – 1 ) ) * C.

We assume that Y can represent YAC + YDC, the following relationship is derived;

Y’( t ) = α( t ) * YAC + dc( t ),								(4)

Y’( t – 1 ) = α( t – 1 ) * YAC + dc( t – 1 ),							(5)

where, AC component and DC component of the observed pictures Y’( t ) and Y’( t – 1 ) have following relationships;

AC(Y’( t )) = α( t ) * YAC,		DC(Y’( t )) = dc( t )					(6)

AC(Y’( t – 1 )) = α( t – 1 ) * YAC ,	DC(Y’( t – 1 )) = dc( t – 1 ),				(7)

Finally, using AC component and DC component, the weighting factor and the offset factor can be derived in the following equations;

w0=NAC(Y'(t))NAC(Y'(t−1))=α(t)α(t−1),								(8)

o0=DC(Y'(t))−w0×DC(Y'(t−1))=dc(t)−w0×dc(t−1),				(9)

where, we can calculate AC component and DC component of the observed pictures Y’( t );

DC(Y'(t))=1Nx,y∈NY'x,y(t)								(10)

AC(Y'(t))=1Nx,y∈NY'x,y(t)−DC(Y'(t))						(11)

In this model, we can derive WP parameter from only image characteristics, which means AC and DC components of two observed pictures. However, the local decoded picture of the target picture cannot be used for its prediction process owing to decoding dependency. Therefore, we predict the image characteristics of the target picture using the temporal distances from two different reference pictures stored in the decoded picture buffer (DPB). We assume POC0 and POC1 as POC number of these different reference pictures. AC component and DC component of the target picture could be predicted from the following equations. These equations (12)-(15) have been already defined on current WD [3].

tb = Clip3( -128, 127, curPOC – POC0) 						(12)

td = Clip3( -128, 127, POC1– POC0) 							(13)

tx = ( 16384 + abs( td / 2 ) ) / td								(14)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 ) 				(15)

DCP = (DistScaleFactor * DC1 (256 - DistScaleFactor) * DC0+ Offset) >> (Shift) 	(16)

ACP = (DistScaleFactor * AC1+ (256 - DistScaleFactor) * AC0+ Offset) >> (Shift) 	(17)

where, curPOC means POC number of the target picture. DC0 and AC0 mean DC component and AC component for reference picture with POC0 respectively. DC1 and AC1 mean DC component and AC component for reference picture with POC1 respectively.

Let us explain how to select two reference pictures used for derivation of (ACP, DCP) from reference pictures. Since this proposed implicit WP is designed based on an alpha-blending model, it is important to select two temporally different reference pictures with high correlation in order to derive the optimal predictor for the target picture. In addition, it is known that the interpolated prediction has higher accuracy than the extrapolated prediction. Therefore, the following conditions are considered:

1.	Temporal distance from the target picture,

2.	Availability of interpolated prediction.

In accordance with these conditions, the reference pictures for list 0 and list 1 with the smallest differential values between POCX and curPOC are selected, respectively. If the selected POC0 is equal to the selected POC1, the reference picture for list 0 or list 1 with the second-smallest differential value between POCX and curPOC is selected. This process is repeated until two different reference pictures are determined. When the prediction structure does not have reordering reference pictures, two reference pictures are selected from only list 0. If a temporally different reference picture is unavailable, the proposed implicit WP cannot be applied to this target picture and conventional MC prediction is performed. By linearly predicting the image characteristics of the target picture using the temporal distance between two different reference pictures stored in DPB using equations (16)-(17), WP parameters of both uni-prediction and bi-prediction can also be derived. Fig.1 shows the flowchart of derivation process of weighting factors and offsets for the proposed implicit WP scheme.

Fig 1. Flowchart of derivation process of weighting factors and offsets for the proposed implicit WP

# Syntax and decoding process

The detailed syntax change and the derivation process of the weighting factors and offsets are shown in appendix of this contribution.

# Experimental Results

HM software version 6.0 (HM-6.0) with the common configuration Error: Reference source not found is used in these experiments. Test sequences with black/white-fade are provided by WP AHG [6].

Error: Reference source not found1 and Table 2 show BD-Rate and the relative average encoding/decoding time for explicit WP, implicit WP and the proposed implicit WP compared to HM6.0 anchor. The negative value of BD-Rate indicates gain. Table 1 shows the results for black-fade sequences and Table 2 shows the results for white-fade sequences. It observed that proposed Implicit WP can achieve the gain of 16% to 34% on average for RA, LDB and LDP cases and that gain is the almost similar to the explicit WP. Moreover, this scheme can reduce the decoding time for LDB and LDP cases.

Table 1. BD-Rate[%] and relative encoding/decoding time[%] of explicit WP, implicit WP and proposed Implicit WP compared to HM6.0 anchor for black-fade sequences

| Black-fade case | Random Access Main |  |  |  |  | Random Access HE10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc[%] | Dec[%] | Y | U | V | Enc[%] | Dec[%] |
| ExplicitWP | -16.9% | -19.6% | -19.1% | 143% | 100% | -17.2% | -19.6% | -19.4% | 140% | 98% |
| ImplicitWP | -0.2% | -0.4% | -0.6% | 172% | 100% | -0.2% | -0.1% | -0.1% | 167% | 101% |
| Proposed | -16.3% | -17.9% | -17.9% | 142% | 100% | -16.1% | -18.5% | -17.9% | 140% | 101% |
| Black-fade case | Low delay B Main |  |  |  |  | Low delay B HE10 |  |  |  |  |
|  | Y | U | V | Enc[%] | Dec[%] | Y | U | V | Enc[%] | Dec[%] |
| ExplicitWP | -32.3% | -40.5% | -39.2% | 140% | 83% | -32.0% | -43.2% | -41.1% | 134% | 81% |
| ImplicitWP | 1.4% | 3.2% | 2.2% | 181% | 105% | 1.9% | 2.6% | 2.4% | 175% | 104% |
| Proposed | -31.9% | -36.0% | -34.5% | 141% | 86% | -31.4% | -37.4% | -35.6% | 135% | 86% |
| Black-fade case | Low delay P Main |  |  |  |  | Low delay P HE10 |  |  |  |  |
|  | Y | U | V | Enc[%] | Dec[%] | Y | U | V | Enc[%] | Dec[%] |
| ExplicitWP | -32.0% | -39.5% | -38.2% | 123% | 79% | -31.3% | -42.7% | -41.3% | 116% | 76% |
| ImplicitWP | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Proposed | -31.6% | -35.2% | -32.1% | 122% | 83% | -30.8% | -36.3% | -35.0% | 117% | 83% |

Table 2. BD-Rate[%] and relative encoding/decoding time[%] of explicit WP, implicit WP and proposed Implicit WP compared to HM6.0 anchor for white-fade sequences

| White-fade case | Random Access Main |  |  |  |  | Random Access HE10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc[%] | Dec[%] | Y | U | V | Enc[%] | Dec[%] |
| ExplicitWP | -20.3% | -23.4% | -23.0% | 137% | 97% | -20.5% | -22.9% | -23.1% | 136% | 96% |
| ImplicitWP | -0.3% | -0.6% | -0.5% | 180% | 101% | -0.2% | -0.2% | -0.2% | 174% | 101% |
| Proposed | -19.3% | -21.8% | -21.6% | 137% | 100% | -19.0% | -21.4% | -21.4% | 137% | 100% |
| White-fade case | Low delay B Main |  |  |  |  | Low delay B HE10 |  |  |  |  |
|  | Y | U | V | Enc[%] | Dec[%] | Y | U | V | Enc[%] | Dec[%] |
| ExplicitWP | -34.2% | -42.3% | -41.5% | 136% | 80% | -33.7% | -44.7% | -43.2% | 129% | 79% |
| ImplicitWP | 1.5% | 2.5% | 2.8% | 185% | 104% | 2.0% | 3.1% | 2.7% | 178% | 104% |
| Proposed | -33.4% | -38.5% | -36.3% | 136% | 84% | -32.7% | -38.1% | -36.7% | 131% | 85% |
| White-fade case | Low delay P Main |  |  |  |  | Low delay P HE10 |  |  |  |  |
|  | Y | U | V | Enc[%] | Dec[%] | Y | U | V | Enc[%] | Dec[%] |
| ExplicitWP | -33.8% | -40.8% | -39.7% | 114% | 76% | -33.1% | -44.2% | -42.4% | 109% | 75% |
| ImplicitWP | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Proposed | -33.4% | -37.1% | -34.7% | 115% | 81% | -32.3% | -38.0% | -36.1% | 111% | 82% |

# Conclusion

In this contribution, an improved Implicit WP scheme based on local decoded picture is proposed. This method can be applicable to not only bi-prediction in B-slices but also uni-prediction in P/B-slices. Experimental results show that the gain of proposed method can achieve the average BD-rate gain of 16% to 34% for RA, LDB and LDP cases.

It’s suggested that this proposal is introduced in the text and the software of HEVC test model (HM).

# References

[1]	P. Bordes, "Weighted Prediction," Joint Collaborative Team on Video Coding, JCTVC-F265, Torino, July 2011.

[2]	A. Tanizawa, "Explicit Weighted Prediction with simple WP parameter estimation," Joint Collaborative Team on Video Coding, JCTVC-F326, Torino, July 2011.

[3]	AHG report, " WD4: Working Draft 4 of High-Efficiency Video Coding," Joint Collaborative Team on Video Coding, JCTVC-F803, Torino, July 2011.

[4]	Y. Ye and E-S. Ryu, "Improved Weighted Prediction," Joint Collaborative Team on Video Coding, JCTVC-G065, Geneva, Nov. 2011.

[5]	F. Bossen ," Common test conditions and software reference configurations ", Joint Collaborative Team on Video Coding, JCTVC-H1100, San Jose, Feb. 2012.

[6]	AHG report, " JCT-VC AHG report: Weighted prediction (AHG 18)," Joint Collaborative Team on Video Coding, JCTVC-F018, Torino, July 2011.

# Patent rights declaration(s)

TOSHIBA Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

## WD text for implicit weighted prediction

#### 7.3.2.2 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| sign_data_hiding_flag | u(1) |
| if( sign_data_hiding _flag ) |  |
| sign_hiding_threshold | u(4) |
| cabac_init_present_flag | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 | se(v) |
| constrained_intra_pred_flag | u(1) |
| enable_temporal_mvp_flag | u(1) |
| slice_granularity | u(2) |
| max_cu_qp_delta_depth | ue(v) |
| cb_qp_offset | se(v) |
| cr_qp_offset | se(v) |
| weighted_unipred_idc | u(2) |
| weighted_bipred_idc | u(2) |
| output_flag_present_flag | u(1) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| tile_info_present_flag | u(1) |
| tile_control_present_flag | u(1) |
| if( tile_info_present_flag) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| } |  |
| if( tile_control_present_flag ) |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } else if( tiles_or_entropy_coding_sync_idc = = 2 ) |  |
| num_substreams_minus1 | ue(v) |
| deblocking_filter_control_present_flag | u(1) |
| if(slice_type = =P ¦ ¦ slice_type = = B) |  |
| log2_parallel_merge_level_minus2 | ue(v) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### 7.4.2.2 Picture parameter set RBSP semantics

pic_parameter_set_id identifies the picture parameter set that is referred to in the slice header. The value of pic_parameter_set_id shall be in the range of 0 to 255, inclusive.

seq_parameter_set_id refers to the active sequence parameter set. The value of seq_parameter_set_id shall be in the range of 0 to 31, inclusive.

sign_data_hiding_flag equal to 0 specifies that sign bit hiding is disabled. sign_data_hiding_flag equal to 1 specifies that sign bit hiding is enabled

sign_hiding_threshold specifies a threshold to be compared with the number of coefficients that are between the last non-zero coefficient and the first non-zero coefficient along the scanning path within a coefficient group to determine whether the sign bit of the first nonzero coefficient is present in the bitstream or whether it will be inferred.

cabac_init_present_flag equal to 1 specifies that cabac_init_flag is present in the slice header.

num_ref_idx_l0_default_active_minus1 specifies how num_ref_idx_l0_active_minus1 is inferred for P and B slices with num_ref_idx_active_override_flag equal to 0. The value of num_ref_idx_l0_default_active_minus1 shall be in the range of 0 to 15, inclusive.

num_ref_idx_l1_default_active_minus1 specifies how num_ref_idx_l1_active_minus1 is inferred for B slices with num_ref_idx_active_override_flag equal to 0. The value of num_ref_idx_l1_default_active_minus1 shall be in the range of 0 to 15, inclusive.

pic_init_qp_minus26 specifies the initial value minus 26 of SliceQPY for each slice. The initial value is modified at the slice layer when a non-zero value of slice_qp_delta is decoded, and is modified further when a non-zero value of cu_qp_delta is decoded at the coding unit layer. The value of pic_init_qp_minus26 shall be in the range of −(26 + QpBdOffsetY ) to +25, inclusive.

constrained_intra_pred_flag equal to 0 specifies that intra prediction allows usage of residual data and decoded samples of neighbouring macroblocks coded using Inter macroblock prediction modes for the prediction of macroblocks coded using Intra macroblock prediction modes. constrained_intra_pred_flag equal to 1 specifies constrained intra prediction, in which case prediction of macroblocks coded using Intra macroblock prediction modes only uses residual data and decoded samples from I macroblock types.

enable_temporal_mvp_flag specifies whether temporal motion vector predictor can be used or not. If enable_temporal_mvp_flag is equal to 1, temporal motion vector predictors are used; otherwise (enable_temporal_mvp_flag is equal to 0), the temporal motion vector predictors are not used. enable_temporal_mvp_flag equal to 0 together with temporal_id equal to 0 also indicates that the marking process will occur before decoding the first picture with an output order greater than the current non-TMVP picture.

slice_granularity indicates the slice granularity within a picture. The value of slice_granularity shall not be larger than Min( Log2CtbSize − 4, log2_diff_max_min_coding_block_size ). The variable SliceGranularity is set to the value of ( slice_granularity << 1 ).

shared_pps_info_enabled_flag specifies the shared information in picture parameter set RBSP shall be used for the referred slices. If shared_pps_info_enabled_flag is equal to 1, the alf_param() in picture parameter set RBSP shall be applied for the referred slices; otherwise, the  alf_param() in slice header(s) shall be applied.

max_cu_qp_delta_depth indicates the control granularity for QPY values for coding unit within a picture. Its a requirement of bitstream conformance that the value of max_cu_qp_delta_depth shall not be greater than slice_granularity + 1.

The variable Log2MinCUDQPSize specifying the minimum size of coding units that convey cu_qp_delta, is derived as follows.

Log2MinCUDQPSize = Log2MaxCUSize − (max_cu_qp_delta_depth − 1)	(7-1)

The value of log2MinCUDQPSize should be larger than or equal to the CU size specified by the value of slice_granurality.

cb_qp_offset, cr_qp_offset are specifying the offset to the luma quantization parameter QP’Y used for deriving QP’Cb and QP’Cr , respectively.

weighted_unipred_idc equal to 0 specifies that the default weighted prediction is applied to P slices. weighted_unipred_idc equal to 1 specifies that explicit weighted prediction is applied to P slices. weighted_unipred_idc equal to 2 specifies that implicit weighted prediction shall be applied to P slices.  The value of weighted_unipred_idc shall be in the range of 0 to 2, inclusive.

weighted_bipred_idc equal to 0 specifies that the default weighted prediction is applied to B slices. weighted_bipred_idc equal to 1 specifies that explicit weighted prediction is applied to B slices. weighted_bipred_idc equal to 2 specifies that implicit weighted prediction shall be applied to B slices.  The value of weighted_bipred_idc shall be in the range of 0 to 2, inclusive.

output_flag_present_flag equal to 1 indicates that the pic_output_flag syntax element is present in the associated slice headers. output_flag_present_flag equal to 0 indicates that the pic_output_flag syntax element is not present in the associated slice headers.

tile_info_present_flag equal to 1 specifies that the syntax elements num_tile_columns_minus1, num_tile_rows_minus1 are present and the syntax elements uniform_spacing_flag, column_width[ i ], and row_height[ i ] may be present in the picture parameter set. tile_info_present_flag equal to 0 specifies that syntax elements num_tile_columns_minus1, num_tile_rows_minus1, uniform_spacing_flag, column_width[ i ], and row_height[ i ] are not present in the picture parameter set.

tile_control_present_flag equal to 1 specifies that the syntax element loop_filter_across_tiles_enabled_flag is present in the picture parameter set.  tile_control_present_flag equal to 0 specifies that syntax element loop_filter_across_tiles_enabled_flag is not present in the picture parameter set.

num_substreams_minus1 plus 1 specifies the maximum number of subsets included in a slice when tiles_or_entropy_coding_sync_idc is equal to 2. When not present, the value of num_substreams_minus1 is inferred to be equal to 0.

deblocking_filter_control_present_flag equal to 1 specifies that a set of syntax elements controlling the characteristics of the deblocking filter is present in the slice header. deblocking_filter_control_present_flag equal to 0 specifies that the set of syntax elements controlling the characteristics of the deblocking filter is not present in the slice headers and their inferred values are in effect.

log2_parallel_merge_level_minus2 specifies the parallel processing level of merge/skip mode. The value of log2_parallel_merge_level_minus2 shall be in the range of 0 to log2_min_coding_block_size_minus3 + 1 + log2_diff_max_min_coding_block_size, inclusive.

pps_extension_flag equal to 0 specifies that no pps_extension_data_flag syntax elements are present in the picture parameter set RBSP syntax structure. pps_extension_flag shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value of 1 for pps_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for pps_extension_flag in a picture parameter set NAL unit.

pps_extension_data_flag may have any value. Its value does not affect decoder conformance to profiles specified in this Recommendation | International Standard.

###### 8.5.2.2.3.2 Weighted sample prediction process

Inputs to this process are:

–	the same as specified in subclause Error: Reference source not found.

–	variables for weighted prediction logWDC, logWDC, w0C, w1C, o0C, o1C with C being replaced by L for luma samples and, when chroma_format_idc is not equal to 0, Cb and Cr for chroma samples.

Outputs of this process are:

–	the same as specified in subclause Error: Reference source not found.

The prediction sample predSamples[ x, y ] with x = 0..(nPSW)−1 and y = 0..(nPSH)−1, with H being replaced by Y for luma samples and by C for chroma samples are derived as follows:

–	If the predFlagL0 is equal to 1 and predFlagL1 is equal to 0, the prediction samples are derived as follows:

if( logWDC >= 1 ) 
	predSamples[ x, y ] = Clip1H( ( ( predSamplesL0 [ x, y ] * w0C + 2logWDC − 1 ) >> logWDC ) + o0C ) 	 (8-2)
else
	predSamples[ x, y ] = Clip1H( predSamplesL0 [ x, y ] * w0C + o0C ) 	(8-3)

–	Otherwise, if the predFlagL0 is equal to 0 and predFlagL1 is equal to 1, the final predicted sample values predSamples [ x, y ] are derived by

if( logWDC >= 1 )
	predSamples[ x, y ] = Clip1H( ( ( predSamplesL1 [ x, y ] * w1C + 2logWDC − 1 ) >> logWDC ) + o1C ) 	(8-4)
else
	predSamples[ x, y ] = Clip1H( predSamplesL1 [ x, y ] * w1C + o1C ) 	(8-5)

–	Otherwise (both predFlagL0 and predFlagL1 are equal to 1), if RefPicOrderCnt( currPic, refIdxL0, L0) is equal to RefPicOrderCnt( currPic, refIdxL1, L1) and mvL0 is equal to mvL1, the final predicted sample values predSamples [ x, y ] are derived by

predSamples[ x, y ] = Clip1H( ( predSamplesL0 [ x, y ] * ( w0C + w1C ) + 
			 ( ( o0C + o1C + 1 ) << logWDC ) ) >> ( logWDC + 1 ) )	(8-6)

–	Otherwise, the final predicted sample values predSamples[ x, y ] are derived by

predSamples[ x, y ] = Clip1H( ( predSamplesL0 [ x, y ] * w0C + predSamplesL1 [ x, y ] * w1C + 
			 ( ( o0C + o1C + 1 ) << logWDC ) ) >> ( logWDC + 1 ) )	(8-7)

Where the variables logWDC, o0C, o1C, and w0C, w1C are derived as follows.

–	If weighted_bipred_idc is equal to 2 in B-slices, implicit mode weighted prediction is used as follows:

logWDc = 7+shift1		(8-8)

o0C = 0				(8-9)

o1C = 0				(8-10)

The variable WeightScaleFactor is derived from the values currPoc, refIdxL0 and refIdxL1 as follows:

tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListL0( refIdxL0 ) ) )	(8-11)

td = Clip3( −128, 127, PicOrderCnt( RefPicListL1( refIdxL1 ) ) 	 – PicOrderCnt( RefPicListL0( refIdxL0 ) ) )	(8-12)

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td	(8-13)

WeightScaleFactor = Clip3( −1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-14)

The variables w0C and w1C are derived as follows.

–	If PicOrderCnt( RefPicListL0( refIdxL0 ) ) is equal to PicOrderCnt( RefPicListL1( refIdxL1 ) ) or ( WeightScaleFactor >> 2 ) < −64 or ( WeightScaleFactor >> 2 ) > 128, the following applies.

w0C=128			(8-15)

w1C=128			(8-16)

–	Otherwise;

The variables IntShift, IntOffset, LeftShft, LeftOffset, IntDenom and DenomOffset are derived as follows;

–	The variable IntShift is equal to 6 and the variable IntOffset is set to 1<<(IntShift - 1).

–	The variable LeftShft is equal to 2 and the variable LeftOffset is set to 1<<( LeftShift - 1).

–	Variable bitDepth is set equal to BitDepthC with C being replaced by L for luma samples and, when chroma_format_idc is not equal to 0, Cb and Cr for chroma samples

–	The variable IntDenom is set to (logWDc + (bitDepth - 8) + LeftShft + 4) and the variable DenomOffset is set to 1<<( IntDenom - 1).

–	The variable refIdxForPred0 and refIdxForPred1 are initialy set to 0.

–	The variable refIdxForPred0 or refIdxForPred1 are incremented alternately as refIdxForPred0 first, when the following condition is satisfied. If equation (8-xxx) and (8-xxx) are satisfied simultaneously, then equations (8-228) and (8-229) are applied, and both o0C and o0C are set equal to 0;

RefPicOrderCnt( currPic , refIdxForPred0, List0 ) = RefPicOrderCnt( currPic , refIdxForPred1, List1 ), 														(8-199)

refIdxForPredX < num_ref_idx_lX_active_minus1, 						(8-199)

The variables ACForPred and DCForPred are derived as follows;

DC0 = DcForRefPicList[ List0 ][ refIdxForPred0 ], 						(8-199)

AC0 = AcForRefPicList[ List0 ][ refIdxForPred0 ],	 					(8-200)

DC1 = DcForRefPicList[ List1 ][ refIdxForPred1 ],						(8-17)

AC1 = AcForRefPicList[ List1 ][ refIdxForPred1 ],						(8-202)

DCForPred = (DistScaleFactor * DC1+ (256 - DistScaleFactor) * DC0+ IntOffset) >> (IntShift), 	(8-203)

ACForPred = (DistScaleFactor * AC1 + (256 - DistScaleFactor) * AC0 + IntOffset) >> (IntShift), 	(8-204)

The variables w0C, w1C, o0C, o1C with C being replaced by L for luma samples and, when chroma_format_idc is not equal to 0, Cb and Cr for chroma samples are derived as follows;

DCForList0 = DcForRefPicList[List0 ][ refIdx0]<<LeftShift, 					(8-199)

ACForList0 = AcForRefPicList[List0 ][ refIdx0] <<LeftShift,	 				(8-200)

DCForList1 = DcForRefPicList[List1 ][ refIdx1] <<LeftShift, 					(8-199)

ACForList1 = AcForRefPicList[List1 ][ refIdx1] <<LeftShift,	 				(8-200)

w0C = ( ( (ACForPred << logWDc) ) / (ACForList0 ) ) 						(8-205)

o0C = ( ( (DCForPred << logWDc) - (w0C * (DCForList0 ) ) + DenomOffset) >> IntDenom) 	(8-206)

w1C = ( ( (ACForPred << logWDc) ) / (ACForList1 ) ) 						(8-207)

o1C = ( ( (DCForPred << logWDc) - (w1C * (DCForList1 ) ) + DenomOffset) >> IntDenom) 	(8-208)

–	Otherwise, if both weighted_bipred_idc and ref_pic_list_combination_flag are equal to 1 in B slice, combined explicit mode weighted prediction is used as follows:

–	If C is equal to L for luma samples,

logWDc = luma_log2_weight_denom+ shift1		(8-18)

w0C = LumaWeightLC[ RefIdxLCToRefIdxLx[ refIdxL0 ] ]	(8-19)

w1C = LumaWeightLC[ RefIdxLCToRefIdxLx[ refIdxL1 ] ]	(8-20)

o0C = luma_offset_lc[ RefIdxLCToRefIdxLx[ refIdxL0 ] ] * ( 1 << ( BitDepthY − 8 ) )	(8-21)

o1C = luma_offset_lc[ RefIdxLCToRefIdxLx[ refIdxL1 ] ] * ( 1 << ( BitDepthY − 8 ) )	(8-22)

–	Otherwise (C is equal to Cb or Cr for chroma samples, with iCbCr = 0 for Cb, iCbCr = 1 for Cr),

logWDc = ChromaLog2WeightDenom + shift1	(8-23)

w0C = ChromaWeightLC[ RefIdxLCToRefIdxLx[ refIdxL0 ] ][ iCbCr ]	(8-24)

w1C = ChromaWeightLC[ RefIdxLCToRefIdxLx[ refIdxL1 ] ][ iCbCr ]	(8-25)

o0C = ChromaOffsetLC[ RefIdxLCToRefIdxLx[ refIdxL0 ] ][ iCbCr ] * ( 1 << ( BitDepthC − 8 ) )	(8-26)

o1C = ChromaOffsetLC[ RefIdxLCToRefIdxLx[ refIdxL1 ] ][ iCbCr ] * ( 1 << ( BitDepthC − 8 ) )	(8-27)

–	Otherwise (weighted_pred_flag is equal to 1 in P slice or ref_pic_list_combination_flag is equal to 0 in B-slice) explicit mode weighted prediction is used as follows:

–	If C is equal to L for luma samples,

logWDc = luma_log2_weight_denom+ shift1		(8-28)

w0C = LumaWeightL0[refIdxL0]		(8-29)

w1C = LumaWeightL1[refIdxL1]		(8-30)

o0C = luma_offset_l0[refIdxL0] * ( 1 << ( BitDepthY − 8 ) )	(8-31)

o1C = luma_offset_l1[refIdxL1] * ( 1 << ( BitDepthY − 8 ) )	(8-32)

–	Otherwise (C is equal to Cb or Cr for chroma samples, with iCbCr = 0 for Cb, iCbCr = 1 for Cr),

logWDc = ChromaLog2WeightDenom + shift1	(8-33)

w0C = ChromaWeightL0[refIdxL0][ iCbCr ]		(8-34)

w1C = ChromaWeightL1[refIdxL1][ iCbCr ]		(8-35)

o0C = ChromaOffsetL0[refIdxL0][ iCbCr ] * ( 1 << ( BitDepthC − 8 ) )	(8-36)

o1C = ChromaOffsetL1[refIdxL1][ iCbCr ] * ( 1 << ( BitDepthC − 8 ) )	(8-37)

###### Derivation process of DC component for implicit weighted prediction

Inputs of this process are:

–	a sample value pSample[xC][yC] specifying the top-left sample of the target coding unit relative to the top left sample of the current picture,

–	a CUWidthC and CUHeightC specifying the width and height of the target coding unit,

–	a MaxCUNum specifying the maximum number of the coding unit in the slice,

Output of this process is the array DcForRefPicList of the target picture.

      - The array DcValueForRefPicList is initially set equal to 0.

      - The variable currCU is initially set equal to 0.

      - The following process is repeated until currCU is greater than (MaxCUNum – 1):

NormFactor = (CUWidthC * CUHeightC) >> 4,

DcValueForRefPicList += xC,yC∈[nS,nS]pSample[xC][yC]/ (NormFactor)

–	If the variable currCU is equal to MaxCUNum, then the array DcForRefPicList is normalized as follows;

DcForRefPicList[ ListX ][ refIdxX ] = DcValueForRefPicList / MaxCUNum

###### Derivation process of AC component for implicit weighted prediction

Inputs of this process are:

–	a sample value pSample[xC][yC] specifying the top-left sample of the current coding unit relative to the top left sample of the current picture,

–	a CUWidthC and CUHeightC specifying the width and height of the target coding unit,

–	a MaxCUNum specifying the maximum number of the coding unit in the slice,

–	the array DcForRefPicList specifying the DC component of the target picture,

Output of this process is the array AcForRefPicList of the target picture.

- The array AcValueForRefPicList is initially set equal to 0.

- The variable currCU is initially set equal to 0.

- The following process is repeated until currCU is greater than (MaxCUNum – 1):

NormFactor = (CUWidthC * CUHeightC) >> 4,

AcValueForRefPicList += xC,yC∈[nS,nS]abs(pSample[xC][yC]−DcForRefPicList)/ (NormFactor)

–	If the variable currCU is equal to MaxCUNum, then the array AcForRefPicList is normalized as follows;

AcForRefPicList[ ListX ][ refIdxX ] = AcValueForRefPicList / MaxCUNum