| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0112 |
| --- | --- |

| Title: | Long-term picture signalling for error-free environments |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karsten Suehring, Heiko Schwarz, Thomas Wiegand Einsteinufer 37 10587 Berlin Germany | Email: | karsten.suehring@hhi.fraunhofer.de heiko.schwarz@hhi.fraunhofer.de thomas.wiegand@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes a modified long-term picture coding using picture indexes to identify long-term pictures in the decoded picture buffer. The coding scheme is proposed as a low-bitrate alternative coding for environments in which encoder and decoder picture buffers are synchronized (i.e. no picture loss occurs) and can be switched at SPS level. The scheme allows saving between 7 and 17 bits per slice header under the “Common conditions for reference picture marking and list construction proposals” (JCTVC-H0725).

# Introduction

With the adopted AHG21 reference picture set (RPS) coding method ([2], [3]) each coded pictures contains a list of the pictures marked “used for reference” in the decoded picture buffer (DPB). The signalling distinguishes between “short-term” references which can be signaled very efficiently using templates in the sequence parameter set and “long-term” references which need to be signaled in each slice header. The adopted long-term signalling uses the POC LSB difference between the current picture and the first long term picture and the difference between the long-term pictures for the following long-term pictures. Additional bits are required if POC LSB wraps and long-term pictures share the same POC LSB value. For a large POC LSB range with increasing distance between the current picture and the first long-term picture or increasing distance between the long-term pictures, the coded POC differences increase and thus, also the number of bits required for signalling the long-term pictures increases. For a small POC LSB range, value conflicts might occur frequently which requires additional syntax elements to be sent.

# DPB picture index based signalling

When no picture loss occurs, both encoder and decoder share the same decoded picture buffer (DPB) state. Thus both can create a POC ordered list of reference pictures in the DPB. For marking a picture as long-term reference picture, the index of the picture in the list will be signaled in the bitstream.

For coding efficiency reasons, pictures that are contained as short-term reference pictures in the used reference picture set, will not be included in the POC ordered list of pictures in the DPB. Also pictures of higher temporal layers are not included to avoid diverging buffers when temporal layer sub-streams are extracted.

The proposed scheme can be switched at the sequence parameters set (SPS).

# Syntax

Following is the changed section of the SPS header:

| num_short_term_ref_pic_sets | ue(v) |
| --- | --- |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) |  |
| long_term_idx_flag | u(1) |

Following is the changed section of the slice header:

| if( long_term_ref_pics_present_flag ) { |  |
| --- | --- |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| if( long_term_idx_flag ) |  |
| long_term_idx[ i ] | ue(v) |
| else { |  |
| delta_poc_lsb_lt[ i ] | ue(v) |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt_minus1[ i ] | ue(v) |
| } |  |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |

# Semantics

long_term_idx_flag indicates that long-term reference pictures are identified using a POC ordered reference picture list.

long_term_idx specifies the index of a long-term reference picture in the POC ordered reference picture list.

# Decoding Process (for each reference picture)

### Decoding process for reference picture set

This process is invoked once per picture, after decoding of a slice header but prior to the decoding of any coding unit and prior to the decoding process for reference picture list construction of the slice as specified in subclause Error: Reference source not found. The process may result in marking one or more reference pictures as "unused for reference".

NOTE 1 – The reference picture set is an absolute description of the reference pictures used in the decoding process of the current and future coded pictures. The reference picture set signalling is explicit in the sense that all reference pictures included in the reference picture set are listed explicitly and there is no default reference picture set construction process in the decoder that depends on the status of the decoded picture buffer.

Short-term reference pictures are identified by their PicOrderCntVal values. Long-term reference pictures are identified by their pic_order_cnt_lsb values.

Five lists of picture order count values are constructed to derive the reference picture set; PocStCurrBefore, PocStCurrAfter, PocStFoll, PocLtCurr, and PocLtFoll with NumPocStCurrBefore, NumPocStCurrAfter, NumPocStFoll, NumPocLtCurr, and NumPocLtFoll number of elements, respectively.

- If the current picture is an IDR picture, PocStCurrBefore, PocStCurrAfter, PocStFoll, PocLtCurr, and PocLtFoll are all set to empty, and NumPocStCurrBefore, NumPocStCurrAfter, NumPocStFoll, NumPocLtCurr, and NumPocLtFoll are all set to 0.

- Otherwise, the following applies for derivation of the five lists of picture order count values and the numbers of entries.

for( i = 0, j = 0, k = 0; i < NumNegativePics[ StRpsIdx ] ; i++ )
	if( UsedByCurrPicS0[ StRpsIdx ][ i ] )
		PocStCurrBefore[ j++ ] = PicOrderCntVal + DeltaPocS0[ StRpsIdx ][ i ]
	else
		PocStFoll[ k++ ] = PicOrderCntVal + DeltaPocS0[ StRpsIdx ][ i ]
NumPocStCurrBefore = j

for( i = 0, j = 0; i < NumPositivePics[ StRpsIdx ]; i++ )
	if( UsedByCurrPicS1[ StRpsIdx ][ i ] )
		PocStCurrAfter[ j++ ] = PicOrderCntVal + DeltaPocS1[ StRpsIdx ][ i ]
	else
		PocStFoll[ k++ ] = PicOrderCntVal + DeltaPocS1[ StRpsIdx ][ i ]
NumPocStCurrAfter = j
NumPocStFoll = k		(8-1)

When long_term_idx_flag is equal to zero, the lists PocLtCurr and PocLtFoll are derived as follows:

for( i = 0, j = 0, k = 0; i < num_long_term_pics; i++ )
	if( delta_poc_msb_present_flag[ i ] )
		if( used_by_curr_pic_lt_flag[ i ] )
			PocLtCurr[ j++ ] = ( ( PicOrderCntVal − DeltaPocLt[ i ] + MaxPicOrderCntLsb ) % 
				MaxPicOrderCntLsb ) – ( DeltaPocMSBCycleLt[ i ] )* MaxPicOrderCntLsb
		else
			PocLtFoll[ k++ ] = ( ( PicOrderCntVal − DeltaPocLt[ i ] + MaxPicOrderCntLsb ) %
				MaxPicOrderCntLsb ) – ( DeltaPocMSBCycleLt[ i ] ) * MaxPicOrderCntLsb
	else
		if( used_by_curr_pic_lt_flag[ i ] )
			PocLtCurr[ j++ ] = ( PicOrderCntVal − DeltaPocLt[ i ] + MaxPicOrderCntLsb ) % 
				MaxPicOrderCntLsb
		else
			PocLtFoll[ k++ ] = ( PicOrderCntVal − DeltaPocLt[ i ] + MaxPicOrderCntLsb ) %
				MaxPicOrderCntLsb
NumPocLtCurr = j
NumPocLtFoll = k

where PicOrderCntVal is the picture order count of the current picture as specified in subclause 8.2.1.

NOTE 2 – A value of StRpsIdx in the range from 0 to num_short_term_ref_pic_sets − 1, inclusive, indicates that a short-term reference picture set from the active sequence parameter set is being used, where StRpsIdx is the index of the short-term reference picture set to the list of short-term reference picture sets in the order in which they are signalled in the sequenceparameter set. StRpsIdx equal to num_short_term_ref_pic_sets indicates that a short-term reference picture set explicitly signalled in the slice header is being used.

The reference picture set consists of five lists of reference pictures; RefPicSetStCurrBefore, RefPicSetStCurrAfter, RefPicSetStFoll, RefPicSetLtCurr and RefPicSetLtFoll. The variable NumPocTotalCurr is set equal to NumPocStCurrBefore + NumPocStCurrAfter + NumPocLtCurr. When decoding a P or B slice, it is a requirement of bitstream conformance that the value of NumPocTotalCurr shall not be equal to 0.

NOTE 3 – RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetLtCurr contains all reference pictures that may be used in inter prediction of the current picture and that may be used in inter prediction of one or more of the pictures following the current picture in decoding order. RefPicSetStFoll and RefPicSetLtFoll consists of all reference pictures that are not used in inter prediction of the current picture but may be used in inter prediction of one or more of the pictures following the current picture in decoding order.

The marking of a reference picture can be "unused for reference", "used for short-term reference", or "used for long-term reference", but only one among these three. When a reference picture is referred to as being marked as "used for reference", this collectively refers to the picture being marked as "used for short-term reference" or "used for long-term reference" (but not both). A reference picture that is marked as "used for short-term reference" is referred to as a short-term reference picture. A reference picture that is marked as "used for long-term reference" is referred to as a long-term reference picture.

The derivation process for the reference picture set and picture marking are performed according to the following ordered steps, where DPB refers to the decoded picture buffer as described in Annex Error: Reference source not found:

- When long_term_idx_flag is equal to zero, the following applies:

for( i = 0; i < NumPocLtCurr; i++ ) {
	if( !delta_poc_msb_present_flag[ i ] ) {
		if( there is a long-term reference picture picX in the DPB 
				with pic_order_cnt_lsb equal to PocLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picX
		else if( there is a short-term reference picture picY in the DPB
				with pic_order_cnt_lsb equal to PocLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picY
		else 
			RefPicSetLtCurr[ i ] = "no reference picture"
	} else { 
		if( there is a long-term reference picture picX in the DPB
				with PicOrderCntVal equal to PocLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picX
		else if( there is a short-term reference picture picY in the DPB
				with PicOrderCntVal equal to PocLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picY
		else 
			RefPicSetLtCurr[ i ] = "no reference picture"
	}
}																						(8-2)

for( i = 0; i < NumPocLtFoll; i++ ) {
	if( !delta_poc_msb_present_flag[ i ] ) {
		if( there is a long-term reference picture picX in the DPB
					with pic_order_cnt_lsb equal to PocLtFoll[ i ] )
				RefPicSetLtFoll[ i ] = picX
		else if( there is a  short-term reference picture picY in the DPB
					with pic_order_cnt_lsb equal to PocLtFoll[ i ] )
				RefPicSetLtFoll[ i ] = picY
		else 
				RefPicSetLtFoll[ i ] = "no reference picture"
	} else {
		if( there is a long-term reference picture picX in the DPB
					with PicOrderCntVal to PocLtFoll[ i ] )
				RefPicSetLtFoll[ i ] = picX
			else if( there is a short-term reference picture picY in the DPB
					with PicOrderCntVal equal to PocLtFoll[ i ] )
				RefPicSetLtFoll[ i ] = picY
		else
				RefPicSetLtFoll[ i ] = "no reference picture"
	}
}

All reference pictures included in RefPicSetLtCurr and RefPicSetLtFoll are marked as "used for long-term reference"

- The following applies:

for( i = 0; i < NumPocStCurrBefore; i++ )
	if( there is a short-term reference picture picX in the DPB
			           with PicOrderCntVal equal to PocStCurrBefore[ i ])
			RefPicSetStCurrBefore[ i ] = picX
	else
		RefPicSetStCurrBefore[ i ] = "no reference picture"

for( i = 0; i < NumPocStCurrAfter; i++ )
	if( there is a short-term reference picture picX in the DPB
			           with PicOrderCntVal equal to PocStCurrAfter[ i ])
			RefPicSetStCurrAfter[ i ] = picX
	else
		RefPicSetStCurrAfter[ i ] = "no reference picture"	(8-3)

for( i = 0; i < NumPocStFoll; i++ )
	if( there is a short-term reference picture picX in the DPB
			           with PicOrderCntVal equal to PocStFoll[ i ])
			RefPicSetStFoll[ i ] = picX
	else
		RefPicSetStFoll[ i ] = "no reference picture"

- All reference pictures included in RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetStFoll are marked as "used for short-term reference".

- When long_term_idx_flag is equal to one, the following applies:

- all pictures in the DPB that are marked “used for reference” and that are not included in RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetStFoll and that have a value of temporal_id equal to or smaller than the value temporal_id of the current picture are included in the list LtRefCandidates

- the list LtRefCandidates is ordered by increasing value of PicOrderCntVal

- the following pseudo code is applied to generate the list RefPicSetLtCurr

for( i = 0, j = 0, k = 0; i < num_long_term_pics; i++ ) {
		if( used_by_curr_pic_lt_flag[ i ] )
			RefPicSetLtCurr[ j++ ] = LtRefCandidates[ lt_ref_idx[ i ] ]
		else
			RefPicSetLtFoll[ k++ ] = LtRefCandidates[ lt_ref_idx[ i ] ]
}

- All reference pictures that are included in RefPicSetLtCurr or RefPicSetLtFoll are marked as "used for long-term reference"

- All reference pictures in the decoded picture buffer that are not included in RefPicSetLtCurr, RefPicSetLtFoll, RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetStFoll are marked as "unused for reference".

NOTE 4 – There may be one or more reference pictures that are included in the reference picture set but that are not present in the decoded picture buffer. When the first coded picture in the bitstream is an IDR picture or the current coded picture is not a leading picture of the first coded picture in the bitstream, entries in RefPicSetStFoll or RefPicSetLtFoll that are equal to "no reference picture" should be ignored. When the first coded picture in the bitstream is not a CRA picture or the current coded picture is not a leading picture of the first coded picture in the bitstream, an unintentional picture loss should be inferred for each entry in RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetLtCurr that is equal to "no reference picture".

It is a requirement of bitstream conformance that the reference picture set is restricted as follows:

- There shall be no reference picture with temporal_id greater than that of the current picture included in RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetLtCurr.

- There shall be no reference picture included in the reference picture set that precedes, in output order, any CRA picture that precedes the current picture both in decoding order and output order.

- When the first coded picture in the bitstream is an IDR picture or the current coded picture is not a leading picture of the first coded picture in the bitstream, there shall be no entry in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr that is equal to "no reference picture".

NOTE 5 – A reference picture cannot be included in more than one of the five reference picture set lists.

# Results

The following results were obtained using the Common conditions for reference picture marking and list construction proposals [4]. Only the long-term picture relevant clauses 2.6 and 3.3 have been tested, although the scope of this proposal is rather the 2.6 test case. There should be no impact on short-term coding, except for one bit in the SPS.

In the following tables only the bits for the actual long-term syntax in the slice header have been counted. Please see the attached excel sheet for full SPS/PPS/Slice header reporting.

For section 2.6 two whole scene sequences were coded, i.e. 10-30-10-30 and 20-60-20-60 seconds. To keep the test material simple, only a single low-resolution sequence was looped. This was executed for only 24 fps as the lowest and 60 fps as the highest frame rate. Similar gains were achieved.

| 60 fps |  |  |  | 24 fps |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 10/30 with 4800 frames | 20/60 with 9600 frames |  |  | 10/30 with 1920 frames | 20/60 with 3840 frames |
| HM 6.1 | 87937 | 190867 |  | HM 6.1 | 32137 | 68763 |
| proposal | 21527 | 43127 |  | proposal | 8567 | 17207 |
|  | -75,5% | -77,40% |  |  | -73,34 | -74,98% |
| saved bits per slice header | 13,8 | 15,4 |  | saved bits per slice header | 12,3 | 13,4 |

For simplifying section 3.3, one second of the test sequence was encoded using only one slice per picture. This allowed coding of a very low-resolution sequence with only counting the necessary high level bits. The relative results should be similar when the slice header syntax is repeated once for each slice.

| 60 fps |  |  |  | 24 fps |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 200 ms RTT 600 frames | 1000 ms RTT 600 frames |  |  | 200 ms RTT 240 frames | 1000 ms RTT 240 frames |
| HM 6.1 | 11890 | 15590 |  | HM 6.1 | 3916 | 5296 |
| proposal | 5419 | 5155 |  | proposal | 2215 | 2059 |
|  | -54,42% | -66,93% |  |  | -43,44% | -61,12% |
| saved bits per slice header | 10,8 | 17,4 |  | saved bits per slice header | 7,1 | 13,5 |

# References

[1] 	JCTVC-H1003, High efficiency video coding (HEVC) text specification draft 6, Benjamin Bross, Woo-Jin Han, Jens-Rainer Ohm, Gary J. Sullivan, Thomas Wiegand

[2]	JCTVC-G1002, JCT-VC break-out report: Reference picture buffering and list construction (AHG21), Jill Boyce, Rickard Sjöberg, Ye-Kui Wang

[3]	JCTVC-G788, Proposal on Decoded Picture Buffer Description Syntax Relating to AHG21 and JCTVC-F493, Gary Sullivan, Yongjun Wu, Jizheng Xu, Bin Li

[4]	JCTVC-H725, Common conditions for reference picture marking and list construction proposals, Ye-Kui Wang, Miska M. Hannuksela, TK Tan, Rickard Sjöberg, Yan Ye

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).