| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0113 |
| --- | --- |

| Title: | High level syntax parsing issues |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karsten Suehring Fraunhofer HHI | Email: | Karsten.Suehring@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

Two high level syntax parsing issues have been discovered after the last JCT-VC meeting and been discussed on the JCT-VC email reflector: 1) a parsing order issue in the slice header (bug tracker issue #391) and 2) a parsing dependency between SPS and PPS (bug tracker issue #428). This contribution discusses possible solutions. For issue 1) the author suggests reordering the syntax elements (solution B) and for issue 2) the author suggest removing the tile parameter overwrite mechanism (solution A).

# Slice header parsing order

The slice header syntax starts with:

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag = = 0 ) |  |
| slice_address | u(v) |
| slice_type | ue(v) |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| pic_parameter_set_id | ue(v) |
| if( output_flag_present_flag ) |  |

The problem here is u(v) the coding of the slice_address syntax element:

The length of the slice_address syntax element is ( Ceil( Log2( PicWidthInCtbs * PicHeightInCtbs ) ) + SliceGranularity ) bits.

Where PicWidthInCtbs, PicHeightInCtbs  and SliceGranularity  are determined using syntax elements from SPS and PPS. With pic_parameter_set_id following slice_address, it is not possible to calculate these values because it is not known, which PPS and SPS to use.

Note: It would be possible to use the pic_parameter_set_id value from the first slice in the picture, which can be parsed due to the absence of slice_address. But this would be undesirable for error resilience cases where the first slice in a picture could be lost. It also creates a slice header dependency which is very similar to entropy slices.

## Solution A: Change coding to ue(v)

The problem could be solved by changing the coding of slice_address to ue(v) while losing some coding efficiency. With this coding, the first_slice_in_pic_flag could be removed. This information would be coded in the first bit of slice_address. Thus the whole parsing process would be simplified.

Syntax:

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag = = 0 ) |  |
| slice_address | ue(v) |
| slice_type | ue(v) |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| pic_parameter_set_id | ue(v) |
| if( output_flag_present_flag ) |  |

## Solution B: reorder syntax elements

The syntax elements could be reordered moving pic_parameterset_id before slice_address. I would suggest including pic_parameter_set_id into all entropy slices because otherwise slice_address would have to be moved before entropy_slice_flag and another condition on entropy_slice_flag would have to be added. Also first_slice_in_pic_flag is kept as the first syntax element, to allow easy parsing access.

Syntax:

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( first_slice_in_pic_flag = = 0 ) |  |
| slice_address | u(v) |
| slice_type | ue(v) |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| pic_parameter_set_id | ue(v) |
| if( output_flag_present_flag ) |  |

## Performance analysis and conclusions

Changing the coding of slice_address to ue(v) creates a performance loss, especially when large slice addresses need to be coded. The following table shows number of bits necessary for coding first_slice_in_pic_flag and slice_address in both schemes with different numbers of slices (which have each the same number of coding tree blocks). An Excel sheet is attached to this contribution which allows recalculating the values for different input parameters.

| image width | image height | ctb size | slice granularity | # slices | u(1)+u(v) per picture | ue(v) per picture |
| --- | --- | --- | --- | --- | --- | --- |
| 7680 | 4320 | 64 | 0 | 10 | 118 | 210 |
| 4096 | 2160 | 64 | 0 | 10 | 109 | 176 |
| 832 | 480 | 64 | 0 | 10 | 64 | 98 |
| 7680 | 4320 | 16 | 0 | 10 | 154 | 282 |
| 4096 | 2160 | 16 | 0 | 10 | 145 | 248 |
| 832 | 480 | 16 | 0 | 10 | 100 | 170 |
| 320 | 240 | 16 | 0 | 10 | 82 | 124 |
| 7680 | 4320 | 64 | 0 | 20 | 248 | 440 |
| 4096 | 2160 | 64 | 0 | 20 | 229 | 370 |
| 832 | 240 | 64 | 0 | 20 | 134 | 196 |
| 7680 | 4320 | 16 | 0 | 20 | 324 | 592 |
| 4096 | 2160 | 16 | 0 | 20 | 305 | 522 |
| 832 | 240 | 16 | 0 | 20 | 210 | 354 |
| 320 | 240 | 16 | 0 | 20 | 172 | 264 |

The table shows that even with SliceGranularity equal to zero and large coding tree block size, ue(v) coding would add a significant number of bits to the slice headers.

The original reason for moving slice_address to the beginning of the slice header was that a gateway should be able to easily determine the beginning of a slice. With the u(v) coding the gateway would have to parse and store PPS and SPS and follow their activation for finding out the actual slice address, but it would still be able to identify the first slice of a picture.

Weighting the parsing simplification against the coding loss, the author suggests following solution B.

# SPS/PPS parsing dependency on tiles_or_entropy_coding_sync_idc

The syntax element tiles_or_entropy_coding_sync_idc is defined in the SPS. In the PPS an overwrite mechanism exists for the syntax elements associated with tiles_or_entropy_coding_sync_idc equal to one and a new syntax element exists for tiles_or_entropy_coding_sync_idc equal to two.

This creates a parsing dependency between SPS and PPS which conflicts with our high level syntax design principles.

| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| --- | --- |
| tile_info_present_flag | u(1) |
| tile_control_present_flag | u(1) |
| if( tile_info_present_flag) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| } |  |
| if( tile_control_present_flag ) |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } else if( tiles_or_entropy_coding_sync_idc = = 2 ) |  |
| num_substreams_minus1 | ue(v) |

## Solution A: remove the overwrite functionality and completely move the tiles related syntax to the PPS

SPS:

| tiles_or_entropy_coding_sync_idc | u(2) |
| --- | --- |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |

PPS:

| tiles_or_entropy_coding_sync_idc | u(2) |
| --- | --- |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| tile_info_present_flag | u(1) |
| tile_control_present_flag | u(1) |
| if( tile_info_present_flag) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| } |  |
| if( tile_control_present_flag ) |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } else if( tiles_or_entropy_coding_sync_idc = = 2 ) |  |
| num_substreams_minus1 | ue(v) |

If it is the functionality of changing the tiles parameters is not required the SPS level, the suggested PPS syntax could also be moved to the SPS.

## Solution B: create a complete overwrite process that can be parsed independently

SPS:

| tiles_or_entropy_coding_sync_idc | u(2) |
| --- | --- |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } else if( tiles_or_entropy_coding_sync_idc = = 2 ) |  |
| num_substreams_minus1 | ue(v) |

PPS:

| tile_parameters_override_flag | u(1) |
| --- | --- |
| if( tile_parameters_override_flag ) { |  |
| tiles_or_entropy_coding_sync_idc | u(2) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| tile_info_present_flag | u(1) |
| tile_control_present_flag | u(1) |
| if( tile_info_present_flag) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| } |  |
| if( tile_control_present_flag ) |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } else if( tiles_or_entropy_coding_sync_idc = = 2 ) |  |
| num_substreams_minus1 | ue(v) |
| } |  |

## Conclusion

Solution A provides a simplification of the decoding process. Solution B is only useful when picture parameter sets will be updated frequently without changing tile parameters and many tiles are used. This seems unlikely in typical coding scenarios.

Thus the author has a preference for solution A.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).