| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27-07 APRIL , 2012 | Document: JCTVC-I0124 |
| --- | --- |

| Title: | Simplification of Golomb-Rice Parameter Update |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chanyul Kim Jaehyun Kim JungHun Park | Tel: Email: | dionism@samsung.com jhgim@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Simplification of Golomb Rice parameter update is presented in this proposal. The required table is reduced from “g_aauiGoRiceUpdate[5][24]” to “g_auiUpTbl[5]” or a formula in the software, which means that no more tables are needed. The gain -0.1% AI for E, and no loss is observed in common condition in Noral QP. -0.1 AI, 0.0 RA, 0.0 LB is obtained in low QP condition (QP 12,17,22,27). More gain (-0.2%) is observed in F-sequences.

# Introduction

The current HM uses adaptive truncated Golomb-Rice parameter update using LUT value. It allows large transition of the Golomb-Rice parameter in the update processing. It has weak-point when large variation of Golomb-Rice parameter is occurred. That is, Golomb-Rice parameter can be changed from 0 to 3 when single large coefficient is occurred. Gradual changing of Golomb-Rice parameter is allowed only by one in this proposal.

# Algorithm description

The current HM has the table based Golomb-Rice parameter update using table “g_aauiGoRiceUpdate[5][24]” or following equations.

cRiceParam = cLastRiceParam

+ (cLastSE >= 3   &&  cLastRiceParam = = 0)

+ (cLastSE >= 6   &&  cLastRiceParam <= 1)

+ (cLastSE >= 12  &&  cLastRiceParam <= 2)

+ (cLastSE >= 23  &&  cLastRiceParam <= 3)

For example, when the coding of twelve is needed, cRiceParam is also changed from 0 to 3 as shown in figure 1. However the large value of cRiceParam is not efficient when there are small coeff_abs_level_minus3 after twelve. Therefore, gradual changing is more preferable. The proposed method increases only cRiceParam by one even though large coefficient is appeared. It gives a simplification of cRiceParam update by not using min operation between the symbol and maximum value and 120 bytes table. The proposed method uses only “g_auiUpTbl[5]”.

Figure 1. The difference of coeff_abs_level_remaining between current HM and the proposed one

Figure 2. Software analysis

g_auiUpTbl[5] ={3,6,12,24,0xfffff} is chosen instead of {3,5,12,23,0xfffffff} which can be represented as a simple formula. No more tables are needed.

The pseudo code of simple formula

If uiSymbol+baseLevel > 3*(1<<ruiGoRiceParam)

ruiGoRiceParam = ruiGoRiceParam +1

# Simulation Results

In previous meeting, using cRiceParam is extended to 4 due to performance at low QP cases [1].

The proposal focus on the update process keeping cRiceParam is 4. We tested two cases which are normal QPs in common condition test and low QP as shown in Table 1. and Table 2.

Table 1. Rate Distortion performance and speed comparison for QP 22 27 32 37

Table 2.  Rate Distortion performance and speed comparison for QP 12 17 22 27

# Conclusion

The proposed method has target for simplification of Golomb Rice parameter update The required table is reduced from “g_aauiGoRiceUpdate[5][24]” to “g_auiUpTbl[5]” or a formula. The -0.1% AI for E, and no loss is observed. The run time is reduced by 4% in the decoder side. -0.1 AI, 0.0 RA, 0.0 LB is obtained in low QP condition (QP 12,17,22,27).

# References

- Jian Lou., “Improving coeff_abs_level_minus3 coding performance under high bitrate” JCT-VC document JCTVC-H0497, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 8th meeting, San Jose, February 2012.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft Change (JCTVC-H1003_dK)

#### 9.2.2.8  Binarization process for coeff_abs_level_remaining

Input to this process is a request for a binarization for the syntax element coeff_abs_level_remaining[ n ].

Output of this process is the binarization of the syntax element.

The variables cLastSE and cLastRiceParam are derived as follows.

- If n is equal to 0 or all previous syntex elements coeff_abs_level_remaining[ pos ] with pos less than n are derived to be equal to 0 instead of being explicitly parsed, cLastSE and cLastRiceParam are set equal to 0.

- Otherwise ( n is not equal to 0 ), cLastSE is set equal to sum of coeff_abs_level_remaining[ n  − 1 ] and its baseLevel. cLastRiceParam is set equal to the value of cRiceParam that has been derived during the invocation of this subclause for the syntax element coeff_abs_level_remaining[ n − 1 ] of the same transform block.

The variable cRiceParam is derived from cLastSE and cLastRiceParam equivalent to the following pseudo-code

If cLastSE > 3*(1<<cLastRiceParam)

cRiceParam = Min(cLastRiceParam +1, 4)