| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0125 |
| --- | --- |

| Title: | On Reference List Combination |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tammy Lee Jeonghoon Park | Email: | tammy.lee@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Reference list combination combines List 0 and List 1 for uni-directional prediction in B-slice to eliminate duplicated pictures in both lists and the syntax for uni-directional prediction reference index is designed to be adequate to the encoder restriction on search. In this proposal, the syntax of reference list combination is modified to be consistent with syntaxes for bi-directional prediction as the conventional video codec while maintaining non-normative part of reference list combination and the combined list specified syntaxes are proposed to be removed. The experimental shows that 0.1% gain in random access and no loss in low delay B condition when the number of context models is maintained as the same and 0.1% gain in random access main, random access he10 and low delay B condition and no loss in main low delay B he10 condition when one additional context model is used.

# Introduction

Reference list combination of List 0 and List 1 greatly reduces encoder complexity to search uni-directional prediction in List 0 and List 1 by giving the restriction on searching duplicated pictures on both lists. Its complexity reduction is proportional to the number of duplicated pictures in List 0 and List 1 and the low delay B condition in the current common test condition has most benefit from it since List 1 uni-directional prediction is inefficient in the condition where List 0 and List 1 are set to identical in the current HM.

Besides encoder complexity reduction by reference list combination for uni-directional search, several syntaxes are specialized for reference list combination in uni-directional prediction in B slice. In slice data, reference index and inter prediction mode are specialized to reflect encoder restriction by reference list combination. For example, reference index for uni-directional is the order of reference frame in the combined list even though actual reference frame is from either List 0 or List 1 while in bi-directional prediction, reference index is the order of reference frame in either List 0 or List 1. It is also noted that uni-directional prediction in P slice type follows List 0 reference list index coding while uni-directional prediction in B slice type follows combined reference list index coding.

In this proposal, it is tested to unify reference index coding for uni-directional and bi-directional prediction in B slice type while maintaining encoder search restriction by reference list combination. With this modification, there is no differentiation on reference index coding for uni-directional prediction between P and B slice types. And therefore, there is no need to convert the reference index of Pred_LC to the reference index of either Pred_L0 or Pred_L1 for consistent motion prediction and etc. Moreover, reference picture lists combination in B slice are not needed in decoding process.

# Proposal

There are two syntaxes affected in slice data: Reference index and Inter prediction mode.

In reference index coding, the branch to differentiate between uni-directional in combined list and others is removed.

In inter prediction mode coding, the following change is made and the change is the same as HM 1.0 [1].

| slice_type | inter_pred_flag | Name of inter_pred flag |  | slice_type | inter_pred_idc | Name of inter_pred idc |
| --- | --- | --- | --- | --- | --- | --- |
| P | inferred | Pred_L0 |  | P | inferred | Pred_L0 |
| B | 0 | Pred_LC |  | B | 0 | Pred_L0 |
|  | 1 | Pred_BI |  |  | 1 | Pred_L1 |
|  |  |  |  |  | 2 | Pred_BI |

Table 1. The proposed inter prediction mode change

Since the number of bins to be coded for inter prediction mode is changed from 1 to 2 in maximum, the context modeling for the second bin should be considered. The current context model index derivation is assigned by the CU depth which can have values range 0..3 for the first bin

ctxIdx = cuDepth

The following experiments are tested for the second bin coding in inter prediction mode. In the following tests, the first bin indicates whether inter prediction mode is uni-directional (value=0) or bi-directional  (value=1) prediction. The second bin is conditionally coded only if the first bin indicates uni-directional prediction and when the second bin is equal to 0, it is List 0 uni-directional and otherwise (when the second bin is equal to 1), it is List 1 uni-directional. The parsing process is the same as HM 1.0 too [1].

Parse first bin

Pred_BI

Parse second bin

Pred_L0

Pred_L1

Figure 1. The proposed inter prediction mode parsing

## Maintaining the number of context model

In this experiment, the context model index determination for the first bin is changed to save context models and assign those to the second bin. This proposed technique maintains the same number of context model for inter prediction mode coding as previous.

For the first bin, the context model index is restricted in the range of 0..2

ctxIdx = (cuDepth > 2? 2 : cuDepth)

And for the second bin, the context model index is set to 3

The following is the result of this experiment. Since combined list only affects to random access and low delay-B condition, those configurations are tested.

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class B | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | -0.1% |
| Class C | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | -0.1% | 0.1% | -0.1% | -0.2% | -0.3% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% |
| Class F | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | 0.0% |
| Class C | 0.0% | 0.1% | 0.2% | 0.0% | 0.2% | 0.2% |
| Class D | 0.1% | -0.2% | 0.2% | 0.0% | -0.1% | -0.8% |
| Class E | -0.1% | -0.4% | 0.1% | 0.0% | -0.7% | 0.0% |
| Overall | 0.0% | -0.1% | 0.1% | 0.0% | -0.2% | -0.2% |
| Class F | 0.0% | 0.2% | 0.3% | 0.0% | -0.4% | 0.4% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |

## Increase the number of context model by 1

In this experiment, the context model index determination for the first bin is unchanged and the context model index is set to 4 with additional context model for the second bin.

The following is the result of this experiment.

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% |
| Class B | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% |
| Class C | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% |
| Class D | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% |
|  | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class F | -0.1% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.3% | 0.2% | 0.0% | 0.0% | -0.2% |
| Class C | 0.0% | 0.1% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class D | 0.0% | -0.3% | 0.0% | 0.0% | -0.6% | -0.8% |
| Class E | -0.1% | 0.2% | 0.2% | -0.1% | -0.6% | -0.3% |
| Overall | 0.0% | -0.1% | 0.1% | -0.1% | -0.3% | -0.3% |
|  | 0.0% | -0.1% | 0.1% | 0.0% | -0.2% | -0.4% |
| Class F | 0.0% | -0.3% | 0.0% | 0.0% | -0.6% | 0.5% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

# Alternative configuration

In the common test configuration, (Normal) B slice type uses two reference pictures for each List 0 and List1 while (Generalized and low-delay) B slice type uses four reference pictures for both lists (which are identical). To maximize the effect of the combined list, the number of reference pictures for B slice type of non-zero temporal level is set to 4 for both List 0 and List 1 and tested under random access condition (because low delay condition already uses 4 reference pictures for both directions).

For testing under RPS, the following change is made in random access configuration:

|  | Type | POC | ref_buf_size | #ref_pics | Reference pictures | predict |
| --- | --- | --- | --- | --- | --- | --- |
| Frame1 | B | 8 | 4 | 4 | -8 -10 -12 -16 | 0 |
| Frame2 | B | 4 | 4 | 5 | -4 -6 -8 -12 4 | 0 |
| Frame3 | B | 2 | 4 | 6 | -2 -4 -6 -10 2 6 | 0 |
| Frame4 | B | 1 | 4 | 7 | -1 -3 -5 -9 1 3 7 | 0 |
| Frame5 | B | 3 | 4 | 6 | -1 -3 -5 -7 1 5 | 0 |
| Frame6 | B | 6 | 4 | 5 | -2 -4 -6 -8 2 | 0 |
| Frame7 | B | 5 | 4 | 6 | -1 -3 -5 -7 1 3 | 0 |
| Frame8 | B | 7 | 4 | 5 | -1 -3 -7 -9 1 | 0 |

The following is the result when maintaining the number of context model in this alternative configuration:

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class B | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% |
| Class C | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
|  | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |

The following is the result when using one additional context model in this alternative configuration:

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class B | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
| Class C | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% |
| Class D | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Overall | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
|  | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

It is observed that the results of alternative configuration are similar to the results of common configuration.

# Conclusion

By removing combined list, several syntaxes and decoding process are cleaned up and syntaxes are unified with P slice and Bi-directional prediction in B slice. Additionally, performance improvement is also observed as 0.1% gain in random access and no loss in low delay B condition. Therefore, it is proposed to remove normative parts of combined list while keeping non-normative parts in SW.

# Working draft modification

WD change on I0030[2] is accompanied with this proposal in the zip file, in which the version having the same number of context models is enclosed. The following is the WD difference when the number of context model is increased by 1.

## Maintaining the number of context model

Table 9-2 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values

| inter_pred_flag | Table 9-3 | 0 | (cuDepth > 2? 2 : cuDepth) | na3 | na | na | na |
| --- | --- | --- | --- | --- | --- | --- | --- |

## Increase the number of context model by 1

Table 9-3 – Values of variable initValue for inter_pred_flag ctxIdx

| Initialisation variable | inter_pred_flagidc ctxIdx |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 |
| initValue | 95 | 79 | 63 | 31 | 31 |

Table 9-4 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values

| inter_pred_flag | Table 9-3 | 0 | cuDepth | na4 | na | na | na |
| --- | --- | --- | --- | --- | --- | --- | --- |

# References

- T. Wiegand, W.-J. Han, J.-R. Ohm, and G. J. Sullivan, “High Efficiency Video Coding (HEVC) text specification Working Draft 1”, JCTVC-C403, 3th Meeting: Guangzhou, CN, 7-15 October, 2010

- B. Bross, “Suggested bug-fixes for HEVC text specification draft 6”, JCTVC-I0030, 9th Meeting: Geneva, CH, 27 April – 7 May 2012

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).