| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0125 |
| --- | --- |

| Title: | On Reference List Combination |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tammy Lee Jeonghoon Park | Email: | tammy.lee@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Reference list combination combines List 0 and List 1 for uni-directional prediction in B-slice to eliminate duplicated pictures in both lists and the syntax for uni-directional prediction reference index is designed to be adequate to the encoder restriction on search. In this proposal, syntax of reference list combination is modified to be consistent with syntaxes for bi-directional prediction as the conventional video codec while maintaining non-normative part of reference list combination. The experimental shows that 0.1% gain in random access and no loss in low delay B condition and combined list specified syntaxes are proposed to be removed.

# Introduction

Reference list combination of List 0 and List 1 greatly reduces encoder complexity to search uni-directional prediction in List 0 and List 1 by giving the restriction on searching duplicated pictures on both lists. Its complexity reduction is proportional to the number of duplicated pictures in List 0 and List 1 and the low delay B condition in the current common test condition has most benefit from it since List 1 uni-directional prediction is inefficient in the condition where List 0 and List 1 are set to identical in the current HM.

Besides encoder complexity reduction by reference list combination for uni-directional search, several syntaxes are specialized for reference list combination in uni-directional prediction in B slice. In slice data, reference index and inter prediction mode are specialized to reflect encoder restriction by reference list combination. For example, reference index for uni-directional is the order of reference frame in the combined list even though actual reference frame is from either List 0 or List 1 while in bi-directional prediction, reference index is the order of reference frame in either List 0 or List 1. It is also noted that uni-directional prediction in P slice type follows List 0 reference list index coding while uni-directional prediction in B slice type follows combined reference list index coding.

In this proposal, it is tested to unify reference index coding for uni-directional and bi-directional prediction in B slice type while maintaining encoder search restriction by reference list combination. With this modification, there is no differentiation on reference index coding for uni-directional prediction between P and B slice types. And therefore, there is no need to convert the reference index of Pred_LC to the reference index of either Pred_L0 or Pred_L1 for consistent motion prediction and etc. Moreover, reference picture lists combination in B slice are not needed in decoding process.

# Proposal

There are two syntaxes affected in slice data: Reference index and Inter prediction mode.

In reference index coding, the branch to differentiate between uni-directional in combined list and others is removed.

In inter prediction mode coding, the following change is made and the change is the same as HM 1.0 [1].

| slice_type | inter_pred_flag | Name of inter_pred flag |  | slice_type | inter_pred_idc | Name of inter_pred idc |
| --- | --- | --- | --- | --- | --- | --- |
| P | inferred | Pred_L0 |  | P | inferred | Pred_L0 |
| B | 0 | Pred_LC |  | B | 0 | Pred_L0 |
|  | 1 | Pred_BI |  |  | 1 | Pred_L1 |
|  |  |  |  |  | 2 | Pred_BI |

Table 1. The proposed inter prediction mode change

Since the number of bins to be coded for inter prediction mode is changed from 1 to 2 in maximum, the context modeling for the second bin should be considered. The current context model index derivation is assigned by the CU depth which can have values range 0..3 for the first bin

ctxIdx = cuDepth

The following experiments are tested for the second bin coding in inter prediction mode. In the following tests, the first bin indicates whether inter prediction mode is uni-directional (value=0) or bi-directional  (value=1) prediction. The second bin is conditionally coded only if the first bin indicates uni-directional prediction and when the second bin is equal to 0, it is List 0 uni-directional and otherwise (when the second bin is equal to 1), it is List 1 uni-directional. The parsing process is the same as HM 1.0 too [1].

Parse first bin

Pred_BI

Parse second bin

Pred_L0

Pred_L1

Figure 1. The proposed inter prediction mode parsing

## Maintaining the number of context model

In this experiment, the context model index determination for the first bin is changed to save context models and assign those to the second bin. This proposed technique maintains the same number of context model for inter prediction mode coding as previous.

For the first bin, the context model index is restricted in the range of 0..2

ctxIdx = (cuDepth > 2? 2 : cuDepth)

And for the second bin, the context model index is set to 3

The following is the result of this experiment. Since combined list only affects to random access and low delay-B condition, those configurations are tested.

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class B | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | -0.1% |
| Class C | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | -0.1% | 0.1% | -0.1% | -0.2% | -0.3% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% |
| Class F | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | 0.0% |
| Class C | 0.0% | 0.1% | 0.2% | 0.0% | 0.2% | 0.2% |
| Class D | 0.1% | -0.2% | 0.2% | 0.0% | -0.1% | -0.8% |
| Class E | -0.1% | -0.4% | 0.1% | 0.0% | -0.7% | 0.0% |
| Overall | 0.0% | -0.1% | 0.1% | 0.0% | -0.2% | -0.2% |
| Class F | 0.0% | 0.2% | 0.3% | 0.0% | -0.4% | 0.4% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |

## Increase the number of context model by 1

In this experiment, the context model index determination for the first bin is unchanged and the context model index is set to 4 with additional context model for the second bin.

The following is the result of this experiment.

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class B | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% |
| Class C | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | -0.1% |
| Class F | -0.1% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% | 0.2% |
| Class D | 0.0% | -0.1% | 0.4% | -0.1% | -0.3% | -0.4% |
| Class E | 0.0% | 0.1% | -0.2% | -0.2% | -0.2% | 0.0% |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | -0.1% |
| Class F | -0.1% | -0.3% | 0.1% | 0.0% | 0.3% | 0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |

# Conclusion

Since removing combined list specified syntaxes and decoding process, 0.1% gain in random access and no loss in low delay B condition are observed. By this way, the syntax is clean up and unified with P slice and Bi-directional prediction in B slice. Therefore, it is proposed to remove normative parts of combined list while keeping non-normative parts in SW.

# Working draft modification

Proposed working draft text is included below using the following text conventions:

- Removed text from JCTVC-H1003_dK is shown with strikethrough.

- Newly added text is shown in red.

- Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( lists_modification_present_flag ) { |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| … |  |

- Reference picture lists combination syntax

| ref_pic_list_combination( ) { | Descriptor |
| --- | --- |
| if( slice_type = = B ) { |  |
| ref_pic_list_combination_flag | u(1) |
| if( ref_pic_list_combination_flag ) { |  |
| num_ref_idx_lc_active_minus1 | ue(v) |
| ref_pic_list_modification_flag_lc | u(1) |
| if( ref_pic_list_modification_flag_lc ) |  |
| for( i =0; i <= num_ref_idx_lc_active_minus1; i++ ) { |  |
| pic_from_list_0_flag | u(1) |
| if( ( pic_from_list_0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !pic_from_list_0_flag && num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_list_curr | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |

- Prediction weight table syntax

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| … |  |
| if( slice_type = = B ) { |  |
| if( ref_pic_list_combination_flag = = 0 ) { |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) { |  |
| luma_weight_l1_flag | u(1) |
| if( luma_weight_l1_flag ) { |  |
| delta_luma_weight_l1[ i ] | se(v) |
| luma_offset_l1[ i ] | se(v) |
| } |  |
| if( chroma_format_idc != 0 ) { |  |
| chroma_weight_l1_flag | u(1) |
| if( chroma_weight_l1_flag ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l1[ i ][ j ] | se(v) |
| delta_chroma_offset_l1[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } else { |  |
| for( i = 0; i <= num_ref_idx_lc_active_minus1; i++ ) { |  |
| luma_weight_lc_flag | u(1) |
| if( luma_weight_l1_flag ) { |  |
| delta_luma_weight_lc[ i ] | se(v) |
| luma_offset_lc[ i ] | se(v) |
| } |  |
| if( chroma_format_idc != 0 ) { |  |
| chroma_weight_lc_flag | u(1) |
| if( chroma_weight_lc_flag ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_lc[ i ][ j ] | se(v) |
| delta_chroma_offset_lc[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

- Prediction unit syntax

| prediction_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| … |  |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_flag[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_LC ) { |  |
| if( num_ref_idx_lc_active_minus1 > 0 ) |  |
| ref_idx_lc[ x0 ][ y0 ] | ae(v) |
| mvd_coding(mvd_lc[ x0 ][ y0 ][ 0 ], mvd_lc[ x0 ][ y0 ][ 1 ]) |  |
| mvp_lc_flag[ x0 ][ y0 ] | ae(v) |
| } else { /* Pred_L0 or Pred_BI */ |  |
| if( inter_pred_flag[ x0 ][ y0 ] ! = Pred_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding(mvd_l0[ x0 ][ y0 ][ 0 ], mvd_l0[ x0 ][ y0 ][ 1 ]) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_BI ) { |  |
| if( inter_pred_flag[ x0 ][ y0 ] ! = Pred_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag ) { |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] = 0 |  |
| mvd_l1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } else |  |
| mvd_coding( mvd_l1[ x0 ][ y0 ][ 0 ], mvd_l1[ x0 ][ y0 ][ 1 ] ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

inter_pred_flag[ x0 ][ y0 ] specifies whether uni-prediction, list 0, list 1, or bi-prediction is used for the current prediction unit according to Table 7-2. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

Table 7-2 – Name association to inter prediction mode

| slice_type | inter_pred_flag | Name of inter_pred_flag |
| --- | --- | --- |
| P | inferred | Pred_L0 |
| B | 0 | Pred_LC Pred_L0 |
|  | 1 | Pred_L1 |
|  | 12 | Pred_BI |

When inter_pred_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to Pred_L0 when slice_type is equal to P and Pred_BI when slice_type is equal to B.

      - Mapping process for reference picture lists combination in B slices

This initialization process is invoked when decoding a B slice header.

Input to this process are the reference picture list RefPicListX and num_ref_idx_lX_active_minus1 with X being 0 or 1.

Outputs of this process are arrays PredLCToPredLx and RefIdxLCToRefIdxLx.

When the current slice is a B slice and ref_pic_list_modification_flag_lc is equal to 0, the following ordered steps apply:

      - Let refIdxL0 and refIdxL1 be indices into the reference picture lists RefPicListL0 and RefPicListL1. They are initially set equal to 0.

      - Let refIdxLC be an index into PredLCToPredLx and RefIdxLCToRefIdxLx. It is initially set equal to 0.

      - The following process is repeated until refIdxL0 and refIdxL1 are both greater than num_ref_idx_l0_active_minus1 and num_ref_idx_l1_active_minus1, respectively:

–	If refIdxL0 is less than or equal to num_ref_idx_l0_active_minus1,

–	If the entry RefPicListL0[ refIdxL0 ] is the first occurance of the reference picture,

PredLCToPredLx[ refIdxLC ] = Pred_L0, 	(8-1)
			RefIdxLCToRefIdxLx[ refIdxLC++ ] = refIdxL0.

–	refIdxL0++.

–	If refIdxL1 is less than or equal to num_ref_idx_l1_active_minus1 and ref_pic_list_combination_flag equal to 1,

–	If the entry RefPicListL1[ refIdxL1 ] is the first occurance of the reference picture,

PredLCToPredLx[ refIdxLC ] = Pred_L1, 	(8-2)			RefIdxLCToRefIdxLx[ refIdxLC++ ] = refIdxL1.

–	refIdxL1++.

When the current slice is a B slice and ref_pic_list_modification_flag_lc is equal to 1, the following ordered steps apply:

- Let refIdxLC be an index into the reference picture list PredLCToPredLx and RefIdxLCToRefIdxLx. It is initially set equal to 0.

- The corresponding syntax elements pic_from_list_0_flag and ref_idx_list_curr are processed in the order they occur in the bitstream. For each of these syntax elements pairs, the following applies.

–	If pic_from_list_0_flag is equal to 1,

PredLCToPredLx[ refIdxLC ] = Pred_L0, 	(8-3)

–	Otherwise,

PredLCToPredLx[ refIdxLC ] = Pred_L1	(8-4)

–	RefIdxLCToRefIdxLx[ refIdxLC++ ] = ref_idx_list_curr

When refIdxLC is greater than num_com_ref_list_active_minus1+ 1, the extra entries past position num_com_ref_list_active_minus1 are discarded from PredLCToPredLx and RefIdxLCToRefIdxLx.

When refIdxLC is less than num_com_ref_list_active_minus1 + 1, the remaining entries in PredLCToPredLx and RefIdxLCToRefIdxLx are set equal to Pred_L0 and 0, respectively.

          - Weighted sample prediction process

Inputs to this process are:

–	the same as specified in subclause Error: Reference source not found.

–	variables for weighted prediction logWDC, logWDC, w0C, w1C, o0C, o1C with C being replaced by L for luma samples and, when chroma_format_idc is not equal to 0, Cb and Cr for chroma samples.

Outputs of this process are:

–	the same as specified in subclause Error: Reference source not found.

The prediction sample predSamples[ x, y ] with x = 0..(nPSW)−1 and y = 0..(nPSH)−1, with H being replaced by Y for luma samples and by C for chroma samples are derived as follows:

–	If the predFlagL0 is equal to 1 and predFlagL1 is equal to 0, the prediction samples are derived as follows:

if( logWDC >= 1 ) 
	predSamples[ x, y ] = Clip1H( ( ( predSamplesL0 [ x, y ] * w0C + 2logWDC − 1 ) >> logWDC ) + o0C ) 	 (8-5)
else
	predSamples[ x, y ] = Clip1H( predSamplesL0 [ x, y ] * w0C + o0C ) 	(8-6)

–	Otherwise, if the predFlagL0 is equal to 0 and predFlagL1 is equal to 1, the final predicted sample values predSamples [ x, y ] are derived by

if( logWDC >= 1 )
	predSamples[ x, y ] = Clip1H( ( ( predSamplesL1 [ x, y ] * w1C + 2logWDC − 1 ) >> logWDC ) + o1C ) 	(8-7)
else
	predSamples[ x, y ] = Clip1H( predSamplesL1 [ x, y ] * w1C + o1C ) 	(8-8)

–	Otherwise (both predFlagL0 and predFlagL1 are equal to 1), if RefPicOrderCnt( currPic, refIdxL0, L0) is equal to RefPicOrderCnt( currPic, refIdxL1, L1) and mvL0 is equal to mvL1, the final predicted sample values predSamples [ x, y ] are derived by

predSamples[ x, y ] = Clip1H( ( predSamplesL0 [ x, y ] * ( w0C + w1C ) + 
			 ( ( o0C + o1C + 1 ) << logWDC ) ) >> ( logWDC + 1 ) )	(8-9)

–	Otherwise, the final predicted sample values predSamples[ x, y ] are derived by

predSamples[ x, y ] = Clip1H( ( predSamplesL0 [ x, y ] * w0C + predSamplesL1 [ x, y ] * w1C + 
			 ( ( o0C + o1C + 1 ) << logWDC ) ) >> ( logWDC + 1 ) )	(8-10)

Where the variables logWDC, o0C, o1C, and w0C, w1C are derived as follows.

–	If weighted_bipred_idc is equal to 2 in B-slices, implicit mode weighted prediction is used as follows:

logWDc = 5+shift1		(8-11)

o0C = 0				(8-12)

o1C = 0				(8-13)

The variable WeightScaleFactor is derived from the values currPoc, refIdxL0 and refIdxL1 as follows:

tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListL0( refIdxL0 ) ) )	(8-14)

td = Clip3( −128, 127, PicOrderCnt( RefPicListL1( refIdxL1 ) ) 
		 – PicOrderCnt( RefPicListL0( refIdxL0 ) ) )	(8-15)

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td	(8-16)

WeightScaleFactor = Clip3( −1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-17)

The variables w0C and w1C are derived as follows.

–	If PicOrderCnt( RefPicListL0( refIdxL0 ) ) is equal to PicOrderCnt( RefPicListL1( refIdxL1 ) ) or ( WeightScaleFactor >> 2 ) < −64 or ( WeightScaleFactor >> 2 ) > 128, the following applies.

w0C=32				(8-18)

w1C=32				(8-19)

–	Otherwise;

w0C = 64 – (WeightScaleFactor >> 2)		(8-20)

w1C = WeightScaleFactor >> 2		(8-21)

–	Otherwise, if both weighted_bipred_idc and ref_pic_list_combination_flag are equal to 1 in B slice, combined explicit mode weighted prediction is used as follows:

–	If C is equal to L for luma samples,

logWDc = luma_log2_weight_denom+ shift1		(8-22)

w0C = LumaWeightLC[ RefIdxLCToRefIdxLx[ refIdxL0 ] ]	(8-23)

w1C = LumaWeightLC[ RefIdxLCToRefIdxLx[ refIdxL1 ] ]	(8-24)

o0C = luma_offset_lc[ RefIdxLCToRefIdxLx[ refIdxL0 ] ] * ( 1 << ( BitDepthY − 8 ) )	(8-25)

o1C = luma_offset_lc[ RefIdxLCToRefIdxLx[ refIdxL1 ] ] * ( 1 << ( BitDepthY − 8 ) )	(8-26)

–	Otherwise (C is equal to Cb or Cr for chroma samples, with iCbCr = 0 for Cb, iCbCr = 1 for Cr),

logWDc = ChromaLog2WeightDenom + shift1	(8-27)

w0C = ChromaWeightLC[ RefIdxLCToRefIdxLx[ refIdxL0 ] ][ iCbCr ]	(8-28)

w1C = ChromaWeightLC[ RefIdxLCToRefIdxLx[ refIdxL1 ] ][ iCbCr ]	(8-29)

o0C = ChromaOffsetLC[ RefIdxLCToRefIdxLx[ refIdxL0 ] ][ iCbCr ] * ( 1 << ( BitDepthC − 8 ) )	(8-30)

o1C = ChromaOffsetLC[ RefIdxLCToRefIdxLx[ refIdxL1 ] ][ iCbCr ] * ( 1 << ( BitDepthC − 8 ) )	(8-31)

–	Otherwise (weighted_pred_flag is equal to 1 in P slice or ref_pic_list_combination_flag is equal to 0 in B-slice) explicit mode weighted prediction is used as follows:

–	If C is equal to L for luma samples,

logWDc = luma_log2_weight_denom+ shift1		(8-32)

w0C = LumaWeightL0[refIdxL0]		(8-33)

w1C = LumaWeightL1[refIdxL1]		(8-34)

o0C = luma_offset_l0[refIdxL0] * ( 1 << ( BitDepthY − 8 ) )	(8-35)

o1C = luma_offset_l1[refIdxL1] * ( 1 << ( BitDepthY − 8 ) )	(8-36)

–	Otherwise (C is equal to Cb or Cr for chroma samples, with iCbCr = 0 for Cb, iCbCr = 1 for Cr),

logWDc = ChromaLog2WeightDenom + shift1	(8-37)

w0C = ChromaWeightL0[refIdxL0][ iCbCr ]		(8-38)

w1C = ChromaWeightL1[refIdxL1][ iCbCr ]		(8-39)

o0C = ChromaOffsetL0[refIdxL0][ iCbCr ] * ( 1 << ( BitDepthC − 8 ) )	(8-40)

o1C = ChromaOffsetL1[refIdxL1][ iCbCr ] * ( 1 << ( BitDepthC − 8 ) )	(8-41)

- Derivation process for motion vector components and reference indices

…

- The variables LcToLx, refIdxLX and predFlagLX are derived as follows.

- If inter_pred_flag[ xP ][ yP ] is equal to Pred_LC and PredLCToPredLx[ ref_idx_lc[ xP ][ yP ] ] is equal to Pred_LX,

refIdxLX = RefIdxLCToRefIdxLx[ ref_idx_lc[ xP ][ yP ] ]	(8-42)					predFlagLX = 1		(8-43)					mvd_lX[ xP ][ yP ][ 0 ] = mvd_lc[ xP ][ yP ][ 0 ]	(8-44)
					mvd_lX[ xP ][ yP ][ 1 ] = mvd_lc[ xP ][ yP ][ 1 ]	(8-45)
					mvp_lX_flag[ xP ][ yP ][ 0 ] = mvp_lc_flag[ xP ][ yP ]	(8-46)
					LcToLx = LX		(8-47)

- Otherwise, if inter_pred_flag[ xP ][ yP ] is equal to Pred_LX or Pred_BI,

refIdxLX = ref_idx_lX[ xP ][ yP ]		(8-48)
					predFlagLX = 1		(8-49)

- Otherwise, the variables refIdxLX and predFlagLX are specified by

refIdxLX = −1		(8-50)
					predFlagLX = 0		(8-51)

…

| Table 9-3 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element |  | initialisationType |  | Type of binarization |  |  | maxBinIdxCtx | ctxIdxTable |  | ctxIdxOffset |  |  |
| inter_pred_flag |  | 1 |  | FL, cMax = 12 |  |  | 0 | Error: Reference source not found |  | 0 |  |  |
|  |  | 2 |  |  |  |  | 0 | Error: Reference source not found |  | 0 |  |  |
| ref_idx_lc |  | 1 |  | TU, cMax = num_ref_idx_lC_active_minus1 |  |  | 2 | Error: Reference source not found |  | 0 |  |  |
|  |  | 2 |  |  |  |  | 2 | Error: Reference source not found |  | 3 |  |  |
| Table 9-4 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |  |  |  |  |  |
| Syntax element | ctxIdxTable, ctxIdxOffset |  |  |  | binIdx |  |  |  |  |  |  |  |
|  |  |  |  |  | 0 | 1 |  |  | 2 |  | 3 | >=4 |
| inter_pred_flag | Error: Reference source not found |  | 0 |  | (cuDepth > 2? 2 : cuDepth) | na3 |  |  | na |  | na | na |

# References

- T. Wiegand, W.-J. Han, J.-R. Ohm, and G. J. Sullivan, “High Efficiency Video Coding (HEVC) text specification Working Draft 1”, JCTVC-C403, 3th Meeting: Guangzhou, CN, 7-15 October, 2010

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, and T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 6”, JCTVC-H1003, 8th Meeting: San José, CA, USA, 1–10 February, 2012

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).