| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0130 |
| --- | --- |

| Title: | Simplification on SAO syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tammy Lee Elena Alshina Jeonghoon Park | Email: | tammy.lee@samsung.com elena_a.alshina@samsung.net jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

There are two coding modes for SAO parameters: APS mode and interleaving mode. In this proposal, syntaxes regarding one unit flags and repeat row flags were tested in APS mode. In the case of LCU size equal to 64, removing either one unit flags or repeat row flags gives 0.0% drop in RA main and 0.0% drop in RA HE10 and removing both gives 0.1% drop in RA main and 0.1% drop in RA HE10.

# Introduction

In APS mode for SAO parameters signalling, there are special flags: “one unit flags (sao_one_luma_unit_flag, sao_one_cb_unit_flag, sao_one_cr_unit_flag)” to signal whether all the parameters are the same within a picture and “repeat row flags (sao_repeat_row_flag[0], sao_repeat_row_flag[1], sao_repeat_row_flag[2])” to signal whether one LCU row’s parameters are the same as the above LCU row.

      - Sample adaptive offset parameter syntax

| aps_sao_param( ) { | Descriptor |
| --- | --- |
| sao_cb_enable_flag | u(1) |
| sao_cr_enable_flag | u(1) |
| sao_num_lcu_in_width_minus1 | ue(v) |
| sao_num_lcu_in_height_minus1 | ue(v) |
| sao_one_luma_unit_flag | u(1) |
| if(sao_one_luma_unit_flag) |  |
| sao_offset_vlc( 0, 0, 0 ) |  |
| if( sao_cb_enable_flag ){ |  |
| sao_one_cb_unit_flag | u(1) |
| if( sao_one_cb_unit_flag ) |  |
| sao_offset_vlc( 0, 0, 1 ) |  |
| } |  |
| if( sao_cr_enable_flag ){ |  |
| sao_one_cr_unit_flag | u(1) |
| if( sao_one_cr_unit_flag ) |  |
| sao_offset_vlc( 0, 0, 2 ) |  |
| } |  |
| for( ry = 0; ry <= sao_num_lcu_in_height_minus1; ry++ ) { |  |
| for( rx = 0; rx <= sao_num_lcu_in_width_minus1; rx++ ) { |  |
| if( aps_sample_adaptive_offset_flag && ! sao_one_luma_unit_flag ) { |  |
| if( ry > 0 && rx = = 0 ) |  |
| sao_repeat_row_flag[ 0 ] | u(1) |
| sao_unit_vlc(rx, ry, 0 ) |  |
| } |  |
| if( sao_cb_enable_flag && ! sao_one_cb_unit_flag ) { |  |
| if( ry > 0 & & rx = = 0 ) |  |
| sao_repeat_row_flag[ 1 ] | u(1) |
| sao_unit_vlc(rx, ry, 1 ) |  |
| } |  |
| if( sao_cr_enable_flag & & !sao_one_cr_unit_flag ) { |  |
| if( ry > 0 & & rx = = 0 ) |  |
| sao_repeat_row_flag[ 2 ] | u(1) |
| sao_unit_vlc( rx, ry, 2 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

sao_one_luma_unit_flag equal to 1 specifies that all of coding treeblocks in luma component of current slice are processed using the same SAO parameters; equal to 0 specifies that the SAO parameters for the luma component are signalled on a coding treeblock basis.

sao_one_cb_unit_flag equal to 1 specifies that all of coding treeblocks in Cb component of current slice are processed using the same SAO parameters; equal to 0 specifies that the SAO parameters for the Cb component are signalled on a coding treeblock basis.

sao_one_cr_unit_flag equal to 1 specifies that all of coding treeblocks in Cr component of current slice are processed using the same SAO parameters; equal to 0 specifies that the SAO parameters for the Cr component are signalled on a coding treeblock basis.

sao_repeat_row_flag[ cIdx ] equal to 1 specifies that the SAO parameters of coding treeblocks in the current coding treeblock row are the same as those of above coding treeblocks for the colour component cIdx; equal to 0 specifies that the current coding treeblock row has new SAO parameters. When sao_repeat_row_flag is not present, it is inferred to be equal to 0.

# Experimental results

The experiments in this proposal uses the CE1 SW where the most right and bottom pixels (which are modified during deblocking filter) are not used for statistics collection as the base SW and follows the CE1 test condition. The anchor data is SAO off and the test conditions are RA Main and RA HE10 for LCU size of 64, 32 and 16.

The following results summarize overall performance difference from SAO on case.

|  | No One Unit Flags |  | No Repeat Row Flags |  | No One Unit & No Repeat Row |  |
| --- | --- | --- | --- | --- | --- | --- |
| LCU Size | RA_Main | RA_HE10 | RA_Main | RA_HE10 | RA_Main | RA_HE10 |
| 64 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 |
| 32 | 0.1 | 0.1 | 0.2 | 0.1 | 0.3 | 0.2 |
| 16 | 0.1 | 0.1 | 0.5 | 0.3 | 0.7 | 0.6 |

# Conclusion

Since there is negligible performance drop by removing “one unit flags” while keeping syntax as clean, it is proposed to remove one unit flags. However, “repeat row flags” shows coding efficiency in small LCU sizes.

# Working draft modification

Proposed working draft text is included below using the following text conventions:

- Removed text from JCTVC-H1003_dK is shown with strikethrough.

- Newly added text is shown in red.

      - Sample adaptive offset parameter syntax

| aps_sao_param( ) { | Descriptor |
| --- | --- |
| sao_cb_enable_flag | u(1) |
| sao_cr_enable_flag | u(1) |
| sao_num_lcu_in_width_minus1 | ue(v) |
| sao_num_lcu_in_height_minus1 | ue(v) |
| sao_one_luma_unit_flag | u(1) |
| if(sao_one_luma_unit_flag) |  |
| sao_offset_vlc( 0, 0, 0 ) |  |
| if( sao_cb_enable_flag ){ |  |
| sao_one_cb_unit_flag | u(1) |
| if( sao_one_cb_unit_flag ) |  |
| sao_offset_vlc( 0, 0, 1 ) |  |
| } |  |
| if( sao_cr_enable_flag ){ |  |
| sao_one_cr_unit_flag | u(1) |
| if( sao_one_cr_unit_flag ) |  |
| sao_offset_vlc( 0, 0, 2 ) |  |
| } |  |
| for( ry = 0; ry <= sao_num_lcu_in_height_minus1; ry++ ) { |  |
| for( rx = 0; rx <= sao_num_lcu_in_width_minus1; rx++ ) { |  |
| if( aps_sample_adaptive_offset_flag && ! sao_one_luma_unit_flag ) { |  |
| if( ry > 0 && rx = = 0 ) |  |
| sao_repeat_row_flag[ 0 ] | u(1) |
| sao_unit_vlc(rx, ry, 0 ) |  |
| } |  |
| if( sao_cb_enable_flag && ! sao_one_cb_unit_flag ) { |  |
| if( ry > 0 & & rx = = 0 ) |  |
| sao_repeat_row_flag[ 1 ] | u(1) |
| sao_unit_vlc(rx, ry, 1 ) |  |
| } |  |
| if( sao_cr_enable_flag & & !sao_one_cr_unit_flag ) { |  |
| if( ry > 0 & & rx = = 0 ) |  |
| sao_repeat_row_flag[ 2 ] | u(1) |
| sao_unit_vlc( rx, ry, 2 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

sao_one_luma_unit_flag equal to 1 specifies that all of coding treeblocks in luma component of current slice are processed using the same SAO parameters; equal to 0 specifies that the SAO parameters for the luma component are signalled on a coding treeblock basis.

sao_one_cb_unit_flag equal to 1 specifies that all of coding treeblocks in Cb component of current slice are processed using the same SAO parameters; equal to 0 specifies that the SAO parameters for the Cb component are signalled on a coding treeblock basis.

sao_one_cr_unit_flag equal to 1 specifies that all of coding treeblocks in Cr component of current slice are processed using the same SAO parameters; equal to 0 specifies that the SAO parameters for the Cr component are signalled on a coding treeblock basis.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).