| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0133 |
| --- | --- |

| Title: | POC signalling for CRA picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Chanyul Kim Jaehyun Kim Jeonghoon Park | Email: | b.d.choi@samsung.com dionism@samsung.com jhgim@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document proposes to signal the POC MSB explicitly in CRA pictures for error robustness, random access detection, and multiview extension. By signalling the cycle of POC MSB in the slice header of CRA pictures, the occurrence of frame loss or random access can be detected by comparing the MSB inferred from the previously decoded pictures and the explicitly signalled MSB, and then the POC MSB can be corrected if the POC MSB is corrupted.

In consideration of the multiview extension of HEVC, especially, if the POC MSBs of non-base views are reset to zero when the view-point switching or layer switching occurs in CRA pictures, the POC MSB of the non-base view could be different from the POC MSB of the base view, because the POC MSB of the base view will increase continuously in a bit-stream. By explicitly signalling the POC MSB value in CRA picture, the mismatch of POC values between the base view and the enhancement views can be prevented. It is asserted that the explicit signalling of POC MSB in CRA picture would have a negligible effect on coding efficiency.

# Problem

Picture order count (POC) is used to identify pictures, for deriving motion parameters in temporal or spatial direct mode, to represent picture order differences between pictures for motion vector derivation, for implicit mode weighted prediction in B slices, and for decoder conformance checking. If we further consider the multiview extension, the POC is used for confirming the corresponding reference frame for the inter-view prediction.

In the current draft of HEVC, the decoder calculates POC MSB values from the POC MSB value of the previously decoded picture and the difference between the previous POC LSB and the current POC LSB. The loss of some pictures can result in the corruption of POC MSB values, causing out-of-order playback and incorrect reference picture list construction. At least, the corruption of POC MSB vales should be detected and corrected in random access points.

In another aspect, when a random access happens in CRA pictures, a system should notify the decoder to discard all leading pictures without any attempt to decode them. In stand-alone, the decoder cannot detect the occurrence of random access.

Figure 1. POC mismatch in multiview extension

In consideration of the multiview or future scalable extension of HEVC, the POC of each view/layer can be calculated independently from the MSB inferred from the previous MSB and the explicitly signalled LSB of each view/layer. Whether the POC of each view/layer is calculated independently or the POC of the non-base view/layer is inferred from the base view/layer is depending on how to design the extension specification. At least, however, the POC of each view is calculated independently in H.264/MVC and the current design of HEVC 3D extension in MPEG 3DV [1].

While the base view or layer is continually decoded from the start of a bit-stream, non-base views/layers can be selectively transmitted/extracted and decoded according to the network status or the special quality of devices. Then, if non-base views/layers start newly from CRA pictures in a middle of the bit-stream for view-point switching or layer switching, their POC MSBs would have a different value from the POC MSB of the base view. For example, the POC MSB of the base layer is k (non-zero) while the POC MSBs of the non-base layers are zero in Figure 1. It may cause incorrect inter-view/layer prediction and asynchronous outputs of decoded pictures of each view/layer.

# Proposal

To solve those problems, our proposal is to explicitly signal the POC MSB for CRA pictures. In the slice header of the CRA picture, the cycle of POC MSB (pic_order_cnt_msb_cycle) is signalled. From the cycle of POC MSB, the POC MSB can be directly calculated. By comparing the MSB calculated from the pic_order_cnt_msb_cycle and the POC MSB inferred from the previous POC MSB, the frame loss or the random access can be detected, because those POC MSBs should be same each other without the frame loss or the occurrence of random access. If different, the current POC MSB shall be corrected by the MSB calculated from the cycle of MSB signalled in slice headers. It is asserted that error propagation should be stopped at random access points and the explicit signalling of POC MSB in CRA pictures would have a negligible effect on coding efficiency. For random access, the detection of random access in CRA pictures without a notification from systems is helpful for discarding unused leading pictures at decoders.

In the multiview/scalable extension of HEVC, the POC MSBs of enhancement views/layers could be different from the MSB of base view/layer when the layer switching or view-point switching happens in CRA pictures. If the difference between the POC MSB inferred from the previous POC MSB and the POC MSB signalled explicitly is detected, then the POC MSBs of enhancement view/layers are corrected by using the cycle of POC MSB (pic_order_cnt_msb_cycle) explicitly signalled in slice headers. The same POC value in the pictures of all view/layers in an access unit are mandatory for the inter-view/layer prediction and synchronized outputs.

# Changes to Specification Text

We propose to change the specification text as follows:

| slice_header( ) { | Descriptor |
| --- | --- |
| …………… | u(1) |
| if( IdrPicFlag ) { |  |
| idr_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } else { |  |
| if( nal_unit_type == 4 ) |  |
| pic_order_cnt_msb_cycle | u(v) |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| …………… |  |
| } |  |

pic_order_cnt_msb_cycle is used to determine the value of the most significant bits of the picture order count value of the CRA.

Current specification text on POC MSB:

PicOrderCntMsb of the current picture is derived as specified by the following pseudo-code:

if( ( pic_order_cnt_lsb <  prevPicOrderCntLsb )  &&
		( ( prevPicOrderCntLsb − pic_order_cnt_lsb )  >=  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb + MaxPicOrderCntLsb	(8-1)
else if( (pic_order_cnt_lsb  >  prevPicOrderCntLsb )  &&
		( (pic_order_cnt_lsb − prevPicOrderCntLsb )  >  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb − MaxPicOrderCntLsb
else
	PicOrderCntMsb = prevPicOrderCntMsb

Proposed specification text on POC MSB:

PicOrderCntMsb of the current picture is derived as follows

- If the current picture is a CDR picture,

PicOrderCntMsb = pic_order_cnt_msb_cycle* MaxPicOrderCntLsb

- Otherwise as specified by the following pseudo-code:

if( ( pic_order_cnt_lsb <  prevPicOrderCntLsb )  &&
		( ( prevPicOrderCntLsb − pic_order_cnt_lsb )  >=  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb + MaxPicOrderCntLsb	(8-2)
else if( (pic_order_cnt_lsb  >  prevPicOrderCntLsb )  &&
		( (pic_order_cnt_lsb − prevPicOrderCntLsb )  >  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb − MaxPicOrderCntLsb
else
	PicOrderCntMsb = prevPicOrderCntMsb

# References

- Heiko Schwarz, Krzysztof Wegner, “Test Model under Consideration for HEVC based 3D video coding”, document N12559, Feb. 2012.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).