| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0135_r1 WG11 Number: m24374 |
| --- | --- |

| Title: | AHG15: Modification on picture marking process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Toshiyasu Sugio Takahiro Nishi 1006 Kadoma, Kadoma Osaka, Japan Sue Mon Thet Naing ChongSoon Lim Block 1022 Tai Seng Avenue #06-3530 Singapore 534415 | Tel: Email: | +81-50-3587-2047 { sugio.toshiyasu, nishi.takahiro }@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

In this contribution, a mismatch on picture marking process between CD text and HM6.1 software was reported. In order to avoid the mismatch, it was proposed to ignore short tem picture assignment for a picture which got duplicated assignment of long term and short term reference by RPS syntax at the same time. Moreover, it was proposed to replace the parsing process on list_entry_lX parameters with the one which is independent from the variable NumPocTotalCurr.

# Problem Statement

In current CD text, a picture marking process is described in section 8.3.2 such as below. In this process, a long term picture marking process is conducted in step 1 and 2 at first. After that, a short term picture marking process is conducted in step 3 and 4. According to this process in CD text, if one picture is assigned to both long term picture and short term picture at the same time, “no reference picture” would be assigned to either RefPicSetStCurrBefore or RefPicSetStCurrAfter in step 3, and then it would not satisfy the bitstream conformance which is highlighted by yellow color.

----------- section 8.3.2 Decoding process for reference picture set -----------

The derivation process for the reference picture set and picture marking are performed according to the following ordered steps, where DPB refers to the decoded picture buffer as described in Annex Error: Reference source not found:

- The following applies:

for( i = 0; i < NumPocLtCurr; i++ ) {
	if( !delta_poc_msb_present_flag[ i ] ) {
		if( there is a long-term reference picture picX in the DPB 
				with pic_order_cnt_lsb equal to PocLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picX
		else if( there is a short-term reference picture picY in the DPB
				with pic_order_cnt_lsb equal to PocLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picY
		else 
			RefPicSetLtCurr[ i ] = "no reference picture"
	} else { 
		if( there is a long-term reference picture picX in the DPB
				with PicOrderCntVal equal to PocLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picX
		else if( there is a short-term reference picture picY in the DPB
				with PicOrderCntVal equal to PocLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picY
		else 
			RefPicSetLtCurr[ i ] = "no reference picture"
	}
}																						(8-1)

for( i = 0; i < NumPocLtFoll; i++ ) {
	if( !delta_poc_msb_present_flag[ i ] ) {
		if( there is a long-term reference picture picX in the DPB
					with pic_order_cnt_lsb equal to PocLtFoll[ i ] )
				RefPicSetLtFoll[ i ] = picX
		else if( there is a  short-term reference picture picY in the DPB
					with pic_order_cnt_lsb equal to PocLtFoll[ i ] )
				RefPicSetLtFoll[ i ] = picY
		else 
				RefPicSetLtFoll[ i ] = "no reference picture"
	} else {
		if( there is a long-term reference picture picX in the DPB
					with PicOrderCntVal to PocLtFoll[ i ] )
				RefPicSetLtFoll[ i ] = picX
			else if( there is a short-term reference picture picY in the DPB
					with PicOrderCntVal equal to PocLtFoll[ i ] )
				RefPicSetLtFoll[ i ] = picY
		else
				RefPicSetLtFoll[ i ] = "no reference picture"
	}
}

- All reference pictures included in RefPicSetLtCurr and RefPicSetLtFoll are marked as "used for long-term reference"

- The following applies:

for( i = 0; i < NumPocStCurrBefore; i++ )
	if( there is a short-term reference picture picX in the DPB
			           with PicOrderCntVal equal to PocStCurrBefore[ i ])
			RefPicSetStCurrBefore[ i ] = picX
	else
		RefPicSetStCurrBefore[ i ] = "no reference picture"

for( i = 0; i < NumPocStCurrAfter; i++ )
	if( there is a short-term reference picture picX in the DPB
			           with PicOrderCntVal equal to PocStCurrAfter[ i ])
			RefPicSetStCurrAfter[ i ] = picX
	else
		RefPicSetStCurrAfter[ i ] = "no reference picture"	(8-2)

for( i = 0; i < NumPocStFoll; i++ )
	if( there is a short-term reference picture picX in the DPB
			           with PicOrderCntVal equal to PocStFoll[ i ])
			RefPicSetStFoll[ i ] = picX
	else
		RefPicSetStFoll[ i ] = "no reference picture"

- All reference pictures included in RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetStFoll are marked as "used for short-term reference".

- All reference pictures in the decoded picture buffer that are not included in RefPicSetLtCurr, RefPicSetLtFoll, RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetStFoll are marked as "unused for reference".

NOTE 4 – There may be one or more reference pictures that are included in the reference picture set but that are not present in the decoded picture buffer. When the first coded picture in the bitstream is an IDR picture or the current coded picture is not a leading picture of the first coded picture in the bitstream, entries in RefPicSetStFoll or RefPicSetLtFoll that are equal to "no reference picture" should be ignored. When the first coded picture in the bitstream is not a CRA picture or the current coded picture is not a leading picture of the first coded picture in the bitstream, an unintentional picture loss should be inferred for each entry in RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetLtCurr that is equal to "no reference picture".

It is a requirement of bitstream conformance that the reference picture set is restricted as follows:

- There shall be no reference picture with temporal_id greater than that of the current picture included in RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetLtCurr.

- There shall be no reference picture included in the reference picture set that precedes, in output order, any CRA picture that precedes the current picture both in decoding order and output order.

- When the first coded picture in the bitstream is an IDR picture or the current coded picture is not a leading picture of the first coded picture in the bitstream, there shall be no entry in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr that is equal to "no reference picture".

NOTE 5 – A reference picture cannot be included in more than one of the five reference picture set lists.

------------------------------------------------------------------------------------------

In other hand, a picture marking process is implemented in HM6.1 software such as Figure 1. In this implementation, a short term picture marking process is conducted at first, and after that, a long term picture marking process is conducted. According to this implementation, if one picture is assigned to both long term picture and short term picture at the same time, that picture would be assigned to both RefPicSetStCurrX and RefPicSetLtCurr, and then it would not satisfy the NOTE5 in CD text which is highlighted by blue color.

Figure 1: picture marking process in HM6.1 software

As seen above, it seems that there would be a mismatch between CD text and HM6.1 software regarding picture marking process, so it should be modified. Moreover, it seems that bitstream conformance error would be happened if one picture is assigned to both long term picture and short term picture at the same time in both cases.

To prevent bitstream conformance error noted in CD text, DelataPoc for short term picture should be modified every time long term picture is used. Figure2 shows an example for this case. In HB7 case, DeltaPoc for short term picture could be reused base on the position of coding structure. However, for example, when POC_A is marked as long term picture in coding POC_B, DelatPoc for short term picture would be changed in POC_B, POC_C, POC_D and POC_E to prevent bitstream conformance error. It seems redundant and needs some bits to change them.

Figure 2: example for changing DelatPoc for short term picture

# Proposed Method

In this contribution, one modification on picture marking process and parsing process on reference picture list modification syntax is proposed. This proposal is constructed as follows.

- Modified picture marking process

- Modified parsing process on list_entry_lX parameters

## Modified picture marking process

In this proposal, one picture could be assigned to both long term picture and short term picture at the same time. However, long term assignment would be taken priority. Figure3 is an example of this proposal. In Figure3, it would be no need to change DeltaPoc for short term picture, so it could be reused base on the position of coding structure even if long term picture is used. CD text would be modified such as below (Modified parts are highlighted). In modified CD text, the assignment on short term picture would be ignored if a picture has been already marked as long term picture.

Figure 3: proposed method

-----proposed method: section 8.3.2 Decoding process for reference picture set -----------

The derivation process for the reference picture set and picture marking are performed according to the following ordered steps, where DPB refers to the decoded picture buffer as described in Annex Error: Reference source not found:

- The following applies:

for( i = 0; i < NumPocLtCurr; i++ ) {
	if( !delta_poc_msb_present_flag[ i ] ) {
		if( there is a long-term reference picture picX in the DPB 
				with pic_order_cnt_lsb equal to PocLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picX
		else if( there is a short-term reference picture picY in the DPB
				with pic_order_cnt_lsb equal to PocLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picY
		else 
			RefPicSetLtCurr[ i ] = "no reference picture"
	} else { 
		if( there is a long-term reference picture picX in the DPB
				with PicOrderCntVal equal to PocLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picX
		else if( there is a short-term reference picture picY in the DPB
				with PicOrderCntVal equal to PocLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picY
		else 
			RefPicSetLtCurr[ i ] = "no reference picture"
	}
}																						(8-3)

for( i = 0; i < NumPocLtFoll; i++ ) {
	if( !delta_poc_msb_present_flag[ i ] ) {
		if( there is a long-term reference picture picX in the DPB
					with pic_order_cnt_lsb equal to PocLtFoll[ i ] )
				RefPicSetLtFoll[ i ] = picX
		else if( there is a  short-term reference picture picY in the DPB
					with pic_order_cnt_lsb equal to PocLtFoll[ i ] )
				RefPicSetLtFoll[ i ] = picY
		else 
				RefPicSetLtFoll[ i ] = "no reference picture"
	} else {
		if( there is a long-term reference picture picX in the DPB
					with PicOrderCntVal to PocLtFoll[ i ] )
				RefPicSetLtFoll[ i ] = picX
			else if( there is a short-term reference picture picY in the DPB
					with PicOrderCntVal equal to PocLtFoll[ i ] )
				RefPicSetLtFoll[ i ] = picY
		else
				RefPicSetLtFoll[ i ] = "no reference picture"
	}
}

- All reference pictures included in RefPicSetLtCurr and RefPicSetLtFoll are marked as "used for long-term reference"

- The following applies:

for( i = 0, j = 0; i < NumPocStCurrBefore; i++, j++ )
	if( there is a short-term reference picture picX in the DPB
			           with PicOrderCntVal equal to PocStCurrBefore[ i ])
			RefPicSetStCurrBefore[ j ] = picX
	else if( there is a long-term reference picture picX in the DPB
			           with PicOrderCntVal equal to PocStCurrBefore[ i ])
		j− −
    else
		RefPicSetStCurrBefore[ j ] = "no reference picture"

NumPocStCurrBefore = j

for( i = 0, j = 0; i < NumPocStCurrAfter; i++, j++ )
	if( there is a short-term reference picture picX in the DPB
			           with PicOrderCntVal equal to PocStCurrAfter[ i ])
			RefPicSetStCurrAfter[ j ] = picX
     else if( there is a long-term reference picture picX in the DPB
			           with PicOrderCntVal equal to PocStCurrAfter[ i ])
         j− −
	else
		RefPicSetStCurrAfter[ j ] = "no reference picture"	(8-4)

NumPocStCurrAfter = j

for( i = 0, j = 0; i < NumPocStFoll; i++, j++)
	if( there is a short-term reference picture picX in the DPB
			           with PicOrderCntVal equal to PocStFoll[ i ])
			RefPicSetStFoll[ j ] = picX
	else if( there is a long-term reference picture picX in the DPB
			           with PicOrderCntVal equal to PocStFoll[ i ])
         j− −
       else
		RefPicSetStFoll[ j ] = "no reference picture"

NumPocStFoll = j

- All reference pictures included in RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetStFoll are marked as "used for short-term reference".

- All reference pictures in the decoded picture buffer that are not included in RefPicSetLtCurr, RefPicSetLtFoll, RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetStFoll are marked as "unused for reference".

- The variable NumPocTotalCurr is set equal to NumPocStCurrBefore + NumPocStCurrAfter + NumPocLtCurr.

NOTE 4 – There may be one or more reference pictures that are included in the reference picture set but that are not present in the decoded picture buffer. When the first coded picture in the bitstream is an IDR picture or the current coded picture is not a leading picture of the first coded picture in the bitstream, entries in RefPicSetStFoll or RefPicSetLtFoll that are equal to "no reference picture" should be ignored. When the first coded picture in the bitstream is not a CRA picture or the current coded picture is not a leading picture of the first coded picture in the bitstream, an unintentional picture loss should be inferred for each entry in RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetLtCurr that is equal to "no reference picture".

## Modified parsing process on list_entry_lX parameters

In the current CD text, the decoding process for reference picture set in section 8.3.2 is invoked once per picture, after decoding of a slice header. However, the variable NumPocTotalCurr, which is calculated in section 8.3.2, is used to parse list_entry_lX parameters in the slice header. It would seem inconsistent and this dependency might cause increasing waiting time for decoding a slice header. Especially, in this proposal, the variable NumPocTotalCurr would be calculated after picture marking process as it’s showed in above modified text, so it seems that this dependency should be removed.

In proposed method, list_entry_lX parameters are decoded by using ue(v) instead of u(v) and the condition “NumPocTotalCurr > 1” is removed in order to remove above dependency. CD text would be modified such as below.

#### 6.3.3.2  Reference picture list modification syntax

| ref_pic_list_modification( ) { | Descriptor |
| --- | --- |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| ref_pic_list_modification_flag_l0 | u(1) |
| if( ref_pic_list_modification_flag_l0 && NumPocTotalCurr > 1 ) |  |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) |  |
| list_entry_l0[ i ] | ue (v) |
| } |  |
| if( slice_type = = B ) { |  |
| ref_pic_list_modification_flag_l1 | u(1) |
| if( ref_pic_list_modification_flag_l1 && NumPocTotalCurr > 1 ) |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) |  |
| list_entry_l1[ i ] | ue (v) |
| } |  |
| } |  |

#### 6.4.3.2  Reference picture list modification semantics

ref_pic_list_modification_flag_lX (with X equal to 0 or 1) equal to 1 indicates that reference picture list X is specified explicitly as a list of list_entry_lX[ i ] values (with X being 0 or 1). ref_pic_list_modification_flag_lX equal to 0 indicates that reference picture list X is determined implicitly. When ref_pic_list_modification_flag_lX is not present, it is inferred to be equal to 0.

list_entry_lX[ i ] (with X equal to 0 or 1) specifies the index of the reference picture in RefPicSetCurrTempListX to be placed at the current position of reference picture list LX (with X being 0 or 1). The length of the list_entry_lX[ i ] syntax element is Ceil( Log2( NumPocTotalCurr) ) bits. The value of list_entry_lX[ i ] shall be in the range of 0 to NumPocTotalCurr − 1, inclusive. If the syntax element list_entry_lX[ i ] is not present, it is inferred to be equal to 0.

# Experimental result

Proposed method was implemented on HM6.1-dev-ahg15 software and it was evaluated whether bitstream conformance error would be happened or not if one picture is assigned to both long term picture and short term picture at the same time. The test case 2.7 in the common conditions for reference picture marking and list construction described in JCTVC-H0725 [1] was used in this evaluation. The test case 2.7 includes two scenes, first case considers scene intervals of 10s – 30s – 10s – 30s, and second case considers scene intervals of 20s – 60s – 20s – 60s. For these two cases, class D sequences (BasketballPass, BQSquare, BlowingBubbles, RaceHorses) were used to evaluate it. In HM6.1-dev-ahg15 software, it is controlled in encoder side that if RPS includes the POC of the picture which should be assigned to long term picture, the assignment for long term picture would be cancelled. To evaluate without such kind of encoder based restriction, the part of encoder restriction was removed in this evaluation (the function “rpsContainsShortTermReference()” was comment out).

Figure 1 shows one result of decoding bitstreams in case of assignment both short term and long term to one picture by using HM6.1-dev-ahg15 software without encoder based restriction in the test case 2.7 (scene intervals of 10s – 30s – 10s – 30s). Test sequence is BasketballPass, 416x240, 50fps, thus the picture with POC 496 was marked as long term picture. However, the picture with POC 496 was marked as short term picture by RPS at the same time. Therefore, an error such as Figure 1 would be happened in the current HM6.1-dev-ahg15 software without encoder based restriction.

Figure 2 shows one result of decoding bitstreams in case of assignment both short term and long term to one picture by using HM6.1-dev-ahg15 software with proposal in the test case 2.7. Even though the picture with POC 496 was marked as both long term and short term picture at the same time, it was confirmed that any errors would not be happened by applying this proposal. It means that an encoder would not be needed to take care of whether or not RPS includes the POC of the picture which should be assigned to long term picture. Moreover, even if an encoder would illegally set them, the error would be avoidable with this proposal.

Figure 4: result of decoding bitstreams in case of assignment both short term and long term to one picture in HM6.1-dev-ahg15 software without encoder based restriction

Figure 5: result of decoding bitstreams in case of assignment both short term and long term to one picture in HM6.1-dev-ahg15 software with proposal

# Conclusion

In this contribution, a mismatch on picture marking process between CD text and HM6.1 software was reported. In order to avoid the mismatch, it was proposed to ignore short tem picture assignment for a picture which got duplicated assignment of long term and short term reference by RPS syntax at the same time. Moreover, it was proposed to replace the parsing process on list_entry_lX parameters with the one which is independent from the variable NumPocTotalCurr.

In order to fix the mismatch between the CD text and HM software and to reduce the retransmission of DeltaPoc required by the usage of long term reference, we suggest the proposed methods be adopted in the DIS of HEVC.

# References

- Ye-Kui Wang et al, “Common conditions of reference picture marking and list construction proposals”, JCTVC-H0725, San Jose, February 2012.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).