### Sample adaptive offset process

A sample adaptive offset process shall be conditionally performed after the completion of the deblocking filter process for the decoded picture. This process is invoked when both sample_adaptive_offset_enabled_flag and aps_sample_adaptive_offset_flag are equal to 1.

This process is performed on a region basis which is aligned with the coding treeblock boundaries after the completion of the picture construction process prior to sample adaptive offset process for the entire decoded process.

The sample adaptive offset process is invoked with the region equal to entire picture here.

Depending on slice_sample_adaptive_offset_flag, sao_cb_enable_flag and sao_cr_enable_flag, the following applies:

–	When slice_sample_adaptive_offset_flag is equal to 1 the following applies:

  - The sample adaptive offset process for luma region as specified in subclause 0.1.1.1 is invoked with the component index set equal to 0, the coding treeblock location ( 0, 0 ), the largest CU location ( 0, 0 ), the variable saoWidthInCtbs set equal to PicWidthInCtbs, and the variable saoHeightInCtbs set equal to PicHeightInCtbs, and the modified luma picture buffer of current region as output.

  - When sao_cb_enable_flag is equal to 1, the sample adaptive offset process for colour component Cb as specified in subclause 0.1.1.1 is invoked with the component index set equal to 1, the coding treeblock location ( 0, 0 ), the largest CU location ( 0, 0 ), the variable saoWidthInCtbs set equal to PicWidthInCtbs, and the variable saoHeightInCtbs set equal to PicHeightInCtbs, and the modified chroma picture buffer for Cb of current region as output.

  - If sao_cb_enable_flag is equal to 1, the sample adaptive offset process for colour component Cr as specified in subclause 0.1.1.1 is invoked with the component index set equal to 2, the coding treeblock location ( 0, 0 ), the largest CU location ( 0, 0 ), the variable saoWidthInCtbs set equal to PicWidthInCtbs, and the variable saoHeightInCtbs set equal to PicHeightInCtbs, and the modified chroma picture buffer for Cr of current region as output.

#### Modification process for sample adaptive offset

Inputs to this process are:

–	a variable cIdx specifying colour component index,

–	a pair of variables ( rx, ry ) specifying the coding treeblock location,

–	a sample location ( x0, y0 ) specifying the top-left sample of the current largest coding unit,

–	a variable saoWidthInCtbs specifying the number of largest CUs in width of the current region

–	a variable saoHeightInCtbs specifying the number of largest CUs in height of the current region.

Output of this process is a modified picture buffer for the colour component cIdx.

The modified picture buffer for the colour component cIdx is derived as follows.

- Variable nS is set equal to ( 1 << Log2CtbSize ).

- Modification process for samples in a largest coding unit covering ( iC, jC ) with iC = x0 + k*nS for k = 0..saoWidthInCtb − 1 and jC = y0 + j*nS for j = 0..saoHeightInCtb − 1 specified in subclause 0.1.1.1.1 is invoked with the colour component index cIdx, the top-left sample of current largest coding unit ( iC, jC ), the coding treeblock location ( rx, ry ), the offset array saoValueArray set equal to SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ] and the largest coding unit size nS as inputs and the output is the modified picture buffer of colour component cIdx.

##### Modification process for luma and chroma samples

Inputs to this process are:

–	a variable cIdx specifying colour component index,

–	a sample position ( xC, yC ),

–	a pair of variables ( rx, ry ) specifying the coding treeblock location,

–	an array saoValueArray specifying offset values,

–	a block size nS.

Output of this process is a modified picture buffer for the colour component cIdx.

Let recSaoPicture represents the processed sample array of the current picture of colour component cIdx and saoTypeIdx is set equal to sao_type_idx[ cIdx ][ rx ][ ry ].

Variable bitDepth is set equal to BitDepthY if cIdx is equal to 0, otherwise, set equal to BitDepthC.

Depending on the value of saoTypeIdx, the following applies:

–	If saoTypeIdx is equal to one of the values of 1, 2, 3 or 4, the following ordered steps apply:

- Arrays hPos[2] and vPos[2] are specified in Table 8-1.

- A variable edgeIdx is derived as follows.

edgeIdx = 2 + ∑k( Sign( recPicture[ xC + i, yC + j ] –

recPicture[ xC + i + hPos[ k ], yC + j + vPos[ k ] ] ) ) with k = 0..1	(8-1)

- When any of the following conditions is true, edgeIdx is set equal to 2.

- The pixel at position  (xC + i + hPos[ k ], yC + j + vPos[ k ]) is ouside picture boundary

- slice_loop_filter_across_slices_enabled_flag is equal to 0 and the pixel at position (xC + i + hPos[ k ], yC + j + vPos[ k ]) belongs to different slice.

- loop_filter_across_tiles_enabled_flag is equal to 0 and the pixel at position (xC + i + hPos[ k ], yC + j + vPos[ k ]) belongs to different tile.

- The reconstructed picture buffer is modified as follows:

- If pcm_loop_filter_disable_flag is equal to 1 and the recPicture[ xC + i, yC + j ] is a reconstructed sample of an I_PCM block or qpprime_y_zero_transquant_bypass_flag is equal to 1 and QP’Y is equal to 0, the recSaoPicture[ xC + I yC + j ] is derived as

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ]	(8-2)

with i = 0..nS−1 and j = 0..nS−1 where 	edgeTable[5] = { 1, 2, 0, 3, 4}.

- Otherwise,

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] + saoValueArray[ edgeTable[ edgeIdx ] ]	(8-3)

with i = 0..nS−1 and j = 0..nS−1 where 	edgeTable[5] = { 1, 2, 0, 3, 4}.

- Otherwise, , if saoTypeIdx is equal to 5, the following ordered steps apply:

- If cIdx is equal to 0, the variable bandShift is set equal to BitDepthY – 5 and variable offsetValShift is set equal to 2 and variable offsetLine is set equal to 4.

- Otherwise (cIdx is not equal to 0), bandShift is set equal to BitDepthC − 5 and variable offsetValShift is set equal to 1 and variable offsetLine is set equal to 2.

- The variable saoLeftClass is set equal to sao_band_position[ cIdx ][ rx ][ ry ].

- The vector bandTable is defined with 32 elements and all elements are initially set to 0. Then, 4 of its elements (indicating the position of bands for which an offset is explicitly transmitted) are modified as follows:

for( i = 0; i < 4; i++ )
	bandTable[ (i + saoLeftClass) & 0x1F ]  = i + 1		(8-4)

- If  rx is not equal to 0 and run[cIdx][rx-1][ry] is equal to 0, sx is set to xC. Otherwise, sx is set to xC-1. If rx is not equal to saoWidthInCtb - 1 and run[cIdx][rx][ry] is equal to 0, ex is set to xC + nS - 1. Otherwise, ex is set to xC + nS. The variable runLeftEnd is set to the rx of the coding treeblock which locates nearest the current coding treeblock and whose rx is equal to 0 or run[cIdx][runLeftEnd-1][ry] is equal to 0. If ry is not equal to 0 and mergeUpFlag[cIdx][runLeftEnd][ry] is equal to 0, sy is set to yC. Otherwise, sy is set to yC-1. If ry is not equal to saoHeightInCtb - 1 and mergeUpFlag[cIdx][rx][ry+1] is equal to 0, sy is set to yC + nS. Otherwise, ey is set to yC + nS + 1.

5.6.The reconstructed picture buffer recSaoPicture is modified as follows.

bandIdx = ( recPicture[ xC + i, yC + j ] >> bandShift )

The variable offset1 and offset2 are derived as follows:

- If ( recPicture[ xC + i, yC + j ] % ( 1 << bandShift ) ) is equal to one of the values of  0 or ( ( 1 << bandShift ) – 1),

offset1 = Sign(saoValueArray[ bandTable[ bandIdx ] ] ) * ( Abs(saoValueArray[ bandTable[ bandIdx ] ] ) >> 1 )	(8-5)

- Otherwise, if ( recPicture[ xC + i, yC + j ] % ( 1 << bandShift ) ) is equal to one of the values of ( ( 1 << bandShift ) – 2) or ( ( 1 << bandShift ) + 1),

offset1 = Sign(saoValueArray[ bandTable[ bandIdx ] ] ) * ( Abs(saoValueArray[ bandTable[ bandIdx ] ] ) >> 2 )	(8-6)

- Otherwise,

offset1 = saoValueArray[ bandTable[ bandIdx ] ]	(8-7)

- If (xC + i = = sx + m || xC + i = = ex – m || yC + i = = sy + m || yC + i = = ey – m  ) then,

offset2 = Sign( offset1 ) * ( Abs(offset1 ) >> 2 )	(8-8)

- Otherwise,

offset2 = offset1	(8-9)

with m = 0..offsetLine – 1.

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] + offset2saoValueArray[ bandTable[ bandIdx ] ]	(8-10)
	with i = 0..nS−1 and j = 0..nS−1

–	Otherwise (sao_type_idx[ cIdx ][ rx ][ ry ] is equal to 0), the following applies:

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] with i = 0..nS−1 and j = 0..nS−1	(8-11)

Table 8-1 – Specification of hPos[2] and vPos[2] according to the type of sample adaptive offset process

| sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| hPos[0] | −1 | 0 | −1 | 1 |
| hPos[1] | 1 | 0 | 1 | −1 |
| vPos[0] | 0 | −1 | −1 | −1 |
| vPos[1] | 0 | 1 | 1 | 1 |