| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0136 WG11 Number: m24375 |
| --- | --- |

| Title: | CE1: Method of removing boundary artifacts for SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Toru Matsunobu Toshiyasu Sugio Hisao Sasai Takahiro Nishi | Tel: Email: | +81-6-6900-9689 {matsunobu.toru, sugio.toshiyasu, sasai.hisao, nishi.takahiro}@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

In this contribution, visual artifact caused by SAO[1] in HM6.0 was reported and an approach to remove the artifact was proposed. The artifact was reportedly observed at SAO LCU boundaries and category boundaries. In the proposal, a weak offset value is added to reconstructed samples along with the boundaries. It was reported that the artifact was reduced when this proposal is applied on top of Core Experiment 1 (CE1) anchor based on HM6.0. Experimental results reportedly showed that the proposed changes can be introduced on top of CE1 Anchor with 0.x%-0.x% average BD-rate loss for the test conditions of CE1.

# Introduction

We observed that it occasionally results in two types of visual artifact. Fig.1 shows an example of the first type of visual artifact. BasketballDrill sequence is compressed using low delay B main configuration with QP 37. Fig.1 (a) is the original x th frame. (b) is the part of original x th frame. (c) is the result of CE1 Anchor. (d) is the result of CE1 anchor without SAO. There is a vertical sharp line in (c). This is a visible LCU boundary of SAO. Fig.2 shows an example of the second type of visual artifact. SlideEditing sequence is compressed using low delay B main configuration  configuration with QP 37. Fig.2 is indicated in the same way as Fig. 1. There is an artifact like ‘spot’ in (c). It is a visible category boundary of SAO. The issues should be fixed if SAO would be available in HEVC.

- Original

(b) Original                (c) CE1 anchor             (d) (c) without SAO

Fig.1. Subjective image quality comparison – BasketballDrill (20th frame, QP37, low delay B Main  configuration)

- Original

(b) Original                (c) CE1 anchor              (d) (c) without SAO

Fig.2. Subjective image quality comparison – SlideEditing (46th frame, QP37, low delay B Main  configuration)

# Problems

SAO adds offset values to reconstructed samples at each LCU and category individually. Fig.3 shows an example of LCU of a picture. Fig.4 shows an example of classifying samples in a LCU by band offset. By SAO, spatial continuity is broken on the LCU boundary and the category boundary. Fig.5 shows an example of the discontinuity that occurs on a LCU boundary. A blue line indicates intensities before SAO, and a red line indicates intensities after SAO. The difference between neighboring samples on the partition boundary is increased by addition of very different offset values. This can be visible as a visual artifact. Especially such a artifact is easily observed in flat and gradation.

Fig.3. LCU of picture             Fig.4. Classifying samples by band offset

Fig.5. Discontinuity on LCU boundary

# Solutions

Because it is difficult to compensate reconstructed samples without LCU based processing and classifying, we propose to add weak offset values to samples near a LCU boundary and a category boundary.

Proposal 1. weakening offset for LCU boundary for BO

Fig.6 shows how to weaken offset values near the LCU boundary for BO. The cells indicate samples of a reconstructed image, and a figure in each cell indicates a weight coefficient that is multiplied by offset values. The nearer the LCU boundary, the smaller the offset value becomes.

Fig.6. LCU boundary

In SAO, bordering LCUs share offset parameters by referring flags. Proposal 1 processes only boundaries of bordering LCUs which don’t share offset parameters. Proposal 1 selects a target boundary whether to add weak offset values by using the flags. SAO in CE1 anchor has two signaling mode, the APS mode and the interleaving mode. Each modes use different flags for sharing. Fig.7 (a) shows the case of the APS mode. Then Proposal 1 uses mergeUpFlag[cIdx][x][y] and run[cIdx][x][y]. cIdx indicates color component. x indicates LCU horizontal location and x = 0,…,(the number of LCUs in width)-1. y indicates LCU vertical location and y = 0,…,(the number of LCUs in height)-1. The current LCU locates (rx, ry) indicated in blue. The mergeUpFlag[cIdx][rx][ry] and run[cIdx][rx][ry] are referred for the upper boundary and the right boundary. If the mergeUpFlag[cIdx][rx][ry] is OFF, weak offset values are added to samples near the upper boundary. If run[cIdx][rx][ry] is zero, weak offset values are added to samples near the right boundary. For the upper boundary, the mergeUpFlag[cIdx][runLeftEnd][ry] is used. runLeftEnd is the horizontal location of the LCU whose rx is not equal to 0 or run[cIdx][runLeftEnd-1][ry] is equal to 0. For the lower boundary, mergeUpFlag[cIdx][rx][ry+1] is used. Fig.7 (b) shows the case of the interleaving mode. Then Proposal 1 uses mergeUpFlag[cIdx][rx][ry] and mergeLeftFlag[cIdx][rx][ry]. The current LCU is indicated in blue. The mergeUpFlag[cIdx][rx][ry] and mergeLeftFlag[cIdx][rx][ry] are referred for decision of the upper boundary and the left boundary. If the mergeUpFlag[cIdx][rx][ry] is OFF, weak offset values are add to samples near the upper boundary. If the mergeLeftFlag[cIdx][rx][ry] is OFF, weak offset values are added to samples near the left boundary. For the right boundary, the mergeLeftFlag[cIdx][rx+1][ry] is used. For the lower boundary, mergeUpFlag [cIdx][rx][ry+1] is used.

(a)APS mode                            (b)Interleaving mode

Fig.7. Selection of LCU boundary

Proposal 2. weakening offset for Category boundary for BO

Fig.8 shows how to weaken offset values near category boundary of band offset. The nearer the category boundary, the smaller the offset value becomes.

Fig.8. Category boundary

# Experimental Results

BD-rate performance

The proposed method performance relative to CE1 Anchor was checked under the CE1 test condition described in JCTVC-Hxxx[3]. The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3. Table 1.1 shows BD-rate of non-normative method which limits the offset magnitude by half for BO. Table 1.2 shows BD-rate of proposal 1. Table 1.3 shows BD-rate of proposal 2. Table 1.4 shows BD-rate of the combination of proposal 1 and 2.

Subjective image quality

Fig. 9 and Fig. 10 shows comparison with the reconstructed images. They are results coded on QP37 for low delay B Main configuration. (f) is the result of the non-normative method. (g) is the result of proposal 1. (h) is the result of proposal 2. (i) is the result of the combination of proposal 1 and 2. The sequence of Fig. 9 is BasketballDrill, and the sequence of Fig.10 is SlideEditing. Image quality of the proposed method has made improvement comparing with the CE1 anchor.

# Conclusion

Visual artifact caused by SAO in CE1 anchor was found and an approach to remove the artifact was proposed in this contribution. We confirmed that the artifact was reduced by this proposal on top of CE1 anchor. The result shows the subjective quality improvement and the not significant BR-rate loss (the range of 0.2% in average). However, the proposal has the problem as described in JCTVC-I0137[3]. We suggest to review whether to adopt JCTVC-I0137 instead of the proposal.

Table.1.1. BD-rate results of the non-normative method

Table.1.2. BD-rate results of proposal 1 for LCU boundary

Table.1.3. BD-rate results of proposal 2 for category boundary

Table.1.4. BD-rate results of the combination of proposal 1 and 2

(c) CE1 anchor          (e) non-normative method            (f) proposal 1

(g) proposal 2         (h) combination of (f) and (g)

Fig.9. Subjective image quality comparison – BasketballDrill (20th frame, QP37, low delay B Main configuration)

(c) CE1 anchor           (e) non-normative method            (f) proposal 1

(g) proposal 2         (h) combination of (f) and (g)

Fig.10. Subjective image quality comparison – SlideEditing (46th frame, QP37, low delay B Main configuration)

# Reference

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 6”, JCTVC-H1003, JCT-VC 8th Meeting, San José, CA, USA, 1-10 February, 2012.

- Y.W. Huang, E. Alshina, I.S. Chong, W. Wan, M. Zhou, “Description of Core Experiment 1 (CE1) Sample adaptive offset filtering”, JCTVC-H1101, JCT-VC 8th Meeting, San José, CA, USA, 1-10 February, 2012.

- T.Matsunobu, T.Sugio, H.Sasai, T.Nishi, “Non-CE1: Removing SAO boundary artifacts by weighted offset values”, JCTVC-I0137, 9th Meeting: Geneva, CH, 27 April – 7 May 2012

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).