| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0138 |
| --- | --- |

| Title: | Syntax on entropy slice information |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seungsoo Jeong Tammy Lee Chanyul Kim JaeHyun Kim JeongHoon Park | Email: | ss00.jeong@samsung.com tammy.lee@samsung.com dionism@samsung.com jhgim@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In current HEVC design, the usage of Tile and WPP is signaled by the index named by “tiles_or_entropy_coding_sync_idc” in Sequence Parameter Set (SPS). However, in the case of Entropy Slice, decoder knows the usage of Entropy Slice only after parsing the syntax “entropy_slice_flag” in Slice Header. It is proposed that the syntax “tiles_or_entropy_coding_sync_idc” has to indicate the case of Entropy Slice as other parallel processing support tools like Tile and WPP. This syntax design is also effective to write syntax bits related to Entropy Slice information in Slice Header.

# Introduction

Currently, HEVC has many tools to support parallel processing in multi-core environments such as Tile, WPP and Entropy Slice. In the last meeting, it was adopted that those tools shall be used exclusively and any combination of those is disallowed since the purpose of those tools is regarded as similar each other.

In this sense, current design syntax uses the common syntax which can control this kind of parallel processing tools. The syntax“tiles_or_entropy_coding_sync_idc” in Sequence Parameter Set indicates which parallel processing tool is used ( or no tool is used ).

When the syntax “tiles_or_entropy_coding_sync_idc” equals to 0, no tool is used. When the syntax “tiles_or_entropy_coding_sync_idc” equals to 1 in SPS level, Tile tool is used. After that, tile parallel information is sent by checking syntax “tiles_or_entropy_coding_sync_idc” in SPS and PPS ( Picture Parameter Set ). When the syntax “tiles_or_entropy_coding_sync_idc” equals to 2 in SPS level, Wavefront tool is used. After that, Wavefront parallel information is sent by checking syntax “tiles_or_entropy_coding_sync_idc” in PPS. Currently, “tiles_or_entropy_coding_sync_idc” is ranged between 0 and 2. So, the case of value 3 will be able to be used for expressing useful indication.

To clarify that the combination of any of tiles, WPP and entropy slices is disallowed and those shall be used exclusively, the syntax “tiles_or_entropy_coding_sync_idc” is proposed to indicate the case of Entropy Slice in the same sense of tiles and WPP.

In the case of current design, decoder doesn’t know whether this bitstream is coded by using Entropy Slice or not before parsing the syntax “entropy_slice_flag” in Slice Header is parsed. On the other hand, in the case of Tile and Wavefront, indication information is sent in SPS level. Parallel information of each tool can be overwritten according to syntax of picture or slice level. If the syntax related to the usage of Entropy Slice is sent in higher syntax level, decoder can know the usage of Entropy Slice earlier and therefore, decoder system doesn’t have to wait to know whether entropy slice is used in the next slice or not and only need to parse multiple slice headers to allocate multiple cabac decoders for parallel parsing.

The syntax “entropy_slice_flag” is always sent to inform whether this slice is entropy slice or not. However, if the usage informaton of Entropy Slice is sent in higher level, the syntax “entropy_slice_flag” is not always sent. To save bits for overhead of Slice Header, it is proposed to selectively signal the syntax “entropy_slice_flag” according to the usage of Entropy Slice which is informed in higher level.

# Proposal

The changed slice header is as follows, wherein the changed parts are highlighted.

## Syntax for Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag = = 0 ) |  |
| slice_address | u(v) |
| slice_type | ue(v) |
| if( tiles_or_entropy_coding_sync_idc = = 3 ) |  |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| pic_parameter_set_id | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| … |  |
| } |  |
| … |  |
| if( (tiles_or_entropy_coding_sync_idc = = 1) ¦¦ (tiles_or_entropy_coding_sync_idc = = 2) ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| } |  |

If tiles_or_entropy_coding_sync_idc is considered to be moved from SPS to PPS and PPS id is signaled before sending entropy slice flag as it is proposed as a combination of Solution B for Slice header parsing order and Solution A for SPS/PPS parsing dependency on tiles_or_entropy_coding_sync_idc in [1], the proposed slice header syntax is as follows:

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( first_slice_in_pic_flag = = 0 ) |  |
| slice_address | u(v) |
| slice_type | ue(v) |
| if( tiles_or_entropy_coding_sync_idc = = 3 ) |  |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| pic_parameter_set_id | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| … |  |
| } |  |
| … |  |
| if( (tiles_or_entropy_coding_sync_idc = = 1) ¦¦ (tiles_or_entropy_coding_sync_idc = = 2) ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| } |  |

## Semantics

tiles_or_entropy_coding_sync_idc equal to 0 specifies that there is only one tile in each picture in the coded video sequence, and no specific synchronization process for context variables is invoked before decoding the first coding treeblock of a row of coding treeblocks.

tiles_or_entropy_coding_sync_idc equal to 1 specifies that there may be more than one tile in each picture in the coded video sequence, and no specific synchronization process for context variables is invoked before decoding the first coding treeblock of a row of coding treeblocks. tiles_or_entropy_coding_sync_idc equal to 2 specifies that there is only one tile in each picture in the coded video sequence, a specific synchronization process for context variables is invoked before decoding the first coding treeblock of a row of coding treeblocks, and a specific memorization process for context variables is invoked after decoding two coding treeblocks of a row of coding treeblocks

tiles_or_entropy_coding_sync_idc equal to 3 specifies that there is only one tile in each picture in the coded video sequence, and no specific synchronization process for context variables is invoked before decoding the first coding treeblock of a row of coding treeblocks. And there may be more than one slice whose slice header syntax elements not present is inferred to be equal to the value of slice header syntax elements in a proceeding slice in each picture in the coded video sequence. entropy_slice_flag is inferred to zero

The value of tiles_or_entropy_coding_sync_idc shall be in the range of 0 to 3, inclusive.

entropy_slice_flag equal to 1 specifies that the value of slice header syntax elements not present is inferred to be equal to the value of slice header syntax elements in a proceeding slice, where a proceeding slice is defined as the slice containing the coding treeblock with location (SliceCtbAddrRS − 1). entropy_slice_flag shall be equal to 0 when SliceCtbAddrRS equal to 0.

- Conclusion

The consistent syntax for parallel processing information signalling is proposed. To clear that only on parallel tool can be used at the same time, the syntax “tiles_or_entropy_coding_sync_idc” has to indicate the case of Entropy Slice conceptually. The design of parallel tools is consistent and more effective in terms of writing syntax bits.

# References

- K. Suehring, “High level syntax parsing issues”, JCTVC-I0113, 9th Meeting: Geneva, CH, 27 April – 7 May 2012

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).