| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0139 |
| --- | --- |

| Title: | Syntax on wavefront information |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seungsoo Jeong Tammy Lee Chanyul Kim JaeHyun Kim JeongHoon Park | Email: | ss00.jeong@samsung.com tammy.lee@samsung.com dionism@samsung.com jhgim@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

The current syntax design for Tile and WPP supporting parallel processing is not unified in the location of sending the detailed information and it is not efficient. It is proposed to signal WPP information as the same level of parameter set as Tiles, which is SPS level having overriding flag in PPS level.

# Introduction

Currently, HEVC has many tools to support parallel processing in multi-core environments such as Tile, WPP and Entropy Slice. In the last meeting, it was adopted that those tools shall be used exclusively and any combination of those is disallowed since the purpose of those tools is regarded as similar each other. In this sense, the usage of Tile and WPP is signaled by the index named by “tiles_or_entropy_coding_sync_idc” in Sequence Parameter Set (SPS).

However, the current syntax structures of Tile and WPP are different from each other. In Tile case where “tiles_or_entropy_coding_sync_idc” is equals to 1, the number of Tiles is signaled right after signaling the syntax “tiles_or_entropy_coding_sync_idc” in SPS level. Later, the number of Tiles is able to be overwritten in Picture Parameter Set (PPS) level when “tile_info_present_flag” is sent having the value of 1 in PPS level.

On the other hand, in WPP case where “tiles_or_entropy_coding_sync_idc” is equals to 2, the number of substreams is not signaled right after signaling the syntax “tiles_or_entropy_coding_sync_idc” in SPS level. Instead, the number of substreams “num_substreams_minus1” is sent in Picture Parameter Set (PPS) level.

Therefore, the current syntax design for Tile and WPP is not unified in the location of sending the detailed information and it is not efficient when the number of streams of WPP is the greater than 2 and is not frequently changed in terms of overhead bits. To have unified design and save bits for overhead where the number of substreams are not changed frequently, it is proposed to signal WPP information as the same level of parameter set as Tiles, which is SPS level having overriding flag in PPS level.

Moreover, if the number of substreams is signalled right after “tiles_or_entropy_coding_idc” which indicates WPP enabling in SPS level or overriding WPP information in PPS level, it can be inferred that WPP has two or more substreams. So, it is also proposed to signal the number of substreams as “num_substreams_minus2” instead of “num_substreams_minus1”. The semantics should be changed according to the name.

# Proposal

The changed sequence parameter set and picture parameter set are as follows, wherein the changed parts are highlighted.

## Syntax for Sequence Parameter Set

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| … |  |
| tiles_or_entropy_coding_sync_idc | u(2) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } else if( tiles_or_entropy_coding_sync_idc = = 2 ) |  |
| num_substreams_minus2 | ue(v) |
| … |  |
| } |  |

## Syntax for Picture Parameter Set

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| … |  |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| tile_info_present_flag | u(1) |
| tile_control_present_flag | u(1) |
| if( tile_info_present_flag) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| } |  |
| if( tile_control_present_flag ) |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } else if( tiles_or_entropy_coding_sync_idc = = 2 ){ |  |
| substreams_info_present_flag | u(1) |
| If( substreams_info_present_flag ) |  |
| num_substreams_minus2 | ue(v) |
| } |  |
| … |  |
| } |  |

## Semantics

num_substreams_minus2  plus 2 specifies the maximum number of subsets included in a slice when tiles_or_entropy_coding_sync_idc is equal to 2. When not present, the value of num_substreams_minus2 is inferred to be equal to 0.

substreams_info_present_flag equal to 1 specifies that the syntax elements num_substreams_minus2 is present in the picture parameter set. substreams_info_present_flag equal to 0 specifies that syntax elements num_substreams_minus2 is not present in the picture parameter set.

# Conclusion

The unified syntax for indication of tools supporting parallel processing is proposed. This contribution changes current syntax design to more consistent and effective design. This contribution also proposes new flags of overriding flag for WPP in PPS level and changes the semantics related to the number of substreams according to current effective design.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).