| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0141_r1 |
| --- | --- |

| Title: | AHG4: Intra mode prediction at entropy slice boundary |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Houqiang Li (USTC) Haitao Yang (Huawei) | Tel: Email: | +86 755 28976972 haitao.yang@huawei.com |
| Source: | University of Science and Technology of China Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

Entropy slice is a light-weight parallel mechanism which breaks the entropy decoding status. The intra sample prediction and motion prediction can cross the entropy slice boundary. This contribution discusses the possibility of also making the intra mode prediction across the entropy slice boundary.

# Introduction

Entropy slice is a light-weight parallel mechanism which breaks the entropy decoding status. The intra sample prediction and motion prediction can cross the entropy the entropy slice boundary. This contribution discusses the possibility of also making the intra mode prediction across the entropy slice boundary.

# Solution

In the San José meeting, it was agreed that motion prediction could cross entropy slice boundary [1] because the parsing process of motion information is fully independent with the surrounding motion information.

Also in the San José meeting, the cleanup on intra mode coding has also been adopted [2]. For intra luma mode coding, three fixed MPMs are used. So the parsing process of intra mode is also fully independent with the surrounding intra modes after the adoption of [2].

To keep consistency with the behavior of intra sample prediction and motion prediction, we propose that the intra mode prediction can cross the entropy slice boundary too.

The current Draft, i.e. JCTVC-H1003 dK, does not clearly describe the availability verification process when obtaining the intra prediction mode (MPM). We only find that the derivation process for the availability specified at 6.4 does not prohibit using the intra mode in the other entropy slices. So we consider it as a bug fix in the software.

The software change is also very simple, when getting MPMs, the parameter bEnforceEntropySliceRestriction should be set to false instead of default true.

We propose to enable intra mode prediction across entropy slice boundary as a bug fix in the software.

# References

- JCTVC-H0362, “Motion prediction at entropy slice boundary,” San José, CA, USA, 1–10 February, 2012.

- JCTVC-H0712, “BoG Report on Intra mode coding cleanup and simplification,” San José, CA, USA, 1–10 February, 2012.

- JCTVC-H1003, “High Efficiency Video Coding (HEVC) text specification draft 6,” San José, CA, USA, 1–10 February, 2012.

# Patent rights declaration(s)

University of Science and Technology of China may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).