| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0143_r2 |
| --- | --- |

| Title: | AHG11: High level syntax modifications |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Houqiang Li (USTC) Haitao Yang (Huawei) | Tel: Email: | +86 755 28976972 haitao.yang@huawei.com |
| Source: | University of Science and Technology of China Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This document presents several modifications of the high level syntax of the current HEVC. The proposed modifications include: (1) the nal_ref_flag restriction of CRA picture; (2) differentially signaling of per-temporal layer information; (3) the position of signaling per-temporal information in SPS; and (4) the derivation of POC when num_reorder_pics is 0.

# The nal_ref_flag restriction of CRA picture

In 7.4.1 of JCTVC-H1003_dK, nal_ref_flag shall be equal to 1 for NAL units with nal_unit_type equal to 5 (IDR). But there is no restriction of the nal_ref_flag of CRA (4) picture.

CRA picture, which is “clean random access” picture, can be the start of decoding of a sequence. Consider the following case that the coding starts from a CRA picture (picture A) but with nal_ref_flag equaling to 0. When decoding the first picture after the CRA picture (picture B), the POC of picture B should be derived first. According to 8.3.1, we need to find prevRefPic first. PrevRefPic is the previous reference picture in decoding order that has temporal_id equal to 0. But the only previous picture, the CRA picture (picture A), is non-reference picture. In such a case, the POC derivation of picture B cannot proceed.

To avoid that case happens, this contribution proposes to add the restriction of nal_ref_flag of CRA picture to be 1. (We already have the constraint that the temporal_id of CRA (and IDR) picture shall be 0.)

The corresponding text modifications are as follows.

7.4.1

nal_ref_flag

……

nal_ref_flag shall be equal to 1 for NAL units with nal_unit_type equal to 4 or 5.

# Differentially signaling of per-temporal layer information

It was agreed in the 8th JCT-VC meeting that the max_dec_pic_buffering, num_reorder_pics, and max_latency_increase are signaled according to temporal layers. The below table shows the corresponding syntax elements at SPS.

| for( i = 0; i <= max_temporal_layers_minus1; i++ ) { |  |
| --- | --- |
| max_dec_pic_buffering[ i ] | ue(v) |
| num_reorder_pics[ i ] | ue(v) |
| max_latency_increase[ i ] | ue(v) |
| } |  |

Here is part of the semantics.

max_dec_pic_buffering[ i ] specifies the required size of the HRD decoded picture buffer (DPB) in units of picture storage buffers for the bitstream subset as specified in subclause Error: Reference source not found with  i as input.

num_reorder_pics[ i ] indicates the maximum number of pictures preceding any picture in decoding order and succeeding that picture in output order for the bitstream subset as specified in subclause Error: Reference source not found with  i as input.

We can know that, max_dec_pic_buffering is non-decreasing,
max_dec_pic_buffering[i] >= max_dec_pic_buffering[i-1].

It is similar for the num_reorder_pics.

So this contribution proposes differentially signaling of those two syntax elements to save bits.

The corresponding text modifications are as follows.

7.3.2.1

| for( i = 0; i <= max_temporal_layers_minus1; i++ ) { |  |
| --- | --- |
| max_dec_pic_buffering_increase[ i ] | ue(v) |
| num_reorder_pics_increase[ i ] | ue(v) |
| max_latency_increase[ i ] | ue(v) |
| } |  |

7.4.2.1

max_dec_pic_buffering_increase[ i ] specifies the difference between MaxDecPicBuffering[i] and MaxDecPicBuffering[i-1]. MaxDecPicBuffering[i] is derived as follows:

–	If i is equal to 0, MaxDecPicBuffering[i] = max_dec_pic_buffering_increase[ i ].

–	Otherwise, MaxDecPicBuffering[i] = MaxDecPicBuffering[i−1] + max_dec_pic_buffering_increase[ i ].

MaxDecPicBuffering[i] specifies the required size of the HRD decoded picture buffer (DPB) in units of picture storage buffers for the bitstream subset as specified in subclause Error: Reference source not found with  i as input. The coded video sequence shall not require a decoded picture buffer with size of more than Max( 1, max_dec_pic_buffering MaxDecPicBuffering ) picture storage buffers to enable the output of decoded pictures at the output times specified by dpb_output_delay of the picture timing SEI messages. The value of max_dec_pic_buffering[ i ] MaxDecPicBuffering[i] shall be in the range of 0 to MaxDpbSize (as specified in subclause Error: Reference source not found), inclusive.

- num_reorder_pics_increase[ i ] specifies the difference between NumReorderPics[i] and NumReorderPics[i-1]. NumReorderPics[i] is derived as follows:

–	If i is equal to 0, NumReorderPics [i] = num_reorder_pics_increase [ i ].

–	Otherwise, NumReorderPics [i] = NumReorderPics [i−1] + num_reorder_pics_increase [ i ].

- NumReorderPics[i] indicates the maximum number of pictures preceding any picture in decoding order and succeeding that picture in output order for the bitstream subset as specified in subclause Error: Reference source not found with  i as input. The value of num_reorder_picsNumReorderPics [ i ] shall be in the range of 0 to max_dec_pic_bufferingMaxDecPicBuffering [ i ], inclusive.

And from here, replace all max_dec_pic_buffering (from here, there are 17 remaining) with MaxDecPicBuffering; num_reorder_pics (from here, there are 3 remaining) with NumReorderPics in the text.

# Position of signaling per-temporal layer information in SPS

In the current HEVC, the per-temporal layer information is signal in SPS. When a decoder is not able to decode the whole sequence because the DPB size does not satisfy the requirement of max_dec_pic_buffering, it can choose to only decode the temporal layers satisfying the constraint such as DPB size constraint.

The bitstream extracting can also be applied at the media gateway. The decoder can tell the max DPB size in number of pictures to the media gateway, and the media gateway can discard all the pictures not satisfying the constraint. So the per-temporal layer information may be parsed at the media gateway. But in the current design of SPS, before the per-temporal layer information, there are some other syntax elements, which are not needed at media gateway. So this document propose

- Move max_temporal_layers_minus1 and per-temporal layer information (the loop of signaling max_dec_pic_buffering[ i ], num_reorder_pics[ i ], and max_latency_increase[ i ]) together in SPS.

- And place max_temporal_layers_minus1 and per-temporal layer information at earlier position in SPS, such as

7.3.2.1

| max_temporal_layers_minus1 | u(3) |
| --- | --- |
| for( i = 0; i <= max_temporal_layers_minus1; i++ ) { |  |
| max_dec_pic_buffering[ i ] | ue(v) |
| num_reorder_pics[ i ] | ue(v) |
| max_latency_increase[ i ] | ue(v) |
| } |  |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| pic_cropping_flag | u(1) |
| if( pic_cropping_flag ) { |  |
| pic_crop_left_offset | ue(v) |
| pic_crop_right_offset | ue(v) |
| pic_crop_top_offset | ue(v) |
| pic_crop_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_bit_depth_luma_minus1 | u(4) |
| pcm_bit_depth_chroma_minus1 | u(4) |
| } |  |
| qpprime_y_zero_transquant_bypass_flag | u(1) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| for( i = 0; i <= max_temporal_layers_minus1; i++ ) { |  |
| max_dec_pic_buffering[ i ] | ue(v) |
| num_reorder_pics[ i ] | ue(v) |
| max_latency_increase[ i ] | ue(v) |
| } |  |

# POC derivation when num_reorder_pics is 0

In 8.3.1,

PicOrderCntMsb of the current picture is derived as specified by the following pseudo-code:

if( ( pic_order_cnt_lsb <  prevPicOrderCntLsb )  &&
		( ( prevPicOrderCntLsb − pic_order_cnt_lsb )  >=  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb + MaxPicOrderCntLsb	(8-1)
else if( (pic_order_cnt_lsb  >  prevPicOrderCntLsb )  &&
		( (pic_order_cnt_lsb − prevPicOrderCntLsb )  >  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb − MaxPicOrderCntLsb
else
	PicOrderCntMsb = prevPicOrderCntMsb

It means that PicOrderCntMsb can be equal to, greater than, and smaller than prevPicOrderCntMsb. But for the case num_reorder_pics is 0, the POC of current picture should not be smaller than the POC of previous picture. So in such a case, we propose to refine the deriving process to

If num_reorder_pics[temporal_id] is equal to 0, PicOrderCntMsb of the current picture is derived as specified by the following pseudo-code:

if( pic_order_cnt_lsb < prevPicOrderCntLsb )
	PicOrderCntMsb = prevPicOrderCntMsb + MaxPicOrderCntLsb
else
	PicOrderCntMsb = prevPicOrderCntMsb

Otherwise (num_reorder_pics[temporal_id] is not equal to 0), PicOrderCntMsb of the current picture is derived as specified by the following pseudo-code:

if( ( pic_order_cnt_lsb <  prevPicOrderCntLsb )  &&
		( ( prevPicOrderCntLsb − pic_order_cnt_lsb )  >=  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb + MaxPicOrderCntLsb	(8-2)
else if( (pic_order_cnt_lsb  >  prevPicOrderCntLsb )  &&
		( (pic_order_cnt_lsb − prevPicOrderCntLsb )  >  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb − MaxPicOrderCntLsb
else
	PicOrderCntMsb = prevPicOrderCntMsb

And this contribution propose the bitstream conformance in C.3.2 to be modified to

It is a requirement of bitstream conformance that when num_reorder_pics[max_temporal_layers_minus1] is not equal to 0, the value of maxPicOrderCnt – minPicOrderCnt shall be less than MaxPicOrderCntLsb / 2.

In the default low delay coding structure, at least 5 bits are needed to represent the LSB of POC. With the refined POC derivation process and bitstream conformance, 4 bits are enough.

# Counter of Combined merge candidates

This aspect has already been reported in ticket #395 and #396. JCTVC-I0414 seems to propose the same aspect.

In HM-6.0, the combined merge candidates are generated according to the following process.

Int iCombinedCount = 0;
...
for (Int idx=0; idx<uiCutoff*(uiCutoff-1) && uiArrayAddr!=MRG_MAX_NUM_CANDS && iCombinedCount<5; idx++)
{
 ...
 if (...)
 {}
 else
 {
   uiArrayAddr++;
   iCombinedCount++;
 }
}

As MRG_MAX_NUM_CANDS is no greater than 5 and the initial value of uiArrayAddr is no less than the initial value of iCombinedCount (0), when the condition of
uiArrayAddr!=MRG_MAX_NUM_CANDS
is true, the condition of
iCombinedCount<5
is always true.

So iCombinedCount is not necessary to be there. There should not be any performance changing.

Text Modifications

8.5.2.1.3

When numOrigMergeCand is greater than 1 and less than MaxNumMergeCand, the variable numInputMergeCand is set to numMergeCand, the variables combIdx and combCnt are is set to 0, the variable combStop is set to FALSE and the following steps are repeated until combStop is equal to TRUE.

……

    - The variable combCnt is incremented by 1.

……

5.	When combIdx is equal to ( numOrigMergeCand * ( numOrigMergeCand − 1 ) ) or numMergeCand is equal to MaxNumMergeCand or combCnt is equal to 5, combStop is set to TRUE.

# Patent rights declaration(s)

University of Science and Technology of China may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).