| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0144 |
| --- | --- |

| Title: | Removal of input picture size constraint and picture cropping offset parameters in sequence parameter set |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Il-Koo Kim Youngo Park Jaehyun Kim JeongHoon Park | Email: | ilkoo.kim@samsung.com youngo.park@samsung.com jhgim@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this contribution, removal of a constraint on picture width and height (picture width and height shall be integer multiple of minimum CU size) is proposed. Based on the proposal, it is also suggested to move the position of picture cropping offset parameters from sequence parameter set (SPS) to supplemental enhancement information (SEI). By this modification, general cropping functionality is kept as optional and only mandatory cropping process is kept in the decoding process.

# Cropping window syntax in the committee draft (CD)

In the current committee draft (CD) text, picture width and height are signalled in sample unit by pic_width_in_luma_samples and pic_height_in_luma_samples in sequence parameter set (SPS). pic_width_in_luma_samples and pic_height_in_luma_samples shall be integer multiple of minimum coding unit (CU) size. When either width or height of input picture is not multiple of minimum CU size, input picture should be extended to meet the constraint and fed to encoder. Picture cropping offset parameter may be specified in the bitstream and applied for output in order to get original picture region at decoder. Since applying the cropping is not mandatory according to the current CD text, encoder can choose to crop or not even in this case. This is same approach adopted in H.264/AVC [1].

Current design of picture cropping syntax supports general functionality by signalling four picture cropping offset parameters (pic_crop_left_offset, pic_crop_right_offset, pic_crop_top_offset, and pic_crop_bottom_offset) in SPS [2]. Arbitrary rectangular region can be obtained by those four syntax elements. Signalling of picture cropping offset parameters is controlled by pic_cropping_flag syntax element. When pic_cropping_flag is equal to 1, the picture cropping offset parameters follow next in the SPS. pic_cropping_flag equal to 0 indicates that the picture cropping offset parameters are not present. Table I is a snippet of SPS brought from the CD text to illustrate current syntax design for picture cropping.

Although pic_cropping_flag may be set by 0 in current syntax design since no constraints are specified in CD, cropping is required to get original picture region when either width or height of input picture is not multiple of minimum coding unit (CU) size. In this case, encoder should set pic_cropping_flag by 1 and signal four syntax elements to indicate original picture region. It should be noted that the cropping operation is applied outside the coding loop for output only and doesn't affect to the decoding process.

Table I. Syntax elements for picture cropping in CD text

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| max_temporal_layers_minus1 | u(3) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| pic_cropping_flag | u(1) |
| if( pic_cropping_flag ) { |  |
| pic_crop_left_offset | ue(v) |
| pic_crop_right_offset | ue(v) |
| pic_crop_top_offset | ue(v) |
| pic_crop_bottom_offset | ue(v) |
| } |  |

# Removal of picture size constraint

In this contribution, removal of constraint on picture width and height is proposed. Based on the proposal, position of picture cropping offset parameters is moved from SPS to supplemental enhancement information (SEI). And mandatory picture cropping is conducted in the decoding process when either width or height of input picture is not multiple of minimum CU size. By this modification, general cropping functionality is kept as optional and only mandatory cropping process is kept in the decoding process.

Figure 1 illustrates an example when resolution of input picture is 176x142 and minimum CU size is 8. Although width and height are signalled in sample units, the picture is extended to 176x144 internally since the height of picture is not multiple of minimum CU size (8). At decoder, cropping process is applied to extended picture to get original picture region. Original picture region is easily obtained since width and height of input picture are specified in a sample unit. Additional cropping may be done by signalling the region through the SEI.

Figure 1. Proposed encoding and decoding process without picture size constraint

# Suggested modifications of CD text

Table II is suggested syntax table in SPS. Five syntax elements are moved to SEI as shown Table III and Table IV. In addition, internal process to specify the internal picture buffer size is added in the text. Complete modifications are found in the separate document.

## Suggested syntax tables

Table II. Proposed syntax table in SPS

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| max_temporal_layers_minus1 | u(3) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| pic_cropping_flag | u(1) |
| if( pic_cropping_flag ) { |  |
| pic_crop_left_offset | ue(v) |
| pic_crop_right_offset | ue(v) |
| pic_crop_top_offset | ue(v) |
| pic_crop_bottom_offset | ue(v) |
| } |  |

Table III. SEI payload syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| else if( payloadType = = 1 ) |  |
| pic_timing( payloadSize ) |  |
| else if( payloadType = = 2 ) |  |
| pan_scan_rect( payloadSize ) |  |
| else if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) |  |
| else if( payloadType = = 4 ) |  |
| user_data_registered_itu_t_t35( payloadSize ) |  |
| else if( payloadType = = 5 ) |  |
| user_data_unregistered( payloadSize ) |  |
| else if( payloadType = = 6 ) |  |
| recovery_point( payloadSize ) |  |
| else if( payloadType = = 9 ) |  |
| scene_info( payloadSize ) |  |
| else if( payloadType = = 15 ) |  |
| full_frame_snapshot( payloadSize ) |  |
| else if( payloadType = = 16 ) |  |
| progressive_refinement_segment_start( payloadSize ) |  |
| else if( payloadType = = 17 ) |  |
| progressive_refinement_segment_end( payloadSize ) |  |
| else if( payloadType = = 19 ) |  |
| film_grain_characteristics( payloadSize ) |  |
| else if( payloadType = = 20 ) |  |
| deblocking_filter_display_preference( payloadSize ) |  |
| else if( payloadType = = 22 ) |  |
| post_filter_hint( payloadSize ) |  |
| else if( payloadType = = 23 ) |  |
| tone_mapping_info( payloadSize ) |  |
| else if( payloadType = = 45 ) |  |
| frame_packing_arrangement( payloadSize ) |  |
| else if( payloadType = = 46 ) |  |
| picture_cropping_offset_parameter( payloadSize ) |  |
| else if( payloadType = = 47 ) |  |
| display_orientation( payloadSize ) |  |
| else if( payloadType = = 128 ) |  |
| sop_description( payloadSize ) |  |
| else if( payloadType = = 129 ) |  |
| field_indication( payloadSize ) |  |
| else if( payloadType = = 130) |  |
| decoded_picture_hash( payloadSize ) |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| if( !byte_aligned( ) ) { |  |
| bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

Table IV. Picture cropping offset parameter SEI message syntax

| picture_cropping_offset_parameter( payloadSize ) { | Descriptor |
| --- | --- |
| pic_cropping_flag | u(1) |
| if( pic_cropping_flag ) { |  |
| pic_crop_left_offset | ue(v) |
| pic_crop_right_offset | ue(v) |
| pic_crop_top_offset | ue(v) |
| pic_crop_bottom_offset | ue(v) |
| } |  |
| } |  |

## Sequence parameter set RBSP semantics

pic_width_in_luma_samples specifies the width of each decoded picture in units of luma samples. pic_width_in_luma_samples shall not be equal to 0.

The variable PicWidthInSamplesL is derived as follows.

PicWidthInSamplesL = Ceil( pic_width_in_luma_samples ÷ ( 1 << Log2MinCbSize ) )*( 1 << Log2MinCbSize )

pic_height_in_luma_samples specifies the height of each decoded picture in units of luma samples. pic_height_in_luma_samples shall not be equal to 0.

The variable PicHeightInSamplesL is derived as follows.

PicHeightInSamplesL = Ceil( pic_height_in_luma_samples ÷ ( 1 << Log2MinCbSize ) )*( 1 << Log2MinCbSize )

pic_cropping_flag equal to 1 indicates that the picture copping offset parameters follow next in the sequence parameter set. pic_cropping_flag equal to 0 indicates that the picture cropping offset parameters are not present.

pic_crop_left_offset, pic_crop_right_offset, pic_crop_top_offset, and pic_crop_bottom_offset specify the samples of the pictures in the coded video sequence that are output from the decoding process, in terms of a rectangular region specified in picture coordinates for output.

The variables CropUnitX and CropUnitY are derived as follows:

–	If ChromaArrayType is equal to 0, CropUnitX and CropUnitY are derived as:

CropUnitX = 1		(7-1)
CropUnitY = 1		(7-2)

–	Otherwise (ChromaArrayType is equal to 1, 2, or 3), CropUnitX and CropUnitY are derived as:

CropUnitX = SubWidthC		(7-3)
CropUnitY = SubHeightC		(7-4)

The picture cropping rectangle contains luma samples with horizontal picture coordinates from CropUnitX * pic_crop_left_offset to PicWidthInSamplesL − ( CropUnitX * pic_crop_right_offset + 1 ) and vertical picture coordinates from CropUnitY * pic_crop_top_offset to PicHeightInSamplesL − ( CropUnitY * pic_crop_bottom_offset + 1 ), inclusive. The value of pic_crop_left_offset shall be in the range of 0 to ( PicWidthInSamplesL / CropUnitX ) − ( pic_crop_right_offset + 1 ), inclusive; and the value of pic_crop_top_offset shall be in the range of 0 to ( PicHeightInSamplesL / CropUnitY ) − ( pic_crop_bottom_offset + 1 ), inclusive.

When pic_cropping_flag is equal to 0, the values of pic_crop_left_offset, pic_crop_right_offset, pic_crop_top_offset, and pic_crop_bottom_offset is inferred to be equal to 0.

When ChromaArrayType is not equal to 0, the corresponding specified samples of the two chroma arrays are the samples having picture coordinates ( x / SubWidthC, y / SubHeightC ), where ( x, y ) are the picture coordinates of the specified luma samples.

NOTE – The picture cropping parameters are only applied at the output. All internal decoding processes are applied to the uncropped picture size.

## SEI payload semantics

    - Picture cropping offset parameter SEI message semantics

pic_cropping_flag equal to 1 indicates that the picture cropping offset parameters follow next in the sequence parameter set. pic_cropping_flag equal to 0 indicates that the picture cropping offset parameters are not present.

pic_crop_left_offset, pic_crop_right_offset, pic_crop_top_offset, and pic_crop_bottom_offset specify the samples of the pictures in the coded video sequence that are output from the decoding process, in terms of a rectangular region specified in picture coordinates for output.

The variables CropUnitX and CropUnitY are derived as follows:

–	If ChromaArrayType is equal to 0, CropUnitX and CropUnitY are derived as:

CropUnitX = 1		(7-5)
CropUnitY = 1		(7-6)

–	Otherwise (ChromaArrayType is equal to 1, 2, or 3), CropUnitX and CropUnitY are derived as:

CropUnitX = SubWidthC		(7-7)
CropUnitY = SubHeightC		(7-8)

The picture cropping rectangle contains luma samples with horizontal picture coordinates from CropUnitX * pic_crop_left_offset to PicWidthInSamplesL − ( CropUnitX * pic_crop_right_offset + 1 ) and vertical picture coordinates from CropUnitY * pic_crop_top_offset to PicHeightInSamplesL − ( CropUnitY * pic_crop_bottom_offset + 1 ), inclusive. The value of pic_crop_left_offset shall be in the range of 0 to ( PicWidthInSamplesL / CropUnitX ) − ( pic_crop_right_offset + 1 ), inclusive; and the value of pic_crop_top_offset shall be in the range of 0 to ( PicHeightInSamplesL / CropUnitY ) − ( pic_crop_bottom_offset + 1 ), inclusive.

When pic_cropping_flag is equal to 0, the values of pic_crop_left_offset, pic_crop_right_offset, pic_crop_top_offset, and pic_crop_bottom_offset is inferred to be equal to 0.

When ChromaArrayType is not equal to 0, the corresponding specified samples of the two chroma arrays are the samples having picture coordinates ( x / SubWidthC, y / SubHeightC ), where ( x, y ) are the picture coordinates of the specified luma samples.

NOTE – The picture cropping parameters are only applied at the output. All internal decoding processes are applied to the uncropped picture size.

    - Picture decoding and output

When output, the picture shall be cropped, using pic_width_in_luma_samples and pic_height_in_luma_samples or the cropping rectangle specified in the picture cropping offset parameter SEI message.

# Conclusions

In this contribution, removal of a constraint on picture width and height was proposed. Based on the proposal, it was also suggested to move the position of picture cropping offset parameters from SPS to SEI. By this modification, general cropping functionality is kept as optional and only mandatory cropping process is kept in the decoding process. It is recommended to adopt this proposal in the HEVC standard.

# References

- Joint Video Team of ITU-T and ISO/IEC JTC 1, “Draft ITU T Recommendation and Final Draft International Standard of Joint Video Specification (ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC),” Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG, JVT-G050, March 2003

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm and T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 6,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1103, 8th Meeting: San José, CA, USA, 1–10 February, 2012

- K. Suehring and B. Bross, " Picture size signaling and cropping operation," Document of Joint Collaborative Team on Video Coding, JCTVC-H0485, 8th Meeting: San José, CA, USA, 1–10 February, 2012

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).