| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0147 |
| --- | --- |

| Title: | AHG4: Parallel Processing Entry Point Indication For Low Delay Applications |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Stewart Worrall Adrian Wise Aspex House, 44-45 Oxford Street, High Wycombe, Buckinghamshire, HP11 2DJ | Tel: Email: | +44 1494 558121 stewart.worrall@aspex-semi.com |
| Source: | Aspex Semiconductor |  |  |

_____________________________

# Abstract

To permit parallel decoding of tile or wavefront substreams it is necessary to include indicators in the bitstream, so that the decoder is able to access these substreams. Two approaches currently exist in the Committee Draft [1]: an entry point offset table in the slice header, and tile markers. The entry point offset table approach in general requires fewer bits, but incurs delay. Tile markers allow low delay encoding, but require a 24 bit marker code to be inserted before each substream. This proposal introduces a technique that claims to have lower delay than the entry point table scheme, and requires less overhead than the marker code scheme. The technique is compatible with both tiles and wavefront parallel processing, and it is recommended that this technique is used to replace the two separate schemes that currently exist in the CD.

[Changes v1->r1: Typos, additional results, clearer figure]

# Introduction

This proposal aims to provide a single entry point indicator scheme for tiles and wavefronts, providing low delay, and compression efficiency that is comparable to, or better than, the entry point offset table specified in the committee draft [1].

The scheme will now be described in more detail. In the description, the following definitions will be used:

- A substream is used to indicate either a wavefront substream, or a complete tile

- An entry point marker is a 24-bit unique code, currently known as entry_point_marker_two_3bytes in the CD

- An entry point offset gives the distance from the start of the offset to the start of the next substream

A slice may consist of a number of parallel encoded substreams. In the proposed scheme, the first substream in a slice is encoded without any entry point offset. The second substream in a slice is encoded with an entry point marker at the beginning, along with an entry point offset, which tells the decoder the location of the next substream. All other substreams are encoded with entry point offsets pointing to the next substream. The final substream in a slice may be encoded with a zero offset to reduce the number of bits used. This will allow a decoder to locate the first substream by searching for the 24-bit marker. All other substreams can be located by using the entry point offsets to jump forwards in the bitstream.

This arrangement means that the first substream can be written to the encoder output buffer without delay. As it is not possible to write the remaining substreams to the encoder output buffer until the first substream has been written, the delay incurred by using entry points offsets is close to zero for parallel encoders.

In addition to these changes, it is also proposed that entry points be inserted for every tile, and for every CTB line (when WPP is used). This is required to enable flexible parallel decoding, and also means that some parameters (such as number of substreams) can be inferred, rather than explicitly signalled.

Figure 1: Location of entry point syntax in proposed scheme for an example featuring four substreams.

Table 1: Example of impact on slice level syntax for a 1080p video (CTB size of 64x64, one slice per picture).

| Entry point index table [1] |  | Entry point markers [1] |  | Proposed scheme |  |
| --- | --- | --- | --- | --- | --- |
| Num | Syntax element | Num | Syntax element | Num | Syntax element |
| 1 | num_entry_point_offsets ue(v) | 1 | num_entry_point_offsets ue(v) | 1 | entry_point_marker_two_3bytes f(24) |
| 1 | offset_len_minus1 ue(v) | 16 | entry_point_marker_two_3bytes f(24) | 16 | offset_len_idc f(3) |
| 16 | entry_point_offset[ i ] u(v) | 16 | tile_idx_minus_1 u(v) | 16 | entry_point_offset u(v) |

# Revised Syntax

## Syntax Tables

It is proposed that the existing entry point entry index table in the slice header be removed, and that all syntax related to entry points be moved to the slice_data(). Table 1 shows the existing syntax included in [1], while Table 2 describes the updated syntax.

It should be noted that Table 2 includes an offset_len_idc syntax element. This is equivalent to the offset_len_minus1 syntax element in [1]. It was found to be more efficient to represent the offset length using a fixed length element, (see section 2.2), rather than as a ue(v) element. The length of the entry_point_offset syntax element is calculated using:

EntryPointOffsetLength = (offset_len_idc + 1) << 2

This means that bit lengths from 4-32 can be used to code the entry point offset, in jumps of four bits.

Table 2: Existing slice data syntax for entry point markers.

| if( moreDataFlag && ( ( tiles_or_entropy_coding_sync_idc = = 1 && TileId[ CtbAddrTS ] != TileId[ CtbAddrTS − 1 ] ) ¦ ¦ ( tiles_or_entropy_coding_sync_idc = = 2 && num_substream_minus1 > 0 && CtbAddrTS / PicWidthInCtbs <= num_substream_minus1 && CtbAddrTS % PicWidthInCtbs = = 0 ) ) ) { |  |
| --- | --- |
| rbsp_trailing_bits( ) |  |
| if( nextbits(24) = = 0x000002 ) { |  |
| entry_point_marker_two_3bytes | f(24) |
| tile_idx_minus_1 | u(v) |
| } |  |
| } |  |

Table 3: Revised slice data syntax for proposed scheme.

| if( moreDataFlag && ( ( tiles_or_entropy_coding_sync_idc = = 1 && TileId[ CtbAddrTS ] != TileId[ CtbAddrTS − 1 ] ) ¦ ¦ ( tiles_or_entropy_coding_sync_idc = = 2 && CtbAddrTS % PicWidthInCtbs = = 0 ) ) ) { |  |
| --- | --- |
| rbsp_trailing_bits( ) |  |
| if((TileId[ CtbAddrTS ] - TileId[ CtbAddrRStoTS[ SliceCtbAddrRS ] ] == 1) ¦¦ ((CtbAddrRS / PicWidthInCtbs) – ( SliceCtbAddrRS / PicWidthInCtbs) == 1)) { |  |
| entry_point_marker_two_3bytes | f(24) |
| } |  |
| offset_len_idc | f(3) |
| entry_point_offset[ i ] | u(v) |
| } |  |

## Syntax Description

The updated syntax is intended to replace existing syntax for offset_len_minus1 and for entry_point_offset[i], and has been checked for compatibility with the text in section 9.2, which specifies how to derive the Bitstream pointers. As well as updating to reflect the new scheme, the text has been rewritten in a form that the authors believe provides more clarity.

### Original syntax descriptions

num_entry_point_offsets specifies the number of entry_point_offset[ i ] syntax elements in the slice header. When tiles_or_entropy_coding_sync_idc is equal to 1, the value of num_entry_point_offsets shall be in the range of 0 to ( num_tile_columns_minus1 + 1 ) * ( num_tile_rows_minus1 + 1 ) − 1, inclusive. When tiles_or_entropy_coding_sync_idc is equal to 2, the value of num_entry_point_offsets shall be in the range of 0 to num_substreams_minus1, inclusive. When not present, the value of num_entry_point_offsets is inferred to be equal to 0.

offset_len_minus1 plus 1 specifies the length, in bits, of the entry_point_offset[ i ] syntax elements.

entry_point_offset[ i ] specifies the i-th entry point offset, in bytes and shall be represented by offset_len_minus1 plus 1 bits. The coded slice NAL unit consists of num_entry_point_offsets + 1 subsets, with subset index values ranging from 0 to num_entry_point_offsets, inclusive. Subset 0 consists of bytes 0 to entry_point_offset[ 0 ] − 1, inclusive, of the coded slice NAL unit, subset k, with k in the range of 1 to num_entry_point_offsets - 1, inclusive, consists of bytes entry_point_offset[ k − 1 ] to entry_point_offset[ k ] + entry_point_offset[ k − 1 ] − 1, inclusive, of the coded slice NAL unit, and the last subset (with subset index equal to num_entry_point_offsets) consists of the remaining bytes of the coded slice NAL unit.

NOTE – The NAL unit header and the slice header of a coded slice NAL unit are always included in subset 0.

When tiles_or_entropy_coding_sync_idc is equal to 1 and num_entry_point_offsets is greater than 0, each subset shall contain all coded bits of one or multiple complete tiles, and the number of subsets shall be equal to or less than the number of tiles in the slice.

When tiles_or_entropy_coding_sync_idc is equal to 2 and num_entry_point_offsets is greater than 0, subset k, for each of all the possible k values, shall contain all bits to be used during the initialization process for the current bitstream pointer k.

### Updated syntax descriptions

offset_len_idc is used to derive the length in bits of the entry_point_offset syntax element that immediately follows the offset_len_idc. The number of bits for the entry_point_offset syntax element is derived as:

EntryPointOffsetLength = (offset_len_idc + 1) << 2

entry_point_offset[ i ] specifies the i-th entry point offset, in bytes and shall be represented by EntryPointOffsetLength bits. The coded slice NAL unit consists of NumEntryPointOffsets subsets.

When tiles_or_entropy_coding_sync_idc is equal to 1:

NumEntryPointOffsets = (num_tile_columns_minus1+1) * (num_tile_rows_minus1+1)

When tiles_or_entropy_coding_sync_idc is equal to 2:

NumEntryPointOffsets = PicHeightInCtbs

The slice NAL unit subset index range is defined as SubsetIndexRange, where:

SubsetIndexRange = 0…NumEntryPointOffsets-1

Subset 0 consists of Subset[0] bytes of the coded slice NAL unit, where:

Subset[0] = 0…entry_point_offset[ 0 ] − 1

Subset k, consists of Subset[k] bytes of the coded slice NAL unit, where:

Subset[k] = entry_point_offset[ k − 1 ]… entry_point_offset[ k ] + entry_point_offset[ k − 1 ] − 1

The last subset, Subset[NumEntryPointOffsets] consists of the remaining bytes of the coded slice NAL unit. If entry_point_offset[ i ] is equal to zero, then the decoder shall infer that the following bytes belong to the last subset, Subset[NumEntryPointOffsets].

NOTE – The NAL unit header and the slice header of a coded slice NAL unit are always included in subset 0.

When tiles_or_entropy_coding_sync_idc is equal to 1 and NumEntryPointOffsets is greater than 1, each subset shall contain all coded bits of one complete tile, and the number of subsets shall be equal to the number of tiles in the slice. The variable, i, shall be derived as:

i = TileId

When tiles_or_entropy_coding_sync_idc is equal to 2, subset k, for each of all the possible k values, shall contain all bits to be used during the initialization process for the current bitstream pointer k, and it shall be possible to initialize bitstream pointers for all possible values of k. The variable, i, shall be derived as:

i = CtbAddrTS / PicWidthInCtbs

# Experimental Results

Results showing the difference between existing entry point schemes have been provided in previous contributions [4], although these results use fewer entry points than tested in this proposals, so do not fully indicate the differences between entry markers and entry point tables.

To illustrate the differences here, tests were carried out using:

- HM-6.1 – existing entry point table scheme compared to proposed scheme

- Wavefront parallel processing – one entry point substream per CTB line, flushing for every CTB line

The results show no differences in compression efficiency for the All Intra case between the proposed low delay scheme and the entry point table scheme. For the Random Access case, 0.1-02% increase in BD-PSNR is observed.

Table 4: Experimental results, where the reference is HM-6.1 with one slice header entry offset per line.

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | - |  |  |
| Dec Time[%] | - |  |  |

|  | Random Access Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.1% | 0.1% | 0.1% |
| Class B | 0.1% | 0.1% | 0.1% |
| Class C | 0.2% | 0.2% | 0.2% |
| Class D | 0.3% | 0.3% | 0.3% |
| Class E |  |  |  |
| Overall | 0.2% | 0.2% | 0.2% |
|  | 0.2% | 0.2% | 0.2% |
| Class F | 0.2% | 0.1% | 0.1% |
| Enc Time[%] | - |  |  |
| Dec Time[%] | - |  |  |

|  | Low delay B Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.1% | 0.1% | 0.1% |
| Class C | 0.2% | 0.1% | 0.1% |
| Class D | 0.3% | 0.2% | 0.2% |
| Class E | 1.1% | 0.9% | 0.9% |
| Overall | 0.4% | 0.3% | 0.3% |
|  | 0.4% | 0.3% | 0.3% |
| Class F | 0.3% | 0.2% | 0.2% |
| Enc Time[%] | - |  |  |
| Dec Time[%] | - |  |  |

# Summary and Recommendations

The proposed changes can be summarised as:

- Removal of the entry point index table in slice_header()

- Impose a constraint where entry points must be specified for all LCU lines for wavefronts, and entry points must be specified for all tiles

- Remove num_substreams_minus1 from PPS, as it can be inferred from the number of LCU lines

- Remove tile_idx_minus1, as this can be inferred by mandating an entry point for every tile

- Modify slice_data() syntax to implement the proposed scheme

- Update syntax description text to reflect updated slice_data() syntax and to improve clarity

# References

[1] Bross et al., “High efficiency video coding (HEVC) text specification draft 6,” JCTVC-H1003, 9th Meeting: San Jose, USA, 1–10 February, 2012.

[2] Misra et al., “Harmonization of Entry Points for tiles and Wavefront Processing,” JCTVC-G722, 7th JCT-VC Meeting, 7th Meeting: Geneva, CH, 21-30 November, 2011.

[3] Hendry et al., “AHG4: Harmonized Method for Signalling Entry Points of tiles and WPP Substreams,” JCTVC-H0556, 9th Meeting: San Jose, USA, 1–10 February, 2012.

[4] Kiran Misra, Andrew Segall, “New results for parallel decoding for Tiles”, JCTVC-F594, JCT-VC Meeting, 6th Meeting: Torino, IT, 14-22 July, 2011.

# Patent rights declaration(s)

Aspex Semiconductor does not have any current or pending patent rights relating to the technology described in this contribution.