| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0148 |
| --- | --- |

| Title: | LM Mode Clean-Up |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-VC] |  |  |
| Purpose: | [Proposal] |  |  |
| Author(s) or Contact(s): | Lingzhi Liu (HiSilicon Technologies) Rik Allen (Altera Europe Limited) Xingyu Zhang (The Hong Kong University of Science & Technology) Pieter Kapsenberg (Intel Corp) Edouard Francois (Canon Research Centre France) | Tel: Email: | +1 4083304996 lingzhi.liu@huawei.com rallen@altera.com eexyzhang@ust.hk pieter.kapsenberg@intel.com edouard.francois@crf.canon.fr |
| Source: | HiSilicon Technologies, Altera Europe Limited, The Hong Kong University of Science & Technology, Intel Corp and Canon Research Centre France |  |  |

_____________________________

# Abstract

It has been observed that several mismatches between WD and HM software for Luma-based-Chroma (LM) mode. This contribution proposes the necessary clean-up work for these mismatches.

# Problem Statement

For chroma blocks, the current design of High Efficiency Video Coding (HEVC) has a chroma from luma intra prediction mode, which is also referred as LM mode, in addition to the directional modes and the DC mode. [1][2][3].

According to the HEVC working draft [1], the current design of LM mode uses reconstructed luma samples to generate chroma samples. The values of the prediction are derived as the following ordered steps [1]:

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS−1, are derived as the following ordered steps:

- Variable k3 and the sample array pY’ are derived as:

k3 = Max( 0, BitDepthC + Log2( nS ) − 14 )		(8-1)

pY’[ x, −1 ] = ( PLM[ 2x−1, −1 ] + 2*PLM[ 2x, −1 ] + PLM[ 2x+1, −1 ] + 2 ) >> 2, with x = 0..nS−1	(8-2)

pY’[ −1, y ] = ( PLM[ −1, 2y ] + PLM[ −1, 2y+1 ] ) >> 1, with y = 0..nS−1	(8-3)

pY’[ x, y ] = ( recSamplesL[ 2x, 2y ] + recSamplesL[ 2x, 2y+1 ] ) >> 1, with x, y = 0..nS−1	(8-4)

- Variables L, C, LL, LC and k2 are derived as follows:

L = y=0nS−1pY'[−1, y]+x=0nS−1pY'[x, −1]>>k3		(8-5)

C = y=0nS−1p[−1, y]+x=0nS−1p[x, −1]>>k3		(8-6)

LL = y=0nS−1pY'[−1, y]2+x=0nS−1pY'[x, −1]2>>k3	(8-7)

LC = y=0nS−1pY'[−1, y]∗p[−1, y]+y=0nS−1pY'[x, −1]∗p[x,−1]>>k3	(8-8)

k2 = Log2( (2*nS) >> k3 )		(8-9)

- Variables a, b and k are derived as:

a1 	= ( LC << k2 ) – L*C		(8-10)
a2 	= ( LL << k2 ) – L*L		(8-11)
k1 	= Max( 0, Log2( abs( a2 ) ) − 5 ) – Max( 0, Log2( abs( a1 ) ) − 14 ) + 2	(8-12)
a1s	= a1 >> Max(0, Log2( abs( a1 ) ) − 14 )		(8-13)
a2s	= abs( a2 >> Max(0, Log2( abs( a2 ) ) − 5 ) )		(8-14)
a3	= a2s < 1 ? 0 : Clip3( −215, 215−1, a1s*lmDiv + ( 1 << ( k1 − 1 ) ) >> k1 )	(8-15)

a	= a3 >> Max( 0, Log2( abs( a3 ) ) − 6 )		(8-16)
k	= 13 – Max( 0, Log2( abs( a ) ) − 6 )		(8-17)

b	= ( L – ( ( a*C ) >> k1 ) + ( 1 << ( k2 − 1 ) ) ) >> k2	(8-18)

where lmDiv is specified in Table 8-2 with the input a2s.

- The values of the prediction samples predSamples[ x, y ] are derived as:

predSamples[ x, y ] = Clip1C( ( ( pY’[ x, y ] * a ) >> k ) + b ), with x, y = 0..nS−1	(8-19)

# Mismatches between WD and HM6.0

It has been observed that several mismatches between WD and HM software, some are listed as bellowing:

| Mismatch/Problem | Solution | Change |
| --- | --- | --- |
| Input parameters should specify both Luma and Chroma neighbouring samples and their locations. | Change accordingly | WD |
| C and L should be swapped in (8-75) | Change accordingly | WD |
| Equations of (8-69) to (8-74) use the Log2 function, which is defined as the base-2 logarithm , thus it appears to be a floating point operation and it isn’t clear how to convert to integer | Using mathematic function Floor(log2 (x)) defined in 5.7 in WD; using equivalent function GetFloorLog2( x ) in HM | WD &HM |
| In (8-72), k1 can be zero, (k1-1) can be negative, and the operation 1 << ( k1 − 1 ) is undefined. | Change accordingly | WD |
| The operation “+ ( 1 << ( k1 − 1 ) ) >> k1” in (8-72) is not used in HM6.0 | Change accordingly | WD |
| (8-73) in HM6.0 uses the function of CountLeadingZerosOnes(x), which is different from Log2(x) in the WD | Using GetFloorLog2() in HM and WD | WD &HM |
| CountLeadingZerosOnes(x) in HM is equal to 14 - Floor(log2(abs( x) + (sign(x) -1)/2 )) when x!=0. | Change accordingly to simplify the description and remove CountLeadingZerosOnes(x) | WD&HM |
| GetMSB (x) in HM is equal to Floor(log2(x))+1 when x!=0. | Change accordingly to simplify the description and remove GetMSB (x) | WD &HM |
| The calculation of iCountShift in HM6.0 could be simplified as g_aucConvertToBit[ uiWidth ] + 3 | Change accordingly | HM |

# WD Changes

The WD changes are listed here as well as in Appendix 1 with highlights.

8.4.3.1.8 Specification of Intra_FromLuma prediction mode

Inputs to this process are:

–	a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

–	 luma neighbouring samples PLM [ x, y ], with x=-1, y = 0... 2*nS−1; and x=0…2*nS-1, y=-1.

–	chroma neighbouring samples p [ x, y ], with x=-1, y = 0... nS−1; and x=0…nS-1, y=-1.

–	a variable nS specifying the prediction size.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS−1.

This intra prediction mode is invoked when intraPredMode is equal to 35.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS−1, are derived as the following ordered steps:

- Variable k0 and the sample array pY’ are derived as:

k0 = Max( 0, BitDepthC + Log2( nS ) − 14 )		(8-20)

pY’[ x, −1 ] = ( PLM[ 2x−1, −1 ] + 2*PLM[ 2x, −1 ] + PLM[ 2x+1, −1 ] + 2 ) >> 2, with x = 0..nS−1	(8-21)

pY’[ −1, y ] = ( PLM[ −1, 2y ] + PLM[ −1, 2y+1 ] ) >> 1, with y = 0..nS−1	(8-22)

pY’[ x, y ] = ( recSamplesL[ 2x, 2y ] + recSamplesL[ 2x, 2y+1 ] ) >> 1, with x, y = 0..nS−1	(8-23)

- Variables L, C, LL, LC and k1 are derived as:

L = y=0nS−1pY'[−1, y]+x=0nS−1pY'[x, −1]>>k0		(8-24)

C = y=0nS−1p[−1, y]+x=0nS−1p[x, −1]>>k0		(8-25)

LL = y=0nS−1pY'[−1, y]2+x=0nS−1pY'[x, −1]2>>k0	(8-26)

LC = y=0nS−1pY'[−1, y]∗p[−1, y]+y=0nS−1pY'[x, −1]∗p[x,−1]>>k0	(8-27)

k1 = Log2( (2*nS) >> k0 )		(8-28)

- Variables a, b and k are derived as:

a1 	= ( LC << k1 ) – L*C		(8-29)
a2 	= ( LL << k1 ) – L*L		(8-30)

if (a1==0), k2=0

otherwise, k2= Max( 0, Floor(log2(abs(a1)))− 14 )	(8-69)

if (a2==0), k3=0

otherwise, k3= Max( 0, Floor(log2(abs(a2))) − 5 )	(8-70)

k4 = k3 – k2 + 2 		(8-71)

a1s = a1 >> k2 		(8-72)

a2s = a2 >> k3 		(8-73)

if (a2s < 1), a3 = 0

otherwise, if (k4 >= 0), a3 = (a1s*lmDiv) >> k4)

otherwise, (k4 < 0), a3 = (a1s*lmDiv) << (-k4)		(8-74)

a4  = Clip3( −215, 215−1, a3);		(8-75)

if (a4 == 0), k5 = 0

otherwise, k5 = Floor(log2(abs( a4) + (sign(a4) -1)/2 ))– 5	(8-76)

if ( a4 >= -26 && a4 <= 26-1), k = 13

otherwise, k = 13 - k5		(8-77)

a=a4>> (13 – k)		(8-78)

b	= ( C – ( ( a*L ) >> k ) + ( 1 << ( k1 − 1 ) ) ) >> k1	(8-79)

where lmDiv is specified in Table 8-2 with the input a2s.

- The values of the prediction samples predSamples[ x, y ] are derived as:

predSamples[ x, y ] = Clip1C( ( ( pY’[ x, y ] * a ) >> k ) + b ), with x, y = 0..nS−1	(8-80)

Table 8-1 – Specification of lmDiv

| a2s | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lmDiv | 32768 | 16384 | 10923 | 8192 | 6554 | 5461 | 4681 | 4096 | 3641 | 3277 | 2979 | 2731 | 2521 |
| a2s | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| lmDiv | 2341 | 2185 | 2048 | 1928 | 1820 | 1725 | 1638 | 1560 | 1489 | 1425 | 1365 | 1311 | 1260 |
| a2s | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| lmDiv | 1214 | 1170 | 1130 | 1092 | 1057 | 1024 | 993 | 964 | 936 | 910 | 886 | 862 | 840 |
| a2s | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 |
| lmDiv | 819 | 799 | 780 | 762 | 745 | 728 | 712 | 697 | 683 | 669 | 655 | 643 | 630 |
| a2s | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 |  |
| lmDiv | 618 | 607 | 596 | 585 | 575 | 565 | 555 | 546 | 537 | 529 | 520 | 512 |  |

# Simulation Results

Table 1 shows the result of comparing the cleaned-up version with HM6.0 software anchor under encoder_intra_he10 configuration. It is observed that they are exactly the same.

Table 1. Testing results of cleaned-up version vs. HM 6.0 AI_HE10

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 98% |  |  |

Table 2 shows the result of comparing the cleaned-up version with HM6.0 software anchor under encoder_intra_mian configuration with LMChroma switched on. It is observed that they are exactly the same.

Table 2. Testing results of cleaned-up version vs. HM 6.0 AI_MAIN + LMChroma=1

|  | All Intra MAIN |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 98% |  |  |

# Conclusion

It is proposed to consider adopting the clean-up of LM mode into WD and HM7.0 software.

# Reference

[1] Benjamin Bross, et. Al, “WD6: Working Draft 6 of High-Efficiency Video Coding”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-H1003

[2] Jianle Chen, et. Al, “Complexity reduction of chroma intra LM prediction mode”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F494

[3] L. Liu, G. Li, Y. Lin, “Non-CE6a: Remove the large multiplier for LM mode calculation”, JCTVC-H0491, San Jose, Feb., 2012.

[3] Frank Bossen, “Common HM test conditions and software reference configurations”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-H1100

# Patent rights declaration(s)

HiSilicon Technologies, Altera Europe Limited, The Hong Kong University of Science & Technology, Intel Corp and Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix 1 (WD Changes)

##### 8.4.3.1.8 Specification of Intra_FromLuma prediction mode

Inputs to this process are:

–	a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

–	 luma neighbouring samples PLM [ x, y ], with x=-1, y = 0... 2*nS−1; and x=0…2*nS-1, y=-1.

–	chroma neighbouring samples p [ x, y ], with x=-1, y = 0... nS−1; and x=0…nS-1, y=-1.

–	a variable nS specifying the prediction size.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS−1.

This intra prediction mode is invoked when intraPredMode is equal to 35.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS−1, are derived as the following ordered steps:

- Variable k0 and the sample array pY’ are derived as:

k0 = Max( 0, BitDepthC + Log2( nS ) − 14 )		(8-31)

pY’[ x, −1 ] = ( PLM[ 2x−1, −1 ] + 2*PLM[ 2x, −1 ] + PLM[ 2x+1, −1 ] + 2 ) >> 2, with x = 0..nS−1	(8-32)

pY’[ −1, y ] = ( PLM[ −1, 2y ] + PLM[ −1, 2y+1 ] ) >> 1, with y = 0..nS−1	(8-33)

pY’[ x, y ] = ( recSamplesL[ 2x, 2y ] + recSamplesL[ 2x, 2y+1 ] ) >> 1, with x, y = 0..nS−1	(8-34)

- Variables L, C, LL, LC and k1 are derived as:

y=0nS−1pY'[−1, y]+x=0nS−1pY'[x, −1]>>k3		-

y=0nS−1p[−1, y]+x=0nS−1p[x, −1]>>k3		-

y=0nS−1pY'[−1, y]2+x=0nS−1pY'[x, −1]2>>k3	-

y=0nS−1pY'[−1, y]∗p[−1, y]+y=0nS−1pY'[x, −1]∗p[x,−1]>>k3	-

L = y=0nS−1pY'[−1, y]+x=0nS−1pY'[x, −1]>>k0		(8-39)

C = y=0nS−1p[−1, y]+x=0nS−1p[x, −1]>>k0		(8-40)

LL = y=0nS−1pY'[−1, y]2+x=0nS−1pY'[x, −1]2>>k0	(8-41)

LC = y=0nS−1pY'[−1, y]∗p[−1, y]+y=0nS−1pY'[x, −1]∗p[x,−1]>>k0	(8-42)

k1 = Log2( (2*nS) >> k0 )		(8-43)

- Variables a, b and k are derived as:

a1 	= ( LC << k1 ) – L*C		(8-44)
a2 	= ( LL << k1 ) – L*L		(8-45)
		-
			-
			-
		-

if (a1==0), k2=0

otherwise, k2= Max( 0, Floor(log2(abs(a1)))− 14 )	(8-69)

if (a2==0), k3=0

otherwise, k3= Max( 0, Floor(log2(abs(a2))) − 5 )	(8-70)

k4 = k3 – k2 + 2 		(8-71)

a1s = a1 >> k2 		(8-72)

a2s = a2 >> k3 		(8-73)

if (a2s < 1) a3 = 0

otherwise if (k4 >= 0), a3 = (a1s*lmDiv) >> k4)

otherwise (k4 < 0), a3 = (a1s*lmDiv) << (-k4)		(8-74)

a4  = Clip3( −215, 215−1, a3);		(8-75)

-
			-

if (a4 == 0), k5 = 0

otherwise, k5 = Floor(log2(abs( a4) + (sign(a4) -1)/2 ))– 5	(8-76)

if ( a4 >= -26 && a4 <= 26-1), k = 13

otherwise, k = 13 - k5		(8-77)

a=a4>> (13 – k)		(8-78)

-

b	= ( C – ( ( a*L ) >> k ) + ( 1 << ( k1 − 1 ) ) ) >> k1	(8-79)

where lmDiv is specified in Table 8-2 with the input a2s.

- The values of the prediction samples predSamples[ x, y ] are derived as:

predSamples[ x, y ] = Clip1C( ( ( pY’[ x, y ] * a ) >> k ) + b ), with x, y = 0..nS−1	(8-80)

Table 8-2 – Specification of lmDiv

| a2s | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lmDiv | 32768 | 16384 | 10923 | 8192 | 6554 | 5461 | 4681 | 4096 | 3641 | 3277 | 2979 | 2731 | 2521 |
| a2s | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| lmDiv | 2341 | 2185 | 2048 | 1928 | 1820 | 1725 | 1638 | 1560 | 1489 | 1425 | 1365 | 1311 | 1260 |
| a2s | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| lmDiv | 1214 | 1170 | 1130 | 1092 | 1057 | 1024 | 993 | 964 | 936 | 910 | 886 | 862 | 840 |
| a2s | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 |
| lmDiv | 819 | 799 | 780 | 762 | 745 | 728 | 712 | 697 | 683 | 669 | 655 | 643 | 630 |
| a2s | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 |  |
| lmDiv | 618 | 607 | 596 | 585 | 575 | 565 | 555 | 546 | 537 | 529 | 520 | 512 |  |