| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0149 |
| --- | --- |

| Title: | Coherent transform split pattern for non square transform units |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungsun Kim, Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea | Tel: Email: | +82-10-2201-9774 jungsun.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In this contribution, the transform split pattern of non square transform TUs is restrained consistent regardless of the size of CU. Currently a TU in a CU larger than the maximum transform size is split into four square TUs until the TU size is equal to the maximum TU size. And then each square TU is split into four non square TUs. In this contribution, a transform unit is always split into four non square TUs regardless TU or CU sizes. It stats 0.0%, -0.1%, 0.0% gain for RAHE10 and 0.1%, -0.1% -0.3% gain for LDHE10 under the common condition.

# Introduction

When non square transform is applicable, the transform split pattern is as Fig. 1. The numbers in blocks indicate the order of coding:

Fig. 1 Transform split pattern for non square transform

On the layer of Tr Depth=0, square TU is used. On the layer of Tr Depth=1, the square TU is split into four non square transform one of whose width and height is a quarter of the TU from top to bottom. On the layer of Tr Depth=2, a non square TU is split into four non square transform whose width and height are half of the TU. For example, if the left most block in Fig 1. is 32x32, it splits into four 32x8 blocks on the second layer of Tr Depth=1 and then each 32x8 block is split into four 16x4 blocks on the third layer of Tr Depth=2.

But when the CU size is larger than the maximum transform size, a transform unit is split into four square transform units until the TU size is equal to the maximum TU size. An example of 64x64 CU with 32x32 maximum transform size is shown in Fig. 2. The TU keep being split into four square TUs until the size of TU is equal to 32x32. Then each 32x32 TU is split into 32x8 TUs as shown in Fig. 2.

Fig. 2 Transform split pattern for non square transform when the maximum TU size is smaller than CU size

# Proposed Methods

We propose to use coherent split pattern regardless CU or TU sizes. When non square transform is applicable to a CU, its transform unit always split into non square transform. Transform units are split into four non square transform units until non square transform is available. Fig. 4 depicts an example of 64x64 CU when maximum TU size is 32x32.

Fig. 3 Non square transform in 64x64 CU with coherent transform split pattern

On the first layer of Tr Depth=0, the TU of 64x64 is split into four 64x16 TUs. Since 64 point transform kernel is not available they should be split again into 32x8 TU. Therefore the split pattern is coherent with that of other combinations that have different transform size and CU size. In this case, the transform split flag for intermediate 64x16 transform unit should be always derived as true since 64x16 transform is not available. Therefore CBFs for transform units on the layer of Tr Depth=1 does not need to be coded.

# Results

The proposed method is implemented on top of HM6.1 under the configuration of RAHE10 and LBHE10. The Experimental results are shown in Table 1. It shows 0.0%, 0.1%, 0.0% and 0.1%, -0.1%, -0.2% in RAHE10 and LBHE10 case, respectively. Detailed results are described in the accompanied excel file.

Table 1 the BD rate of the proposed method

|  | Random Access HE10 |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.2% | 0.0% |  |  |  |
| Class B | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | -0.3% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.2% |
| Class D | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | -0.2% |
| Class E |  |  |  | 0.1% | -0.6% | -0.4% |
| Overall | 0.0% | -0.1% | 0.0% | 0.1% | -0.1% | -0.3% |
|  | 0.0% | -0.1% | 0.0% | 0.1% | -0.1% | -0.2% |
| Class F | 0.0% | -0.1% | 0.0% | 0.1% | -0.5% | -0.3% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

# References

- F. Bossen, “Common HM test conditions”, JCTVC-H1100, 8th JCT-VC Meeting, San Jose, USA, Mar. 2012.

# Patent rights declaration(s)

LG electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).