| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0150 |
| --- | --- |

| Title: | Virtual large transform unit over maximum transform size with zero CBF |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungsun Kim, Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea | Tel: Email: | +82-10-2201-9774 jungsun.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes virtual transform units whose width and height are twice larger than the maximum transform size. When the virtual transform is applied, the luma CBF of the TU is derived as zero and the maximum size transform is applied to chroma components. It states 0.1%, -0.4%, -0.6% and 0.1%, -0.5%, -0.8% BDrate respectively in AIMAIN and AIHE10 condition. It also states 0.1%, -0.6%, -0.6% and 0.1%, -0.5%, -0.5% BDrate respectively in RAMAIN and RAHE10 condition. It states 0.1%, -0.7%, -0.9% and 0.1%, -0.9%, -1.0% BDrate respectively in the LBMAIN and RBHE10 condition.

# Introduction

In the current HM, a TU should be split when it is larger than the maximum transform size. Since the transform split pattern of chroma TUs should be same as that of luma TUs, chroma TUs are also split even though their size is smaller than or equal to the maximum transform size. When transform split is unavoidable, transform split flag is not coded but derived as true. Fig. 1 depicts an example of 64x64 CU with 32x32 maximum TU size. Since 64x64 transform is not available, a transform unit should be split. Because the transform split for chroma components is identical to that of luma component, the transform unit of chroma components should be split into 16x16 TU even though 32x32 transform is available.

Fig. 1 Available transform unit for 64x64 CU with 32x32 maximum TU size

# Proposed Methods

We propose to use virtual transform units whose width and height are twice larger than the maximum transform size while CBF for luma component of the TU is set to zero. For example, a virtual transform unit has size of 64x64 when the maximum transform size is 32x32, and 32x32 when the maximum transform size is 16x16. Since CBF of luma component of the TU is always zero, actual transform is not required. When virtual transform is applied to luma blocks, actual transform that is half the size of virtual TU is applied to chroma blocks. Fig.2 depicts an example of 64x64 CU with 32x32 maximum TU. The gray block indicates virtual 64x64 transform block. On the first layer of Tr Depth=0, virtual 64x64 TU always has zero CBF for luma components, and does not require actual transform. The actual 32x32 transform is applied to the chroma components

In the proposed scheme, transform split flag for the first layer of Tr Depth=0 should be coded to indicate whether virtual TU is used or not.

Fig. 2 Available transform unit for 64x64 CU with 32x32 maximum TU size

# Results

The proposed method is implemented on top of HM6.1. The Experimental results are shown in Table 1. Table 2 shows the results when the maximum transform size is 16x16 and the maximum transform size is 64x64. It shows that the proposed scheme prevents severe losses in chroma component when the maximum transform size is small. Detailed results are described in the accompanied excel file.

Table 1 BDrate of the proposed method

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.8% | -1.0% | 0.1% | -0.7% | -1.5% |
| Class B | 0.1% | -0.2% | -0.4% | 0.1% | -0.3% | -0.5% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.1% | -1.4% | -1.8% | 0.2% | -1.8% | -2.2% |
| Overall | 0.1% | -0.4% | -0.6% | 0.1% | -0.5% | -0.8% |
|  | 0.1% | -0.4% | -0.6% | 0.1% | -0.5% | -0.8% |
| Class F | 0.0% | -0.4% | -0.5% | 0.1% | -0.6% | -0.9% |
| Enc Time[%] | 96% |  |  | 96% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -1.1% | -1.1% | 0.1% | -1.0% | -0.8% |
| Class B | 0.1% | -0.8% | -0.6% | 0.1% | -0.7% | -0.8% |
| Class C | 0.0% | -0.3% | -0.2% | 0.0% | -0.3% | -0.2% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | -0.6% | -0.5% | 0.1% | -0.5% | -0.5% |
|  | 0.0% | -0.6% | -0.5% | 0.1% | -0.5% | -0.4% |
| Class F | 0.1% | -0.4% | -0.2% | 0.1% | -0.5% | -0.7% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | -0.9% | -1.0% | 0.1% | -1.3% | -1.7% |
| Class C | 0.0% | -0.6% | -0.6% | 0.0% | -0.4% | -0.6% |
| Class D | 0.0% | -0.3% | -0.3% | 0.0% | 0.1% | 0.1% |
| Class E | 0.1% | -1.0% | -2.6% | 0.1% | -2.2% | -1.5% |
| Overall | 0.1% | -0.7% | -0.9% | 0.1% | -0.9% | -1.0% |
|  | 0.1% | -0.6% | -0.9% | 0.1% | -0.9% | -1.0% |
| Class F | 0.1% | -0.4% | -1.2% | 0.1% | -1.7% | -0.4% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 2 BDrate of the proposed method when maximum TU size is 16x16

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.3% | -2.3% | -3.4% | 0.3% | -1.8% | -2.9% |
| Class B | 0.5% | -1.9% | -2.4% | 0.5% | -2.0% | -2.6% |
| Class C | 0.2% | -0.7% | -0.7% | 0.2% | -0.9% | -0.9% |
| Class D | 0.1% | -0.9% | -0.8% | 0.2% | -1.0% | -0.9% |
| Class E | 0.3% | -4.8% | -6.5% | 0.3% | -5.0% | -6.4% |
| Overall | 0.3% | -2.0% | -2.6% | 0.3% | -2.0% | -2.6% |
|  | 0.3% | -2.0% | -2.6% | 0.3% | -2.0% | -2.6% |
| Class F | 0.2% | -1.5% | -1.3% | 0.2% | -1.6% | -1.6% |
| Enc Time[%] | 91% |  |  | 92% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.6% | -3.0% | -2.3% | 0.5% | -2.4% | -2.2% |
| Class B | 0.4% | -2.7% | -2.4% | 0.4% | -2.7% | -2.6% |
| Class C | 0.2% | -1.3% | -1.6% | 0.2% | -1.2% | -1.3% |
| Class D | 0.1% | -0.8% | -0.6% | 0.1% | -0.7% | -0.7% |
| Class E |  |  |  |  |  |  |
| Overall | 0.3% | -2.0% | -1.7% | 0.3% | -1.8% | -1.7% |
|  | 0.4% | -2.0% | -1.7% | 0.3% | -1.8% | -1.7% |
| Class F | 0.2% | -1.0% | -0.9% | 0.2% | -1.5% | -1.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.5% | -4.5% | -5.0% | 0.5% | -4.7% | -5.1% |
| Class C | 0.3% | -2.5% | -2.2% | 0.3% | -2.1% | -2.2% |
| Class D | 0.1% | -1.8% | -1.4% | 0.1% | -1.8% | -1.2% |
| Class E | 0.4% | -5.9% | -9.5% | 0.4% | -6.6% | -8.0% |
| Overall | 0.3% | -3.4% | -4.0% | 0.3% | -3.7% | -4.0% |
|  | 0.3% | -3.3% | -4.0% | 0.3% | -3.6% | -3.9% |
| Class F | 0.1% | -1.1% | -1.2% | 0.3% | -1.6% | -2.8% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

# References

- F. Bossen, “Common HM test conditions”, JCTVC-H1100, 8th JCT-VC Meeting, San Jose, USA, Mar. 2012.

# Patent rights declaration(s)

LG electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).