| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0151 |
| --- | --- |

| Title: | LM mode with Uniform Bit-width Multipliers |  |  |
| --- | --- | --- | --- |
| Status: | [ Input Document to JCT-VC] |  |  |
| Purpose: | [Proposal] |  |  |
| Author(s) or Contact(s): | Lingzhi Liu (HiSilicon Technologies) Yongbing Lin (HiSilicon Technologies) Guichun Li (Santa Clara University) | Tel: Email: | +14083304556 lingzhi.liu@huawei.com Linyongbing@huawei.com g1li@scu.edu |
| Source: | HiSilicon Technologies |  |  |

_____________________________

# Abstract

This proposal suggests unifying the bit-widths of the multipliers used in the Chroma from Luma intra prediction (LM mode) in HEVC standard. Instead of using multiple bit-width multipliers, the proposal method only use unified bit-width multipliers according to the InternalBitDepth. The complexity is reduced from the modification. The BD-Rate for this optimization is: AI-HE10: Y: 0.0%, U: 0.0%, V: 0.1%.

# Introduction…

For chroma blocks, the current design of High Efficiency Video Coding (HEVC) has a chroma from luma intra prediction mode, which is also referred as LM mode, in addition to the directional modes and the DC mode. [1][2][3].

To get the parameter of a and b, several multipliers are use as listed bellows

LL = y=0nS−1pY'[−1, y]2+x=0nS−1pY'[x, −1]2>>k3	(1)

LC = y=0nS−1pY'[−1, y]∗p[−1, y]+y=0nS−1pY'[x, −1]∗p[x,−1]>>k3	(2)

a1 	= ( LC << k2 ) – L*C		(3)
a2 	= ( LL << k2 ) – L*L		(4)
a3	= a2s < 1 ? 0 : Clip3( -215, 215-1, a1s*lmDiv )		(5)

The multipliers in (1) and (2) has the same bit-widths as InternalBitDepth, i.e. the bit-width is 8 or 10 if InternalBitDepth equals to 8 or 10.

If the input bit-width is 8, both LC and LL will have maximum 21 bits. Each calculation of (3) and (4) then will have a 13-bit multiplier.

If the input bit-width is 10, both L and C will have maximum 15 bits. Each calculation of (3) and (4) will have a 15-bit multiplier.

The bit-width of the multiplier in (5) is always 15 bit signed.

We can see that LM mode uses multiple bit-width multipliers. It is desired from the hardware that only unified bit-width multipliers are used in the design because the design can be simplified and hardware cost can be reduced by reusing some of the multipliers.

# Unify the Bit-widths of the Multipliers

## Unify the Multipliers in a1, a2 and b

The bit-widths of (3), (4) and parameter “b” calculation are unified using the methods descript in JCTVC-H0491, where they can be calculated through the formulas bellow:

a1 	= LC– (2*nS*avgY’*avgC+avgY’*RErrC+avgC*RErrY’)					(6)
a2 	= LL – (2*nS*avgY’2+2*avgY’*RErrY’)

b	=  avgC – ( ( a * avgY’ ) >> k )                                                                                                        (7)

where avgY’, RErrY’, avgC, and RErrC are defined as:

avgY'=floory=0nS−1pY'[−1, y]+x=0nS−1pY'[x, −1]2∗nS=L>>(log2(2∗nS))				(8)

avgC=floory=0nS−1p[−1, y]+x=0nS−1p[x, −1]2∗nS=C>>(log2(2∗nS))				(9)

RErrY'=(y=0nS−1pY'[−1, y]+x=0nS−1pY'[x, −1])−(avgY'<<log2(2∗nS))=L∧((1<<log2(2∗nS))−1)	(10)

RErrC=(y=0nS−1p[−1, y]+x=0nS−1p[x, −1])−(avgC<<log2(2∗nS))=C∧((1<<log2(2∗nS))−1)	(11)

avgY’ and avgC mean the floored average value of all the luma prediction samples and the reconstructed neighboring chroma samples respectively, while RErrY’ and RErrC denote the corresponding rounding errors. They can be obtained by shifting and simple logic operations.

## Another implementation for a1 and a2 calculation

JCTVC-H0491 provided another implementation for a1 and a2 calculation which achieves the same result as 2.1.

Set variables avgY’, avgC as (8) and (9).

And set ΔpY’[ -1, y ]= pY’[ -1, y ] - avgY’, ΔpY’[ x, -1 ]= pY’[ x, -1 ] - avgY’, Δp[ -1, y ]= p[ -1, y ] – avgC and Δp[ x, -1 ]= p[ x, -1 ] – avgC, with x=0..nS-1, y = 0..nS-1, and derive variables LL and LC as follows:

LL = y=0nS−1ΔpY'[−1, y]2+x=0nS−1ΔpY'[x, −1]2>>k3		(16)

LC = y=0nS−1ΔpY'[−1, y]∗Δp[−1, y]+y=0nS−1ΔpY'[x, −1]∗Δp[x,−1]>>k3	(17)

Variables a1, a2 can be derived as:

a1 	= LC		(18)
a2 	= LL		(19)

The results are identical to the method in 2.1. An encoder can chose to use less multiplier by using this method. The original LM design could not provide this option.

## Unify the Multipliers in a3

The calculation of a3 in (5) has a multiplication of a1s*lmDiv, where currently a1s is in the range of (-215, 215-1), lmDiv is in the range of (0, 215) specified in a look-up-table with the input a2s.

To unify the multipliers with InternalBitDepth, the bit-width of a1s is restrained in the range of ((-2InternalBitDepth-1, 2InternalBitDepth-1-1) by the following operations:

if (a1==0), k2=0

otherwise, k2= Max( 0, Floor(log2(abs(a1))) – BitDepthC + 2 )					(12)

a1s	= a1 >> k2										(13)

The output value of lmDiv is restrained in the range of ((0, 2InternalBitDepth-1) as follows

lmDiv = a2s < 1 ? 0 : ( ( lmDiv + ( 1 << ( 15 - InternalBitDepth - 1 ) ) ) >> (15 - InternalBitDepth) ) ) 	(14)

lmDiv = Clip1C (lmDiv)		(15)

The multiplication in (5) is then aligned with InternalBitDepth.

# Simulation results

Simulations have been performed to test the performance of the proposed algorithms.

Table 1 shows the result of comparing the new algorithm described in Section 2 with HM6.0 software anchor under encoder_intra_he10 configuration.

Table 1. Testing results of proposed algorithm in Section 3 vs. HM 6.0 AI_HE10

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.1% | 0.2% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.1% |
| Class F | 0.0% | 0.2% | 0.4% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 2 shows the result of comparing the new algorithm described in Section 2 with HM6.0 software anchor under encoder_intra_main configuration with LMChroma switched on.

Table 2. Testing results of proposed algorithm in Section 3 vs. HM 6.0 AI_MAIN + LMChroma=1

|  | All Intra MAIN |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 1.0% | 0.7% |
| Class B | 0.0% | 0.3% | 0.0% |
| Class C | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | -0.1% |
| Overall | 0.0% | 0.3% | 0.1% |
|  | 0.0% | 0.3% | 0.1% |
| Class F | 0.0% | 0.2% | 0.4% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

It is observed that for AI_MAIN case, the BR-rate increase for Class A is more than that of AI_HE10 configuration. The largest coding increase is from the sequence of SteamLocomotive, which are Y: 0.0%, U: 2.58%, V: 2.42% respectively. Additional QP values are simulated to further compare the performance. Figure 1 shows the RD curve of SteamLocomotive

(a) U componant                                                   (b) V componant

Figure 1. RD curve for SteamLocomotive of AI_MAIN

Figure 1 shows that both U and V component have very similar performances compared to the anchor.

We also found that, in the common testing condition, the Y-PSNR values of SteamLocomotive for the testing QP {22, 27, 32, 37} are relatively high compared with the Y-PSNR values of other sequences under same QP values, which implies that the testing QP values are not optimal for the performance comparing.

We calculate the corresponding average Y-PSNR of a dedicate QP from all the sequences other than SteamLocomotive and Nebuta, and get the values of {42.34dB, 39.06dB, 36.07dB, 33.13dB}. We then choose QP values of {25, 34, 42, 48 } for SteamLocomotive because the corresponding Y-PSNR is close to these average values.

Table 3. Testing Results of SteamLocomotive on QP={25, 34, 42, 48} on AI_MAIN

|  | Anch |  |  |  | Prep |  |  |  | Y | U | V | Y | U | V |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25 | 60825.44 | 42.09 | 46.12 | 45.74 | 60858.82 | 42.09 | 46.07 | 45.71 | -0.1% | -0.9% | -3.1% | -0.1% | -0.5% | -3.0% |
| 34 | 20753.13 | 39.13 | 44.89 | 44.79 | 20743.68 | 39.13 | 44.88 | 44.78 |  |  |  |  |  |  |
| 42 | 6999.92 | 36.03 | 43.94 | 43.81 | 6996.92 | 36.03 | 43.97 | 43.88 |  |  |  |  |  |  |
| 48 | 2203.28 | 33.36 | 43.51 | 43.12 | 2201.61 | 33.35 | 43.54 | 43.18 |  |  |  |  |  |  |

Table 3 implies that in normal Y-PSNR, the proposed method works better than the Anchor.

Since the performances of the 10-bit sequences are varying in AI_MAIN configuration, we compare the performance without the two 10-bit sequences which is the same methodology as HM5.0. The results are shown in Table 4.

Table 2. Testing results without 10-bit sequences of proposed algorithm vs. HM 6.0 AI_MAIN

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8-b) | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.3% | 0.0% |
| Class C | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | -0.1% |
| Overall | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.2% | 0.0% |
| Class F | 0.0% | 0.2% | 0.4% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

The results are consistent with Intra_HE10 configuration.

# Conclusion

It is proposed to consider adopting the optimization of unifying the bit-width of multipliers in LM mode.

# Reference

[1] Benjamin Bross, et. Al, “WD6: Working Draft 6 of High-Efficiency Video Coding”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-H1003

[2] Jianle Chen, et. Al, “Complexity reduction of chroma intra LM prediction mode”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F494

[3] L. Liu, G. Li, Y. Lin, “Non-CE6a: Remove the large multiplier for LM mode calculation”, JCTVC-H0491, San Jose, Feb., 2012.

[3] Frank Bossen, “Common HM test conditions and software reference configurations”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-H1100

# Patent rights declaration(s)

Hisilicon Technologies and Satnta Clara University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix 1 (WD Changes)

##### 8.4.3.1.8 Specification of Intra_FromLuma prediction mode

Inputs to this process are:

–	a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

–	 luma neighbouring samples PLM [ x, y ], with x=-1, y = 0... 2*nS−1; and x=0…2*nS-1, y=-1.

–	chroma neighbouring samples p [ x, y ], with x=-1, y = 0... nS−1; and x=0…nS-1, y=-1.

–	a variable nS specifying the prediction size.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS−1.

This intra prediction mode is invoked when intraPredMode is equal to 35.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS−1, are derived as the following ordered steps:

- Variable k0 and the sample array pY’ are derived as:

k0 = Max( 0, BitDepthC + Log2( nS ) − 14 )		(8-1)

pY’[ x, −1 ] = ( PLM[ 2x−1, −1 ] + 2*PLM[ 2x, −1 ] + PLM[ 2x+1, −1 ] + 2 ) >> 2, with x = 0..nS−1	(8-2)

pY’[ −1, y ] = ( PLM[ −1, 2y ] + PLM[ −1, 2y+1 ] ) >> 1, with y = 0..nS−1	(8-3)

pY’[ x, y ] = ( recSamplesL[ 2x, 2y ] + recSamplesL[ 2x, 2y+1 ] ) >> 1, with x, y = 0..nS−1	(8-4)

- Variables L, C, LL, LC and k1 are derived as follows:

L = y=0nS−1pY'[−1, y]+x=0nS−1pY'[x, −1]>>k3		(8-5)

C = y=0nS−1p[−1, y]+x=0nS−1p[x, −1]>>k3		(8-6)

LL = y=0nS−1pY'[−1, y]2+x=0nS−1pY'[x, −1]2>>k3	(8-7)

LC = y=0nS−1pY'[−1, y]∗p[−1, y]+y=0nS−1pY'[x, −1]∗p[x,−1]>>k3	(8-8)

L = y=0nS−1pY'[−1, y]+x=0nS−1pY'[x, −1]>>k0		(8-9)

C = y=0nS−1p[−1, y]+x=0nS−1p[x, −1]>>k0		(8-10)

LL = y=0nS−1pY'[−1, y]2+x=0nS−1pY'[x, −1]2>>k0	(8-11)

LC = y=0nS−1pY'[−1, y]∗p[−1, y]+y=0nS−1pY'[x, −1]∗p[x,−1]>>k0	(8-12)

k1 = Log2( (2*nS) >> k0 )		(8-13)

- Variables a, b and k are derived as:

-
			-
avgY’	=  L >> k1 		(8-66)
RErrY’= L & ( ( 1 << k1 ) - 1 ) 		(8-67)
avgC	=  C  >> k1 		(8-68)
RErrC= C & ( ( 1 << k1 ) - 1 )  		(8-69)
a1 	= LC– (2*nS*avgY’*avgC+avgY’*RErrC+avgC*RErrY’)	(8-70)
a2 	= LL – (2*nS*avgY’2+2*avgY’*RErrY’)		(8-71)
		-
			-
			-
		-

if (a1==0), k2=0

otherwise, k2= Max( 0, Floor(log2(abs(a1))) – BitDepthC + 2 )	(8-72)

if (a2==0), k3=0

otherwise, k3= Max( 0, Floor(log2(abs(a2))) − 5 )	(8-73)

k4 = k3 – k2 + BitDepthC – 13		(8-74)

a1s	= a1 >> k2		(8-75)

a2s	= a2 >> k3		(8-76)

if, (a2s < 1), a2t = 0

otherwise,  a2t =  ( lmDiv + ( 1 << ( 15 - BitDepthC - 1 ) ) ) >> (15 - BitDepthC) 	(8-77)

a2t = Clip1C ( a2t )		(8-78)

if (a2t < 1) a3 = 0

otherwise if (k4 >= 0), a3 = (a1s*a2t) >> k4)

otherwise (k4 < 0), a3 = (a1s*a2t) << (-k4)		(8-79)

a4  = Clip3( −215, 215−1, a3);		(8-80)

-
			-

if (a4 == 0), k5 = 0

otherwise, k5 = Floor(log2(abs( a4) + (sign(a4) -1)/2 ))– 5	(8-81)

if ( a4 >= -26 && a4 <= 26-1), k = 13

otherwise, k = 13 - k5		(8-82)

a=a4>> (13 – k)		(8-83)

-

b	=  avgC – ( ( a * avgY’ ) >> k ) 		(7-84)

where lmDiv is specified in Table 8-1 with the input a2s.

- The values of the prediction samples predSamples[ x, y ] are derived as:

predSamples[ x, y ] = Clip1C( ( ( pY’[ x, y ] * a ) >> k ) + b ), with x, y = 0..nS−1	(8-85)

Table 8-1 – Specification of lmDiv

| a2s | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lmDiv | 32768 | 16384 | 10923 | 8192 | 6554 | 5461 | 4681 | 4096 | 3641 | 3277 | 2979 | 2731 | 2521 |
| a2s | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| lmDiv | 2341 | 2185 | 2048 | 1928 | 1820 | 1725 | 1638 | 1560 | 1489 | 1425 | 1365 | 1311 | 1260 |
| a2s | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| lmDiv | 1214 | 1170 | 1130 | 1092 | 1057 | 1024 | 993 | 964 | 936 | 910 | 886 | 862 | 840 |
| a2s | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 |
| lmDiv | 819 | 799 | 780 | 762 | 745 | 728 | 712 | 697 | 683 | 669 | 655 | 643 | 630 |
| a2s | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 |  |
| lmDiv | 618 | 607 | 596 | 585 | 575 | 565 | 555 | 546 | 537 | 529 | 520 | 512 |  |