| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0152 |
| --- | --- |

| Title: | CBF coding without derivation process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungsun Kim, Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea | Tel: Email: | +82-10-2201-9774 jungsun.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes to code each Coded Block Flag for each TU without any derivation. In the current HM, a CBF of the last TU among four TUs in a transform depth is conditionally derived. In this contribution all CBFs are coded without exception. As a result, the contribution removes several if-else conditions and several lines in the Text with 0.0%, -0.2%, -0.3% and 0.0%, -0.3%, -0.3% BD-rate respectively in AIMAIN and AIHE10 condition. It also states 0.1%, -0.2%, -0.3% and 0.1%, -0.3%, -0.2% BD-rate respectively in RAMAIN and RAHE10 condition. It reports 0.1%, -0.2%, -0.3% and 0.2%, 0.0%, -0.4% BD-rate respectively under the LBMAIN and LBHE10 condition.

# Introduction

A Coded Block Flag indicates whether a corresponding TU has non zero residual coefficients or not. In the current HM, not all of CBF are coded but a CBF of the last TU among four TUs in a transform depth is conditionally derived as one. The conditions to decide the derivation are as follows.

A chroma CBF is derived as 1 when the following conditions are true:

- Current TU size is less than maximum TU size

- Current TU is the last TU of four TUs in the same RQT

- CBFs of previous three TUs are all zero.

These conditions can be depicted as shown in Fig. 1

Fig. 1 The condition of CBF derivation : CBF of the shaded TU is derived as 1

A luma CBF is derived as 1 when the following conditions are true:

- Current TU is the last TU of four TUs in a same RQT

- CBFs of previous three TUs are all zero.

- CBFs of corresponding chroma TUs are all zero

- Current CU is not intra CU

- Current CU size is less than or equal to 2*maximum transform size or current transform size is less than maximum transform size

These conditions can be depicted as shown in Fig. 2.

CBFs can be derived since the transform splits imply that there exists at least one non zero transform unit in the lower depth level. But when a TU size is greater than the maximum TU size, the TU should be split whether there does not exist any non zero transform unit. Condition #5 indicates the case and it is depicted in Fig. 3

Fig. 2 The condition of CBF derivation : CBF of the shaded TU is derived as 1

Fig. 3 The condition of CBF derivation : CBFs of the shaded TUs are not derived

In Fig. 3, thick lines indicate 64x64 CU boundaries and thin lines indicate 16x16 TU boundaries. If the maximum TU size is not 16x16, CBFs of shaded blocks can be derived. However, if the maximum TU size is 16x16, CBFs should be coded.

# Proposed Methods and Results

We propose to code all CBFs for every TUs without any exception since the conventional derivation adds several process and lines in both the software and the text without benefits. The experimental result is shown in Table 1. The proposed method is implemented on top of HM6.1. Detailed results are described in the accompanied excel file. Table 2 shows the results when the proposed scheme is applied only to luma components, i.e. all CBFs for every luma TU is coded. And Table 3 shows the results when the proposed scheme is applied only to chroma components, i.e. all CBFs for every chroma TU is coded.

Table 1 BD rate when the proposed method applied to both luma and chroma blocks

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.04% | -0.20% | -0.21% | 0.04% | -0.20% | -0.25% |
| Class B | 0.05% | -0.27% | -0.32% | 0.05% | -0.29% | -0.38% |
| Class C | 0.05% | -0.26% | -0.24% | 0.05% | -0.25% | -0.25% |
| Class D | 0.03% | -0.14% | -0.16% | 0.03% | -0.17% | -0.11% |
| Class E | 0.07% | -0.32% | -0.38% | 0.07% | -0.36% | -0.30% |
| Overall | 0.05% | -0.24% | -0.26% | 0.05% | -0.25% | -0.26% |
|  | 0.05% | -0.23% | -0.25% | 0.05% | -0.24% | -0.25% |
| Class F | 0.05% | -0.18% | -0.14% | 0.04% | -0.24% | -0.18% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.06% | -0.19% | -0.19% | 0.10% | -0.20% | 0.01% |
| Class B | 0.09% | -0.38% | -0.44% | 0.12% | -0.37% | -0.45% |
| Class C | 0.06% | -0.29% | -0.33% | 0.15% | -0.43% | -0.38% |
| Class D | 0.07% | -0.02% | -0.32% | 0.04% | -0.05% | -0.03% |
| Class E |  |  |  |  |  |  |
| Overall | 0.07% | -0.23% | -0.33% | 0.10% | -0.27% | -0.23% |
|  | 0.07% | -0.23% | -0.34% | 0.10% | -0.28% | -0.25% |
| Class F | 0.09% | 0.00% | 0.01% | 0.11% | -0.33% | -0.04% |
| Enc Time[%] | 116% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.17% | -0.18% | -0.01% | 0.20% | -0.35% | -0.39% |
| Class C | 0.09% | -0.40% | -0.45% | 0.16% | -0.05% | -0.48% |
| Class D | 0.15% | -0.13% | -0.60% | 0.18% | 0.36% | -0.40% |
| Class E | 0.17% | -0.26% | -0.23% | 0.22% | 0.16% | -0.44% |
| Overall | 0.13% | -0.22% | -0.29% | 0.19% | 0.00% | -0.42% |
|  | 0.13% | -0.19% | -0.27% | 0.19% | -0.03% | -0.38% |
| Class F | 0.02% | -0.34% | -0.40% | 0.24% | -0.18% | 0.06% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

Table 2 BD rate when the proposed method applied to luma blocks

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.06% | -0.08% | 0.17% | 0.03% | -0.16% | -0.08% |
| Class B | 0.04% | 0.04% | -0.06% | 0.08% | 0.09% | -0.04% |
| Class C | 0.04% | 0.00% | 0.00% | 0.04% | 0.01% | -0.06% |
| Class D | 0.05% | 0.04% | 0.11% | 0.07% | 0.04% | 0.22% |
| Class E |  |  |  |  |  |  |
| Overall | 0.05% | 0.00% | 0.05% | 0.06% | 0.00% | 0.01% |
|  | 0.05% | 0.02% | 0.05% | 0.05% | 0.02% | 0.01% |
| Class F | 0.04% | 0.08% | 0.12% | 0.05% | -0.04% | -0.05% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.10% | 0.05% | 0.31% | 0.10% | 0.09% | -0.32% |
| Class C | 0.05% | -0.26% | -0.14% | 0.07% | -0.06% | -0.24% |
| Class D | 0.11% | -0.16% | -0.45% | 0.11% | 0.15% | 0.28% |
| Class E | 0.11% | 0.08% | 0.20% | 0.11% | -0.50% | -0.02% |
| Overall | 0.09% | -0.07% | -0.01% | 0.10% | -0.04% | -0.10% |
|  | 0.08% | -0.01% | 0.04% | 0.10% | -0.08% | -0.06% |
| Class F | -0.01% | 0.37% | -0.29% | 0.05% | -0.63% | 0.32% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3 BD rate when the proposed method applied to chroma blocks

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.04% | -0.20% | -0.21% | 0.04% | -0.20% | -0.25% |
| Class B | 0.05% | -0.27% | -0.32% | 0.05% | -0.29% | -0.38% |
| Class C | 0.05% | -0.26% | -0.24% | 0.05% | -0.25% | -0.25% |
| Class D | 0.03% | -0.14% | -0.16% | 0.03% | -0.17% | -0.11% |
| Class E | 0.07% | -0.32% | -0.38% | 0.07% | -0.36% | -0.30% |
| Overall | 0.05% | -0.24% | -0.26% | 0.05% | -0.25% | -0.26% |
|  | 0.05% | -0.23% | -0.25% | 0.05% | -0.24% | -0.25% |
| Class F | 0.05% | -0.18% | -0.14% | 0.04% | -0.24% | -0.18% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.05% | -0.21% | -0.22% | 0.07% | -0.42% | -0.09% |
| Class B | 0.02% | -0.42% | -0.44% | 0.05% | -0.33% | -0.44% |
| Class C | 0.04% | -0.26% | -0.24% | 0.07% | -0.41% | -0.36% |
| Class D | -0.02% | -0.02% | -0.27% | 0.01% | -0.33% | -0.23% |
| Class E |  |  |  |  |  |  |
| Overall | 0.02% | -0.24% | -0.30% | 0.05% | -0.37% | -0.29% |
|  | 0.03% | -0.24% | -0.31% | 0.05% | -0.36% | -0.32% |
| Class F | 0.04% | -0.02% | -0.04% | 0.05% | -0.35% | -0.03% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 98% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.04% | -0.17% | -0.04% | 0.08% | -0.19% | -0.66% |
| Class C | 0.05% | -0.59% | -0.28% | 0.06% | -0.37% | -0.28% |
| Class D | 0.05% | 0.07% | -0.78% | 0.05% | 0.01% | 0.17% |
| Class E | 0.17% | -0.15% | -0.03% | 0.06% | -0.30% | -0.38% |
| Overall | 0.06% | -0.20% | -0.27% | 0.06% | -0.21% | -0.31% |
|  | 0.06% | -0.15% | -0.28% | 0.06% | -0.21% | -0.22% |
| Class F | -0.10% | -0.20% | -0.26% | 0.09% | -1.21% | 0.55% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

# Working Draft

In the proposed scheme, yellow highlighted lines in the current text are removed without any additions.

| transform_tree( x0, y0, xC, yC, log2CbSize, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| … |  |
| split_transform_flag[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if( log2TrafoSize <= Log2MaxTrafoSize ) { |  |
| firstChromaCbf = ( log2TrafoSize = = Log2MaxTrafoSize ¦ ¦ trafoDepth = = 0 ) ? 1 : 0 |  |
| if( firstChromaCbf ¦ ¦ log2TrafoSize > Log2MinTrafoSize ) { |  |
| if( firstChromaCbf ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| readCbf = TRUE |  |
| if( blkIdx = = 3 && log2TrafoSize < Log2MaxTrafoSize ) |  |
| readCbf = cbf_cb[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] |  |
| if( !readCbf ) |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| if( firstChromaCbf ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| readCbf = TRUE |  |
| if( blkIdx = = 3 && log2TrafoSize < Log2MaxTrafoSize ) |  |
| readCbf = cbf_cr[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cr[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cr[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] |  |
| if( !readCbf ) |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| } |  |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| if( InterTUSplitDirection = = 2 ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1 = y0 |  |
| x2 = x0 |  |
| y2 = y0 + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| x3 = x1 |  |
| y3 = y2 |  |
| } else { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x2 = x1 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x3 = x2 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| log2TrafoHeight = log2TrafoHeight + 2 * InterTUSplitDirection − 1 |  |
| log2TrafoWidth = log2TrafoWidth − 2 * InterTUSplitDirection + 1 |  |
| } |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 0 ) |  |
| transform_tree( x1, y1, x0, y0, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 1 ) |  |
| transform_tree( x2, y2, x0, y0, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 2 ) |  |
| transform_tree( x3, y3, x0, y0, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 3 ) |  |
| } else { |  |
| if( PredMode = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| readCbf = TRUE |  |
| if( blkIdx = = 3 && PredMode != MODE_INTRA && ( ( log2CbSize <= Log2MaxTrafoSize+1 ) ¦ ¦ ( log2TrafoSize < Log2MaxTrafoSize ) ) |  |
| readCbf = cbf_luma[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_luma[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_luma[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1] ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1] |  |
| if( !readCbf ) |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| transform_unit (x0, y0, xC, yC, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx) |  |
| } |  |
| } |  |
| } |  |

# References

- F. Bossen, “Common HM test conditions”, JCTVC-H1100, 8th JCT-VC Meeting, San Jose, USA, Mar. 2012.

# Patent rights declaration(s)

LG electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).