| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0153 |
| --- | --- |

| Title: | AHG8: Inclusion Of Parallel Processing Schemes In The Main Profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Stewart Worrall Adrian Wise Aspex House, 44-45 Oxford Street, High Wycombe, Buckinghamshire, HP11 2DJ | Tel: Email: | +44 1494 558121 stewart.worrall@aspex-semi.com |
| Source: | Aspex Semiconductor |  |  |

_____________________________

# Abstract

This contribution provides an analysis of the different high level parallel processing tools. It summarises some of the key advantages and disadvantages of tiles and Wavefront Parallel Processing (WPP). The key issues focussed on are Motion Estimation (ME) bandwidth, parallel processing efficiency, compression efficiency, delay issues, profile specification, and parallel decoder implementation. Based on the analysis, the authors argue that WPP has a number of significant advantages over tiles. The authors contend that advantages in terms of compression efficiency, flexibility, and profiling simplicity, make it a stronger candidate than tiles for inclusion in the Main Profile of HEVC.

# Introduction

To facilitate practical implementation of parallel decoders and encoders, it is better to include only one parallel processing tool in the Main Profile. This contribution represents a summary of an analysis of some of the main parallel processing issues for tiles [3] and WPP [4]. In this analysis, the following key issues are considered:

- Motion Estimation (ME) bandwidth

- Parallel processing efficiency

- Compression efficiency

- Delay

- Profile specifications

The contribution begins by stating the case for parallel processing in general, and specifically why it can be beneficial for inclusion in the Main Profile.

# Parallel Processing Requirements

Parallel processing is already exploited for encoding AVC and MPEG-2/H.262 video. Both WPP and tiles have two important features to justify their inclusion in a Main Profile:

- Facilitation of decoder side parallelism

- Parallel CABAC encoding

These two features are only made possible by including either tiles or WPP in the Main Profile.

Decoder side parallelism will be particularly useful for handling 4K and 8K content. However, it is also useful for transcoding operations, permitting implementation of both parallel encoders and decoders. For transcoding, the parallelism will allow more flexible allocation of resources between encoder and decoder, based on the input and output resolutions.

Parallel encoding of AVC may be achieved using FMO in the Baseline profile. This approach involves degradation of compression efficiency. The FMO approach can only be used in applications permitting use of the Baseline profile (e.g. cannot be used for broadcasting). Note that no FMO is present in HEVC.

Multiple slices per picture may be used for parallel encoding, but this is not desirable, due to significant compression efficiency overhead.

Alternatively, Wavefront parallel processing (WPP) may be used with AVC, which is compatible with all profiles, and does not impact compression efficiency. However, if multiple slices per picture are avoided (for best compression efficiency), then it is necessary to buffer the symbols before CABAC encoding. The CABAC encoder in AVC must produce a bitstream in raster scan order. This results in spikes in CABAC throughput. For example, in WPP, the first line may be CABAC encoded as soon as the encoded coefficients are ready. The second line is only two blocks behind, but CABAC encoding of the second line cannot begin until the first line has been completely CABAC encoded. This leads to sudden spikes in bin throughput as each wavefront reaches the righthand boundary of the picture. It also means that larger amounts of memory are required to store the encoded coefficients. If the encoded data is stored off-chip, then memory bandwidth is increased. WPP may also be used at the decoder, although large amounts of symbol buffering are required.

Clearly, the bin throughput issue is one that is handled in existing parallel encoders, but it does place some constraints on the use of parallelism for very high bit-rate applications. It also means that CABAC hardware needs to be designed to meet much higher throughput rates than would be necessary if a parallel encoding feature is adopted in the Main Profile of HEVC. Apart from solving the above issues, parallel CABAC encoding will facilitate some very high bit-rate applications that are challenging using a single, serial CABAC encoder. Both tiles and WPP provide parallel CABAC encoding, as well as parallel generation of encoded symbols.

The main negative aspect of including a particular parallel processing approach is that it will constrain all implementers to a particular type of parallelism, and it may constrain flexibility in the amount of parallelism that can be used (e.g. number of cores) if not specified carefully in the standard.

# Parallel Processing Efficiency

Parallel processing efficiency is defined here as being the ratio between the available processing power, and what can be exploited in practice. In an ideal scenario, equal workloads can be allocated to each processor, which will give 100% efficiency. In practice, for common picture formats, WPP and tile processing both suffer from a degree of parallel processing efficiency loss.

Figure 1: Illustration of processing efficiency issues with WPP, where the final wavefront is half the height of the others.

Figure 2: Example of grouping tiles to enable processing of a nine tile picture with four cores.

Tiles Parallel Processing Efficiency

For tiles, inefficiency can come from having different tile sizes, and from having to group different numbers of tiles together to achieve a particular level of parallelism. These differences can be illustrated by the tile configuration information shown in Table 1. The number of tile columns and rows are modified from those given in JCTVC-F335 [3]. The modifications were made to achieve minimum column widths of 384, as currently defined in the Main Profile of the CD [2]. The table shows differences in the minimum and maximum tile sizes, which can result in inefficiency. Further inefficiency can arise from grouping tiles. Consider the following example: a Class E sequence (9 tiles) must be encoded (or decoded) using four cores. In this example, some cores will have more tiles to encode than others, as illustrated by Figure 2.

The differences in tile groups and tile sizes can be mitigated by using load balancing. With load balancing, smaller tiles can be processed by larger tile groups, and larger tiles by smaller tile groups. However, care has to be taken with load balancing. If “fully flexible” load balancing is used, then tiles can be distributed to any group to maximise efficiency. This has some cost as it may involve processing tiles out of scan order, possibly leading to delay, and additional memory requirements. Therefore, constrained load balancing has also been considered in these calculations, where load balancing can only be achieved when compatible with row by row tile processing.

Tile parallel processing efficiency is calculated by finding the ratio between the number of CTBs in the largest and smallest tile groups after load balancing.

WPP Parallel Processing Efficiency

For WPP, there are two sources of inefficiency. First, the last wavefront may have to be less than the full height, because the picture height is not an integer number of wavefront heights. Secondly, the offset in starting and finishing parts of the wavefront mean that there is always at least a small amount of time where the amount of CTBs being processed is less than a full wavefront height (see Figure 1). To calculate efficiency, we define it as the ratio of the actual number of CTBs processed in a picture and the total number of CTBs that could theoretically be processed by WPP with a particular wavefront height.

Table 1: Tile size configurations used for parallel processing efficiency calculations.

|  |  |  |  | Tile Width in CTBs |  | Tile Height in CTBs |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | CTB Size | Tile Columns | Tile Rows | Min | Max | Min | Max | Min Tile Width (Pixels) |
| A | 64 | 6 | 5 | 6 | 7 | 6 | 5 | 384 |
|  | 32 | 6 | 5 | 13 | 14 | 10 | 10 | 416 |
| B | 64 | 5 | 3 | 6 | 6 | 5 | 6 | 384 |
|  | 32 | 5 | 3 | 12 | 12 | 11 | 12 | 416 |
| E | 64 | 3 | 3 | 6 | 7 | 4 | 4 | 384 |
|  | 32 | 3 | 3 | 13 | 14 | 7 | 8 | 416 |

Table 2: Average parallel processing efficiency for tiles and wavefront parallel processing, where LB is fully flexible load balancing and “CLB” means constrained Load Balancing.

|  |  | Average Efficiency |  |  |
| --- | --- | --- | --- | --- |
| Class | CTB Size | WPP | Tiles: LB | Tiles: CLB |
| A | 64 | 91% | 93% | 93% |
|  | 32 | 95% | 92% | 92% |
| B | 64 | 87% | 83% | 81% |
|  | 32 | 94% | 79% | 78% |
| E | 64 | 88% | 72% | 72% |
|  | 32 | 92% | 69% | 69% |

Parallel Processing Efficiency Discussion

More complete results are shown in section 10, while a summary is shown in Table 2. One point worth noting is the difference between CTB sizes of 64 and 32. For WPP, smaller CTB sizes lead to improvements in parallel processing efficiency, as expected. For tiles, parallel processing efficiency can actually be slightly worse. This is because the tile sizes are more equal for 32 CTB size, and therefore the different tile sizes are not as effective for load balancing against the different sizes of the tile groups.

Overall, it seems reasonable to conclude that the differences in parallel processing efficiency between tiles and WPP are small enough to ignore, although there are some significant differences for Class E.

One issue to consider with tiles, is the need to constrain tile height, as well as tile width to enable efficient parallel decoding. Although an encoder can select an ideal configuration for best efficiency, the configuration may not work well at a decoder that has a different number of cores to the encoder. It would be best to fix minimum and maximum tile heights/widths to ensure that high parallel processing efficiency can be achieved at both encoder and decoder.

Summary: certain configurations of tiles and wavefronts can lead to parallel processing efficiency issues. However, when the efficiency figures are evaluated, there is no clear winner between tiles and WPP, unless Class E resolution sequences are considered at particular levels of parallelism. Minimum and maximum tile heights and widths should be specified to allow good parallel processing efficiency to be achieved at the encoder and decoder.

# Compression Efficiency Issues

Both approaches incur coding efficiency penalties. There are three sources of coding efficiency penalties:

- CABAC resets and byte alignments;

- Coding of entry point data to allow parallel decoding [5];

- Prediction constraints (i.e. Intra prediction limitations at tile boundaries).

Both approaches currently involve increased numbers of CABAC resets, and byte alignments. For tiles, entry points are only required at the end of a tile, while WPP involves CABAC flush and byte alignment at the end of every line. It has been reported that this cost for WPP is relatively minor [1], so would certainly not be an issue for tiles either.

Coding of entry point data is a more difficult issue to assess because the final entry point scheme has not been fully harmonized for tiles and WPP. It is also difficult, because the number of entry points for WPP should ideally always be equal to the number of CTB lines.

For tiles, the number of entry points is equal to the number of tiles. The penalty for using the entry point table, currently defined in the slice header does not impose significant problems. However, the 24-bit marker code currently used in tiles can impose a more significant penalty. The 24-bit marker would need to be used for every tile, or for every CTB line with WPP.

Table 3 shows the number of entry points required for WPP and tiles, where the number of tiles is based on Table 1.

Figure 3: Illustration of requirements for entry point markers for tiles and WPP.

Prediction constraints are only imposed by tiles. For WPP, prediction is independent from the level of parallelism chosen. For tiles, prediction across tile boundaries is constrained for Intra blocks. In most cases the visual impact of this is limited when filtering across tile boundaries is used. There are some sequences (outside of the current set used in HEVC) where column boundaries are visible in smooth, flat areas of the picture, which feature a small amount of gradient (e.g. blue sky).

Table 3: Illustration of number of entry points required for tiles and WPP.

|  | Num Entry Points |  |
| --- | --- | --- |
| Class | Tiles | WPP |
| A | 30 | 25 |
| B | 15 | 17 |
| E | 9 | 12 |

More significant artefacts can be seen with tiles, if rate control algorithms are not designed with tiles in mind. If the rate-control algorithm needs to reduce bit-rate in the middle of a frame, then it may do so for all tiles in parallel. Thus, the bottom rows of the tiles may have much higher QPs than the top rows. Significant differences in QPs of adjacent CTBs can lead to visible tile boundary artefacts. For WPP, the parallel substreams are adjacent to each other, so a simple rate-control algorithm will result in the top left of the picture having a different QP to the bottom right, which is much less visually disturbing. In the case of tiles, this means that rate control algorithms must be designed with tiles in mind for both parallel and single core encoders. This way the differences between QPs in adjacent CTBs can be limited.

Overall, WPP is better in terms of coding efficiency, due to the lack of prediction at tile boundaries. A brief summary of the differences is given in Table 4, while more extensive results are shown in section 15.

Summary: both tiles and WPP have different issues regarding compression efficiency. However, WPP can provide around 1% better BD-PSNR performance (average for Classes A, B, C, E), and would therefore seem to be the better option in terms of coding efficiency.

Table 4: Differences in BD-PSNR between tiles and WPP encoded videos, where a positive difference indicates that WPP is better (in BD-PSNR spreadsheet reference WPP, tested tiles). The comparison is made using HM-6.1.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.9% | -0.1% | 1.0% | 0.1% | -0.4% | 0.1% |
| Class B | 1.0% | 0.6% | 0.7% | 0.5% | -1.5% | -2.0% |
| Class C | 0.7% | 0.6% | 0.6% | 0.3% | -1.8% | -2.5% |
| Class D | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% |
| Class E | 1.9% | 0.3% | 0.6% | 1.3% | -6.2% | -7.8% |
| Overall | 0.8% | 0.3% | 0.6% | 0.4% | -1.8% | -2.2% |
|  | 0.8% | 0.3% | 0.5% | 0.4% | -1.7% | -2.1% |
| Overall (no Class D) | 1.1% | 0.3% | 0.7% | 0.6% | -2.5% | -3.0% |
| Class F | 1.2% | 0.9% | 1.0% | 1.1% | -0.8% | -0.7% |
| Enc Time[%] | - |  |  | - |  |  |
| Dec Time[%] | - |  |  | - |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 2.3% | 2.1% | 2.3% | 2.1% | 4.0% | 4.2% |
| Class B | 1.1% | 1.0% | 0.9% | 1.0% | 1.5% | 0.8% |
| Class C | 0.3% | 0.5% | 0.6% | 0.3% | 0.4% | 0.6% |
| Class D | -0.9% | -0.7% | -0.7% | -0.9% | -0.7% | -0.9% |
| Class E |  |  |  |  |  |  |
| Overall | 0.7% | 0.7% | 0.8% | 0.7% | 1.3% | 1.2% |
|  | 0.7% | 0.7% | 0.8% | 0.7% | 1.3% | 1.2% |
| Overall (no Class D) | 1.3% | 1.2% | 1.3% | 1.1% | 2.0% | 1.9% |
| Class F | 1.6% | 1.7% | 1.8% | 1.5% | 1.8% | 1.4% |
| Enc Time[%] | - |  |  | - |  |  |
| Dec Time[%] | - |  |  | - |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.9% | 0.9% | 0.6% | -0.1% | -2.1% | -2.9% |
| Class C | 0.0% | 0.0% | 0.3% | -0.4% | -1.8% | -1.5% |
| Class D | -1.0% | -1.1% | -0.6% | -1.0% | -1.3% | -1.3% |
| Class E | 0.5% | 0.0% | -0.3% | 0.3% | -5.5% | -5.5% |
| Overall | 0.1% | 0.0% | 0.1% | -0.3% | -2.4% | -2.6% |
|  | 0.1% | -0.1% | 0.1% | -0.3% | -2.3% | -2.5% |
| Overall (no Class D) | 0.5% | 0.3% | 0.2% | -0.1% | -3.1% | -3.3% |
| Class F | -0.8% | -0.7% | -0.4% | 1.1% | 0.0% | 0.3% |
| Enc Time[%] | - |  |  | - |  |  |
| Dec Time[%] | - |  |  | - |  |  |

# ME Bandwidth

Motion Estimation bandwidth is a key issue during the design of real-time encoders. Caches large enough for multiple reference pictures are too expensive to include on-chip. Therefore, most real-time encoders use a smaller local cache, and adaptively fetch reference picture data, as required by the ME algorithm. When the cache is full, old data is overwritten by new data. This can lead to the same data being fetched multiple times, increasing memory bandwidth requirements. Although the achievable data transfer rate increases over time, ME bandwidth is still an issue for systems that process multiple videos at once, and for systems where other applications share the same bus.

Results have previously been presented, demonstrating how tiles can benefit motion estimation [3]. Cache simulations have also been carried out by Aspex, which confirm that ME bandwidth can be reduced using tiles. Investigations have also been carried out into ME bandwidth reductions provided by WPP. The results presented here avoid a direct comparison between tiles and WPP, as there are many different approaches to caching/buffering and ME algorithms, which would all have some impact on a comparison.

Figure 4: Illustration of ME search overlap for WPP.

In theory WPP has an advantage over ME carried out in raster scan order. This is illustrated by Figure 4, which demonstrates how WPP can lower ME bandwidth when a single ME cache is used. Use of WPP results in increased amounts of overlap between ME searches. This means that less data needs to be fetched into the cache than when a non-parallel encoding approach is used.

Cache Simulation Set-Up

To further examine ME bandwidth performance for WPP, some simulations have been run using a motion estimation cache model. The motion estimation search implemented was a deterministic full search, with a search radius of 64, and the simulations assumed that two reference frames are used. The cache model implements a Least Recently Used algorithm for evicting data already in the cache. Fourteen different cache configurations were tested to find a configuration that could be said to provide good results. The different configurations included variations of the following:

- Line buffer length

- Overall buffer size (64Kb, 128Kb, 256Kb tested)

- Number of ways of associativity

For WPP, CTBs were pushed into the ME engine is wavefront order (e.g. current CTB in line 1, current CTB in line 2 etc.). Wavefront heights of 2-6 were evaluated.

Cache Simulation Results

The results shown in Table 5 give an indication of the kinds of reductions in ME bandwidth that can be achieved using WPP.

Table 5: Cache simulation results for 128Kb and 256Kb cache sizes.

| Parallelism | WPP (% of non-parallel) |  |
| --- | --- | --- |
|  | 128Kb | 256Kb |
| 1 | 100% | 100% |
| 2 | 54% | 62% |
| 3 | 37% | 46% |
| 4 | 36% | 42% |
| 5 | 36% | 36% |
| 6 | 38% | 29% |

Summary: WPP results in lower ME bandwidth than raster scan order ME. Tiles are also effective at reducing ME bandwidth. A direct comparison between tiles and WPP is difficult, as results are often dependent on proprietary ME algorithms, and architectures.

# Delay Issues

Parallel processing is useful for low delay applications. Parallel processing can be used to reduce encoding and decoding time, and this potentially reduces the end-to-end delay. WPP and Tiles differ in their delay characteristics. The issues involving delay can be grouped into two areas:

- Processing delay – WPP means that the encoder can only begin to process substream n after two CTB blocks of substream n-1 have been encoded; This delay also exists at the decoder for WPP; No such delay is present for tiles;

- Communications delay – the time taken to transfer data across the communication channel; For WPP the first line in each substream must be transferred to enable all parallel substreams to be decoded; For tiles, the first slice in each tile must be delivered to enable parallel processing of all tiles.

This section considers delay with one slice per picture, and with multiple slices per picture.

## Transmission Assumptions

A few simplifying assumption have been made concerning transmission, which are intended to facilitate concise discussion, without compromising the validity of the presented results. The first simplifying assumption is that transmission delay has been considered to be simply a matter of encoded video bit-rate, and available channel bandwidth. A more realistic model would include some additional variable delay factor. This would likely extend the transmission time, and make the case for WPP stronger than indicated below.

Another simplifying assumption is that no consideration has been made of underlying transport protocols. If an IP-based scheme is used, then the data would be fragmented into packets. In general, an IP packet cannot be sent until its payload has been completed, and decoding cannot begin until the whole IP packet is received. Using smaller IP packets may allow data to be sent and received more regularly, and will allow decoding of the final parallel stream (tile or wavefront) to begin more quickly. However, this would not significantly affect the conclusions below, because to begin decoding the last tile, it is still necessary to receive all of the preceding tiles. Similarly, for WPP it is necessary to receive the first set of lines. The transmission delay for WPP may still therefore be less than for tiles.

## Delay: One Slice Per Picture

As the processing delay for tiles is considered to be zero, the overall delay for tiles is bounded by the transmission delay. Although all tiles may be encoded in parallel, they must be concatenated and transmitted as a single, serial bitstream.

Figure 5: Illustration of delay bounds for wavefront parallel processing.

For WPP, the overall delay bound is slightly more complicated. Because the processing delay is non-zero, the actual bound depends on the relative sizes of the transmission delay and the processing delay, as shown in Figure 5. The transmission delay for WPP will always be less than with tiles, provided that:

Stiles < PicHeightInCtb

Tline > 2PCTB

Stiles is the number of tiles used in a picture, Tline is the time taken to transmit one CTB line to the decoder, and PCTB is the time taken to encode one CTB. Please see section 12 of this contribution for details of the derivation of these equations.

If Tline ≤ 2PCTB, then WPP delay is less than tile delay if:

BR/BC > 2(Swpp - 1) Stiles/(PicWidthInCtb * PicHeightInCtb * M * ptile)

where BR is the average bit-rate of encoded video, BC is the available rate at which the video may be transmitted over the channel, Swpp is the height of the wavefront, ptile is the number of tiles to be processed in parallel, and M represents a real-time speed up factor (M≥1).

Table 6: Example processing delay values for WPP, with a wavefront height of 4.

|  |  |  | Parallel Proc For Low Delay |  | Parallel Proc for More Cycles |  |
| --- | --- | --- | --- | --- | --- | --- |
| Class | Frame Rate | Parallel Streams | 2P_CTB (ms) | WPP Proc. Delay (ms) | 2P_CTB (ms) | WPP Proc. Delay (ms) |
| A | 30 | 4 | 0.07 | 0.10 | 0.27 | 0.40 |
|  | 60 | 4 | 0.03 | 0.05 | 0.13 | 0.20 |
| B | 30 | 4 | 0.13 | 0.20 | 0.52 | 0.78 |
|  | 60 | 4 | 0.07 | 0.10 | 0.26 | 0.39 |
| E | 30 | 4 | 0.28 | 0.42 | 1.11 | 1.67 |
|  | 60 | 4 | 0.14 | 0.21 | 0.56 | 0.83 |

Delay Bounded Example

Table 6 provides some example WPP delay values. Two scenarios are considered: parallel processing for low delay, and parallel processing for more cycles. With the former, parallel processing is effectively used to encode faster than real-time, minimizing processing delay. With the former, parallel processing is used to obtain more cycles to encode each CTB at a particular system clock speed.

As an illustration, for a 1080p30 video, 64x64 CTB, the maximum allowable time for 2PCTB is 0.13ms. We can compare this with the transmission time, assuming that the video is encoded at 12Mbit/s, and is sent over a 50Mbit/s capable channel. In this case, the transmission time is 0.47ms for one CTB line. For comparison, the transmission time for one tile is 0.53ms. This means that the fourth tile cannot start decoding for 2.1ms, if we assume that 15 tiles are used. The fourth WPP substream can start decoding after 1.9ms. Note that increasing the level of parallelism (e.g. more tiles or greater wavefront heights) will accentuate the delay difference between tiles and WPP.

Of course, there are many transmission delay issues left out of this simple calculation, and real-world transmission delay is likely to be higher. However, it seems reasonable to conclude that in many real-world applications, both tiles and WPP delay will be bounded by transmission delay, rather than processing delay. In this case, encoding using WPP will result in lower delay than tiles.

Processing Time Bounded Example

To illustrate the case where WPP is processing time bounded, consider a case with 1080p30 video, Swpp = ptile = 4, Stiles = 15, and M = 1. In this case, delay with WPP is less than tiles if:

BR/BC > 0.044

In this example, tiles will have lower delay than WPP when less than one twentieth of the available channel bandwidth is used. Note that if parallel processing is used to decrease the encoding time (M>1), the problem will either become transmission delay bounded, or an even smaller proportion of channel bandwidth will be required before tiles exhibit lower delay than WPP.

## Impact of Slices on Delay

Slices may include multiple CTB lines for WPP, or multiple tiles. CTB lines and tiles may also be split into multiple slices. If slices are transmitted in Tile scan or raster scan order, then using multiple slices will not change the basic delay scenario presented above. It is only possible to achieve lower delay by presenting the decoder with slices that are not in scan order. This is not possible given that there is currently no Arbitrary Slice Order feature in the CD [2].

## Delay: Single Core Decoding

The above examples assumed that parallel decoding is carried out. However, the delay criteria specified above remain the same for single core decoding. The only effect on the delay for tiles and WPP is that the decoding may take longer. In this case, the delay caused by single core decoding will be the same for tiles and WPP. Therefore, as with parallel decoding, the main issues are transmission delay and encoder processing delay.

## Delay Summary

Although tiles features lower encoding-decoding processing delay than WPP, transmission delay means that WPP may perform better in some low-delay applications. In many scenarios, delay will be bounded by transmission delay, rather than delay between wavefronts. In this case, the delay incurred by tiles and wavefronts depends on the relative size of CTB lines to tile size, as well as the number of streams to be processed in parallel. If we consider the relative sizes of CTB lines and tiles, then:

- The minimum transmission delay for WPP is limited by the time taken to transmit one CTB line, whereas the minimum transmission delay for tiles is limited by the time taken to transmit one tile (current minimum of 6 CTBs, as specified in the CD);

- WPP delay for one CTB line can be lowered by reducing CTB size, at the expense of reducing quality;

- Because using smaller tiles means degrading compression efficiency, it can be argued that for a particular delay requirement, WPP will provide better compression efficiency

A requirement for delay less than that provided by WPPs minimum delay constraint has not been established, therefore the superior compression efficiency of WPP would seem to make it the better option for low delay applications.

# Profile Specification and Decoder Parallelism

WPP is relatively simple to include in the Main Profile, because it requires no further specifications or settings (apart from mandating tiles_or_entropy_coding_sync_idc=2, and num_substreams_minus1= PicHeightInCtb-1). This setting allows encoders and decoders to use different levels of parallelism. The main constraint on the level of parallelism exploited will be the capability and architecture of the devices on which the encoders and decoders are implemented.

Figure 6: Example case where a six tile coded 1080p stream, with 64x64 CTB, is sent to a four core decoder, where the numbers in the boxes represent the capability of each processor core or the demands of a particular tile.

Tiles are somewhat harder to include in a profile. Care has to be taken to ensure that it is practical to build a parallel decoder that can conform to a particular level. With parallel decoding, a particular core may be specified to be able to handle a particular luma pixel rate per tile in real-time. To illustrate this point, consider a case where a 1080p, 30 fps picture is encoded using six tiles, and is then sent to a decoder with four cores. As shown in Figure 6, a four core decoder that is capable of decoding four tiles for 1080p, would not be able to decode six tiles, as two of its cores will be overloaded.

Without careful and precise definitions of per level tile sizes/pixel rates, it will be impractical to design a parallel decoder that meets a particular level specification. This should be done in a manner that is consistent with the rest of the profile/level definitions, and with minimal implications on other parts of the profile/level specifications. The current CD text does not achieve this [2].

One possible solution is to limit the number of tiles to powers of two (i.e. 2, 4, or 8). However, it would still be necessary to ensure that a certain number of tiles are used at a particular level to ensure that a level conformant parallel decoder can be built. Another problem with this solution is that it significantly reduces parallel encoding flexibility, and would introduce additional delay. It will mean that the level of parallel processing cannot be adapted to fit a particular architecture. The level of parallelism is fixed by the level of the bitstream. This solution would mean that HEVC is less flexible for parallel encoding than AVC.

Another solution is to use small tiles (such as those shown in Table 1), but this will lead to significant issues in terms of compression efficiency, and greater burden for single core architectures.

A final issue with tiles is that of load balancing. A profile requiring parallel decoding should specify a regular and well balanced design for parallelism, and for different level of parallel implementation. It is important to have equal size tiles to allow practical parallel decoder implementation. This can only be achieved through large numbers of tiles, which will lead to compression efficiency losses, as discussed in section 5.

Summary: On the basis of practicality and simplicity, there is a clear advantage for making WPP the default in the Main Profile, rather than tiles.

# Summary

There are a number of areas where there is no clear benefit of choosing Tiles over WPP, including parallel processing efficiency. However, WPP has significant advantages in terms of compression efficiency, parallel processing flexibility, and the simplicity of profiling. Previously expressed concerns about WPP regarding delay, do not appear to be a significant issue when transmission delay is considered.

Based on the analysis presented in this contribution, we propose the inclusion of WPP (tiles_or_entropy_coding_sync_idc=2 and num_substreams_minus1 = PicHeightInCtb-1) in the Main Profile. Given the concerns about compression efficiency, and profiling issues (e.g. practicalities of building conforming decoders, possible reduction in flexibility for encoders), we believe that tiles should not be included in the Main Profile. Therefore, if WPP is not included, we consider no parallelism (tiles_or_entropy_coding_sync_idc=0) to be better than including tiles.

If the tiles feature is to be included in the Main Profile, then we would recommend that tiles are always equal size, and strict settings for tile sizes (including tile heights) be introduced to enable level conforming parallel decoders to be implemented.

# References

- Lee et al., “Simplification on WPP”, JCTVC-H0349, JCT-VC 8th Meeting: San José, CA, USA, 1–10 February, 2012.

- Bross et al., “High efficiency video coding (HEVC) text specification draft 6”, JCT-VCH1003_dk, JCT-VC 8th Meeting: San José, CA, USA, 1–10 February, 2012.

- Fuldseth et al., “Tiles”, JCTVC-F335, JCT-VC 6th Meeting: Torino, IT, 14-22 July, 2011.

- Clare et al., “Wavefront Parallel Processing for HEVC Encoding and Decoding”, JCTVC-F274, JCT-VC 6th Meeting: Torino, IT, 14-22 July, 2011.

- Hendry et al., “AHG4: Harmonized Method for Signalling Entry Points of tiles and WPP Substreams”, JCTVC-H0566, JCT-VC 8th Meeting: San José, CA, USA, 1–10 February, 2012.

# Appendix A

This appendix contains calculated parallel processing efficiency values for tiles and wavefront parallel processing. The results for tiles include efficiency values with and without fully flexible Load Balancing (LB).

Table 7: Comparison of parallel processing efficiency for tiles and wavefronts, where LB means Load Balancing, and CLB means Constrained Load Balancing.

# Appendix B

Results are presented for HM-6.1 comparing tiles and WPP against the anchors. The results include an offset entry point for every tile and for every CTB line in the slice header. Note that Class D does not include any tiles. The comparison against WPP may therefore be unfair, as WPP offers parallel processing for up to four cores, whereas tiles offers no parallel processing for Class D. Averages have therefore been calculated with and without the Class D results.

Table 8: Coding results for WPP using HM-6.1.

Table 9: Results of encoding tiles using HM-6.1.

# Appendix C

Delay Equations Derivation

If Tline and Ttile are the transmission times for one CTB line and one tile respectively, then:

Tline = BR/(BC * PicHeightInCtb * FR)

Ttile = BR/(BC * Stiles * FR)

where BR is the average encoded bit-rate, BC is the channel transmission rate, FR is the frame rate, and Stiles is the number of tiles used to code the picture. These calculations make a number of simplifying assumptions (e.g. equal size tiles). Using the above, it is possible to calculate the ratio between tile and WPP transmission delay:

Tline/Ttile = Stiles/PicHeightInCtb

This means that the transmission delay for WPP will always be less than with tiles, provided that:

Stiles < PicHeightInCtb

Tline > 2PCTB

The first condition will often be true (see Table 3 for examples of number of tiles against number of CTB lines). If the second condition is not true, then the delay is bounded by:

Dw = 2(Swpp - 1)PCTB

where Dw is the processing delay incurred from using WPP, and Swpp is the wavefront height (number of substreams). Now it is possible to recalculate the delay ratio between tiles and WPP:

ptileDw/Ttile = 2(Swpp - 1)PCTBBCStilesFR/ BR

where ptile is the number of tiles to be processed in parallel. In this case, WPP delay is less than tile delay if:

BR/BC > 2(Swpp - 1) PCTBStilesFR / ptile

We can assume that a real-time encoding system is designed so that:

PCTB = 1/(PicWidthInCtb * PicHeightInCtb * FR * M)

where M represents a real-time speed up factor (i.e. how much faster than real-time a frame can be encoded), which is always greater than or equal to one.

Now, WPP delay is less than tile delay if:

BR/BC > 2(Swpp - 1) Stiles/(PicWidthInCtb * PicHeightInCtb * M * ptile)

# Patent rights declaration(s)

Aspex Semiconductor does not have any current or pending patent rights relating to the technology described in this contribution.